3GPP TSG RAN WG1 #83	R1- 156490

November 15th  – 22nd, 2015

Anaheim, USA

Agenda item:	6.2.3.1

Source: 	Cisco Systems

Title: 	Views on Multi-Channel Access for LAA

Document for:	Discussion and Decision

# Introduction

This contribution describes Cisco Systems’ views on multi-channel access for LAA.  The agreements in RAN1 #82 are listed below.

Multi-Carrier LBT Agreements in RAN1 #82

- For multi-Carrier LBT on a group carriers

  - Alt1: eNB performs Cat-4 based LBT on only one unlicensed carrier

    - When the eNB completes LBT on a carrier, the eNB can sense other configured carriers for a period, e.g., PIFS (25 microseconds), immediately before the completion of LBT on the carrier.

    - The eNB is allowed to transmit DL data burst(s) on the carriers sensed idle according to above procedure.

    - FFS: How fast the eNB can change the carrier requiring Cat-4 based LBT

    - FFS: Whether to apply the Wi-Fi channel bonding rule

    - FFS: Energy detection threshold used on channels not performing Cat-4 based LBT

  - Alt2: eNB performs Cat-4 based LBT on more than one unlicensed carriers

    - The eNB is allowed to transmit DL data burst(s) on the carriers that has completed Cat-4 based LBT with potential self-deferral (including idle sensing for a single interval) to align transmission over multiple carriers.

    - FFS: If the eNB can receive on a carrier while transmitting on another carrier, freeze backoff counter(s) for the carrier(s) not transmitting while other carrier(s) is transmitting if the carriers are within X MHz apart

      - FFS: X MHz

  - FFS: Whether LAA supports Alt1 + Alt2 or Alt2 only.

# Multi-Carrier LBT

In terms of multi-carrier LBT, two options were identified in RAN1 #82. The first option (Alt1) requires that the LAA eNB perform LBT on only one carrier in a group of carriers. In the second option (Alt2), the LAA eNB performs Cat 4 LBT on more than one carrier. Both Alt1 and Alt2 do not place any restrictions for LAA in terms of which unlicensed SCells an LAA eNB transmits on.

In comparison to either of the above options, Wi-Fi defines rigid rules of multi-channel access. Provided that the CCA-PD procedure has detected that the primary channel is idle, a device can transmit on secondary channels if detected idle for PIFS duration prior to the intended transmission time. Additionally, the channel access in Wi-Fi occurs over contiguous channels. Specifically, for a 80 MHz VHT BSS, 20 MHz transmission occurs over the primary 20 MHz channel, 40 MHz transmission occurs over the primary 40 MHz channel and 80 MHz transmission can happen only after the STA determines the primary 20 MHz is idle following CCA-PD and each secondary 20 MHz/40 MHz channel are idle for a duration of PIFS prior to the intended transmission time. The CCA checks for multi-channel transmission in Wi-Fi are hierarchical as shown below:

- For 20 MHz transmission, preamble detection (CCA-PD = -82 dBm) and energy detection (CCA-ED = -62 dBm) is employed.

- For 40 MHz transmission,

  - Preamble detection (CCA-PD = -82 dBm) is performed on the primary 20 MHz channel

  - One shot energy detection (CCA-ED = -75 dBm) for PIFS (25 us) duration is performed on secondary 20 MHz channel.

- For 80 Mhz transmission

  - Preamble detection (CCA-PD = -82 dBm) is performed on the primary 20 MHz channel

  - One shot Energy Detection (CCA-ED = -72 dBm) is performed on secondary 40 MHz channel

One shot Energy Detection (CCA-ED = -75 dBm) is performed on each secondary 20 MHz channel.

Simulation results [1] have shown that both Alt1 and Alt2 result in the non-replaced 802.11ac network, configured for 80 MH operation, being restricted to 20 MHz channel access. The reason being that the fully independent nature of SCell channel access incentivizes LAA to avoid transmitting on primary Wi-Fi channels and prevent Wi-Fi from 40 MHz/80 Mhz transmissions.

To ensure fair co-existence with Wi-Fi’s channel access, for Alt1, our preference is that LAA follow the Wi-Fi channel bonding principles. Having complete flexibility for choosing the carrier on which to perform LBT would enable the LAA eNB to potentially “crowd” out Wi-Fi secondary channels.  This would prevent Wi-Fi from accessing secondary channels and be forced to transmit only on primary channel.

Proposal 1: Under Alt1, to maximize co-existence with Wi-Fi in the multi-carrier scenario

- For multi-carrier channel access, analogous to Wi-Fi, DL transmission should occur only over contiguous unlicensed channels.

- A single unlicensed channel, choosen semi-statically is chosen as the channel over which the LAA eNB performs Cat-4 based LBT. Designate this as the Assisted Primary Scell.

- The LAA eNB shall semi-statically select one or more contiguous unlicensed Scells, at least one of which is contiguous to the Assisted Primary Scell. Designate such SCells as the Assisted Secondary Scells

- The LAA eNB shall, for purpose of LBT,

  - employ ED threshold of -77 dBm on the Assisted Primary Cell.

  - for each Assisted Secondary Scell, employ identical ED thresholds (-75 dBm over 20 MHz, -72 dBm over 40 Mhz) as Wi-Fi secondary channels.

In the multi-carrier LBT scheme in Alt2, a completely independent selection of eCCA counter on each carrier potentially helps the LAA eNB commence transmission using the carrier that completes eCCA countdown earliest (with possible self-deferral). It is important that during the post-transmission backoff, a common eCCA counter be used for performing Cat 4 countdown across multiple channels. Also, once the LAA eNB commences transmission following completion of Cat 4 countdown and self-deferral, the unused carriers should reset their eCCA counter so that they do not have an advantage in terms of access to the medium for the ensuing TXOP.

Proposal 2: Under Alt2, upon completion of transmission from one or more carriers within a particular group,

- All carriers within that group should perform post transmission backoff with a common eCCA counter value.

- Following completion of the post transmission backoff and self-deferral, the carriers on which transmission does not occur should reset their eCCA counter value.

# Summary

Proposal 1: Under Alt1, to maximize co-existence with Wi-Fi in the multi-carrier scenario

- For multi-carrier channel access, analogous to Wi-Fi, DL transmission should occur only over contiguous unlicensed channels.

- A single unlicensed channel, choosen semi-statically is chosen as the channel over which the LAA eNB performs Cat-4 based LBT. Designate this as the Assisted Primary Scell.

- The LAA eNB shall semi-statically select one or more contiguous unlicensed Scells, at least one of which is contiguous to the Assisted Primary Scell. Designate such SCells as the Assisted Secondary Scells

- The LAA eNB shall, for purpose of LBT,

  - employ ED threshold of -77 dBm on the Assisted Primary Cell.

  - for each Assisted Secondary Scell, employ identical ED thresholds (-75 dBm over 20 MHz, -72 dBm over 40 Mhz) as Wi-Fi secondary channels.

Proposal 2: Under Alt2, upon completion of transmission from one or more carriers within a particular group,

- All carriers within that group should perform post transmission backoff with a common eCCA counter value.

- Following completion of the post transmission backoff and self-deferral, the carriers on which transmission does not occur should reset their eCCA counter value.

# References

- R1-155547, Discussion on Multi-Channel LBT, Broadcom