# 3GPP TSG-RAN1 #83	R1-157407

# 15th – 22th November 2015

# Anaheim, US

Source:                    	Ericsson

Title:  	NB-IoT - Downlink link level coexistence in guard band - interference from LTE

Document for:        	Discussion and decision

Agenda Item:         	6.2.6.1.2

# Introduction

At RAN#69, a new work item named NarrowBand IOT (NB-IOT) was approved, see [1]. The objective is to specify a radio access for cellular internet of things that addresses improved indoor coverage, support for massive number of low throughput devices, low delay sensitivity, ultra-low device cost, low device power consumption and (optimized) network architecture.

NB-IOT should support 3 different modes of operation:

1.	“Stand-alone operation” utilizing for example the spectrum currently being used by GERAN systems as a replacement of one or more GSM carriers

2.	“Guard band operation” utilizing the unused resource blocks within a LTE carrier’s guard-band

3.	“In-band operation” utilizing resource blocks within a normal LTE carrier

Furthermore according to [1], NB-IOT should have a single synchronization signal design for the different modes of operation, including techniques to handle overlap with legacy LTE signals

A narrowband LTE based solution (called NB-LTE) was proposed and included in [3], and is now under investigation under the NB-IOT work item [1]. The document [4] provided an updated high level description of the random access concept in NB-LTE such as the subcarrier spacing and multiplexing.

In this contribution, we provide evaluation results comparing two different carrier-spacing alternatives for NB-IOT in Downlink in Guard band deployments. In particular, the M-PDSCH performance between 15 kHz carrier spacing where full orthogonality with LTE is preserved, and 3.75 kHz carrier spacing with the exposure to interference from the LTE carrier is presented with 10 interfering LTE PRBs adjacent to the NB-IoT band or with a separation of 180 kHz.

No special Tx filter was used. For the NB-IoT receiver, a RRC filter, with a roll-off of 0.1, was used.

The evaluation was done for MCL 144, 154, 164 and with power boosting NB-IoT by 0, 3 and 6 dB, except for MCL 164 where only the 6 dB boosting result is provided.

# Performance Evaluation

In guard band scenarios, the 15 kHz spacing does not need to protect any LTE legacy signalling. So both 15 kHz and 3.75 kHz have the same overhead. Except from the carrier spacing, the same algorithms are used in both carrier spacing, that is mainly: channel estimation, frequency offset and timing offset estimation, which are realistic accordingly to the simulation assumptions.

In this context, the differences between 15 kHz and 3.75 kHz can come either from the performance of the baseband algorithms or from an increase in interference due to the nearby LTE system.

The power boosting is relative 29 dBm which is the assumed Tx power per PRB of the LTE system. Each power boosting corresponds to a specific Tx power which, in its turn, translates to a required SNR to achieve MCL 144, 154 and 164. The required SNR is given in Table 1.

The methodology adopted has been to identify the rate that matches the MCL for 15 kHz and run the same rate for 3.75 kHz.

Table 1

|  | MCL 144 | MCL 154 | MCL 164 |
| --- | --- | --- | --- |
| 6 dB boost | 7.3 | -2.6 | -12.6 |
| 3 dB boost | 4.3 | -5.6 | -15.6 |
| 0 dB boost | 1.3 | -8.6 | -18.6 |

Table 2, below, summarizes the simulation parameters that have been used in the evaluation.

Table 2

| Parameter | Value |
| --- | --- |
| Channel model | TU |
| Doppler spread | 1 Hz |
| Antenna configuration | 2 Tx; 1 Rx |
| Timing uncertainty | Randomly drawn from [-2.5 us, +2.5 us] |
| Frequency error | Uniformly drawn from the set {-50 Hz, 50 Hz} |
| Scenarios | Guard band with 0kHz and 180 kHz separation |
| LTE Tx Power per 180kHz | 29 dBm |
| NB-IoT power boosting | 0 dB, 3 dB, 6 dB |
| Coverage scenarios | MCL 144, 154, 164. See table 5 in [10] |
| RX filter | RRC, roll off 0.1 |
| TX filter | None |
| Nr of Interfering PRB | 10 |

## Guard band with 0 kHz separation

### MCL 144

Note that for 3 dB boosting, the gap between 15 kHz and 3.75 kHz is bigger, most likely due to the fact that the code rate is only 4/7, thus not benefiting from the convolutional coding.

### MCL 154

### MCL 164

## Guard band scenario with 180 kHz separation

### MCL 144

Observe that the MCL 144 curves in the 0 kHz separation Guard band scenario are very similar to those for the 180 kHz separation. This indicates that the differences are mainly due to the baseband algorithms behaviour at 15 kHz and 3.75 kHz.

### MCL 154

We make the same observation as previously at MCL 144.

### MCL 164

Here also, at MCL 164, the 2 separation cases are very similar to each other.

# Conclusions

It has been observed that the curves are very similar between the two guard band separations 0 kHz and 180 kHz. The reason for that is that the difference between 15 kHz and 3.75 kHz is mainly due to the baseband algorithms performance and not to the interference from the LTE system.

In other words, the RRC receiver filter removes most of the LTE interference. This conclusion will be also valid for 0 dB and 3 dB boosting at coverage MCL 164, since the number of repetitions will be even higher, averaging out even more the interference from the LTE system.

# References

- GP-151621, “New Work Item: NarrowBand IOT (NB-IOT),” source Qualcomm Inc., RAN #69.

- GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418),” source VODAFONE Group Plc. GERAN#62.

- 3GPP TR 45.820, “Technical Specification Group GSM/EDGE Radio Access Network; Cellular System; Support for Ultra Low Complexity and Low Throughput Internet of Things; (Release 13),” www.3gpp.org.

- R1-156010, “NB-LTE – General L1 concept description,” source Ericsson…, October 2015, Malmo, Sweden.

- R1-156009, “NB-LTE –  Synchronization channel design and performance,” source Ericsson…, October 2015, Malmo, Sweden.

- 3GPP Technical Specification 36.211, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation,” www.3gpp.org.

- 3GPP Technical Specification 36.300, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 13),” www.3gpp.org.

- 3GPP Technical Specification 36.104, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception,” www.3gpp.org.

- GP-150782, “Narrowband LTE – PDSCH and PUSCH design and performance”