# 3GPP TSG-RAN1 NB-IOT Ad Hoc	R1-160274

# 15-19 February 2016

# St Julian’s, Malta

Source:                    	Ericsson

Title:  	NB-IoT - UL Reference signals

Document for:        	Discussion and Decision

Agenda Item:         	7.2.1.2.1

# Introduction

At GERAN#62, a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1]. At GERAN#67, the FS_IoT_LC study item was completed and the TR 45.820 v2.1.0 [4] approved. Among the candidates included in the TR was the NB-LTE candidate solution.

At RAN#70, the narrow band IoT (NB-IoT) work item was revised and approved. Three different operation modes, i.e., standalone, guard-band, and in-band, are supported. Moreover, a unified downlink design with 15 kHz sub-carrier spacing for all three operation modes is defined. For the UL, both single tone and multi-tone operations can be supported.

To be more specific, the following items have been agreed [5].

•	180 kHz UE RF bandwidth for both downlink and uplink

•	OFDMA on the downlink

- 15 kHz sub-carrier spacing for all the modes of operation (with normal or extended CP).

•	For the uplink:

- Single tone transmissions are supported. Two numerologies should be configurable by the network for single-tone transmission: 3.75 kHz and 15 kHz. A cyclic prefix is inserted. Frequency domain sinc pulse-shaping in the physical layer description.

- Multi-tone transmissions are supported, based on SC-FDMA with 15 kHz UL subcarrier spacing.

- FFS: Additional mechanisms for PAPR reduction.

- The UE shall indicate the support single-tone and/or multi-tone, details to be discussed by RAN WGs

The UL Demodulation Reference Symbol (DMRS) is needed for the receiver at the eNB to perform coherence demodulation. As NB-IoT supports both single-tone and multi-tone transmissions, the LTE UL DMRS cannot be directly applied to the NB-IoT UL. Therefore, in this contribution, we outline some design considerations about how to design the UL DMRS for NB-IoT systems.

# General principles

Two UL subcarrier spacing numerologies, i.e., 3.75 kHz and 15 kHz, are supported by NB-IoT systems, and only 15 kHz UL subcarrier spacing is used for multi-tone transmissions. It is proposed in [2] that for multi-tone NB-PUSCH with 15 kHz subcarrier spacing, it follows the subframe structure of LTE PUSCH. If a transmission uses all 12 subcarriers, it is straightforward to re-use the LTE UL DMRS design.

Proposal 1: If an NB-IoT device is allocated with 12 subcarriers for its UL transmission, the LTE UL DMRS should be re-used.

For the LTE UL DMRS, when 1 or 2 PRBs are allocated to an UE, the UL DMRSs are Computer Generated (CG). The desired criteria for LTE UL DMRS are summarized in [3]. These criteria, e.g., low CM, cross-correlations properties, complexity issues, should also be considered in the design of NB-IoT UL DMRS.

Proposal 2: In the design of NB-IoT UL DMRS, similar design criteria as the LTE UL DMRS should be considered.

In order to minimize the impact of inter-cell interference, the chosen of LTE UL DMRS sequence is initialized based on the cell ID. This is a straightforward way to ensure the pseudo-randomness for the UL DMRS among neighbouring cells, which should also be considered in the design of NB-IoT UL DMRS.

Proposal 3: The chosen NB-IoT UL DMRS sequence should be initialized based on the cell ID to ensure the pseudo-randomness of UL DMRS among neighbouring cells.

# Multi-tone transmissions

Only 15 kHz sub-carrier spacing is used for multi-tone transmissions. In the NB-IoT Ad hoc meeting, the following is agreed for multi-tone tone transmission

- UL multi-tone transmission for the data with 12 tones is supported

  - UL multi-tone transmission for the data also supports followings of numbers of multiple

- {3} with 4 msec resource unit size

- {6} with 2 msec resource unit size

As discussed in Section 2, in the current LTE system the UL DMRS is only defined for the 12-subcarrier case. Therefore, new UL DMRS need to be designed for the other cases. However, since the slot format is the same for NB-IoT in the multi-tone transmissions as it is in LTE, unless otherwise motivated, the same UL DMRS positions should be used in NB-IoT as they are in LTE.

Proposal 4: Unless otherwise motivated, for multi-tone transmissions, the same UL DMRS positions should be used in NB-IoT as they are in LTE.

Furthermore, the DMRS used for 3-tone and 6-tone transmission should have good auto-correlation and cross-correlation properties to facilitate good channel estimation and lower inter-cell interference. Therefore, a brutal force truncation of the CG LTE DMRS is less preferable, as it may use sequences with poor auto-correlation and/or cross-correlation properties. In addition, the CG LTE DMRS for 1 or 2 PRBs are based on QPSK symbols and have been found using computer search. These sequences have been shown to have constant amplitude and a sufficiently low cubic metric, and therefore similar principles should be applied also for NB-IoT UL DMRS.

Proposal 5: For 3/6-tone NB-PUSCH, the generation of UL DMRS is based on length-3 and length-6 base sequences that have good auto-correlation and cross-correlation properties.

Proposal 6: For 3/6-tone NB-PUSCH, the base sequences for UL DMRS are defined using QPSK symbols.

In LTE, a set of 30 base sequences in case of length 12 and length 24 are specified explicitly to be used as UL DMRS for the cases that 1 or 2 PRBs are allocated to a UE. The base sequence used by a particular cell is obtained by taking the modulus of the cell ID with 30, and phase rotation is applied afterwards. This allows sufficiently low inter-cell interference over a wide area.

## To be more specific, in LTE, the UL DMRS for 1 and 2 PRBs is based on QPSK-based sequences, and are generated by computer search. In general, these base sequence is given by

rn=ejφnπ/4, n=0, 1, …, MscRS-1,

where MscRS is the number of subcarriers that  rn is mapped, and the values of φ(n) are given in Table 5.5.1.2-1 and Table 5.5.1.2-2 in [6]. For NB-IoT transmission, it is desirable to use QPSK-based sequences as well due to their constant amplitude and low cubic metric property. Therefore, the task here is to find φ(n) that can generate desired property of the RS which can be used by NB-IoT uplink transmission.

For 3-tone transmission, assuming QPSK symbols are used, each element of a base sequence takes the value of one out of four possible QPSK symbols. Therefore, the total number of possible length-3 base sequences is 4^3 = 64. Of these, half of the base sequences are phase-rotated versions of other base sequences. Therefore, there are only 32 unique base sequences. Of these 32 sequences, i.e., φ(n) = {1, 1, 1} and φ(n) = {3, 3, 3} have poor auto-correlation properties and should be removed. Thus, we end up with exactly 30 length-3 base sequences, any of which can be used for UL DMRS generation. However, of the 30 length-3 base sequences, we have identified 8 sequences that have significantly better auto-correlation properties (shown in Figure 1). These sequences are recommended to be specified explicitly in the standard.

Recommendation 1: At least the 8 sequences that have the auto-correlation property as shown in Figure 1 should be used as base sequence of UL DMRS of NB-IoT for 3-tone transmission.

If it is enough to only use these 8 base sequences, the cell-specific base sequence is then obtained by taking modulus of the cell ID with 8, and similar to LTE, a phase rotation can be applied. Otherwise, all the 30 length-3 base sequences can be used, in which case, the cell-specific base sequence is then obtained by taking modulus of the cell ID with 30, and similar to LTE, a phase rotation can be applied. The configuration of which base sequence is used can be configured, e.g, via higher-layer signalling.

Figure 1 Auto-correlation of the recommended 8 sequences

For 6-tone transmission, following the same reasoning of length-3 base sequences, there are in total 2048 base sequences that can be used. Therefore, there is more flexibility in determining a good set of sequences.

# Single-tone transmissions

Two numerologies should be configurable by the network for single-tone transmissions, i.e., 3.75 kHz and 15 kHz. For single-tone transmissions, the UL frame structure and the modulation formats are still under discussion, which is coupled with the design of UL DMRS for NB-IoT single-tone transmissions. Nevertheless, there still are design considerations worth to be deliberated.

There is a preference to use pi/2-BPSK and pi/4-QPSK for single tone transmission. For pi/2-BPSK and pi/4-QPSK, since the phases of the symbols are continuously rotated to guarantee low PAPR, it is not desirable to specify reference symbols, as it may disrupt the phase rotation. Therefore, instead of using reference symbols, it is preferable to specify reference bit sequences. These sequences are inserted in-between coded bits and then mapped to symbols. In this way, continuous phase rotation can be obtained.

Proposal 7: Instead of using reference symbols, it is preferable to specify reference bit sequences when pi/2-BPSK and pi/4-QPSK are used for single tone transmission.

However, the general principles discussed in Section 2 should still be applied. It is very important that the reference symbols should be generated in a pseudo random way, and when several single-tone UEs are scheduled at the same time in a cell, they do not affect the multi-tone transmission in neighbouring cells.

# Conclusions

Base on the discussions above, we have the following proposals.

Proposal 1: If an NB-IoT device is allocated with 12 subcarriers for its UL transmission, the LTE UL DMRS should be re-used.

Proposal 2: In the design of NB-IoT UL DMRS, similar design criteria as the LTE UL DMRS should be considered.

Proposal 3: The chosen NB-IoT UL DMRS sequence should be initialized based on the cell ID to ensure the pseudo-randomness of UL DMRS among neighbouring cells.

Proposal 4: Unless otherwise motivated, for multi-tone transmissions, the same UL DMRS positions should be used in NB-IoT as they are in LTE.

Proposal 5: For 3/6-tone NB-PUSCH, the generation of UL DMRS is based on length-3 and length-6 base sequences that have good auto-correlation and cross-correlation properties.

Proposal 6: For 3/6-tone NB-PUSCH, the base sequences for UL DMRS are defined using QPSK symbols.

Proposal 7: Instead of using reference symbols, it is preferable to specify reference bit sequences when pi/2-BPSK and pi/4-QPSK are used for single tone transmission.

Recommendation 1: At least the 8 sequences that have the auto-correlation property as shown in Figure 1 should be used as base sequence of UL DMRS of NB-IoT for 3-tone transmission.

# References

- GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

R1-160085, “NB-IoT - NB-PUSCH design”, source Ericsson, RAN1 NB-IoT Ad Hoc, Budapest, Hungary, 18-20 January 2016.

R1-073878, “UL RS: Outcome of Offline Discussions”, source Samsung, Ericsson, Panasonic, LGE, Freescale,    Texas Instruments, Sharp, Qualcomm, Nokia, NSN, Motorola, Nortel, NTT DoCoMo.

3GPP TR 45.820, “Technical Specification Group GSM/EDGE Radio Access Network; Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things,” Aug. 2015, v.2.1.0.

RP-152284, “New Work Item: Narrowband IoT (NB-IoT),” sources Huawei and HiSilicon, RAN #70.

3GPP TS 36.211 V13.0.0 (2015-12), 3rd Generation Partnership Project; Technical Specification Group Radio Access Network;Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation(Release 13)