# 3GPP TSG-RAN1 #84	R1-160276

# 15-19 February 2016

# St Julian’s, Malta

Source:                    	Ericsson

Title:  	NB-IoT – NB-PRACH Evaluations

Document for:        	Discussion and decision

Agenda Item:         	7.2.1.2.2

# Introduction

At RAN#69, a new work item named Narrowband IoT (NB-IoT) was approved, see [1]. The objective is to specify a radio access for cellular internet of things that addresses improved indoor coverage, support for massive number of low throughput devices, low delay sensitivity, ultra-low device cost, low device power consumption and (optimized) network architecture.

In RAN#70, the following items have been agreed [2].

- 180 kHz UE RF bandwidth for both downlink and uplink

- OFDMA on the downlink

  - 15 kHz sub-carrier spacing for all the modes of operation (with normal or extended CP).

- For the uplink:

  - Single tone transmissions are supported. Two numerologies should be configurable by the network for single-tone transmission: 3.75 kHz and 15 kHz. A cyclic prefix is inserted. Frequency domain sinc pulse-shaping in the physical layer description.

  - Multi-tone transmissions are supported, based on SC-FDMA with 15 kHz UL subcarrier spacing.

  - FFS: Additional mechanisms for PAPR reduction.

  - The UE shall indicate the support single-tone and/or multi-tone, details to be discussed by RAN WGs

In RAN1 NB-IoT ad hoc meeting, it was further agreed that “Support one PRACH scheme for all MCL cases [3].” Also, as a working assumption, “PRACH scheme is based on single-tone transmission [3].”

In this contribution, we provide NB-PRACH evaluation results to support the observations and proposals made in [4][5].

# Evaluation Setup and Results

In this section, simulation results are presented to evaluate NB-PRACH design for NB-IoT. The simulation assumptions used are based on the ones outlined in [4][5], and are summarized in Table 1.

Table 1: Simulation parameters

| Parameter | Value |
| --- | --- |
| Channel model | TU |
| Doppler spread | 1 Hz |
| Antenna configuration | 1 Tx; 2 Rx |
| Cell size | 35 km |
| Timing uncertainty | Randomly drawn from [-5.2 us, 233.3+5.2 us] for 35 km cell, or Randomly drawn from [-5.2 us, 53.3+5.2 us] for 8 km cell. See Note 1. |
| Frequency error | Uniformly drawn from the set {-50 Hz, 50 Hz}. See Note 2. |
| MS frequency drift | Uniformly drawn from the set {-22.5 Hz/s, 22.5 Hz/s}. |
| Number of channel realizations | 10,000 |
| Note 1: 233.3 us and 53.3 us are the maximum round-trip delay in the cell of size 35 km and 8 km, respectively. The additional +- 5.2 us accounts for the residual timing errors in the cell search. The timing uncertainty is drawn based on uniform user location in a hexagonal cell of size 35 or 8 km. |  |

Table 2 summarizes the PRACH coverage performance in the SC-FDMA based uplink design for NB-IoT. We can see the required SNR for Formats 0, 1 and 2 are 14.25 dB, 4.25 dB, -5.75 dB, respectively.

Table 2: PRACH coverage performance

| 1) Tx power (dBm) | 23 | 23 | 23 |
| --- | --- | --- | --- |
| 2) PSD (dBm/Hz) | -174 | -174 | -174 |
| 3) Rx noise figure (dB) | 3 | 3 | 3 |
| 4) Interference margin (dB) | 0 | 0 | 0 |
| 5) BW (Hz) | 3750 | 3750 | 3750 |
| 6) Effective noise (dBm) 2)+3)+4)+10*log10(5)) | -135.2597 | -135.2597 | -135.2597 |
| 7) Required SINR (dB) | 14.25 | 4.25 | -5.75 |
| 8) Rx sensitivity (dBm) 6)+7) | -121.0097 | -131.0097 | -141.0097 |
| 9) Rx processing gain (dB) | 0 | 0 | 0 |
| 10) MCL (dB) 1)-8)+9) | 144.0097 | 154.0097 | 164.0097 |

For ease of reference, the set of design configuration parameters proposed in [4] are copied here as Table 3 and are used in the simulation.

Table 3: Example design for single tone frequency hopping NB-PRACH

| Cell size (km) | MCL (dB) | Subcarrier spacing (kHz) | Tcp (us) | Number of symbols per group | Number of groups | Hopping pattern | Pseudo random hopping range |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 35 | 144 | 3.75 | 266.7 | 5 | 8 | 1 tone micro hopping + pseudo random hopping | {8, 12, 16} tones |
|  | 154 | 3.75 | 266.7 | 5 | 24 | 1 tone micro hopping + pseudo random hopping | {8, 12, 16} tones |
|  | 164 | 3.75 | 266.7 | 5 | 120 | 1 tone micro hopping + pseudo random hopping | {8, 12, 16} tones |
| 8 | 144 | 3.75 | 66.7 | 5 | 8 | 1 tone micro hopping + pseudo random hopping | {8, 12, 16} tones |
|  | 154 | 3.75 | 66.7 | 5 | 24 | 1 tone micro hopping + pseudo random hopping | {8, 12, 16} tones |
|  | 164 | 3.75 | 66.7 | 5 | 120 | 1 tone micro hopping + pseudo random hopping | {8, 12, 16} tones |
| Remark 1: Pseudo random hopping is based on LTE PUSCH type 2 hopping, as detailed in [4][5]. |  |  |  |  |  |  |  |

## Time-of-Arrival Estimation (35 km)

### MCL: 144 dB

Figure 1 shows the distributions of time-of-arrival estimation errors for the single-tone frequency hopping with 35 km cell size and 144 dB MCL. The NB-PRACH band may consist of 8, 12, or 16 tones, in which 1-tone fixed hopping and LTE PUSCH type 2 pseudo random hopping are applied. It can be seen high accuracy time-of-arrival estimation accuracy can be achieved with the proposed design.

Figure 1: Time-of-arrival estimation performance with 35 km cell size and 144 dB MCL.

### MCL: 154 dB

Figure 2 shows the distributions of time-of-arrival estimation errors for the single-tone frequency hopping with 35 km cell size and 154 dB MCL. The NB-PRACH band may consist of 8, 12, or 16 tones, in which 1-tone fixed hopping and LTE PUSCH type 2 pseudo random hopping are applied. It can be seen high accuracy time-of-arrival estimation accuracy can be achieved with the proposed design.

Figure 2: Time-of-arrival estimation performance with 35 km cell size and 154 dB MCL.

### MCL: 164 dB

Figure 3 shows the distributions of time-of-arrival estimation errors for the single-tone frequency hopping with 35 km cell size and 164 dB MCL. The NB-PRACH band may consist of 8, 12, or 16 tones, in which 1-tone fixed hopping and LTE PUSCH type 2 pseudo random hopping are applied. It can be seen high accuracy time-of-arrival estimation accuracy can be achieved with the proposed design.

Figure 3: Time-of-arrival estimation performance with 35 km cell size and 164 dB MCL.

## Time-of-Arrival Estimation (8 km)

### MCL: 144 dB

Figure 4 shows the distributions of time-of-arrival estimation errors for the single-tone frequency hopping with 8 km cell size and 144 dB MCL. The NB-PRACH band may consist of 8, 12, or 16 tones, in which 1-tone fixed hopping and LTE PUSCH type 2 pseudo random hopping are applied. It can be seen high accuracy time-of-arrival estimation accuracy can be achieved with the proposed design.

Figure 4: Time-of-arrival estimation performance with 8 km cell size and 144 dB MCL.

### MCL: 154 dB

Figure 5 shows the distributions of time-of-arrival estimation errors for the single-tone frequency hopping with 35 km cell size and 154 dB MCL. The NB-PRACH band may consist of 8, 12, or 16 tones, in which 1-tone fixed hopping and LTE PUSCH type 2 pseudo random hopping are applied. It can be seen high accuracy time-of-arrival estimation accuracy can be achieved with the proposed design.

Figure 5: Time-of-arrival estimation performance with 8 km cell size and 154 dB MCL.

### MCL: 164 dB

Figure 6 shows the distributions of time-of-arrival estimation errors for the single-tone frequency hopping with 35 km cell size and 164 dB MCL. The NB-PRACH band may consist of 8, 12, or 16 tones, in which 1-tone fixed hopping and LTE PUSCH type 2 pseudo random hopping are applied. It can be seen high accuracy time-of-arrival estimation accuracy can be achieved with the proposed design.

Figure 6: Time-of-arrival estimation performance with 8 km cell size and 164 dB MCL.

# Conclusions

In this contribution, we have provided NB-PRACH evaluation results to support the observations and proposals made in [4][5]. The following observations are made from the evaluation results.

Observation 1: Fixed size hopping + pseudo random hopping (based on LTE PUSCH Type 2 hopping) can provide accurate time-of-arrival estimation accuracy.

Observation 2: Pseudo random hopping (based on LTE PUSCH Type 2 hopping) allows the eNB to flexibly configure NB-PRACH bands consisting of different number of tones.

# References

- RP-151621, “New Work Item: NarrowBand IOT (NB-IOT),” source Qualcomm Inc., RAN #69.

- RP-152284, “New Work Item: Narrowband IoT (NB-IoT),” sources Huawei and HiSilicon, RAN #70.

- RAN1 Chairman’s Notes, RAN1 NB-IoT Ad Hoc, January 2016.

- R1-160275, “NB-IoT: Single tone frequency hopping NB-PRACH design,” sources Ericsson, RAN1 #84, February 2016.

- R1-160439, “NB-IoT – NB-PRACH frequency hopping pattern,” source Ericsson, RAN1 #84, February 2016.

- 3GPP TR 45.820, “Technical Specification Group GSM/EDGE Radio Access Network; Cellular System; Support for Ultra Low Complexity and Low Throughput Internet of Things; (Release 13),” www.3gpp.org.