# 3GPP TSG-RAN1#84	R1-160277

# 15-19 February 2016

# St Julian’s, Malta

Source:                    	Ericsson

Title:  	NB-IoT – Near-Far Performance of NB-PRACH

Document for:        	Discussion and decision

Agenda Item:         	7.2.1.2.2

# Introduction

At RAN#69, a new work item named NarrowBand IOT (NB-IoT) was approved, see [1]. The objective is to specify a radio access for cellular internet of things that addresses improved indoor coverage, support for massive number of low throughput devices, low delay sensitivity, ultra-low device cost, low device power consumption and (optimized) network architecture. At RAN#70, a revised work item description was approved, see [2].

NB-IOT should support 3 different modes of operation:

1.	“Stand-alone operation” utilizing for example the spectrum currently being used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment.

2.	“Guard band operation” utilizing the unused resource blocks within a LTE carrier’s guard-band

3.	“In-band operation” utilizing resource blocks within a normal LTE carrier

NB-IoT will support 180 kHz UE RF bandwidth for both downlink and uplink. For the uplink:

o	Single tone transmissions are supported. Two numerologies should be configurable by the network for single-tone transmission: 3.75 kHz and 15 kHz. A cyclic prefix is inserted. Frequency domain sinc pulse-shaping in the physical layer description.

o	Multi-tone transmissions are supported, based on SC-FDMA with 15 kHz UL subcarrier spacing.

o	FFS: Additional mechanisms for PAPR reduction.

o	The UE shall indicate the support single-tone and/or multi-tone, details to be discussed by RAN WGs

To maintain orthogonality between NB-PUSCH’s with 15 kHz subcarrier spacing, accurate timing advance for aligning the uplink subframe boundary among different UEs is needed. Thus, NB-PRACH has to be designed accordingly. In [3-4], a NB-PRACH design is shown to be able to support 15 kHz subcarrier spacing for NB-PUSCH. One remaining open issue is the near-far performance of such a NB-PRACH design. This is analysed in this contribution.

# NB-PRACH preamble formats

In RAN1 NB-IoT ad hoc meeting, it was agreed that “Support one PRACH scheme for all MCL cases [5].” Also, as a working assumption, “PRACH scheme is based on single-tone transmission [5].” Thus, our evaluations are based on single-tone frequency-hopping transmissions [3-4]. Specifically, the designs according to Table 1 are used. More details can be found in [3-4].

Table 1: Example design for single tone frequency hopping NB-PRACH

| Cell size (km) | MCL (dB) | Subcarrier spacing (kHz) | Tcp (us) | Number of symbols per group | Number of groups | Hopping pattern | Pseudo random hopping range |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 35 | 144 | 3.75 | 266.7 | 5 | 8 | 1 tone micro hopping + pseudo random hopping | 12 tones |
|  | 154 | 3.75 | 266.7 | 5 | 24 | 1 tone micro hopping + pseudo random hopping | 12 tones |
|  | 164 | 3.75 | 266.7 | 5 | 120 | 1 tone micro hopping + pseudo random hopping | 12 tones |
| Remark 1: Pseudo random hopping is based on LTE PUSCH type 2 hopping. |  |  |  |  |  |  |  |

# Simulation setup

The simulation assumptions used are based on the ones outlined in [6] and are summarized in Table 2. The only simulation assumption that is different from the ones in [6] is the frequency offset. In [6], 50 Hz was assumed, however to verify the robustness of NB-PRACH design in the presence of a larger frequency offset, we use 100 Hz in our analysis.

Table 2: Simulation parameters

| Parameter | Value |
| --- | --- |
| Channel model | TU |
| Doppler spread | 1 Hz |
| Antenna configuration | 1 Tx; 2 Rx |
| Cell size | 35 km |
| Timing uncertainty | Randomly drawn from [-5.2 us, 233.3+5.2 us] for 35 km cell, or |
| MS frequency drift | Uniformly drawn from the set {-22.5 Hz/s, 22.5 Hz/s}. |
| Number of channel realizations | 10,000 |
| Note 1: 233.3 us is the maximum round-trip delay in the cell of size 35 km. The additional +- 5.2 us accounts for the residual timing errors in the cell search. The timing uncertainty is drawn based on uniform user location in a hexagonal cell of size 35 km. |  |

To analyse the robustness to the near-far problem, two different NB-PRACH preambles are transmitted by two different UEs with different received power levels. Here, two different preambles mean that the two transmissions use two different logical tones in NB-PRACH band. The difference in averaged received power in dB is referred to as the near-far ratio. Since the two NB-PRACH preambles are transmitted through independent fading channels, the difference in instantaneous power levels may be larger than the near-far ratio. The performance of time-of-arrival estimation of the weaker received performance is analysed.

# Simulation results

The near-far performance of preambles designed for 144 dB MCL (see Table 1) is shown in Figure 1. Each curve corresponds to one near-far ratio. It can be seen that the design is robust to near-far ratios up to 20 dB since the ToA estimation error falls within [-5, 5] s with more than 99% probability. The ToA estimation performance with 25 dB near-far ratio (purple line) might be also acceptable, although some small percentage of outliers of ToA estimation error is observed.

Figure 1: Timing advance estimate accuracy of NB-PRACH preamble designed for 144 dB MCL in near-far scenarios. (Performance of weaker received NB-PRACH preamble, 100 Hz frequency offset)

Near-far performance of preambles designed for extended coverage, 154 dB and 164 dB MCL, is shown in Figure 2 and Figure 3. Due to that the number of symbol groups is increased (see Table 1) for preambles designed for extended coverage, they are more robust to the near-far problem. As seen in Figure 2 and Figure 3, preambles designed for 154 dB MCL achieve good ToA estimation accuracy with near-far ratios up to 25 dB, and preambles designed for 164 dB MCL achieve good ToA estimation accuracy with near-far ratios up to 30 dB.

Figure 2: Timing advance estimate accuracy of NB-PRACH preamble designed for 154 dB MCL in near-far scenarios. (Performance of weaker received NB-PRACH preamble, 100 Hz frequency offset)

Figure 3: Timing advance estimate accuracy of NB-PRACH preamble designed for 164 dB MCL in near-far scenarios. (Performance of weaker received NB-PRACH preamble, 100 Hz frequency offset)

# Conclusions

In this contribution, we study the performance of NB-PRACH preamble detection when two different preambles are received at the same time with different power levels. It is found that the NB-PRACH preamble design based on [3-4] has good near-far performance.

Observation 1:

- Preambles designed for 144 dB MCL are robust to a near-far ratio up to 20 dB.

- Preambles designed for 154 dB MCL are robust to a near-far ratio up to 25 dB.

- Preambles designed for 164 dB MCL are robust to a near-far ratio up to 30 dB.

# References

- RP-151621, “New Work Item: NarrowBand IOT (NB-IOT),” source Qualcomm Inc., RAN #69, Sept. 2015.

- RP-152284, “New Work Item: NarrowBand IOT (NB-IOT),” source Huawei and HiSilicon, RAN #70, Dec. 2015.

- R1-160275, “NB-IoT – Singe Tone Frequency Hopping NB-PRACH Design,” source Ericsson, RAN1 #84, Feb. 2016, St Julian’s, Malta.

- R1-160439, “NB-IoT – NB-PRACH Frequency Hopping Pattern,” source Ericsson, RAN1 #84, Feb. 2016, St Julian’s, Malta.

- RAN1 Chairman’s Notes, RAN1 NB-IoT Ad Hoc, January 2016.

- 3GPP TR 45.820, “Technical Specification Group GSM/EDGE Radio Access Network; Cellular System; Support for Ultra Low Complexity and Low Throughput Internet of Things; (Release 13),” www.3gpp.org.