3GPP TSG RAN WG1 Meeting #84	R1-160325

St Julian’s, Malta, 15th-19th February 2016

Agenda Item:	7.2.1.2.1

Source:	Huawei, HiSilicon

Title:	NB-PUSCH design

Document for:	Discussion and decision

# Introduction

At RAN1 NB-IoT Ad-Hoc, it was agreed that (see [1]):

- One resource unit schedulable in PUSCH transmission for the data consists of fixed [X] msec at least for FDD

  - X(115kHz) is 8 for 15 kHz case with single tone transmission

  - X(13.75kHz) is 32 for 3.75 kHz case with single tone transmission

  - In case of multiple tones is allocated for one UE, X{m} is smaller than X(115kHz)

    - In case of 12 tones is allocated for one UE, X{12} is 1

- UL multi-tone transmission for the data with 12 tones is supported

  - UL multi-tone transmission for the data also supports followings of numbers of multiple

    - {3} with 4 msec resource unit size

    - {6} with 2 msec resource unit size

- For 15kHz subcarrier spacing, OFDM/SC-FDMA symbol boundary is no change from LTE

- At least for FDD with the normal CP case,

  - For 3.75kHz subcarrier spacing of uplink,

    - Define a 2ms NB-slot, there are 7 symbols.

    - One symbol consists of [FFS] Ts of symbol with CP length of [FFS] Ts assuming Ts=1/1.92MHz. Editor will take care how to capture Ts value in the spec.

    - The above symbols are located from the beginning of 2ms period. FFS the usage of the remaining time in the NB-slot if there is the remaining time

    - The 2ms NB-slot boundary is aligned with LTE subframe boundary. FFS among odd only or even only or to support both even/odd

    - FFS on collision of LTE SRS for in band mode

      - Opt 1: eNB scheduling or implementation

      - Opt 2: puncture/rate matching of 3.75kHz transmission to avoid the collision

      - Opt 3: define a GP to avoid potential collision

      - Note combination of above options can also be considered.

- When single-tone is allocated, Pi/4-QPSK and Pi/2-BPSK are supported

  - FFS: How rotation is realized

  - FFS: Phase rotation for RS(s)

- When multi-tone is allocated, QPSK is supported

  - FFS: TPSK, 8-BPSK

This document provides some further design considerations for NB-PUSCH.

# Reference signal

In LTE, DMRS is used to acquire and track channel response for PUSCH. It is interesting to understand whether the same design can also apply in NB-IoT, and whether the DMRS density in LTE is sufficient for the support of the 164 dB MCL target for the 15 kHz subcarrier spacing option of NB-PUSCH.

Simulations were performed to investigate the link level performance of NB-PUSCH assuming different DMRS densities with simulation assumptions in Annex A, and the results are shown in Figure 1 of Annex B. It can be seen that by increasing the DMRS density from 1 symbol per slot (i.e. the same as in LTE) to 2 symbols per slot, the BLER performance is only improved by 0.5 dB for TU 1 Hz channel and 0.6 dB for TU 100 Hz channel. The gain cannot even compensate the loss due to the decrease of data symbol density from 6 to 5 per slot, i.e. 10*log10(5/6)=-0.8 dB.

In fact, as shown in Table 7:, the MCL target of 164 dB is comfortably met for NB-PUSCH by reusing the LTE DMRS density. Hence it is proposed to reuse the LTE uplink DMRS pattern for the 15 kHz subcarrier spacing option of NB-PUSCH.

For the 3.75 kHz subcarrier spacing option, it was agreed that there are 7 symbols in a 2ms NB-slot [2]. Similar pattern as LTE DMRS can be used for NB-DMRS, i.e. the third or the fifth symbol of every 7 symbols as NB-DMRS. The little difference from LTE DMRS is to avoid the possible collision between NB-DMRS and LTE SRS, see [2].

- For NB-PUSCH with 15 kHz subcarrier spacing, the same DMRS pattern as in LTE uplink is reused (i.e. the middle symbol of every 7 symbols is used as uplink NB-DMRS).

- For NB-PUSCH with 3.75 kHz subcarrier spacing, the third or the fifth symbol of every 7 symbols is allocated to NB-DMRS.

# Modulation

In RAN1 NB-IoT Ad hoc #1, it was agreed that Pi/4-QPSK and Pi/2-BPSK are supported for single-tone NB-PUSCH transmission. Phase rotation is helpful to reduce PAPR and out-of-band emission for single-tone transmission. To obtain the benefits, phase rotation should be implemented in a contiguous way at symbol level including both data symbol and NB-DMRS symbol.

- For symbol n (n = 0, 1, 2, …) of a single-tone NB-PUSCH transmission using pi/4-QPSK modulation, the phase rotated constellation point for both data and NB-DMRS symbols is obtained by multiplying the unrotated constellation point (as defined in Table 7.1.2-1 of TS 36.211) by exp(+j*n*pi/4).

- For symbol n (n = 0, 1, 2, …) of a single-tone NB-PUSCH transmission using pi/2-BPSK modulation, the phase rotated constellation point for both data and NB-DMRS symbols is obtained by multiplying the unrotated constellation point (as defined in Table 7.1.1-1 of TS 36.211) by exp(+j*n*pi/2).

Since the BPSK constellation points are a subset of the QPSK constellation points, BPSK can be used as the modulation for NB-DMRS symbols (prior to phase rotation) regardless of whether the modulation for data symbols (prior to phase rotation) is BPSK or QPSK, without introducing any adverse impact on PAPR. The NB-DMRS sequence can be generated using the Gold sequence generator in LTE.

- For single-tone transmission of NB-PUSCH, the modulation for NB-DMRS symbols, prior to phase rotation, is BPSK.

- For single-tone transmission of NB-PUSCH, the NB-DMRS sequence is generated using the Gold sequence generator defined in sub-clause 7.2 of TS 36.211.

# Coding and rate matching

The Turbo coding and rate matching schemes in LTE is proposed to be reused for NB-PUSCH, because it provides significantly higher coding gain than convolutional coding. Although the decoding complexity of turbo coding is higher than that of convolutional coding, the decoding is performed at the base station and the complexity is considered acceptable.

- Turbo coding and rate matching in LTE is reused for NB-PUSCH.

To minimize the downlink signaling overhead, it is suggested that only two RVs are supported for NB-PUSCH.

- Two redundancy versions are supported for NB-PUSCH.

As agreed in RAN2#91bis, “1 HARQ process for dedicated transmissions (1 for UL and 1 for DL)” is supported [4]. Therefore, RAN1 should design according to this agreement. Due to the wide variation of the transmission duration between different coverage levels, it is hard to fix the time interval between first transmission and re-transmission. Consequently, asynchronous HARQ is proposed. For more discussion of this, see [5].

- Single process HARQ for NB-PUSCH is realized by adaptive and asynchronous transmission.

# MCS and TBS

## Single-tone transmission

For single-tone NB-PUSCH transmission, it was agreed that Pi/2-BPSK and Pi/4-QPSK are supported. As Pi/2-BPSK is expected to be used mostly for extreme coverage, it is suggested that only one MCS level (i.e. MCS 0) with low code rate is designed for it. For Pi/4-QPSK, considering the measurement in NB-IoT is less accurate than that in LTE, the MCS table for NB-IoT needs to only reuse the even MCS indices for QPSK for LTE PUSCH. The MCS index can be comprised of 3 bits to indicate 5 levels with RV index 0 and 3 levels with RV index 2. The overall modulation and TBS index table is shown in Table 1. The corresponding TBS indices (i.e. 0, 2, 4, 6, 8) in Table 8.6.1-1 of TS 36.213 are also reused.

Table 7.1.7.2.1-1 in TS 36.213 with ITBS=0, 2, 4, 6, and 8 are reused but with the restriction that the TBS is not larger than 1000 bits, see Table 2.

- For single-tone transmission of NB-PUSCH with Pi/4-QPSK modulation, MCS indices 2, 4, 6 and 8 in Table 8.6.1-1 of TS 36.213 are reused.

- For single-tone transmission of NB-PUSCH with Pi/2-BPSK modulation, a new MCS with index 0 is introduced.

- For single-tone transmission of NB-PUSCH, TBS indices 0, 2, 4, 6 and 8 in Table 7.1.7.2.1-1 of TS 36.213 are reused with an appropriately restricted TBS limit.

Table 1: Modulation and TBS index table for single tone transmission of NB-PUSCH

| MCS Index | Modulation Order | TBS Index | Redundancy Version rvidx |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 0 |
| 1 (21) | 2 | 1 (22) | 0 |
| 2 (41) | 2 | 2 (42) | 0 |
| 3 (61) | 2 | 3 (62) | 0 |
| 4 (81) | 2 | 4 (82) | 0 |
| 5 (41) | 2 | 2 (42) | 2 |
| 6 (61) | 2 | 3 (62) | 2 |
| 7 (81) | 2 | 4 (82) | 2 |

(01)es 0, 2, 4, 6, and 8 0, 2, 4, 6, and 8)lso reused.alNote 1: IMCS in Table 8.6.1-1 of TS 36.213.

Note 2: ITBS in Table 8.6.1-1 of TS 36.213.

Table 2: TBS table for single-tone transmission of NB-PUSCH

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 (01) | 16 | 32 | 56 | 88 | 120 | 152 | 176 | 208 | 224 | 256 |
| 1 (21) | 32 | 72 | 144 | 176 | 208 | 256 | 296 | 328 | 376 | 424 |
| 2 (41) | 56 | 120 | 208 | 256 | 328 | 408 | 488 | 552 | 632 | 696 |
| 3 (61) | 88 | 176 | 256 | 392 | 504 | 600 | 712 | 808 | - | - |
| 4 (81) | 120 | 256 | 392 | 536 | 680 | 808 | - | - | - | - |
|  | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| 0 (01) | 288 | 328 | 344 | 376 | 392 | 424 | 456 | 488 | 504 | 536 |
| 1 (21) | 472 | 520 | 568 | 616 | 648 | 696 | 744 | 776 | 840 | 872 |
| 2 (41) | 776 | 840 | - | - | - | - | - | - | - | - |
|  | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| 0 (01) | 568 | 600 | 616 | 648 | 680 | 712 | 744 | 776 | 776 | 808 |
| 1 (21) | - | - | - | - | - | - | - | - | - | - |
|  | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
| 0 (01) | 840 | 872 | - | - | - | - | - | - | - | - |

Note 1: ITBS in Table 7.1.7.2.1-1 of TS 36.213.

## Multi-tone transmission

For multi-tone NB-PUSCH transmission, similarly, the even MCS indices and the corresponding TBS indices for QPSK for LTE PUSCH are reused, as shown in Table 3. This can be revisited if other modulation scheme(s) are introduced for multi-tone.

Table 3: Modulation and TBS index table for multi-tone transmission of NB-PUSCH

| MCS Index | Modulation Order | TBS Index | Redundancy Version rvidx |
| --- | --- | --- | --- |
| 0 (01) | 2 | 0 (02) | 0 |
| 1 (21) | 2 | 1 (22) | 0 |
| 2 (41) | 2 | 2 (42) | 0 |
| 3 (61) | 2 | 3 (62) | 0 |
| 4 (81) | 2 | 4 (82) | 0 |
| 5 (41) | 2 | 2 (42) | 2 |
| 6 (61) | 2 | 3 (62) | 2 |
| 7 (81) | 2 | 4 (82) | 2 |

(01)es 0, 2, 4, 6, and 8 0, 2, 4, 6, and 8)lso reused.alNote 1: IMCS in Table 8.6.1-1 of TS 36.213.

Note 2: ITBS in Table 8.6.1-1 of TS 36.213.

Since the number of REs in one resource unit for NB-PUSCH is the same as that in a LTE PRB, the even TBS indices in the TBS table for LTE PUSCH can also be fully reused, as shown in Table 4.

- For multi-tone transmission of NB-PUSCH, MCS indices 0, 2, 4, 6 and 8 in Table 8.6.1-1 of TS 36.213 are reused.

- For multi-tone transmission of NB-PUSCH, TBS indices 0, 2, 4, 6 and 8 in Table 7.1.7.2.1-1 of TS 36.213 are reused with an appropriately restricted TBS limit.

Table 4: TBS table for multi-tone transmission of NB-PUSCH

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 (01) | 16 | 32 | 56 | 88 | 120 | 152 | 176 | 208 | 224 | 256 |
| 1 (21) | 32 | 72 | 144 | 176 | 208 | 256 | 296 | 328 | 376 | 424 |
| 2 (41) | 56 | 120 | 208 | 256 | 328 | 408 | 488 | 552 | 632 | 696 |
| 3 (61) | 88 | 176 | 256 | 392 | 504 | 600 | 712 | 808 | 936 | - |
| 4 (81) | 120 | 256 | 392 | 536 | 680 | 808 | - | - | - | - |
|  | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| 0 (01) | 288 | 328 | 344 | 376 | 392 | 424 | 456 | 488 | 504 | 536 |
| 1 (21) | 472 | 520 | 568 | 616 | 648 | 696 | 744 | 776 | 840 | 872 |
| 2 (41) | 776 | 840 | - | - | - | - | - | - | - | - |
|  | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| 0 (01) | 568 | 600 | 616 | 648 | 680 | 712 | 744 | 776 | 776 | 808 |
|  | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
| 0 (01) | 840 | 872 | - | - | - | - | - | - | - | - |

Note 1: ITBS in Table 7.1.7.2.1-1 of TS 36.213.

# Repetitions

Repetitions are proposed to be applied at TTI level. The proposed options for the number of repetitions for NB-PUSCH are {1, 2, 4, 8, 16, 32, 64, 128}. DCI can indicate directly from this set, with no need for a higher layer configuration.

- The number of repetitions for a NB-PUSCH transmission is selected from {1, 2, 4, 8, 16, 32, 64, 128}.

# Conclusions

In this contribution, design considerations for NB-PUSCH are described with following proposals:

- For NB-PUSCH with 15 kHz subcarrier spacing, the same DMRS pattern as in LTE uplink is reused (i.e. the middle symbol of every 7 symbols is used as uplink NB-DMRS).

- For NB-PUSCH with 3.75 kHz subcarrier spacing, the third or the fifth symbol of every 7 symbols is allocated to NB-DMRS.

- For symbol n (n = 0, 1, 2, …) of a single-tone NB-PUSCH transmission using pi/4-QPSK modulation, the phase rotated constellation point for both data and NB-DMRS symbols is obtained by multiplying the unrotated constellation point (as defined in Table 7.1.2-1 of TS 36.211) by exp(+j*n*pi/4).

- For symbol n (n = 0, 1, 2, …) of a single-tone NB-PUSCH transmission using pi/2-BPSK modulation, the phase rotated constellation point for both data and NB-DMRS symbols is obtained by multiplying the unrotated constellation point (as defined in Table 7.1.1-1 of TS 36.211) by exp(+j*n*pi/2).

- For single-tone transmission of NB-PUSCH, the modulation for NB-DMRS symbols, prior to phase rotation, is BPSK.

- For single-tone transmission of NB-PUSCH, the NB-DMRS sequence is generated using the Gold sequence generator defined in sub-clause 7.2 of TS 36.211.

- Turbo coding and rate matching in LTE is reused for NB-PUSCH.

- Two redundancy versions are supported for NB-PUSCH.

- Single process HARQ for NB-PUSCH is realized by adaptive and asynchronous transmission.

- For single-tone transmission of NB-PUSCH with Pi/4-QPSK modulation, MCS indices 2, 4, 6 and 8 in Table 8.6.1-1 of TS 36.213 are reused.

- For single-tone transmission of NB-PUSCH with Pi/2-BPSK modulation, a new MCS with index 0 is introduced.

- For single-tone transmission of NB-PUSCH, TBS indices 0, 2, 4, 6 and 8 in Table 7.1.7.2.1-1 of TS 36.213 are reused with an appropriately restricted TBS limit.

- For multi-tone transmission of NB-PUSCH, MCS indices 0, 2, 4, 6 and 8 in Table 8.6.1-1 of TS 36.213 are reused.

- For multi-tone transmission of NB-PUSCH, TBS indices 0, 2, 4, 6 and 8 in Table 7.1.7.2.1-1 of TS 36.213 are reused with an appropriately restricted TBS limit.

- The number of repetitions for a NB-PUSCH transmission is selected from {1, 2, 4, 8, 16, 32, 64, 128}.

# References

Draft Report of 3GPP TSG RAN WG1 #AH_NB-IoT v0.1.0, MCC Support.

R1-160038, “Remaining details of uplink frame structure design”, Huawei, HiSilicon, 3GPP RAN1#84, St Julian’s, Malta, February 2016.

R1-160100, “PAPR reduction for single tone PUSCH transmission”, Huawei, HiSilicon, RAN1 NB-IoT Adhoc, Budapest, Hungary, January 2016.

R2-154893, “Report of the LTE breakout session (NB-IoT)”, Mediatek Inc. (Session Chair), RAN2#91bis.

R1-160326, “Timing relationships”, Huawei, HiSilicon, 3GPP RAN1#84, St Julian’s, Malta, February 2016.

R1-160324, “UCI for NB-IoT”, Huawei, HiSilicon, 3GPP RAN1#84, St Julian’s, Malta, February 2016.

- Simulation assumptions

Table 5:: Simulation assumptions

| Parameter | Value |
| --- | --- |
| System bandwidth | 180 kHz |
| Carrier frequency | 900 MHz |
| Antenna configuration | 1T2R |
| Channel model | TU 1Hz/100Hz |
| Frenquency error | F_offset(t) = F_est_error + (F_drift_active * t) |
| Frenquency error(F_est_error) | Randomly chosen from the set {-50, 50} Hz |
| Frequency drift rate (F_drift_active) | 22.5 Hz/second |
| Timing error | Randomly distributed between [-2.6, 2.6] us |
| HARQ | Not enabled |
| Performance target | 10% BLER |
| Channel estimation | Realistic cross-subframe channel estimation |
| Subcarrier bandwidth (kHz) | 3.75, 15 |
| Number of subcarriers | 1 |
| Modulation | QPSK |
| Coding | Turbo rate-1/3 |
| Repetition | 8 for 3.75 kHz subcarrier spacing, 32 for 15 kHz subcarrier spacing |
| Packet Length (bits) | 800 |

- BLER performance for different DMRS densities

Figure 1: BLER performance for different DMRS densities

- BLER performance for QPSK

Figure 2: BLER performance for QPSK

Table 7: MCL calculation for uplink data channel

|  | PUSCH single-tone(3.75kHz) |  | PUSCH single-tone(15kHz) |  |
| --- | --- | --- | --- | --- |
|  | UL-SCH |  | UL-SCH |  |
|  | TU 1Hz | TU 100Hz | TU 1Hz | TU 100Hz |
| Transmitter |  |  |  |  |
| (1) Total Tx power (dBm) | 23 | 23 | 23 | 23 |
| Receiver |  |  |  |  |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 |
| (3) Receiver noise figure (dB) | 3 | 3 | 3 | 3 |
| (4) Interference margin (dB) | 0 | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 3750 | 3750 | 15000 | 15000 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log((5)) (dBm) | -135.3 | -135.3 | -135.3 | -135.3 |
| (7) Required SINR (dB) | -8.0 | -5.9 | -12.7 | -12.5 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -143.3 | -141.2 | -129.2 | -129.2 |
| (9) Rx processing gain (dB) | 0 | 0 | 0 | 0 |
| Maximum coupling loss |  |  |  |  |
| (10) MCL = (1) – (8) + (9) (dB) | 166.3 | 164.2 | 164.9 | 164.7 |
| Data rate at the SAP to the SNDCP (kbps) | 0.21 | 0.21 | 0.21 | 0.21 |