3GPP TSG RAN WG1 Meeting #84	R1-160411

St Julian’s, Malta, 15th - 19th February 2016

Source: 	Intel Corporation

Title:                     	On NB-IoT Secondary Synchronization Signal Design

Agenda item:	7.2.1.1.4

Document for:	Discussion and Decision

# 1 Introduction

At the RAN WG1 Ah-hoc meeting in January, a number of agreements were reached on the synchronization signal design for NB-IoT [1].

Agreements

- One transmission of NB-PSS, NB-SSS, NB-PBCH, and NB-PDSCH never overlaps between multiple LTE PRB bandwidths for in-band operation

- The number of subcarriers for NB-SSS is 12

- The number of subcarriers for NB-PSS is 12 or 11

  - FFS: exact number of subcarrier

- NB-PSS uses the last 11 OFDM symbols of subframes in which NB-PSS occurs for normal CP

- NB-SSS uses the last [11] or [9] OFDM symbols of subframes  in which NB-SSS occurs for normal CP

No discussion or decision was made on the NB-SSS design. In this contribution, we present our NB-SSS design from [2], and also provide performance results for the same. Note that the NB-PSS design is discussed in our companion contribution in [3]. We also discuss potential issues with the NB-SSS design discussed in [4].

# NB-Secondary Synchronization Signal Design

## 2.1 Secondary Synchronization Signal Design in [4]

The Secondary Synchronization Sequence (NB-SSS) design using two Zadoff Chu sequences with two independent roots was proposed in [4]. The combination of the two root indices was used to indicate physical cell ID.

SSS1n=exp-jπu1nn+1NSSS,                 n=0,1,… , NSSS-1

SSS2n=exp-jπu2nn+1NSSS,                n=0,1,… , NSSS-1

Since each SSS1 and SSS2 carries independent information, both the sequences have to be detected correctly in order for correct SSS detection and thus obtain cell ID information which is represented by the combination of two short codes. If a user is at the cell edge, the user can see similar strength signals from two or possibly three cells. In such a scenario, a user can detect SSS1 from one cell and SSS2 from another cell, leading to a cell-ambiguity problem. The idea of carrying cell ID information using two short codes is the same as proposed earlier in LTE SSS. Such ambiguity and collision problems [5] are well known and that is why two types of scrambling sequences were proposed (PSS based scrambling and SSS1 based scrambling on SSS2 in order to resolve the issue). However, the problem is not completely resolved because the number of SSS1based scrambling codes is limited by 8 – thus, there is still a 3dB sidelobe of the cross-correlation values when one of two codes is identical. In some scenarios, CRS based verification may be needed. Figure 1 shows such an ambiguous cell search scenario and the possible problematic scenarios are shown in Table 1 (copied from [5]).

Figure 1 Cell Search Scenario [5]

Table 1. 8 scenarios for PSC, SSC1, SSC2 triplet [5]

| No. | Pa & Pb | S1a & S1b | S2a & S2b | Problem |
| --- | --- | --- | --- | --- |
| 1 | Same | Same | Same | n/a |
| 2 | Same | Same | Different | Collision + Phase mismatch |
| 3 | Same | Different | Same | Collision + Phase mismatch |
| 4 | Same | Different | Different | Ambiguity + Phase mismatch |
| 5 | Different | Same | Same | Phase mismatch |
| 6 | Different | Same | Different | Collision + Phase mismatch |
| 7 | Different | Different | Same | Collision + Phase mismatch |
| 8 | Different | Different | Different | Ambiguity + Phase mismatch |

Thus, unless the additional scrambling (increasing UE complexity) is introduced, use of two short codes combination to carry cell ID information needs to be avoided.

Proposal 1: The two code NB-SSS sequence is not used to represent cell ID information.

## 2.2 Proposed Secondary Synchronization Signal Design

The Secondary Synchronization Sequence (NB-SSS) is used in NB-IOT to achieve frame synchronization and detect one out of the N = 504 cell IDs. The NB-SSS design we discuss here is slightly different from the one mentioned in [2]. NB-SSS is generated in the frequency domain, occupies 12 subcarriers and is spread across 11 OFDM symbols. The NB-SSS symbols corresponding to the LTE RS are punctured. The NB-SSS is composed of a length NZC= 131 Zadoff-Chu (ZC) sequence cyclically extended to occupy 132 resources (other option would be the truncation of 5 last elements from length-137 ZC sequence). This 131 length ZC sequence can be generated using different roots and scrambled with unique sequences to provide 504 unique cell identities. To support N=504 cell IDs, we propose the use of different root indices and scrambling codes - four scrambling codes would be sufficient to cover 504 cell IDs (130*4=520>504). This NB-SSS sequence is placed at every 20 ms. The time domain allocation is shown in Fig. 1. If additional information needs to be indicated (e.g. the 80ms boundary and subframe location within the 80ms boundary), (e.g. dmax=4) different cyclic time shifts can be applied to the frequency domain SSS sequence.

Figure 2. Time domain location of NB-PSS and NB-SSS transmission for NB-IoT

We next provide the details of the sequence design for the NB-SSS sequence supporting 504 cell IDs, p=0,1,…,503. We use a combination of the root of the ZC sequence and a scrambling code to represent the cell ID of a cell. Specifically, the root index, say rp and scrambling sequence index, say sp uniquely represent a cell ID p, where

rp=1+modp,131 and

sp= p131.

To represent the location at ‘d’ different radio frame locations, we use time-domain cyclic shifts lq, where q =0, 1, 2, …, dmax-1.

Note that rp takes the values from {1,…,131}, while sp takes on the values {0, 1, 2, 3, 4}. The transmitter is shown in Fig. 2.

Figure 3. Transmitter for NB-SSS generation

The NB-SSS sequence is given by

SSSn=e-jπrp(n'n'+1NZC cspn e-j2πlqndmax,       where n'=n mod NZc ,n=0,1,…,NZC,  lq={0, 1, 2, …, dmax-1}

where cspn is the scrambling sequence. If sending NB-SSS dmax=4 times like in Fig. 2, the following values of  can be used.

l0 =0, l1=33, l2=66, l3=99

The scrambling sequence can be a M-sequence or a PN sequence with the 4 different sequences chosen to have good autocorrelation and cross-correlation properties. Note that NZc=131 in the equation above.

Proposal 2: It is proposed for NB-SSS:

- A combination of root index of Zadoff Chu sequence and scrambling is used for carrying the cell ID information.

- To embed the frame number information (e.g. the 80ms boundary and subframe location within the 80ms boundary) different cyclic time shifts can be applied to the frequency domain SSS sequence.

# Simulation Results

The simulation assumptions are tabulated in Table 1. For standalone mode, a transmit filter consisting of 19 taps is used in order that the NB-IoT signal fulfils the GSM PSD mask. This is essential to reduce the leakage outside the transmission bandwidth, thereby causing minimal interference to other systems, e.g., GSM. For guard-band and inband operation, no such filter is required. Three levels of coverage enhancement for each operation mode (i.e. standalone, guard-band and in-band) are considered in the simulations, corresponding to coupling loss values of 164 dB, 154 dB and 144 dB. The correlation outputs are accumulated over multiple 20 ms windows. Rest of the simulation assumptions are summarized in Table 1.

Table 1: Simulation Parameters

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 900 MHz |
| Channel Model | TU 1 Hz |
| Subcarrier Spacing | 15 kHz |
| Coupling loss | 144 dB, 154 dB, 164 dB |
| Sampling Frequency (Fs) | 1.92 MHz (A/D) |
| Cyclic Prefix | 10 samples for the 1st and 7th OFDM symbol within a subframe, 9 samples for the rest |
| Antenna Configuration | 1 Tx, 1 Rx for standalone 2 Tx, 1 Rx for guard-band/inband |
| Number of realizations | 1000 |

We next report the time after timing and frequency offset estimation required to detect the Cell ID correctly. Once the NB-IOT UE has timing and frequency information from NB-PSS, the UE tries look for NB-SSS in the next 20 ms. If needed, the UE can accumulate over multiple 20 ms subframes to detect NB-SSS. Correct NB-SSS detection leads to detection of cell ID as well as frame number information. We report the average, 50th percentile and 95th percentile for the three coupling losses (144 dB, 154 dB, 164 dB) in Tables 2, 3 and 4 for standalone, guard band and inband operation modes respectively.

Table 2: Time required for Cell ID detection (in ms) for standalone operation

| Statistic | 144 dB coupling loss | 154 dB coupling loss | 164 dB coupling loss |
| --- | --- | --- | --- |
| Average | 20 | 20.8 | 34 |
| 50th percentile | 20 | 20 | 20 |
| 95th percentile | 20 | 20 | 110 |

Table 3: Time required for Cell ID detection (in ms) for guard-band operation

| Statistic | 144 dB coupling loss | 154 dB coupling loss | 164 dB coupling loss |
| --- | --- | --- | --- |
| Average | 20 | 23.4 | 58.93 |
| 50th percentile | 20 | 20 | 40 |
| 95th percentile | 20 | 40 | 220 |

Table 4: Time required for Cell ID detection (in ms) for inband operation

| Statistic | 144 dB coupling loss | 154 dB coupling loss | 164 dB coupling loss |
| --- | --- | --- | --- |
| Average | 20.2 | 25.8 | 65.84 |
| 50th percentile | 20 | 20 | 40 |
| 95th percentile | 20 | 60 | 240 |

# Conclusions

We described possible issues with the design in [4] leading to cell-ambiguity problem and proposed NB-SSS sequence design using a combination of Zadoff Chu sequence and sequence scrambling to indicate the cell ID. We also presented simulation results for the same. In conclusion, we propose the following:

Proposal 1: The two code NB-SSS sequence is not used to represent cell ID information.

Proposal 2: It is proposed for NB-SSS:

- A combination of root index of Zadoff Chu sequence and scrambling is used for carrying the cell ID information.

- To embed the frame number information (e.g. the 80ms boundary and subframe location within the 80ms boundary) different cyclic time shifts can be applied to the frequency domain SSS sequence.

# References

- “RAN 1 Chairman notes,” 3GPP TSG-RAN1 NB-IOT Ad Hoc, Budapest, Hungary, Jan 2016.

- R1-160130, “NB-IoT Secondary Synchronization Signal Design,’’ Intel Corporation, 3GPP TSG-RAN1 NB-IOT Ad Hoc, Budapest, Hungary, Jan 2016.

- R1-160410, “On NB-IoT Primary Synchronization Signal Design,” Intel Corporation, 3GPP TSG RAN WG1 Meeting #84, St Julian’s, Malta, Feb 2016.

- R1-160182, “Synchronization Signal Design,’’ 	Huawei, HiSilicon, China Telekom, RAN 1 # 83, 3GPP TSG-RAN1 NB-IOT Ad Hoc, Budapest, Hungary, Jan 2016.

- R1-073434, “Secondary SCH Mapping and Scrambling,’’	Texas Instruments, RAN 1 # 50, Athens, Greece, Aug 2007.