3GPP TSG RAN WG1 Meeting #84			R1-160428

St. Julian’s, Malta, 15th – 19th February 2016

Agenda Item:          7.3.1.5

Source: 		Intel Corporation

Title:	Introduction of the Non-Scheduled UL Operation for eLAA

Document for: 	Discussion/Decision

# Introduction

It was identified that LAA UL can experience significant performance degradation as documented in our companion contributions [1-2].

- One of the main reasons of the performance degradation is due to the “triple” contention for UEs to access the UL. That is, 1) the LBT performed at the eNB before sending UL grant, 2) UE selection by eNB, and 3) the LBT performed at the scheduled UE before transmission.

- In addition, one particular limitation imposed on the LTE system is the four subframe processing delay between UL grant and PUSCH transmission. The four subframe processing delay prevents the initial four subframes to be configured as UL since the UL grants are unavailable for those subframes within the burst.

This contribution provides an overview of the non-scheduled (grant-less) LAA UL transmission that can tackle the above referred UL scheduling starvation issue. To begin with, the non-scheduled UL transmission has advantage over scheduled UL transmission in the following aspects:

- If a UE succeeds the LBT, then it can start transmitting immediately as Wi-Fi. Thus, no more triple contention is imposed on the UL access.

- The non-scheduled transmission does not rely on the UL grant and, thus, the four subframe processing delay has no impact.

- It will naturally well-coexist with Wi-Fi as the UE behaviour is not different from Wi-Fi stations.

# Non-Scheduled UL Operation

## Overview of the design

In this section, necessary enhancements to enable non-scheduled operation will be discussed including UL LBT, UL subframe design, scheduling, link adaptation and the detection of UL burst at the eNB.

Figure 1 illustrates the potential non-scheduled operation. The UE that intends to transmit to the anchored eNB performs channel sensing without the need for UL grant from the eNB. After completing the LBT, the UE then starts its transmission immediately.

Figure 1: An illustration of non-scheduled UL transmission

## UL LBT design

In order to maintain fair co-existence with incumbent systems and with other LAA networks, the non-scheduled UEs should perform full category 4 LBT as the Release 13 DL LAA. This will ensure natural coexistence with neighbour Wi-Fi/LAA networks as it is verified by system-level performance evaluation in Section 3.

## UL subframe design

The non-scheduled UE may sense the channel and perform UL transmission at any time. Similar to the design consideration for Release 13 LAA, a partial subframe for PUSCH transmission can be considered to minimize the implementation impact. Figure 1(a) provides an illustration of non-scheduled PUSCH transmission utilizing partial subframe.

As another alternative, PUSCH can be transmitted without the alignment with PCell subframe boundary (aka floating subframe) as shown in Figure 1(b). The UE may transmit the PUSCH transmission based on the legacy 1 ms subframe design after the completion of LBT.

## PUSCH detection at eNB

For the UE-autonomous transmission, the eNB needs to detect the presence of the UL burst. The detection can be done via:

- Preamble signal (or initial signal)

- DMRS

In Section 3, the DMRS-based presence detection performance is provided.

## Link adaptation

Link adaptation and MCS selection at the UE can be performed based on the CQI feedback from the eNB, or using channel reciprocity. Some information such as UE C-RNTI and the chosen MCS needs to be signalled to the eNB to help decoding. The following options for signalling can be considered.

- Using newly defined preamble signal: The preamble signal can not only be used for detection but also contain information such as UE C-RNTI, MCS, and the length of UL burst, etc.

- Using newly defined control channel for non-scheduled access: The newly defined control channel can be used to indicate the above listed information.

# Performance Evaluation

In this section, we compare the performance of non-scheduled operation to that of the original scheduled operation. In addition, we provide the link-level presence detection performance based on DMRS.

## Comparison between scheduled and non-scheduled UL access

The simulation assumptions are summarized in Table 1 in Appendix, which comply with 3GPP evaluation methodology [3]. Figure 2 provides the DL/UL UPT and the buffer occupancy.

Figure 2: The performance of non-scheduled UL access

Observations and discussion

- Non-scheduled UL performance is significantly improved compared to the scheduled mode of operation. In low load, the performance improvement is up to four times, while at high load the improvement is up to seven times.

- The LAA with non-scheduled UL access well coexists with Wi-Fi.

Observation 1:

- The LAA UL performance is significantly improved by non-scheduled access.

- LAA with non-scheduled UL access well coexists with Wi-Fi.

Proposal:

- Introduce the non-scheduled (or grant-less) mode for eLAA UL access.

## DMRS based UL transmission presence detection

We present the evaluation results for the DMRS based PUSCH presence detection as follows. The simulation assumption is given in Table 2 in appendix.

Figure 3 illustrates the misdetection probability at the given false-alarm probability. From Figure 3, we can observe that the SNR required to achieve detection probability higher than 99% and 99.9% are -14dB and -12.3dB, respectively, when targeting false-alarm probability is 1%. When the false-alarm probability is 0.1%, the SNR required to achieve detection probability higher than 99% and 99.9% are -13.5dB and -11.4dB, respectively. Thus, we can conclude that the DMRS based detection can provide enough performance.

1)  Pfa = 1%                                                                         2)  Pfa = 0.1%

Figure 3: DMRS-based presence detection for non-scheduled UL access

Observation 2:

- The DMRS based detection can provide sufficiently good performance for non-scheduled PUSCH detection.

# Conclusion

In this contribution, non-scheduled (grant-less) UL access for eLAA was introduced. Below we summarize our observations and proposals:

Observation 1:

- The LAA UL performance is significantly improved by non-scheduled access.

- LAA with non-scheduled UL access well coexists with Wi-Fi.

Proposal: Introduce the non-scheduled (or grant-less) mode for eLAA UL access.

Observation 2: The DMRS based detection can provide sufficiently good performance for non-scheduled PUSCH detection.

# References

[1] R1-160425, “On the UL scheduling starvation issue”, 3GPP TSG RAN WG1 #84, Intel Corporation, Feb., 2016

[2] R1-160424, “On the Remaining details for UL LBT”, 3GPP TSG RAN WG1 #84, Intel Corporation, Feb., 2016

[3] TR 36.889

# Appendix

Table 1: Simulation Assumptions for Comparison between Scheduled and Non-scheduled UL LAA

|  | Comments |  |
| --- | --- | --- |
| Scenario | Indoor, 1 channel, 20 UEs/operator , DL:UL = 50:50 traffic |  |
| LAA assumptions | LAA DL | Based on the Release 13 design |
|  | Non-scheduled LAA UL LBT | Cat 4 LBT with DL LBT, [X,Y] = [31, 1023], ED threshold = -72 dBm |
|  | Scheduled UL LBT | Single interval LBT, ED threshold = -72 dBm |
|  | TXOP | MCOT = 8 ms, UL max burst length is 4 ms. |
|  | Other assumptions | 2x2 for DL; 1x2 for UL |
| WiFi Assumptions | TXOP | 4ms |
|  | GI | Short GI |
|  | Other assumptions | 2x2 CL MIMO for DL; 1x2 OL MIMO for UL |

Table 2: Simulation Assumptions for DMRS Based Detection

|  | Comments |  |
| --- | --- | --- |
| Simulation setup | System bandwidth | 20 MHz |
|  | Number of transmitter antennas | 1 |
|  | Number of receiver antennas | 2 |
|  | Channel model | EPA-5Hz |
|  | Number of PRBs for PUSCH transmission | 100 PRBs |
|  | Number of UEs that can simultaneously transmit | At most 1 UE |
|  | Number of UEs anchored at eNB | 12 |
|  | Targeting false-alarm probability | 1%, 0.1% |
| Key performance metrics | False-alarm probability (Pfa) | The probability that no PUSCH is transmitted in the subframe, but eNB detects the presence of the signal. |
|  | Misdetection probability (Pmd) | The probability that the UE transmits, but eNB detects that the UE of interest does not transmit. |