3GPP TSG RAN WG1 Meeting #84	                            R1-160490

St Julian’s, Malta, 15th - 19th February 2016

Agenda Item:	   7.3.1.1

Source: 	   Fujitsu

Title:            Evaluation results for UL LBT and PUSCH Design

Document for:   Discussion/Decision

# Introduction

In RAN1 #82 meeting, the following working assumptions have been agreed [1].

- For self-carrier scheduling, the following UL LBT candidate procedures should be considered

  - A CCA duration of 25 us before the transmission burst

    - The sensing duration can be less than the CCA duration

  - A category 4 LBT scheme with a defer period of 25 µs including a defer duration of 16 us followed by one CCA slot, and a maximum contention window size of X={3, 4, 5, 6, 7}, respectively

    - FFS: The random backoff counter is generated at the eNB and is signalled to the UE

    - The UL maximum contention window size should be smaller than for DL category 4 LBT

    - Note that X = 7 can be revisited later after DL LBT discussions, if necessary

  - FFS: Transmission without LBT when UL transmission burst follows DL transmission burst with a gap of at most 16 µs between the two bursts

During the e-mail discussion [82-06] after RAN1 #82 meeting, above working assumptions have been updated as follows,

•      For self-carrier scheduling, the following UL LBT candidate procedures should be considered

–     A CCA duration of at least 25 us before the transmission burst

•      The sensing duration in a CCA slot can be less than the CCA slot duration

–     A category 4 LBT scheme with a defer period of 25 µs including a defer duration of 16 us followed by one CCA slot, and a maximum contention window size chosen from X={3, 4, 5, 6, 7}

•      FFS: The random backoff counter is generated at the eNB and is signaled to the UE

•      FFS: When a UL grant is subject to LBT with a new random counter, the UL transmissions scheduled by the UL grant also uses a new random counter (previous counter is discarded) irrespective of prior success/failure in accessing the channel.

•      The UL maximum contention window size should be smaller than for DL category 4 LBT

•      Note that X = 7 can be revisited later after DL LBT discussions, if necessary

–     FFS: Energy detection threshold used for UL LBT

•      LBT may be needed for an UL transmission burst that contains PUSCH

•      LBT may or may not be needed for an UL transmission burst without PUSCH that contains control transmissions (PUCCH)

•      FFS: Whether and under what conditions the following option may be used

–     Transmission without LBT when an UL transmission burst on a carrier follows a DL transmission burst on that respective carrier with a gap of at most 16 µs between the two bursts

For further discussion on design of PUSCH transmission, this contribution provides evaluation results for three different options. In the accompanying contribution [2], we analyze design options for UL LBT and PUSCH and summarize our proposals.

# LBT for a UL transmission burst that contains PUSCH

From the viewpoint of UL LBT, three options, as shown in Fig. 1, 2 and 3, can be considered for transmission of PUSCH.

- Option 1: The UE performs CCA. The same as for TDD operation in licensed bands, the timing of PUSCH transmission is fixed according to the corresponding UL grant. Although in Figure 1, the UE transmit only one subframe per LBT procedure, multi-subframe continuous transmission can also be considered.

- Option 2: Piggybacked PUSCH transmission with CCA. The UE performs CCA. PUSCH is transmitted after a DL burst which may be or may not be the DL burst that contains its UL grant. The timing of PUSCH transmission depends on the end time of the DL burst. Although in Figure 2, the UE transmit only one subframe per LBT procedure, multi-subframe continuous transmission can also be considered.

- Option 3: Piggybacked PUSCH transmission without CCA. The UE does not perform CCA. PUSCH follows a DL burst with a gap of at most 16µs. The timing of PUSCH transmission depends on the end time of the DL burst.

Different from Option 1, in Option 2 and Option 3 the transmission timing of PUSCH can be delayed with respect to the UL grant, depending on the length and transmission timing of the DL burst.

In option 1 and 2, multi-subframe continuous transmission can be realized by adding information on number of subframes into the UL grant.

Fig. 1: (Option 1) PUSCH transmissions with a specified timing relative to the timing of their UL grants

Fig. 2: (Option 2) Piggybacked PUSCH transmission with CCA

Fig. 3: (Option 3) Piggybacked PUSCH transmission without CCA

# Evaluation results

We have evaluated UPT performance based on the indoor scenario for LAA coexistence evaluations which is specified in A.1.1 of TR 36.889 [3]. Operator A and Operator B share the same unlicensed carrier. The DL/UL traffic ratio is 80:20. The maximum length of a burst is 4 ms. The UL grant is transmitted over the unlicensed carrier. LBT parameters for LAA DL are based on Rel-13 specification. In option 1 and 2 for PUSCH transmission, the UE performs a category 4 LBT scheme with a defer period of 25 µs including a defer duration of 16 us followed by one CCA slot, and a maximum contention window size X=3.

## Without multi-subframes continuous transmission

Table 1, Table 2 and Table 3 show the simulation results for low, medium and high traffic load cases, respectively. It can be seen that in both Option 1 and 2, as the traffic load increases, more and more of the additional PUSCHs cannot be transmitted due to CCA busy, and the discarding rates of UL grants increase. Thanks to better transmission opportunities, Option 2 can reduce the discarding rates of UL grants compared to Option 1. Thus, Option 2 has a better UPT performance than Option 1. Option 3 has the best UPT performance by eliminating the discarding rate. Table 4 summarize the gain of Option 2 and 3 compared to Option 1 in the case of medium load. It can be seen that option 2 can increase the UPT of LAA by over 50%, option 3 can increase the UPT of LAA by over 60%. The performance of coexisting Wi-Fi also improves accordingly thanks to less congested conditions on the operating carrier.

Observation 1: Option 1 leads to enormous discarding rate of UL grants due to CCA busy as the traffic load increases. Compared to Option 1, Option 2 can reduce the discarding rates, thus improve the UPT performance of LAA and coexisting Wi-Fi. Option 3 can improve the UPT performance further by eliminating discarding rate of UL grants.

Table 1 Low Load

| Reported parameters |  | Low load (Offered Traffic: DL, 4.0 [Mbps/Cell]; UL, 1.0 [Mbps/cell]) BO range for Wi-Fi Opt.1 in Step 1: 10%~25% |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi only |  | Option 1 |  | Option 2 |  | Option 3 |  |
|  |  | Wi-Fi Oprt.1 in step 1 | Wi-Fi Oprt.2 in step 1 | Wi-Fi Oprt. 1 in step 2 | LAA Oprt.2 in step 2 | Wi-Fi Oprt. 1 in step 2 | LAA Oprt.2 in step 2 | Wi-Fi Oprt. 1 in step 2 | LAA Oprt.2 in step 2 |
| DL: UPT CDF [Mbps] | 5% | 10.649 | 10.627 | 10.951 | 23.361 | 18.651 | 24.566 | 18.425 | 28.538 |
|  | 50% | 46.813 | 44.871 | 54.950 | 79.477 | 59.415 | 88.897 | 59.850 | 87.920 |
|  | 95% | 76.513 | 76.714 | 89.692 | 120.215 | 95.306 | 123.854 | 96.519 | 125.016 |
|  | Mean | 45.813 | 44.011 | 54.673 | 78.607 | 59.738 | 84.181 | 59.280 | 84.262 |
| UL: UPT CDF [Mbps] | 5% | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  | 50% | 31.265 | 28.890 | 28.694 | 20.780 | 30.468 | 23.461 | 33.301 | 25.891 |
|  | 95% | 62.496 | 62.520 | 62.566 | 43.245 | 62.590 | 42.555 | 62.578 | 45.586 |
|  | Mean | 29.593 | 28.637 | 29.151 | 20.142 | 30.233 | 21.900 | 31.372 | 23.415 |
| 𝜌DL |  | 0.998 | 0.999 | 0.996 | 0.997 | 0.995 | 0.996 | 0.995 | 1.000 |
| 𝜌UL |  | 0.999 | 0.999 | 0.998 | 0.992 | 0.998 | 0.992 | 0.998 | 0.994 |
| BO [%] |  | 13.5 | 13.9 | 11.8 | 10.7 | 10.6 | 9.5 | 10.6 | 9.2 |
| Discarding rate of UL grants due to CCA busy |  | N/A | N/A | N/A | 29.7 | N/A | 22.7 | N/A | 0 |

Table 2 Medium load

| Reported parameters |  | Medium load (Offered Traffic: DL, 5.24 [Mbps/Cell]; UL, 1.31 [Mbps/cell]) BO range for Wi-Fi Opt.1 in Step 1: 35%~50% |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi only |  | Option 1 |  | Option 2 |  | Option 3 |  |
|  |  | Wi-Fi Oprt.1 in step 1 | Wi-Fi Oprt.2 in step 1 | Wi-Fi Oprt. 1 in step 2 | LAA Oprt.2 in step 2 | Wi-Fi Oprt. 1 in step 2 | LAA Oprt.2 in step 2 | Wi-Fi Oprt. 1 in step 2 | LAA Oprt.2 in step 2 |
| DL: UPT CDF [Mbps] | 5% | 0.473 | 0.437 | 0.798 | 2.749 | 3.828 | 8.887 | 6.309 | 17.386 |
|  | 50% | 14.314 | 15.924 | 14.642 | 28.511 | 35.569 | 58.115 | 37.638 | 63.120 |
|  | 95% | 48.219 | 47.027 | 59.161 | 85.361 | 64.790 | 99.389 | 71.291 | 100.287 |
|  | Mean | 18.135 | 18.488 | 20.796 | 36.444 | 35.432 | 57.304 | 37.765 | 60.922 |
| UL: UPT CDF [Mbps] | 5% | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  | 50% | 7.482 | 6.501 | 7.360 | 3.268 | 17.503 | 11.284 | 19.432 | 13.356 |
|  | 95% | 52.680 | 51.568 | 55.113 | 35.557 | 62.255 | 39.803 | 62.402 | 43.245 |
|  | Mean | 14.412 | 14.100 | 14.201 | 9.202 | 21.576 | 13.974 | 22.875 | 15.484 |
| 𝜌DL |  | 0.890 | 0.883 | 0.940 | 0.986 | 0.989 | 0.997 | 0.991 | 0.998 |
| 𝜌UL |  | 0.960 | 0.953 | 0.973 | 0.888 | 0.992 | 0.961 | 0.989 | 0.981 |
| BO [%] |  | 42.8 | 42.7 | 41.8 | 38.6 | 23.9 | 21.9 | 21.8 | 19.2 |
| Discarding rate of UL grants due to CCA busy |  | N/A | N/A | N/A | 61.5 | N/A | 42.4 | N/A | 0 |

Table 3 High load

| Reported parameters |  | High load, Offered Traffic: DL, 5.6 [Mbps/Cell]; UL, 1.4 [Mbps/cell] BO range for Wi-Fi Opt.1 in Step 1: above 55% |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi only |  | Option 1 |  | Option 2 |  | Option 3 |  |
|  |  | Wi-Fi Oprt.1 in step 1 | Wi-Fi Oprt.2 in step 1 | Wi-Fi Oprt. 1 in step 2 | LAA Oprt.2 in step 2 | Wi-Fi Oprt. 1 in step 2 | LAA Oprt.2 in step 2 | Wi-Fi Oprt. 1 in step 2 | LAA Oprt.2 in step 2 |
| DL: UPT CDF [Mbps] | 5% | 0.308 | 0.473 | 0.459 | 1.755 | 0.660 | 2.631 | 0.973 | 3.049 |
|  | 50% | 2.856 | 2.863 | 7.720 | 18.423 | 22.428 | 34.633 | 34.069 | 54.393 |
|  | 95% | 35.189 | 37.484 | 54.502 | 85.114 | 58.691 | 89.896 | 63.665 | 94.127 |
|  | Mean | 10.279 | 10.670 | 15.991 | 28.926 | 24.462 | 39.064 | 33.091 | 52.666 |
| UL: UPT CDF [Mbps] | 5% | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  | 50% | 3.162 | 2.766 | 5.124 | 1.581 | 10.681 | 5.380 | 15.750 | 12.103 |
|  | 95% | 42.922 | 39.263 | 51.273 | 33.013 | 57.508 | 36.531 | 61.192 | 43.718 |
|  | Mean | 9.683 | 8.512 | 11.706 | 7.125 | 15.349 | 10.139 | 19.631 | 14.630 |
| 𝜌DL |  | 0.786 | 0.803 | 0.892 | 0.986 | 0.933 | 0.986 | 0.962 | 0.995 |
| 𝜌UL |  | 0.936 | 0.934 | 0.973 | 0.798 | 0.980 | 0.853 | 0.986 | 0.949 |
| BO [%] |  | 62.5 | 62.9 | 54.0 | 51.2 | 41.6 | 40.5 | 28.3 | 26.2 |
| Discarding rate of UL grants due to CCA busy |  | N/A | N/A | N/A | 66.4 | N/A | 59.3 | N/A | 0 |

Table 4: The gain in mean UPT compared to Option 1 in case of medium load

|  |  | Option 1 | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Operator 1 (Wi-Fi) | DL | 1.0 | 1.71 | 1.82 |
|  | UL | 1.0 | 1.52 | 1.61 |
| Operator 2 (LAA) | DL | 1.0 | 1.58 | 1.68 |
|  | UL | 1.0 | 1.52 | 1.69 |

## With multi-subframe continuous transmission

Multi-subframe continuous transmission can reduce the overhead of the LBT procedure. In addition, the number of UL grants could be also reduced if multi-subframe continuous transmission can be indicated by a single UL grant. Table 5 shows the result of Option 1 and Option 2 with multi-subframe continuous transmission in the high load case. It can be seen that Option 3 still has better performance than Option 1 and Option 2. Table 6 summarizes the ratio between the mean UPT with and without multi-subframe continuous transmission. It can be seen that Option 1 does not really benefit from multi-subframe continuous transmission whereas Option 2 does. The reason is again related to transmission opportunities of PUSCH. In Option 1 without multi-subframe continuous transmission, stochastically the first single-subframe PUSCH burst has fewer transmission opportunities than the following single-subframe PUSCH bursts due to the probability of interruption by neighbouring networks. On the other hand, in Option 2 the first single-subframe PUSCH burst has a similar transmission opportunity to the following single-subframe PUSCH bursts.

Observation 2: Multi-subframe continuous transmission can reduce the overhead of the LBT procedure in Option 2. In addition, the number of UL grants could be also reduced if multi-subframe continuous transmission can be indicated by a single UL grant. In terms of UPT performance, Option 2 can benefit from multi-subframe continuous transmission whereas Option 1 does not.

Table 5 Multi-subframe continuous transmission

| Reported parameters |  | High load, Offered Traffic: DL, 5.6 [Mbps/Cell]; UL, 1.4 [Mbps/cell] BO range for Wi-Fi Opt.1 in Step 1: above 55% |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Oprt.1 in step 1 | Wi-Fi Oprt.2 in step 1 | Wi-Fi Oprt. 1 in step 2 | LAA Oprt.2 (Method 1) in step 2 | Wi-Fi Oprt. 1 in step 2 | LAA Oprt.2 (Method 2) in step 2 | Wi-Fi Oprt. 1 in step 2 | LAA Oprt.2 (Method 3) in step 2 |
| DL: UPT CDF [Mbps] | 5% | 0.308 | 0.473 | 0.559 | 2.150 | 0.893 | 2.672 | 0.973 | 3.049 |
|  | 50% | 2.856 | 2.863 | 10.910 | 20.284 | 28.627 | 44.006 | 34.069 | 54.393 |
|  | 95% | 35.189 | 37.484 | 55.709 | 82.993 | 62.172 | 94.572 | 63.665 | 94.127 |
|  | Mean | 10.279 | 10.670 | 16.479 | 28.464 | 28.120 | 44.325 | 33.091 | 52.666 |
| UL: UPT CDF [Mbps] | 5% | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
|  | 50% | 3.162 | 2.766 | 5.503 | 1.621 | 13.155 | 8.297 | 15.750 | 12.103 |
|  | 95% | 42.922 | 39.263 | 51.190 | 33.987 | 57.532 | 44.236 | 61.192 | 43.718 |
|  | Mean | 9.683 | 8.512 | 11.507 | 7.156 | 17.189 | 12.263 | 19.631 | 14.630 |
| 𝜌DL |  | 0.786 | 0.803 | 0.913 | 0.986 | 0.952 | 0.990 | 0.962 | 0.995 |
| 𝜌UL |  | 0.936 | 0.934 | 0.975 | 0.798 | 0.983 | 0.888 | 0.986 | 0.949 |
| BO [%] |  | 62.5 | 62.9 | 51.7 | 51.2 | 36.2 | 35.5 | 28.3 | 26.2 |
| Discarding rate of UL grants due to CCA busy |  | N/A | N/A | N/A | 69.8 | N/A | 57.5 | N/A | 0 |

Table 6: The ratio between mean UPT with and without multi-subframe continuous transmission

|  |  | Option 1 | Option 2 |
| --- | --- | --- | --- |
| Operator A (Wi-Fi) | DL | 1.03 | 1.15 |
|  | UL | 0.98 | 1.12 |
| Operator B (LAA) | DL | 0.98 | 1.13 |
|  | UL | 1.02 | 1.05 |

# Conclusions

This contribution provides evaluation results for three different options,

- Option 1: The UE performs CCA. The same as for TDD operation in licensed bands, the timing of PUSCH transmission is fixed according to the corresponding UL grant. Although in Figure 1, the UE transmit only one subframe per LBT procedure, multi-subframe continuous transmission can also be considered.

- Option 2: Piggybacked PUSCH transmission with CCA. The UE performs CCA. PUSCH is transmitted after a DL burst which may be or may not be the DL burst that contains its UL grant. The timing of PUSCH transmission depends on the end time of the DL burst. Although in Figure 2, the UE transmit only one subframe per LBT procedure, multi-subframe continuous transmission can also be considered.

- Option 3: Piggybacked PUSCH transmission without CCA. The UE does not perform CCA. PUSCH follows a DL burst with a gap of at most 16µs. The timing of PUSCH transmission depends on the end time of the DL burst.

The following observations have been made based on the evaluation results,

Observation 1: Option 1 leads to enormous discarding rate of UL grants due to CCA busy as the traffic load increases. Option 2 can reduce the discarding rates, thus improve the UPT performance of LAA and coexisting Wi-Fi. Option 3 can improve the UPT performance further by eliminating discarding rate of UL grants.

Observation 2: Multi-subframe continuous transmission can reduce the overhead of the LBT procedure in Option 2. In addition, the number of UL grants could be also reduced if multi-subframe continuous transmission can be indicated by a single UL grant. In terms of UPT performance, Option 2 can benefit from multi-subframe continuous transmission whereas Option 1 does not.

# References

- “RAN1 Chairman’s Notes”, RAN1 meeting #82, August 2015

- R1- 160489, “UL LBT and PUSCH Design for LAA”, Fujitsu, RAN1 meeting #84, February 2016

- TR36.889

# Annex: Simulation parameters

Table 7 Additional LAA system evaluation assumptions

| Parameters | Value |
| --- | --- |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized. |
| Scheduling | Proportional fair |
| Link adaptation | Realistic |
| CCA-ED | -72dBm (per Rel-13 LAA specification) |
| Maximum length of DL burst (including reservation signals) | 4ms |
| MCS | QPSK/16QAM/64QAM |

Table 8 Additional Wi-Fi system evaluation assumptions

| Parameters |  | Value |
| --- | --- | --- |
| Antenna configuration |  | 2Tx2Rx in DL, Cross-polarized,open loop |
| Frame aggregation |  | A-MPDU |
| Max PPDU duration |  | 4 ms (Asynchronous to LTE timing) |
| MAC | Coordination | DCF |
|  | Detection | Energy detection & preamble detection |
|  | RTS/CTS | No |
| CCA-ED |  | -62dBm |
| MCS |  | 802.11ac MCS table without 256 QAM |