3GPP TSG RAN WG1 Meeting #84                                                                  R1-160693

St Julian’s, Malta, 15th – 19th February 2016

Source:	ZTE, Keysight Technologies

Title:	Channel statistics per deployment scenario

Agenda Item:	7.3.5.1

Document for: 	Discussion and Decision

# Introduction

In RAN#69, the SID for above 6-GHz channel modelling was approved in [1]. Subsequently, the email discussions and the contributions to RAN reflector were summarized in [2]. As for the deployment scenarios, the Indoor and UMi/UMa (with O2O and O2I) attract much attention from most of companies; meanwhile, some other scenarios such as relay backhaul, V2V and D2D are also mentioned in [2].

As given in the RAN email discussion and [3], our views on deployment scenarios as well as their corresponding prioritizations are listed in Table 1. Furthermore, we showed a clear observation in [3] that the accurate channel modelling for different deployments even in the same scenario classification such as indoor or UMi may still lead to different propagation statistics or procedures.

| Scenarios & Freq |  | 6~20 GHz | 20~30 GHz | 30~60 GHz | >60 GHz |
| --- | --- | --- | --- | --- | --- |
| Indoor | Conference | High | High | High | High |
|  | Office | High | High | High | High |
|  | Shopping Mall | High | High | High | High |
|  | Stadium | High | High | High | High |
|  | Gym | High | High | High | High |
| UMi (O2O) | Open square | High | High | High | High |
|  | Street canyon | High | High | High | High |
|  | Campus | High | High | High | High |
| UMa (O2O) |  | High | Low | Low | Low |
| UMi/UMa (O2I) |  | High | Low | No | No |

Table 1 Scenarios considered for channel modelling

Notes: “High” and “Low” indicate high and low priorities, respectively. “No” indicates that the corresponding combination of the deployment scenario and frequency band is not suggested.

This contribution further explores this observation in different indoor scenarios, i.e., shopping mall and conference room, and with different antenna heights in the same indoor conference room deployment, by applying the ray-tracing (RT) method in the evaluation. The field measurements are provided to verify the accuracy of ray-tracing modelling used in this study.

# The studied scenarios

Based on the earlier discussion in RAN plenary, the indoor scenarios seem to be the preferable application cases for the high-frequency communications. Therefore the corresponding deployment scenarios should be covered in the channel modelling. Figure 1 and Figure 2 illustrate the geometric layouts of two typical deployments, namely conference room and shopping mall, respectively. The measurements are conducted in conference room at both 23.5GHz and 45GHz through the VNA-based sounding system. Table 2 gives the Tx antenna locations and the potential Rx antenna heights (parameter Z). More details about the measurement can be found in [5]~[7].

(a)

(b)

Figure 1 Indoor conference room: (a) Panorama (b) Digitalized map

Figure 2 Digitalized map of the indoor shopping mall

| Scenarios | Tx Locations [m] |  | Rx Area [m] | Frequency [GHz] |
| --- | --- | --- | --- | --- |
| Conference Room | Tx1 | (10.32,1.49,1.95) | No.1: Z = 0.5 No.2: Z = 1.0 No.3: Z = 1.5 UE horizontal separation is 0.1m | 23.5, 45 |
|  | Tx2 | (7.1,1.1,1.95) |  |  |
|  | Tx3 | (6,4.5,1.95) |  |  |
|  | Tx4 | (0.8,9,1.95) |  |  |
|  | Tx5 | (2.2,9,1.95) |  |  |
| Shopping Mall | Tx 1 | (124,22,11.8) | No.1: Z = 1.5 No.2: Z = 7.5 UE horizontal separation is 0.1m |  |

Table 2 Geometric configurations for measurement and RT simulation

# Verification of Ray-Tracing modeling

The full-3D RT platform, which is used to conduct extensive simulations in section 4, is firstly verified by the comparison of both power delay profile (PDP) and other ray parameters between the field measurements at 23.5GHz and simulation results based on ray tracing. The comparisons are performed over total of 33 measurement locations. Below is given the comparison for a specific measurement location (labelled as Rx 13) as an example.

- Figure 3 shows the ray-level comparison. The blue arrow lines point to the direction of the ray obtained from the measurement, and the red solid line illustrates the trace for the same path based on RT simulator. It can be found that these dominant paths generated by RT simulations well match with those from measurements. The more comparisons on the relevant rays are summarized in Table 3, where the absolute modelling error on each parameter is shown much less than either the measured parameter itself or the corresponding measurement resolution.

- Figure 4 shows the PDP comparison. It is shown that the (three) dominant components with significant power peaks are all captured by ray-tracing. Moreover, the specular peaks are extended to cluster-alike dispersive components and more realistic PDPs are obtained by adopting scattering in RT method.

The comparisons over all measurement locations draw the similar conclusion, which sufficiently verifies the RT method in use.

(a)                                                                             (b)

Figure 3 Rays at measurement location Rx 13: (a) LOS, (b) Double-bounce reflection

Table 3 Comparisons of ray parameters

| Rx.13 |  | Measurement | RT model | Difference (absolute value) | Measurement resolution |
| --- | --- | --- | --- | --- | --- |
| LOS | Delay[ns] | 13.64 | 13.64 | 0.00 | 1.5152 |
|  | Power[dB] | -42.30 | -38.19 | 4.11 | / |
|  | AoD[°] | 45.00 | 44.75 | 0.25 | 15 |
|  | EoD[°] | -15.00 | -16.21 | 1.21 | 10 |
|  | AoA[°] | 135.00 | 134.75 | 0.25 | 15 |
|  | EoA[°] | 10.00 | 16.21 | 6.21 | 10 |
| Double-bounce reflection | Delay[ns] | 66.67 | 65.01 | 1.65 | 1.5152 |
|  | Power[dB] | -56.25 | -52.81 | 3.45 | / |
|  | AoD[°] | -15.00 | -12.00 | 3.00 | 15 |
|  | EoD[°] | -5.00 | -2.89 | 2.11 | 10 |
|  | AoA[°] | -105.00 | -102.85 | 2.15 | 15 |
|  | EoA[°] | 0.00 | 2.89 | 2.89 | 10 |

Figure 4 PDP comparison at 23.5GHz

Observation 1: The ray-tracing method can be used to reliably reproduce the channel realizations and emulate measurement samples for the studies of high-frequency channel modeling.

# Channel statistics in indoor scenarios

In this section, we use the RT platform, whose accuracy was verified in last section, to simulate the two different indoor deployments, i.e., shopping mall and conference room. The 3GPP-3D channel model [4] alike parameters are collected as in Table 4. The statistics are collected in such a way that, the transmitter is at a horizontal central location (No. 3 Tx location in conference room deployment and No. 1 Tx location in shopping mall deployment) and the receivers are distributed over the No.2 Rx area in both deployments, where all Tx/Rx locations are given in Table 2. In the statistical study, the pathloss model is fitted with equation:

where and  respectively relate to the intercept and slope in Table 4.

The clear differences are observed between these two indoor deployments over quite a few statistics, such as pathloss, spreads of delay and elevation angles, shadowing fading and K-factors. Meanwhile, the numerous cross-correlations and correlation distances also differ. These differences can conclude that a single set of stochastic parameters may not be able to cover all the deployments in one scenario classification.

Table 4 Channel statistics for conference and shopping mall

| Scenarios |  | Conference（Tx3） 23.5 GHz |  | Shopping Mall 23.5 GHz |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| Path Loss | Slope | 1.30 | -0.11 | 1.43 | 2.00 |
|  | Intercept | 63.97 | 89.04 | 67.30 | 67.92 |
| Delay spread (DS) | DS | -8.05 | -8.05 | -7.66 | -7.64 |
| log10([s]) | DS | 0.11 | 0.26 | 0.16 | 0.29 |
| AoD spread (σASD) log10([]) | ASD | 1.63 | 1.57 | 1.47 | 1.52 |
|  | ASD | 0.13 | 0.31 | 0.20 | 0.27 |
| AoA spread (σASA) log10([]) | ASA | 1.79 | 1.60 | 1.67 | 1.68 |
|  | ASA | 0.14 | 0.29 | 0.22 | 0.26 |
| ZoD spread (σZSD) log10([]) | ZSA | 1.66 | 1.21 | 1.34 | 1.39 |
|  | ZSA | 0.07 | 0.47 | 0.21 | 0.32 |
| ZoA spread (σZSA) log10([]) | ZSA | 1.59 | 1.21 | 1.27 | 1.39 |
|  | ZSA | 0.08 | 0.45 | 0.28 | 0.27 |
| Shadow fading (SF) [dB] | SF | 0.65 | 5.58 | 0.68 | 7.41 |
| K-factor (K) [dB] | K | 1.64 | N/A | 3.84 | N/A |
|  | K | 3.66 | N/A | 5.38 | N/A |
| Cross-Correlations | ASD vs DS | 0.67 | 0.45 | 0.68 | 0.52 |
|  | ASA vs DS | 0.20 | 0.50 | 0.46 | 0.42 |
|  | ASA vs SF | -0.37 | 0.43 | -0.26 | -0.41 |
|  | ASD vs SF | 0.27 | 0.39 | 0.04 | -0.08 |
|  | DS vs SF | 0.12 | 0.49 | 0.02 | 0.12 |
|  | ASD vs ASA | 0.25 | 0.47 | 0.50 | 0.56 |
|  | ASD vs K | -0.43 | N/A | -0.69 | N/A |
|  | ASA vs K | -0.80 | N/A | -0.67 | N/A |
|  | DS vs K | -0.33 | N/A | -0.57 | N/A |
|  | SF vs K | 0.31 | N/A | 0.30 | N/A |
| Cross-Correlations | ZSD vs SF | 0.52 | 0.57 | -0.28 | -0.56 |
|  | ZSA vs SF | 0.40 | 0.60 | -0.50 | -0.47 |
|  | ZSD vs K | -0.24 | N/A | -0.89 | N/A |
|  | ZSA vs K | -0.49 | N/A | -0.85 | N/A |
|  | ZSD vs DS | 0.28 | 0.62 | 0.33 | 0.36 |
|  | ZSA vs DS | 0.37 | 0.61 | 0.33 | 0.46 |
|  | ZSD vs ASD | 0.43 | 0.72 | 0.61 | 0.45 |
|  | ZSA vs ASD | 0.57 | 0.74 | 0.53 | 0.49 |
|  | ZSD vs ASA | 0.17 | 0.66 | 0.59 | 0.75 |
|  | ZSA vs ASA | 0.36 | 0.65 | 0.52 | 0.73 |
|  | ZSD vs ZSA | 0.95 | 0.99 | 0.92 | 0.93 |
| Delay distribution |  | Exp | Exp | Exp | Exp |
| AoD and AoA distribution |  | Wrapped Gaussian |  | Wrapped Gaussian |  |
| ZoD and ZoA distribution |  | Laplacian |  | Laplacian |  |
| Delay scaling parameter r |  | 1.22 | 1.63 | 1.55 | 1.34 |
| XPR [dB] |  | / | / | / | / |
|  |  | / | / | / | / |
| Number of clusters |  | 9.00 | 8.00 | 9.00 | 9.00 |
| Number of rays per cluster |  | 12.00 | 14.00 | 15.00 | 58.00 |
| Cluster ASD |  | 14.15 | 10.27 | 7.64 | 10.08 |
| Cluster ASA |  | 19.73 | 12.36 | 13.45 | 14.06 |
| Cluster ZSD |  | 6.77 | 4.62 | 4.81 | 6.24 |
| Cluster ZSA |  | 6.53 | 4.48 | 6.15 | 7.55 |
| Per cluster shadowing std  [dB] |  | 4.22 | 4.48 | 4.18 | 5.10 |
| Correlation distance in the horizontal plane [m] | DS | 3.29 | 1.67 | 14.86 | 31.07 |
|  | ASD | 2.02 | 8.45 | 10.62 | 42.25 |
|  | ASA | 4.60 | 4.65 | 8.94 | 29.23 |
|  | SF | 3.35 | 5.89 | 23.28 | 35.53 |
|  |  | 6.02 | N/A | 45.84 | N/A |
|  | ZSA | 1.77 | 5.85 | 43.19 | 22.05 |
|  | ZSD | 1.54 | 5.96 | 43.66 | 25.37 |

As another interesting observation from the simulations, the different Tx locations in the same deployment also lead to different propagation statistics. This observation is given in Table 5 that contains the channel statistics for the five different Tx locations in the studied conference room deployments. From this table, the clear differences among different Tx locations can be found upon parameters such as pathloss, shadow fading, K-factor, cluster-related parameters and the correlation parameters. This observation tells that, even in the same deployment scenario, a single set of stochastic parameters may not be able to accurately model the channel statistics from every Tx location.

Table 5 Channel statistics on different Tx locations in conference room

| Scenarios |  | Case1（Tx1） |  | Case3（Tx3） |  | Case4（Tx4） |  | Case5（Tx5） |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| Path Loss | Slope | 1.28 | 0.22 | 1.30 | -0.11 | 1.30 | 0.89 | 1.55 | 0.81 |
|  | Intercept | 62.97 | 88.27 | 63.97 | 89.04 | 63.32 | 75.16 | 62.59 | 74.78 |
| Delay spread (DS) | DS | -8.18 | -8.16 | -8.05 | -8.05 | -8.01 | -7.92 | -7.94 | -8.11 |
| log10([s]) | DS | 0.09 | 0.24 | 0.11 | 0.26 | 0.17 | 0.27 | 0.18 | 0.20 |
| AoD spread (σASD) log10([]) | ASD | 1.82 | 1.71 | 1.63 | 1.57 | 1.69 | 1.56 | 1.62 | 1.63 |
|  | ASD | 0.12 | 0.21 | 0.13 | 0.31 | 0.08 | 0.29 | 0.09 | 0.16 |
| AoA spread (σASA) log10([]) | ASA | 1.80 | 1.58 | 1.79 | 1.60 | 1.81 | 1.57 | 1.82 | 1.72 |
|  | ASA | 0.10 | 0.35 | 0.14 | 0.29 | 0.13 | 0.42 | 0.11 | 0.22 |
| ZoD spread (σZSD) log10([]) | ZSA | 1.62 | 1.08 | 1.66 | 1.21 | 1.60 | 1.60 | 1.61 | 1.33 |
|  | ZSA | 0.07 | 0.59 | 0.07 | 0.47 | 0.05 | 0.28 | 0.06 | 0.27 |
| ZoA spread (σZSA) log10([]) | ZSA | 1.58 | 1.02 | 1.59 | 1.21 | 1.56 | 1.60 | 1.55 | 1.29 |
|  | ZSA | 0.09 | 0.63 | 0.08 | 0.45 | 0.06 | 0.27 | 0.09 | 0.27 |
| Shadow fading (SF) [dB] | SF | 0.80 | 7.01 | 0.65 | 5.58 | 0.70 | 9.16 | 0.80 | 4.63 |
| K-factor (K) [dB] | K | -2.95 | N/A | 1.64 | N/A | -2.90 | N/A | 0.45 | N/A |
|  | K | 3.04 | N/A | 3.66 | N/A | 2.68 | N/A | 2.64 | N/A |
| Cross-Correlations | ASD vs DS | 0.15 | 0.01 | 0.67 | 0.45 | 0.08 | -0.08 | 0.66 | 0.46 |
|  | ASA vs DS | -0.27 | 0.53 | 0.20 | 0.50 | 0.00 | -0.18 | 0.00 | 0.25 |
|  | ASA vs SF | -0.01 | 0.68 | -0.37 | 0.43 | -0.29 | -0.29 | -0.15 | -0.33 |
|  | ASD vs SF | -0.28 | -0.11 | 0.27 | 0.39 | 0.13 | -0.30 | -0.22 | -0.18 |
|  | DS vs SF | 0.14 | 0.52 | 0.12 | 0.49 | 0.05 | -0.15 | -0.27 | -0.10 |
|  | ASD vs ASA | 0.23 | 0.10 | 0.25 | 0.47 | 0.06 | 0.57 | 0.28 | 0.11 |
|  | ASD vs  | -0.63 | N/A | -0.43 | N/A | -0.11 | N/A | -0.40 | N/A |
|  | ASA vs  | -0.63 | N/A | -0.80 | N/A | -0.64 | N/A | -0.68 | N/A |
|  | DS vs  | 0.25 | N/A | -0.33 | N/A | 0.26 | N/A | -0.22 | N/A |
|  | SF vs  | 0.38 | N/A | 0.31 | N/A | 0.35 | N/A | 0.54 | N/A |
| Cross-Correlations | ZSD vs SF | 0.48 | 0.75 | 0.52 | 0.57 | 0.54 | -0.30 | 0.28 | 0.04 |
|  | ZSA vs SF | 0.33 | 0.76 | 0.40 | 0.60 | 0.45 | -0.10 | 0.21 | 0.09 |
|  | ZSD vs K | -0.15 | N/A | -0.24 | N/A | 0.33 | N/A | -0.28 | N/A |
|  | ZSA vs K | -0.51 | N/A | -0.49 | N/A | -0.18 | N/A | -0.54 | N/A |
|  | ZSD vs DS | 0.18 | 0.59 | 0.28 | 0.62 | 0.41 | 0.70 | 0.46 | 0.39 |
|  | ZSA vs DS | 0.03 | 0.60 | 0.37 | 0.61 | 0.24 | 0.72 | 0.29 | 0.35 |
|  | ZSD vs ASD | 0.04 | 0.17 | 0.43 | 0.72 | 0.04 | 0.04 | 0.51 | 0.03 |
|  | ZSA vs ASD | 0.27 | 0.17 | 0.57 | 0.74 | 0.16 | -0.06 | 0.49 | -0.02 |
|  | ZSD vs ASA | 0.45 | 0.84 | 0.17 | 0.66 | 0.03 | -0.01 | 0.38 | 0.08 |
|  | ZSA vs ASA | 0.63 | 0.84 | 0.36 | 0.65 | 0.36 | -0.08 | 0.64 | 0.08 |
|  | ZSD vs ZSA | 0.91 | 1.00 | 0.95 | 0.99 | 0.84 | 0.96 | 0.90 | 0.99 |
| Delay distribution |  | Exp | Exp | Exp | Exp | Exp | Exp | Exp | Exp |
| AoD and AoA distribution |  | Wrapped Gaussian |  | Wrapped Gaussian |  | Wrapped Gaussian |  | Wrapped Gaussian |  |
| ZoD and ZoA distribution |  | Laplacian |  | Laplacian |  | Laplacian |  | Laplacian |  |
| Delay scaling parameter r |  | 1.31 | 1.75 | 1.22 | 1.63 | 1.11 | 1.11 | 1.31 | 1.83 |
| XPR [dB] |  | / | / | / | / | / | / | / | / |
|  |  | / | / | / | / | / | / | / | / |
| Number of clusters |  | 10.00 | 6.00 | 9.00 | 8.00 | 8.00 | 7.00 | 7.00 | 6.00 |
| Number of rays per cluster |  | 20.00 | 35.00 | 12.00 | 14.00 | 20.00 | 20.00 | 15.00 | 11.00 |
| Cluster ASD |  | 16.99 | 19.11 | 14.15 | 10.27 | 14.10 | 12.39 | 14.50 | 19.16 |
| Cluster ASA |  | 13.74 | 13.51 | 19.73 | 12.36 | 14.76 | 13.65 | 18.33 | 16.97 |
| Cluster ZSD |  | 7.26 | 6.15 | 6.77 | 4.62 | 6.83 | 5.25 | 6.80 | 6.80 |
| Cluster ZSA |  | 6.47 | 4.78 | 6.53 | 4.48 | 6.49 | 4.82 | 7.13 | 4.36 |
| Per cluster shadowing std  [dB] |  | 4.27 | 5.32 | 4.22 | 4.48 | 3.97 | 4.67 | 3.88 | 4.63 |
| Correlation distance in the horizontal plane [m] | DS | 3.48 | 1.37 | 3.29 | 1.67 | 12.39 | 5.48 | 10.36 | 8.38 |
|  | ASD | 6.34 | 5.46 | 2.02 | 8.45 | 6.78 | 1.35 | 1.58 | 1.81 |
|  | ASA | 5.29 | 1.76 | 4.60 | 4.65 | 6.50 | 1.41 | 7.52 | 8.78 |
|  | SF | 4.42 | 1.96 | 3.35 | 5.89 | 4.81 | 2.21 | 5.79 | 2.83 |
|  |  | 6.71 | N/A | 6.02 | N/A | 12.31 | N/A | 3.55 | N/A |
|  | ZSA | 3.34 | 1.75 | 1.77 | 5.85 | 6.00 | 5.02 | 3.24 | 6.86 |
|  | ZSD | 1.59 | 1.73 | 1.54 | 5.96 | 8.00 | 5.22 | 1.25 | 6.65 |

Observation 2: A single set of stochastic parameters may not be able to cover different deployments in one scenario classification, or to accurately model the channel statistics from every Tx location in the same deployment.

# Conclusion

This contribution provides following observations:

Observation 1: The ray-tracing method can be used to reliably reproduce the channel realizations and emulate measurement samples for the studies of high-frequency channel modeling.

Observation 2: A single set of stochastic parameters may not be able to cover different deployments in one single scenario classification, or to accurately model the channel statistics from every Tx location in the same deployment.

# References

- RP-151606, New SID Proposal: Study on channel model for frequency spectrum above 6 GHz

- RP-151847, Report of RAN email discussion about >6GHz channel modelling, Samsung

- RP-151724, Considerations on high-frequency channel modeling, ZTE

- TR36.873, Study on 3D channel model for LTE

- Zhu, Jin, Haiming Wang, and Wei Hong. "Large-Scale Fading Characteristics of Indoor Channel at 45-GHz Band." Antennas and Wireless Propagation Letters, IEEE 14 (2015): 735-738.

- Dou, J, et al. "45GHz propagation channel modeling for an indoor conference scenario." Personal, Indoor, and Mobile Radio Communications (PIMRC), 2015 IEEE 26th Annual International Symposium on. IEEE, 2015.

- Zhang N, Dou J W, Tian L, et al. Dynamic Channel Modeling for an Indoor Scenario at 23.5 GHz[J]. Access, IEEE. DOI: 10.1109/ ACCESS.2015.2510363

- Degli-Esposti, Vittorio, et al. "Ray-Tracing-Based mm-Wave Beamforming Assessment." Access, IEEE 2 (2014): 1314-1325.

- Fuschini, Franco, et al. "Ray tracing propagation modeling for future small‐cell and indoor applications: a review of current techniques." Radio Science (2015).

- Fernandes, J. J. G., Peter FM Smulders, and José C. Neves. "Simulation results on mm-wave indoor radio channel modelling and comparison with measurements." Communications and Vehicular Technology in the Benelux, 1994., IEEE Second Symposium on. IEEE, 1994