3GPP TSG RAN WG 1 Meeting #84                                                                  R1-160712

St Julian’s, Malta, Feb. 15 - 19, 2016

Title:	Scenarios for >6GHz Channel Modeling

Source:	NTT DOCOMO

Agenda Item:	7.3.5.1

Document for:	Discussion and decision

- Introduction

At the RAN#69, a SI for channel modelling of frequency spectrum above 6 GHz was approved [1]. Subsequently, an e-mail discussion was conducted focusing on analysis of channel modelling activities outside of 3GPP, scenarios of interest, frequency bands, modelling methodologies, etc. [2, 3]. In this contribution, we present our views on scenarios for >6GHz channel modeling.

- Target Scenarios for >6GHz Channel Modeling

## 2.1 Urban Micro (UMi)

This scenario represents an urban micro-cellular environment with high user density focusing on pedestrian and slow vehicular users. The BS is placed below the rooftop level of the surrounding buildings. Examples include placing the BS at a bus stop, lamppost, building wall, or the rooftop of a building. The average height of the surrounding buildings is assumed to be approximately 18 m (corresponding to the height of a typical 6-story building). Figs. 1(a) and 1(b) show example pictures of UMi scenario, i.e., street canyon and open square, respectively.

Considering large pathloss property for mmWave, UMi scenario with relatively small cell radius, becomes more attractive for 5G network. In this sense, UMi is one of the most important scenarios for >6GHz channel modeling.

Figure 1 (a): UMi scenario - Street canyon

Figure 1 (b): UMi scenario - Open square

## 2.2 Indoor

Indoor is a scenario with most interest for >6GHz channel modeling with the same reason as UMi scenario. Possible examples include office and shopping mall environment as shown in Figs. 2 (a) and 2(b). Office scenario can be comprised of a mixture of open and closed areas (e.g., open cubicle areas, walled offices, conference rooms, open areas, elevator shafts, stairwells, etc.). Partition walls are placed in the open-cubicle area with height of approx. 110 cm. The partition walls can be composed of a variety of materials like sheetrock, poured concrete, glass, cinder block, etc. The BS is mounted at a height of 2-3 m either on the ceilings or walls. On the other hand, shopping mall scenario represents a large multiple-story building of, e.g., 3-5 stories, with an open ceiling in the middle. Shops are arranged along the outer wall of the building.

Figure 2(a): Indoor scenario - Office

Figure 2(b): Indoor scenario - Shopping mall

## 2.3 Urban Macro (UMa)

This scenario represents an urban macro-cellular environment targeted towards wider area coverage with higher user density focusing on pedestrian and vehicular users (slow and medium speed vehicular users). The BS is placed at or above rooftop level of the surrounding buildings, which are of mostly uniform height. The typical height of surrounding buildings can be approximately 18-28 m (6-8 story buildings). Example image for UMa scenario is shown in Fig. 3.

Figure 3: UMa scenario

Proposal 1: Target deployment scenarios include UMi, UMa and indoor.

- UMi and indoor scenarios are prioritized, if necessary.

- Detailed Scenario Description

In order to align understanding of each scenario and avoid modelling based on diverged measurement results, it is important to have detailed description for the scenarios. It is also important in terms of the calibration and future evaluation. Table 1 lists some important parameters for UMi and UMa scenarios. For these scenarios, we can follow 3D-UMi and 3D-UMa scenarios in 3GPP 3D channel model.

Table 1: Detailed parameters for UMi and UMa scenarios

|  |  | UMi | UMa |
| --- | --- | --- | --- |
| Carrier frequency |  | 6-100 GHz |  |
| BS antenna height (hBS) |  | 10 m | 25 m |
| MS location | Outdoor/Indoor | Outdoor and indoor | Outdoor and indoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | Height (hMS) | 1.5 m (outdoor) 1.5-22.5 m (indoor) | 1.5 m (outdoor) 1.5-22.5 m (indoor) |

On the other hand, indoor scenarios need indoor layout model, e.g., dimension of the building, rooms, walls, eNB/UE position, for system-level simulation. The layout should be simple, e.g., symmetric, considering subsequent channel modeling and SLS implementation and hopefully, it should be simple enough to apply stochastic based channel modeling. For office scenario, indoor hotspot environment in TR36.814 can be a starting point. In addition, possible examples for shopping mall environments are shown in Fig. 4.

.

Figure 4: Example of shopping mall layout

Proposal 2: Parameters in Table 1 is applied for UMi and UMa scenarios.

Proposal 3: Layout model should be simple, e.g., symmetric, for indoor scenario(s).

- Indoor hotspot environment in TR36.814 can be a starting point for office scenario.

- Summary

In this contribution, we present our views on scenarios for >6GHz channel modeling. Following proposals are made based on the discussion.

Proposal 1: Target deployment scenarios include UMi, UMa and indoor.

- UMi and indoor scenarios are prioritized, if necessary.

Proposal 2: Parameters in Table 1 is applied for UMi and UMa scenarios.

Proposal 3: Layout model should be simple, e.g., symmetric, for indoor scenario(s).

- Indoor hotspot environment in TR36.814 can be a starting point for office scenario.

# References

[1] 3GPP, RP-151606, 	Samsung and Nokia Networks, “New SID Proposal: Study on channel model for frequency spectrum above 6 GHz,” Sep. 2015.

[2] 3GPP, RP-151544, NTT DOCOMO, CMCC, Ericsson, Huawei, HiSillicon, Intel, KT Corporation, Nokia, Qualcomm and Samsung, “Channel modeling for higher frequency bands,” Sep. 2015.

[3] 3GPP, RP-152212, Samsung, “Report of RAN email discussion about >6GHz channel modelling,” Dec. 2015.

[4] 3GPP, RP-151544, NTT DOCOMO, CMCC, Ericsson, Huawei, HiSillicon, Intel, KT Corporation, Nokia, Qualcomm and Samsung, “Channel model for 6GHz-100GHz,” Dec. 2015.

[5] Aalto University, BUPT, CMCC, Ericsson, Huawei, Intel, KT Corporation, Nokia, NTT DOCOMO, New York University, Qualcomm, Samsung, University of Bristol, University of Southern California, “5G Channel Model for bands up to 100 GHz”, Globecom 2015, Dec. 2015 (http://www.5gworkshops.com/5GCM.html).