3GPP TSG RAN WG1 Meeting #84	R1-160753

St Julian’s, Malta, 15th-19th February 2016

Agenda Item:	7.1.5

Source:	Huawei, HiSilicon

Title:	On MPDCCH order specification

Document for:	Discussion and decision

# Introduction

In the RAN1 #83 meeting, M-PDCCH order support was agreed for both CE mode A and CE mode B [1]:

Agreement:

- DCI sizes for M-PDCCH order

  - In CE mode A: the same size to DCI M1A

  - In CE mode B: the same size to DCI M1B

In this contribution, we compare M-PDCCH for normal scheduling to M-PDCCH order DCI format options, and provide our proposal.

# PDCCH scheduling in DCI format 1A

DCI Format 1A is used for random access procedure initiated by a PDCCH order only if format 1A CRC is scrambled with C-RNTI and all the remaining fields are set as follows [2]. The RB assignment field value depending on system bandwidth is summarized in Table 1.

- Localized/Distributed VRB assignment flag – 1 bit is set to ‘0’

- Resource block assignment – bits, where all bits shall be set to 1

- All the remaining bits in format 1A for compact scheduling assignment of one PDSCH codeword are set to zero

Table 1: Resource block assignment field to differentiate PDCCH scheduling and PDCCH order

| Field | Field state |
| --- | --- |
| Resource block assignment | 11111 (system BW=1.4MHz) 1111111(system BW=3MHz) 111111111(system BW=5MHz) 11111111111(system BW=10MHz) 111111111111(system BW=15MHz) 1111111111111(system BW=20MHz) |

Obviously, there are normally many bits can be used for the differentiation of normal scheduling and PDCCH order.

# M-PDCCH scheduling in DCI format 6-1B

If a similar differentiation method is used for DCI format 6-1B, there are only maximum 5 bits in the RB assignments according to the system bandwidth as listed in Table 2.

Table 2: Resource block assignment field to differentiate M-PDCCH scheduling and M-PDCCH order in DCI format 6-1B

| Field | Field state |
| --- | --- |
| Resource block assignment | 1 (system BW=1.4MHz) 11(system BW=3MHz) 111(system BW=5MHz) 1111(system BW=10MHz) 11111(system BW=15MHz) 11111(system BW=20MHz) |

Fewer bits for RB assignments for eMTC would cause more eNB scheduling restriction, especially for small system bandwidth. Thus, using the current PDCCH order differentiation method to differentiate M-PDCCH order in DCI format 6-1B will impose restriction on eNB’s resource scheduling and DCI indication as the UE cannot use those combinations for valid assignments.

Observation 1: Using the current PDCCH order differentiation method to differentiate M-PDCCH order in DCI format 6-1B will impose restriction on eNB’s resource scheduling and DCI indication.

# Method to differentiate M-PDCCH order in DCI format 6-1B

When an M-PDCCH of DCI format 6-1B is used for PDSCH scheduling, only MCS indices 0~10 are valid. Therefore, the two MSB bits in the MCS field cannot be ‘11’. Based on this a LC/CE UE can determine the DCI by reading the first 2-bits of the MCS field and checking whether they are ‘11’. If the value is set to ‘11’, the UE determines it is actually a DCI for M-PDCCH order, then the UE interprets the remaining bits as the fields of an M-PDCCH order. Otherwise, the UE confirms that this is a DCI for normal scheduling and the other bits contain scheduling information. Such a method is preferred to other proposals that may limit the size of the fields used by the M-PDCCH order.

Proposal 1: When an M-PDCCH is used for M-PDCCH order, the two bits corresponding to the location of two MSB bits in the MCS field can be fixed to 11.

# Conclusions

The following observation and proposal are given:

Observation 1: Using the current PDCCH order differentiation method to differentiate M-PDCCH order in DCI format 6-1B will impose restriction on eNB’s resource scheduling and DCI indication.

Proposal 1: When an M-PDCCH is used for M-PDCCH order, the two MSB bits in the MCS field can be fixed to 11.

# References

3GPP RAN1 #83 meeting, Chairman notes, Anaheim, USA, 15th - 22th November 2015.

TS 36.212. “Multiplexing and channel coding”, V 13.0.0