3GPP TSG RAN WG1 Meeting #84 	R1-160766

St Julian’s, Malta, 15th - 19th February 2016

Source:	Sharp

Title:	Downlink resource mapping and multiplexing for NB-IoT

Agenda Item:	7.2.1.1.2

Document for:	Discussion and Decision

Introduction

In the RAN1 NB-IoT ad-hoc meeting, the following agreements were made on NB-PDCCH [1].

Agreements: - A CCE for NB-PDCCH is composed by resources within a subframe - Within a PRB pair, 2 CCEs are defined - Single antenna port transmission is also supported based on the above definition - Same transmission scheme is applied to NB-PDCCH, NB-PBCH, and NB-PDSCH - REG is not defined for NB-PDCCH - Within CCE resource(s) for NB-PDCCH transmission, a SFBC pair is transmitted consecutively available two REs which is within a OFDM symbol - REs within SFBC pair can be separated at most one tone - LTE CRS (if present) is rate matched - NB-RS is rate matched - This has no implication of NB-RS patterns - FFS: handling of CSI-RS REs - For NB-IoT, - One CCE consists of 6 subcarriers per OFDM symbol in a subframe - NB-PDCCH and NB-PDSCH are multiplexed only based on TDM at subframe level at least per UE - It means that only cross subframe scheduling is supported - Different NB-PDCCHs: - TDM at subframe level for extended and extreme coverage. - Can be multiplexed in one subframe for normal coverage - FFS: FDM+TDM

In this contribution, we share our considerations on CCE resource mapping and NB-PDCCH/NB-PDSCH multiplexing between different UEs for NB-IoT.

Discussions

CCE resource mapping

In the NB-IoT ad hoc meeting, it was agreed for reference signals of NB-IoT as follows [1]:

- At least NB-RS is always present and always used for single antenna port and 2 antenna ports transmission schemes

  - FFS: Additional utilization of LTE CRS in in-band operation mode

- For NB-PDCCH/NB-PDSCH,

  - In-band operation

    - NB-RS is present without condition

  - Stand-alone and guard-band operations

    - Only NB-RS is used

Taking into account the achieved agreements on NB-PDCCH, the CCE resource mappings are shown in Figure1 and Figure 2 for stand-alone/guard-band and in-band operations respectively.

Figure 1 shows the CCE resource mapping for stand-alone and guard-band operations. Only NB-RS is used for NB-PDCCH/NB-PDSCH. Considering SFBC was agreed to be used for NB-PDCCH/NB-PDSCH transmission, two resource elements (RE) (consecutive or separated by one RS RE) in one OFDM symbol are used for a SFBC pair. And these two REs are alternately selected for the two CCEs in one OFDM symbol.

Figure 1 CCE resource mapping for stand-alone and guard-band operations

Figure 2 shows the CCE resource mapping for in-band operation. Except the NB-RS used for NB-PDCCH/NB-PDSCH demodulation, LTE CRS is presented in in-band operation. Figure 2(a) shows two LTE CRS ports presented in the NB-IoT PRB pair and Figure 2(b) shows four LTE CRS ports presented in the NB-IoT PRB pair. The same rules for CCE resource mapping in stand-alone and guard-band operations are applied for in-band operation.

- (b)

Figure 2 CCE resource mapping for in-band operation

Proposal 1:

- For CCE resource mapping, two REs (consecutive or separated by one RS RE) in one OFDM symbol are used for a SFBC pair and these two REs are alternately selected for the two CCEs.

NB-PDCCH/NB-PDSCH multiplexing

It was agreed that NB-PDCCH and NB-PDSCH are multiplexed only based on TDM at subframe level at least per UE. However, How to multiplex NB-PDCCH and NB-PDSCH of different UEs and how to multiplex multiple NB-PDSCH are not decided. Two options can be used to multiplex NB-PDCCH and/or NB-PDSCH:

- Option 1: Multiplex NB-PDCCH and NB-PDSCH of different UEs based on TDM at subframe level.

- Option 2: Multiplex NB-PDCCH and NB-PDSCH of different UEs based on CCE in one NB-IoT PRB pair.

For option 1, when a NB-IoT UE in extreme coverage is scheduled, other NB-IoT UEs have to wait for a long time to be scheduled. So the latency will be large even for a NB-IoT UE in a good channel condition. One solution proposed in last meeting is to schedule NB-IoT UEs in extreme coverage in discontinuous subframes. However, this solution needs additional standardization efforts.

For option 2, according to the agreements on NB-PDCCH, CCE should be defined for NB-IoT. It is a straightforward way to multiplex NB-PDCCH and NB-PDSCH of different UEs based on CCE in one NB-IoT PRB pair without additional standardization efforts.

In addition, for IoT applications the data transmission usually contains a small number of bytes (e.g. alarm information, measurement, or some short commands) [2], only a part of the NB-IoT PRB pair can carry such small data for NB-IoT UEs in good channel condition. Therefore, multiple NB-PDSCH should be multiplexed based on CCE in one NB-IoT PRB pair at least for NB-IoT UEs in good channel condition to improve the spectral efficiency.

Proposal 2:

- NB-PDCCH and NB-PDSCH of different UEs should be multiplexed based on CCE in one NB-IoT PRB pair at least for normal coverage UEs.

- Multiple NB-PDSCH should be multiplexed based on CCE in one NB-IoT PRB pair at least for normal coverage UEs.

Conclusion

In this contribution, we have the following proposals:

Proposal 1:

- For CCE resource mapping, two REs (consecutive or separated by one RS RE) in one OFDM symbol are used for a SFBC pair and these two REs are alternately selected for the two CCEs.

Proposal 2:

- NB-PDCCH and/or NB-PDSCH of different UEs should be multiplexed based on CCE in one NB-IoT PRB pair at least for normal coverage UEs.

- Multiple NB-PDSCH should be multiplexed based on CCE in one NB-IoT PRB pair at least for normal coverage UEs.

References

3GPP RAN1 NB-IoT ad-hoc chairman’s notes, January 2016.

R1-160104, NB-PDSCH design, Qualcomm Incorporated, 3GPP TSG RAN WG1 NB-IoT ad-hoc Meeting, Budapest, Hungary, 18th – 20th January 2016.