3GPP TSG RAN WG1 Meeting #84                                         R1-160791

St Julian’s, Malta, 15th - 19th February 2016

Agenda item:		7.3.2.2

Source: 			Beijing Xinwei Telecom Techn.

Title:             Discussion on V2V resource allocation

Document for:		Discussion and Decision

Introduction

In RAN1 #83 meeting, PC5-based V2V were discussed and the following agreements and FFSs about resource allocation are listed in [1]:

For resource allocation of PC5 interface, the following agreements were made:

- The following resource allocation principles are deemed necessary for PC5-based V2V:

  - Scheduling assignment

    - Study the number of transmissions of a given TB

    - Study the number of transmissions of a given SA

- Enhancement to UE autonomous resource selection

  - Collision avoidance based on sensing (P1), enhanced random resource selection (P2), and location-based resource selection (P3) are shown to provide gain when each of them is evaluated individually.

    - Further discussion is needed to identify whether operating a combination of the principles provides more gain than operating an individual principle.

- RAN1 observes potential benefit of adapting transmitter behavior from physical layer viewpoint:

  - FFS

    - Which specific behavior is adapted

    - What the adaptation is based on

In RAN #70 meeting, the Work Item of V2V services based on LTE sidelink was proposed with the following objective [2]:

- To identify what are necessary sidelink resource allocation enhancement option(s) among the ones captured in TR 36.885 for V2V services and specify the identified option(s) [RAN1, RAN2]

This contribution discusses V2V resource allocation principles based on above agreements, including SA/data transmission number, UE autonomous resource selection principles and UE adaptive behaviour.

Discussion on UE resource allocation

SA and data retransmission

In Rel-12 ProSe communication, SA and data messages are blind retransmitted twice and fourth respectively. Similar mechanism is used in PC5-based V2V, but due to high vehicle density in V2V scenarios, the retransmission number of ProSe communication will introduce extra collision and result in impaired system performance. Therefore, we suggest adjusting SA and data retransmission numbers in V2V.

Simulation results of multiple SA and data retransmission number combinations are shown in figure 1. In this simulation, basic V2V transmission mechanism is used without extra enhancements except using DMRS option 3 in [1]. The evaluation of DMRS structure options is described in our other contribution [3]. Detailed simulation parameters are listed in Appendix A.

(a) Urban, 15km/h

(b) Urban, 60km/h

(c) Freeway, 140km/h

Figure 1 Simulation of SA and data transmission number

Figure 1 illustrates the performance of different SA and data transmission number combinations in Urban and Freeway scenarios with different vehicle speeds. For the Urban 15km/h scenario, PRR performance in distance interval [0, 50m] is mainly focused thus 2 SA transmissions and 2 data transmissions can be applied. For the Urban 60km/h scenario, with distance interval [0, 150m], this combination of retransmission number still have best performance.

In the Freeway 140km/h scenario, for short distance (<180m) the traditional mechanism performs best while for large distance (180~320m) 2 SA transmissions and 2 data transmissions performs best. Since for short distance the PRR performance can reach the V2X requirement, and PRR enhancements should mainly focus on large distances, 2 SA transmissions and 2 data transmissions can be a final option.

Therefore, we suggest using 2 SA transmissions and 2 data transmissions in Urban and Freeway scenarios regardless of vehicle speed.

Proposal 1: In both Freeway and Urban scenarios, the transmission numbers of given SA and data can be set to 2 and 2 respectively.

UE autonomous resource selection principles

The UE autonomous resource selection mechanism in Rel-12 ProSe mode 2 communication also needs to be enhanced for the purpose of collision avoidance. In RAN1 #83 meeting, principles including collision based on sensing (P1), enhanced random resource selection (P2) and location-based resource selection (P3) were discussed. In this contribution, combinations of above principles are simulated to evaluate whether combined principles can provide more system gain.

Simulation results of P1+P2, P2+P3 and P1+P2+P3 are shown in figure 2 comparing with individually using P2. In all schemes enhanced random resource selection is introduced, vehicles randomly select multiple independent transmission resources instead of using T-RPT patterns. In the schemes with sensing, vehicles monitor receiving energy level on all available resources and periodically decide whether to reselect transmission resource through comparing the original receiving energy level of chosen transmission resource with the receiving energy level of multiple resources having lowest collision. In the location-based schemes, in Freeway scenario resources are portioned by geo regions, and in Urban scenario resources are portioned by street directions (horizontal or vertical). Detailed simulation parameters are listed in Appendix A.

(a) Urban, 15km/h

(b) Urban, 60km/h

(c) Freeway, 140km/h

Figure 2 Simulation of P1+P2, P2+P3, P1+P2+P3 and P2

Figure 2 illustrates that the performance of P1+P2 keeps better than P2 for both scenarios, while the gain of P2+P3 compared with P2 is less obvious and changes based on transmitter distance. In addition, when an individual principle is combined with other principles, the performance gain has similar trend to adding performance of the individual principle to original performance. Hence we support to operate a combination of the principles when individual principles can introduce system gain.

Proposal 2: Operating a combination of the principles provides more gain when each involved principle can provide gain individually. The combination of sensing and enhanced random resource selection is a competitive option.

UE Adaptive behaviour

The introduction of UE adaptive behaviour is mainly for the purpose of optimizing system performance in severely changed V2V scenarios. Vehicle density, which can be estimated by vehicle speed, receiving energy level, or be informed by network, is the main factor that changes obviously and aggravates collision. Thus behaviours such as transmission number, transmission power (including muting), physical layer parameters, and cooperative transmission in [4] should be considerable.

On the other hand, services with different type (e.g. public safety/non-public safety, periodical or emergency triggered) may have different QoS requirements. Therefore service type based adaptive behaviour will be beneficial to protect high priority services and satisfy specific QoS requirement.

Proposal 3: Adaptive behaviour including transmission number, transmission power, physical layer parameters and cooperative transmission is considerable. The adaptation can be based on vehicle density, vehicle speed, receiving energy level, service type or QoS requirement, and network informing.

Conclusion

Proposal 1: In both Freeway and Urban scenarios, the transmission numbers of given SA and data can be set to 2 and 2 respectively.

Proposal 2: Operating a combination of the principles provides more gain when each involved principle can provide gain individually. The combination of sensing and enhanced random resource selection is a competitive option.

Proposal 3: Adaptive behaviour including transmission number, transmission power, physical layer parameters and cooperative transmission is considerable. The adaptation can be based on vehicle density, vehicle speed, receiving energy level, service type or QoS requirement, and network informing.

References

- RP-151941

- RP-152293

- R1-160792

- R1-157534

- Appendix A

Table 1: simulation assumptions

| Parameters | Value |
| --- | --- |
| Carrier frequency | 6 GHz |
| System bandwidth | 10MHz |
| Number of carriers | one carrier |
| Tx power | 23dBm |
| Antenna gain | 3dBi |
| Synchronization | Ideal |
| Carrier frequency offset | +/-0.1 PPM |
| UE drop and mobility model | Freeway: absolute vehicle speed 140km/h Urban: absolute vehicle speed 60km/h, 15km/h |
| Vehicle density | Average inter-vehicle distance in the same lane is 2.5 sec * absolute vehicle speed |
| SA period | 40ms, SA pool 8ms and data pool 32ms |
| Channel model | As described in [4] |
| Traffic model | periodic traffic with 100ms period |
| Retransmission times | {1, 2} for SA, {1, 2, 4} for periodic traffic |
| DMRS formats | Option 3 described in [1] |
| Modulation | QPSK with coding rate of 0.5 |
| Number of RBs for data | 16 for 190Byte, 25 for 300Byte |
| Resource selection method | Random selection |