3GPP TSG RAN WG1 Meeting #84		R1-160837

St Julian’s, Malta, 15th - 19th February 2016

Agenda Item:	7.2.1.1.4

Source: 	MediaTek Inc.

Title:  	Evaluation results for NB-PSS/NB-SSS

Document for:	Discussion/Decision

Introduction

A unified NB-PSS/NB-SSS synchronization signal design in all three operation modes is described in [1]. In this contribution we take the NB-PSS/NB-SSS synchronization design in [1] to be a base line and compare with the other NB-PSS/NB-SSS synchronization designs. The different NB-PSS sequence design in [4] and [6] are evaluated in our simulator. Synchronization signal sub-frame allocation is evaluated, contiguous NB-PSS sub-frame allocation scheme is proposed in [2][6] and the separated PSS sub-frame allocation scheme is proposed in [4]. Additionally, the NB-SSS sub-frame allocation and density in 80ms as well as different NB-SSS sequences are evaluated. We also consider the channel raster offset (+/-7.5kHz SFO over-compensation issue) on SSS detection and evaluate the SSS detection performance degradation for different SSS density.

The evaluation is based on the four scenarios suggested by Ericsson during Email discussion. These scenarios are listed in Table 1

We applied the same performance indicators as those in former contributions [3][5] to evaluate various NB-PSS/NB-SSS synchronization signal design.

Table 1: Four scenarios in the evaluation

| MCL 164 dB |  | Operation mode |  |  |
| --- | --- | --- | --- | --- |
|  |  | In-band | Stand-alone | Guard-band |
| Cell search type | Initial cell search | 0 Interferer | 0 Interferer | X (Skipped) |
|  | Non-initial cell search | 2 Interferers | 2 Interferers | X (Skipped) |

Simulation

Simulation assumption

The simulation was done on four scenarios in Table 1.

For initial cell search scenario (with +/- 20ppm initial frequency offset), a single cell is simulated. For non-initial cell search scenario (with +/- 2ppm initial frequency offset), three cells with equal transmitted power are simulated (2 interferers).

The carrier frequency offset and sampling timing error are modeled.

In the stand-alone scenario, the transmit filter that fulfils the GSM PSD mask was not modelled yet. Therefore, the stand-alone simulation results would be a little better.

We will treat the design of [1] as the base line model and compare with different NB-PSS/NB-SSS design.

The detail simulation setting is listed in Table 2.

The NB-PSS/NB-SSS synchronization signal setting is listed in Table 3.

The definition of successful detection is the PSS correlation peak exceeding a pre-determined threshold. Among these successful detection realizations, the definition of false alarm is the realization which is failed to find correct cell ID by NB-SSS. Additionally, other statistics like the residual timing error, residual frequency error, and total synchronization time are collected.

Table 2: Simulation setting

|  | In-band, initial cell search | In-band, non-initial cell search | Stand-alone, initial cell search | Stand-alone, non-initial cell search |
| --- | --- | --- | --- | --- |
| Carrier frequency | 900 MHz |  |  |  |
| Channel model | TU |  |  |  |
| Doppler spread | 1 Hz |  |  |  |
| Initial frequency offset1 | +/- 20 ppm | +/- 2 ppm | +/- 20 ppm | +/- 2 ppm |
| SNR | -12.6 dB2 |  | -4.6 dB3 |  |
| Antenna configuration | 1Tx 1Rx |  |  |  |
| Sampling rate | 1.92 MHz (A/D); 240kHz (PSS detection)ing rategurationand chitted filter to four different NB-SSS sequence PSS subframe allocation in |  |  |  |
| Power boost for NB-IoT signal | 6 dB |  | 0 dB |  |
| Tx transmit filter | No |  | No |  |
| Number of interferer | 0 | 2 | 0 | 2 |
| Number of realizations | 1000 |  |  |  |
| Note1: For each simulation realization in initial cell search, a random initial frequency offset is selected from the set {20ppm, -20ppm}. For each simulation realization in non-initial cell search, a random initial frequency offset is selected from the set {2ppm, -2ppm}. Note2: Under MCL 164 dB, the SNR for in-band operation mode is -12.6 dB Note3: Under MCL 164 dB, the SNR for stand-alone operation mode is -4.6 dB |  |  |  |  |

Table 3: NB-PSS/NB-SSS setting

|  | NB-PSS | NB-SSS |
| --- | --- | --- |
| Sub-frame density in 80ms | 8/80 = 0.1 | 4/80 = 0.05 |
| Symbols in a sub-frame | 11 | 11 |
| CRS puncture (one antenna port) | Yes | Yes |
| Source sequence | Length 143 Zadoff-Chu sequence | Length 131 Zadoff-Chu sequence |

Simulation results

A unified NB-PSS/NB-SSS synchronization signal design in all three operation modes is described in [1]. The statistic of simulation results are in Table 4. We can achieve 100% detection rate and <1% false alarm rate in all four scenarios. We can achieve 0% false alarm by increasing the PSS detection threshold but it will increase the required synchronization time. The range of residual timing error is ±3 µs with 95% confidences among four scenarios. The range of residual frequency error is ±50 Hz with 95% confidences among four scenarios. For initial cell search, the synchronization time is no longer than 900ms with 90% confidences. For non-initial cell search, the synchronization time is no longer than 400ms with 90% confidences.

The more detail figure of statistic is in Appendix A.2.

Observation #1: The synchronization signal design in [1] can achieve 100% detection rate and < 1% false alarm rate in all four scenarios

Observation #2: In the synchronization signal design [1], the range of residual timing error is ±3.2 µs with 95% confidences among four scenarios

Observation #3: In the synchronization signal design [1], the range of residual frequency error is ±80 Hz with 95% confidences among four scenarios

Observation #4: In the synchronization signal design [1], the synchronization time is no longer than 4180ms and 920ms respectively for initial cell search and non-initial cell search.

Table 4: Statistics of simulation results

| MCL 164 dB | In-band, initial cell search, 0 interferer | In-band, non-initial cell search, 2 interferer | Stand-alone, initial cell search, 0 interferer | Stand-alone, non-initial cell search, 2 interferer |
| --- | --- | --- | --- | --- |
| Detection rate | 99.9% | 100% | 100% | 100% |
| False alarm rate | 0.7% | 0.1% | 0% | 0% |
| 95%, Residual timing error (µs) | -2.90~2.90 | -1.85~1.85 | -3.11~3.11 | -2.47~2.47 |
| 95%, Residual frequency error (Hz) | -78~78 | -59~59 | -43~43 | -30~30 |
| 50%, synchronization time (ms) | 1506.0 | 420.98 | 308.57 | 223.19 |
| 90%, synchronization time (ms) | 4170.0 | 902.86 | 594.0 | 376.47 |
| Average synchronization time (ms) | 1967.9 | 524.12 | 388.02 | 271.14 |

Comparison

The different NB-PSS/NB-SSS sub-frame allocation, the different NB-PSS/NB-SSS sequence design are evaluated in follows.

NB-PSS sub-frame allocation

The contiguous NB-PSS sub-frame allocation is proposed in [2][6] and the separated PSS sub-frame allocation is proposed in [4]. In [1], we propose the contiguous NB-PSS sub-frame allocation scheme. The NB-PSS contiguous sub-frame allocation in Figure 1 and the NB-PSS separated sub-frame allocation in Figure 2 are evaluated. For separated scheme, the distance between two NB-PSS sub-frames is 5 sub-frames. The simulation results show the required time of NB-PSS detection for these two schemes are almost the same  in Appendix A.1 Figure 9~Figure 12. The statistic of synchronization time is in Table 5. In fact, compared with the sub-frame allocation scheme, the NB-PSS density is the dominant factor on the required time of NB-PSS detection.

Observation #5: Consider the required time of NB-PSS detection, there is no significant difference between contiguous and separated sub-frame allocation schemes.

Table 5: Statistics on two sub-frame allocation schemes for in-band initial cell search

| MCL 164 dB, in-band, initial cell search, 0 interferer | Contiguous sub-frame allocation | Separated sub-frame allocation |
| --- | --- | --- |
| 50%, synchronization time (ms) | 1506 | 1525 |
| 90%, synchronization time (ms) | 4170.0 | 4173.3 |
| Average synchronization time (ms) | 1967.9 | 1987.6 |

Figure 1: Contiguous NB-PSS sub-frame allocation pattern

Figure 2: Separated NB-PSS sub-frame allocation pattern

NB-PSS sequence

The three different NB-PSS sequences in [4][6][1] are evaluated. The NB-PSS sequence generator in [4][6] use length 139 and 141 Zadoff-Chu sequence respectively. In [1] the length 143 Zadoff-Chu sequence is used. We replace the NB-PSS sequence generator by [4][6] to do simulations and compare with [1]. The simulation results are in Table 6~Table 9 for in-band initial cell search, in-band non-initial cell search, stand-alone initial cell search and stand-alone non-initial cell search respectively. The required time of PSS detection is in Appendix A.2 Figure 13, Figure 17, Figure 21, and Figure 25.

Under the initial cell search scenario, NB-PSS sequence generated by the length 143 Zadoff-Chu sequence in [1] is the best among these 3 NB-PSS sequences.

Under the non-initial cell search scenario, the synchronization time for these three NB-PSS sequences is quite close.

The more detail figure of statistic is in Appendix A.2.

Observation #6: Under the initial cell search scenario, NB-PSS sequence generated by the length 143 Zadoff-Chu sequence in [1] is better than the other Zadoff-Chu sequence with length 139 and length 141 in [4][6] respectively

Table 6: Statistics on NB-PSS sequence comparison for in-band initial cell search

| MCL 164 dB, in-band, initial cell search, 0 interferer | Use NB-PSS generator by [4], ZC length 139 | Use NB-PSS generator by [6] , ZC length 141 | Use NB-PSS generator by [1] , ZC length 143 |
| --- | --- | --- | --- |
| 50%, synchronization time (ms) | 1970.0 | 2413.3 | 1506.0 |
| 90%, synchronization time (ms) | 5970 | 7240 | 4170 |
| Average synchronization time (ms) | 2594.7 | 2966.3 | 1967.9 |

Table 7: Statistics on NB-PSS sequence comparison for in-band non-initial cell search

| MCL 164 dB, in-band, non-initial cell search, 2 interferer | Use NB-PSS generator by [4], ZC length 139 | Use NB-PSS generator by [6] , ZC length 141 | Use NB-PSS generator by [1] , ZC length 143 |
| --- | --- | --- | --- |
| 50%, synchronization time (ms) | 408.64 | 411.18 | 420.98 |
| 90%, synchronization time (ms) | 866.67 | 880.0 | 902.86 |
| Average synchronization time (ms) | 512.20 | 516.38 | 524.12 |

Table 8: Statistics on NB-PSS sequence comparison for stand-alone initial cell search

| MCL 164 dB, stand-alone, initial cell search, 0 interferer | Use NB-PSS generator by [4], ZC length 139 | Use NB-PSS generator by [6] , ZC length 141 | Use NB-PSS generator by [1] , ZC length 143 |
| --- | --- | --- | --- |
| 50%, synchronization time (ms) | 335.64 | 343.16 | 308.57 |
| 90%, synchronization time (ms) | 695.0 | 696.67 | 594.0 |
| Average synchronization time (ms) | 421.94 | 428.16 | 388.02 |

Table 9: Statistics on NB-PSS sequence comparison for stand-alone non-initial cell search

| MCL 164 dB, stand-alone, non-initial cell search, 2 interferer | Use NB-PSS generator by [4], ZC length 139 | Use NB-PSS generator by [6] , ZC length 141 | Use NB-PSS generator by [1] , ZC length 143 |
| --- | --- | --- | --- |
| 50%, synchronization time (ms) | 220.73 | 222.45 | 223.19 |
| 90%, synchronization time (ms) | 363.16 | 367.62 | 376.47 |
| Average synchronization time (ms) | 266.64 | 268.44 | 271.14 |

NB-SSS density

Uniform NB-SSS sub-frame allocation is proposed in [3], and the non-uniform NB-SSS sub-frame allocation is proposed in [2][4]. In [1], we propose the uniform NB-SSS sub-frame allocation in low density (4 SSS sub-frames in 80ms) to shorten the SSS detection time. The NB-SSS uniform sub-frame allocations are demonstrated in Figure 3 and Figure 4 for low and high density respectively. The NB-SSS non-uniform sub-frame allocation is shown in Figure 5. The above 3 allocation methods are evaluated. The simulation results show the required time of NB-SSS detection is sub-optimal for uniform NB-SSS sub-frame allocation in low density. The performance is quite close to the one with high density.

Observation #7: Consider the required time of NB-SSS detection, 4 uniform NB-SSS within 80m scheme has the sub-optimal performance with only half density of the optimal solution that require 8 NB-SSS per 80ms.

Figure 3: Uniform NB-SSS sub-frame allocation pattern in low density (4 SSS)

Figure 4: Uniform NB-SSS sub-frame allocation pattern in high density (8 SSS)

Figure 5: Non-uniform NB-SSS sub-frame allocation pattern (4 SSS)

Table 10: Statistics on NB-SSS sub-frame allocation pattern comparison for in-band initial cell search

| MCL 164 dB, in-band, initial cell search, 0 interferer | Use NB-SSS generator by [3], non-uniform 4 SSS | Use NB-SSS generator by [3], uniform 8 SSS | Use NB-SSS generator by[4], uniform 4 SSS |
| --- | --- | --- | --- |
| 50%, synchronization time (ms) | 27 | 14 | 21 |
| 90%, synchronization time (ms) | 224 | 140 | 225 |

Table 11: Statistics on NB-SSS sub-frame allocation pattern comparison for in-band non-initial cell search

| MCL 164 dB, in-band, non-initial cell search, 2 interferer | Use NB-SSS generator by [3], non-uniform 4 SSS | Use NB-SSS generator by [3], uniform 8 SSS | Use NB-SSS generator by[4], uniform 4 SSS |
| --- | --- | --- | --- |
| 50%, synchronization time (ms) | 24 | 12 | 17 |
| 90%, synchronization time (ms) | 71 | 33 | 68 |

Table 12: Statistics on NB-SSS sub-frame allocation pattern comparison for stand-alone initial cell search

| MCL 164 dB, stand-alone, initial cell search, 0 interferer | Use NB-SSS generator by [3], non-uniform 4 SSS | Use NB-SSS generator by [3], uniform 8 SSS | Use NB-SSS generator by[4], uniform 4 SSS |
| --- | --- | --- | --- |
| 50%, synchronization time (ms) | 21 | 10 | 11 |
| 90%, synchronization time (ms) | 39 | 19 | 37 |

Table 13: Statistics on NB-SSS sub-frame allocation pattern comparison for stand-alone non-initial cell search

| MCL 164 dB, stand-alone, non-initial cell search, 2 interferer | Use NB-SSS generator by [3], non-uniform 4 SSS | Use NB-SSS generator by [3], uniform 8 SSS | Use NB-SSS generator by[4], uniform 4 SSS |
| --- | --- | --- | --- |
| 50%, synchronization time (ms) | 21 | 10 | 10 |
| 90%, synchronization time (ms) | 36 | 18 | 18 |

NB-SSS density considered channel raster

In this section we consider the +/-7.5KHz channel raster offset to evaluate the performance degradation of SSS detection for different SSS density. Before UE obtains the operation mode (i.e. initial cell search), the channel raster offset will causes a SFO over-compensation and degrades the receiving channel (e.g. NB-SSS, NB-PBCH) performance. In this simulation three NB-SSS density are evaluated, 1/2/4 NB-SSS sub-frame(s) in 80ms (Figure 6~Figure 8). The statistics for in-band initial cell search with channel raster 0 and +/-7.5 KHz is in Table 14 and Table 15 respectively. The detail SSS detection time is in Appendix A.5 Figure 37 ~ Figure 39.

There is a little performance degradation for 4 NB-SSS sub-frame in 80ms, the NB-SSS detection rate reduce 5.4% (99.3%-->93.9%). However, the NB-SSS performance degradation is huge for cases with 1 or 2 NB-SSS subframe(s) in 80 ms due to the longer combining period. UE should keep NB-PSS timing tracking when detecting the low density NB-SSS (1 or 2 sub-frame in 80ms).

Figure 6 1 NB-SSS sub-frame in 80 ms

Figure 7 2 NB-SSS sub-frame in 80 ms

Figure 8 4 NB-SSS sub-frame in 80 ms

Table 14 Statistics on NB-SSS density comparison with 0 KHz channel raster offset

| MCL 164 dB, in-band, initial cell search, 0 interferer | 1 NB-SSS in 80 ms | 2 NB-SSS in 80 ms | 4 NB-SSS in 80 ms |
| --- | --- | --- | --- |
| SSS detection rate (%) | 99.3% | 99.5% | 99.3% |
| 50%, SSS detection time (ms) | 221 | 74 | 34 |
| 90%, SSS detection time (ms) | 824 | 422 | 290 |

Table 15 Statistics on NB-SSS density comparison with +/-7.5 KHz channel raster offset

| MCL 164 dB, in-band, initial cell search, 0 interferer | 1 NB-SSS in 80 ms | 2 NB-SSS in 80 ms | 4 NB-SSS in 80 ms |
| --- | --- | --- | --- |
| SSS detection rate (%) | 65.3% | 85.6% | 93.9% |
| 50%, SSS detection time (ms) | 248 | 74 | 34 |
| 90%, SSS detection time (ms) | -- | -- | 352 |

Observation #8 Consider +/-7.5KHz SFO over-compensation due to channel raster offset, the performance of NB-SSS detection for low density NB-SSS design (1 or 2 sub-frame in 80ms) is degraded significantly.

NB-SSS sequence

ZC sequence of 61 is proposed for NB-SSS in [3], and ZC sequence of 107 and 139 is proposed for NB-SSS in [2][4]. In [1], we demonstrate the simulation of SSS detection time for various ZC sequence length. We also evaluate the complexity of NB-SSS receiver. The candidate ZC sequence is 61, 71 and 131, which stands for 6, 6 and 11 NB-SSS symbols respectively. As expected, the results showed the ZC 131 is the best sequence among these 3 sequences, in the cost of extra receiver complexity for long sequence.

Observation #9: Consider the required time of NB-SSS detection, ZC sequence with length 131 has the best performance with the cost of doubled receiver complexity.

Table 16: Statistics on NB-SSS sequence comparison for in-band initial cell search

| MCL 164 dB, in-band, initial cell search, 0 interferer | ZC 61 | ZC 71 | ZC 131 |
| --- | --- | --- | --- |
| 50%, synchronization time (ms) | 49 | 43 | 21 |
| 90%, synchronization time (ms) | 393 | 364 | 225 |

Table 17: Statistics on NB-SSS sequence comparison for in-band non-initial cell search

| MCL 164 dB, in-band, non-initial cell search, 2 interferer | ZC 61 | ZC 71 | ZC 131 |
| --- | --- | --- | --- |
| 50%, synchronization time (ms) | 41 | 35 | 17 |
| 90%, synchronization time (ms) | 176 | 140 | 68 |

Table 18: Statistics on NB-SSS sequence comparison for stand-alone initial cell search

| MCL 164 dB, stand-alone, initial cell search, 0 interferer | ZC 61 | ZC 71 | ZC 131 |
| --- | --- | --- | --- |
| 50%, synchronization time (ms) | 14 | 13 | 11 |
| 90%, synchronization time (ms) | 104 | 96 | 37 |

Table 19: Statistics on NB-SSS sequence comparison for stand-alone non-initial cell search

| MCL 164 dB, stand-alone, non-initial cell search, 2 interferer | ZC 61 | ZC 71 | ZC 131 |
| --- | --- | --- | --- |
| 50%, synchronization time (ms) | 11 | 11 | 10 |
| 90%, synchronization time (ms) | 30 | 25 | 18 |

Conclusion

In this contribution, we provide the evaluation results for the synchronization design in [1]. And several different NB-PSS/NB-SSS designs are analyzed. The NB-PSS sub-frame allocation, the NB-PSS sequence, the NB-SSS density, NB-SSS performance degradation due to channel raster offset for different NB-SSS density and different NB-SSS sequence design are evaluated.

Observation #1: The synchronization signal design in [1] can achieve 100% detection rate and < 1% false alarm rate in all four scenarios

Observation #2: In the synchronization signal design [1], the range of residual timing error is ±3.2 µs with 95% confidences among four scenarios

Observation #3: In the synchronization signal design [1], the range of residual frequency error is ±80 Hz with 95% confidences among four scenarios

Observation #4: In the synchronization signal design [1], the synchronization time is no longer than 4180ms and 920ms respectively for initial cell search and non-initial cell search.

Observation #5: Consider the required time of NB-PSS detection, there is no significant difference between contiguous and separated sub-frame allocation schemes.

Observation #6: Under the initial cell search scenario, NB-PSS sequence generated by the length 143 Zadoff-Chu sequence in [1] is better than the other Zadoff-Chu sequence with length 139 and length 141 in [4][6] respectively

Observation #7: Consider the required time of NB-SSS detection, 4 uniform NB-SSS within 80m scheme has the sub-optimal performance with only half density of the optimal solution that require 8 NB-SSS per 80ms.

Observation #8 Consider +/-7.5KHz SFO over-compensation due to channel raster offset, the performance of NB-SSS detection for low density NB-SSS design (1 or 2 sub-frame in 80ms) is degraded significantly.

Observation #9: Consider the required time of NB-SSS detection, ZC sequence with length 131 has the best performance with the cost of doubled receiver complexity.

References

- R1-160188, Synchronization signal design for NB-IoT, Mediatek

- R1-155960, Synchronization Signal Design, Huawei, HiSilicon

- R1-156009, Narrowband LTE - Synchronization Channel Design and Performance, Ericsson

- R1-156464, Synchronization Signal Design, Huawei, HiSilicon

- R1-156465, Synchronization Signal Evaluation, Huawei, HiSilicon

- R1-157418, NB-IoT - Improved Synchronization Channel Design, Ericsson

- Appendix A

- A.1	Statistic results of NB-PSS sub-frame allocation

Figure 9: The required time of NB-PSS detection for in-band initial cell search

Figure 10: The required time of NB-PSS detection for in-band non-initial cell search

Figure 11: The required time of NB-PSS detection for stand-alone initial cell search

Figure 12: The required time of NB-PSS detection for stand-alone non-initial cell search

- A.2	Statistic results of different NB-PSS sequence

- A.2.1	In-band operation mode

- A.2.1.1	Initial cell search

Figure 13: time required of PSS detection for in-band initial cell search

Figure 14: time required of SSS detection for in-band initial cell search

Figure 15: residual timing error for in-band initial cell search

Figure 16: residual frequency error for in-band initial cell search

- A.2.1.2	Non-initial cell search

Figure 17: time required of PSS detection for in-band non-initial cell search

Figure 18: time required of SSS detection for in-band non-initial cell search

Figure 19: residual timing error for in-band non-initial cell search

Figure 20: residual frequency error for in-band non-initial cell search

- A.2.2	Stand-alone operation mode

- A.2.2.1	Initial cell search

Figure 21: time required of PSS detection for stand-alone initial cell search

Figure 22: time required of SSS detection for stand-alone initial cell search

Figure 23: residual timing error for stand-alone initial cell search

Figure 24: residual frequency error for stand-alone initial cell search

- A.2.2.2	Non-initial cell search

Figure 25: time required of PSS detection for stand-alone non-initial cell search

Figure 26: time required of SSS detection for stand-alone non-initial cell search

Figure 27: residual timing error for stand-alone non-initial cell search

Figure 28: residual frequency error for stand-alone non-initial cell search

- A.3	Statistic results of NB-SSS sub-frame allocation

- A.3.1	In-band operation mode

- A.3.1.1	Initial cell search

Figure 29: time required of SSS detection for in-band initial cell search

- A.3.1.2	Non-initial cell search

Figure 30: time required of SSS detection for in-band non-initial cell search

- A.3.2	Stand-alone operation mode

- A.3.2.1	Initial cell search

Figure 31: time required of SSS detection for standalone initial cell search

- A.3.1.2	Non-initial cell search

Figure 32: time required of SSS detection for standalone non-initial cell search

- A.4	Statistic results of NB-SSS sequence

- A.4.1	In-band operation mode

- A.4.1.1	Initial cell search

Figure 33: time required of SSS detection for in-band initial cell search

- A.4.1.2	Non-initial cell search

Figure 34: time required of SSS detection for in-band non-initial cell search

- A.4.2	Stand-alone operation mode

- A.4.2.1	Initial cell search

Figure 35: time required of SSS detection for standalone initial cell search

- A.4.1.2	Non-initial cell search

Figure 36: time required of SSS detection for standalone non-initial cell search

- A.5	Statistic results of NB-SSS density considered +/- 7.5KHz channel raster offset

- A.5.1	In-band operation mode

- A.5.1.1	Initial cell search

Figure 37 Time required of SSS detection for 1 NB-SSS in 80ms in-band initial cell search

Figure 38 Time required of SSS detection for 2 NB-SSS in 80ms in-band initial cell search

Figure 39 Time required of SSS detection for 4 NB-SSS in 80ms in-band initial cell search