3GPP TSG RAN WG1 Meeting #84	                         R1-160876

St Julian’s, Malta

15th - 19th February 2016

Agenda item:    7.2.1.1.2

Source:             Qualcomm Incorporated

Title:                  NB-PDCCH design

Document for:  Discussion and Decision

- Introduction

In RAN1#83, the following agreements have been made:

Agreements:

- A CCE for NB-PDCCH is composed by resources within a subframe

  - within a PRB pair, 2 CCEs are defined

  - Single antenna port transmission is also supported based on the above definition

- Same transmission scheme is applied to NB-PDCCH, NB-PBCH, and NB-PDSCH

- The same DCI size for DL and UL is targeted for all operation modes and all coverage cases

- In in-band, first few OFDM symbols are not used for NB-PDCCH

  - FFS CFI is signaled by SIB or MIB or fixed to 3

- In stand-alone and guard-band, all OFDM symbols are assumed to be available for NB-PDCCH

- REG is not defined for NB-PDCCH

- Within CCE resource(s) for NB-PDCCH transmission, a SFBC pair is transmitted consecutively available two REs which is within a OFDM symbol

  - REs within SFBC pair can be separated at most one tone

  - LTE CRS (if present) is rate matched

  - NB-RS  is rate matched

    - This has no implication of NB-RS patterns

  - FFS: handling of CSI-RS REs

In this contribution, we provide our view on the control channel for NB-IoT.

- Design Considerations for NB-PDCCH

Similarly to LTE and eMTC, there is a need in NB-IoT to have a control channel in order to carry uplink and downlink assignments. Due to the low complexity UEs, large coverage needs and narrowband characteristics of NB-IoT, some new design aspects have to be considered for NB-PDCCH.

- Common design with PDSCH: Due to the low complexity requirements of NB-IoT, it would be desirable for UEs to have to implement a single channel estimation and demodulation mode for PDSCH and NB-PDCCH. We propose to use a new cell-specific reference signal for time/frequency tracking and PDSCH demodulation. This reference signal should also be used for NB-PDCCH. Note that reusing the control channel in eMTC (MPDCCH), for example, would force the UE to implement DMRS-based channel estimation, which is not desirable.

Proposal 1: Consider having the same NB-CRS for PDSCH and NB-PDCCH.

- Multiplexing of multiple UE and channels: NB-PDCCH has to offer enough flexibility to carry assignments for multiple UE, potentially in different coverage enhancement levels. We propose to divide the resource elements in control channel elements (CCE), and use this CCE concept to multiplex PDSCH and NB-PDCCH. This multiplexing can be semi-statically configured (e.g. in SIB) or can be dynamically indicated in the DCI: a UE would blindly decode NB-PDCCH in some CCE, and will receive an indication in the DCI of the CCE to be used for PDSCH.

Proposal 2: Allow multiplexing of NB-PDSCH and NB-PDCCH in the same RB by using different CCE.

- Reduced number of blind decodings: Due to the absence of a turbo-decoder at the UE side, the TBCC decoder represents the peak decoding complexity in NB-IoT. In order to reduce the cost and power consumption of a UE, the number of blind decodings for the control channel should be very small (e.g. 4).

Proposal 3: The maximum number of blind decodings should be reduced with respect to eMTC to a very small value (e.g. 2-4 blind decodes).

- Control-less downlink data transmission

For IoT applications the data transmission usually contains a small number of bytes (e.g. alarm information, measurement, or some short commands). Thus, transmitting a control channel associated with a short data packet might incur in a very large overhead. It is desirable to have the possibility of transmitting a data packet without any control associated with it. Also, note that it is agreed to use TBCC for all downlink channels, so the data and control channels are sharing the coding scheme.

In order to achieve this control-less data transmission effectively and with a reasonable complexity, some constraints have to be introduced in the data transmission.

- Keep transport block to have same size as DCI: A UE would monitor a single payload size (otherwise the UE would need to perform additional blind decodings). Data and control can be differentiated by having a different redundancy check (e.g. data having a CRC of different size, or same size but different RNTI).

- Reduced MAC header: Some fields, like logical channel ID, reserved, etc. can be removed from the MAC header to reduce the overhead of data transmitted over NB-PDCCH.

Proposal 4: Allow transmission of data directly over the control channel to avoid large overhead for small payload sizes.

- Conclusion

Proposal 1: Consider having the same NB-CRS for PDSCH and NB-PDCCH.

Proposal 2: Allow multiplexing of NB-PDSCH and NB-PDCCH in the same RB by using different CCE.

Proposal 3: The maximum number of blind decodings should be reduced with respect to eMTC to a very small value (e.g. 2-4 blind decodes).

Proposal 4: Allow transmission of data directly over the control channel to avoid large overhead for small payload sizes.