3GPP TSG RAN WG1 Meeting #84		R1-160924

St Julian’s, Malta, 15th - 19th February 2016

Agenda item:	7.3.5.1

Source:	InterDigital

Title:	Diamond Shaped Cell Layouts for Above 6 GHz Channel Modeling

Document for:		Discussion and Decision

# 1	Introduction

As part of the SID for above 6GHz channel modeling, which was approved in RP-151606, email discussions occurred, which were summarized in [1] and contain the following topics:

- Analysis of work done outside of 3GPP

- Views on deployment scenarios, and spectrum bands of interest

- Identifications and analysis of channel modeling methodologies

- Identification of additional features required for higher frequency channel modeling

- Identification of measurement capabilities

This document focuses on item 2 from above, and in particular on the nature of the deployment scenarios. This deployment layout is applicable for the UMi scenario. Its applications for other scenarios is also possible.

# 2	Overview and Motivation

Cellular systems require many base stations to be placed throughout a geographical area to assure adequate radio coverage of that area. Realistically, each base station provides radio coverage for an irregularly shaped part (i.e. cell) of the total area. However, in order to perform mathematically convenient system analysis, a common regularly shaped cell is preferred.

Though a circular cell shape seems most appropriate, tessellating cell shapes are more convenient since they can be laid next to each other with no overlap. Examples of tessellating polygons include square, triangle, diamond, and hexagon. Hexagon is chosen for cellular analysis due to its proper approximation of a circular shape.

However, the hexagon deployment layout may not be accurate in certain scenarios. Consider the example of UMi street canyon scenario where base stations are placed near road intersections and below rooftops, e.g., mounted on street furniture or lampposts. It is shown in [2] that diamond shape cells will better approximate typical radio coverage and would be the more appropriate layout for system analysis.

Although the measurements and analyses in [2] were done for below 6 GHz, the same conclusions are expected for the above 6 GHz frequency band. Actually, the higher penetration loss and diffraction loss at higher frequency band would make the diamond layout even more appropriate. Furthermore, the above 6 GHz deployments may focus more on small cells. Hence, it is an appropariate assumption of placing base stations below rooftops in many cases.

As an example, Figure 1 shows the “Madrid grid”, which is an environmental model specified in [3] used to model a dense urban area. For this scenario a typical above 6 GHz deployment would place base stations at the intersections with the intent of providing coverage along the four cardinal directions, as penetration through building as well as diffraction around buildings will be very limited.

Observation 1: In certain scenarios like street canyon scenario with below rooftop base stations, the diamond shaped cells are a better approximate deployment.

- This is even more appropariate for above 6 GHz frequency band.

- This fits well if the intention is to provide coverage along the four cardinal directions in grid-like urban areas

It’s also well known that beamforming gains are required for above 6 GHz systems to be feasible, and this fact provides additional motivation for using diamond shaped cells. Above 6 GHz base stations will be equipped with multiple, relatively large, phased array antennas in order to form steerable beamformed links. Each antenna element in the phased array will likely be of a printed planar patch element. The beamforming gain will then drop off as a function of the radiation pattern of an individual antenna element.

As an illustration, Figure 2 shows the azimuth radiation pattern for a single patch element. The pattern was computed with ArrayCalc [5], a Matlab toolbox that uses idealized mathematical models. As shown there is an approximate 6 dB loss from 45 to 60 degrees. As such, steering over a range of +/- 60 degrees as implied from a three sector hexagonal cell shape will be challenging. Using a diamond layout however, implies four sectors (i.e. four phased array antennas) each requiring only +/- 45 degrees of steering.

Additional phased arrays can be utilized, each requiring even less degrees of steering, however this comes with increased cost per base station. Furthermore, as mentioned above the diamond cell structure lends itself to the typical urban deployment scenario as shown in Figure 1. Finally, the diamond cell shape also allows wrapping, as described in [4] for hexagonal shape, to be used so that unbiased performance results can be obtained for all simulated users.

Observation 2: Arrays built from printed planar patch elements will be gain limited for larger steering angles, hence a smaller viewing angle per cell would likely be prefered.

Figure 1. Madrid Grid Outdoor Layout [3]

Figure 2. Radiation Pattern for Patch Antenna

# 3	Diamond Shaped Cell Layout

For the more familiar hexagonal shaped cell we start with one base station (site) and create three sectors, each represented by a hexagon as shown in Figure 3 by the site labelled 1. In this case we would have three separate antenna arrays, each facing at angles 0, 120, and 240 degrees respectively. We can then begin to tessellate this shape by translating and rotating by 60 degrees to obtain six additional sites. This is referred to as a 7-site cluster and is the simplest cluster size commonly used for system analysis. This process of translating and rotating can be repeated to obtain the next common cluster size, which is a 19-site cluster. Normally, the inter-site distance (ISD) is specified as the design parameter. In the hexagonal case this ISD is common across the entire cluster and can be written as:

ISD=3S

where S represents the hexagon side length.

Wraparound can be supported by creating copies of the original 7/19-site cluster as described in [4]. After appropriate wraparound processing the deployment scenario can be simulated to obtain unbiased performance results for all UEs located in the original centre cluster.

A corresponding deployment scenario can be constructed using instead a diamond shaped cell, which is also shown in Figure 3. Starting again with the centre site we now create four sectors, each represented by a diamond. In this case we would have four separate antenna arrays, each facing at angles 0, 90, 180, and 270 degrees respectively. Next we tessellate this shape, similarly to what was done in the hexagonal case but instead rotating by 45 degrees so that we create an additional 8 sites. In this case we have a 9-site cluster, which is the simplest cluster size using diamond shape cells. The next larger size would be a 25-site cluster. It’s important to point at that for the diamond shaped layout there are multiple ISD values. However since we desire symmetrical coverage per cell, we further propose to use only the special case of a square. In this case there are only two ISD values, referred to in Figure 3 as ISDmax and ISDmin. From simple geometry the following relationships exists:

ISDmin=2S
ISDmax=2IDSmin= 22S

where S represents the side length of the diamond (square) cells.

Finally, wraparound is similarly supported so that unbiased performance results can also be obtained for a diamond cell shape layout.

Figure 3. Basic Hexagonal and Diamond Shaped Cluster

# 4	Conclusion

In this contribution, we discussed the cell layout for typical urban micro scenario with below rooftop base stations. From the discussions and obersvations, we propose the following:

Proposal: The diamond shaped cell layout instead of hexagonal shaped is considered for the deployment scenarios with below rooftop base stations for above 6 GHz depolyement scenarios.

The justifications specified are as follows:

- In certain scenarios like street canyon scenario with below rooftop base stations, the diamond shaped cells are a better approximate deployment.

  - This is even more appropariate for above 6 GHz frequency band.

  - This fits well if the intention is to provide coverage along the four cardinal directions in grid-like urban areas.

- Arrays built from printed planar patch elements will be gain limited for larger steering angles, hence a smaller viewing angle per cell would likely be prefered.

# References

## 3GPP, RP-152212, Samsung, “Report of RAN email discussion about >6GHz channel modelling,” Dec. 2015. http://www.3gpp.org/ftp/TSG_RAN/TSG_RAN/TSGR_70/Docs/RP-152212.zip

## Goldsmith, A.J.; Greenstein, L.J., "A measurement-based model for predicting coverage areas of urban microcells," in Selected Areas in Communications, IEEE Journal on , vol.11, no.7, pp.1013-1023, Sep 1993. http://ieeexplore.ieee.org/stamp/stamp.jsp?tp=&arnumber=233214&isnumber=6028&tag=1

## Jose F. Monserrat, Michael Fallgren, “ICT-317669-METIS/D6.1 METIS simulation guidelines,” Tech. Rep., 2013. https://www.metis2020.com/wp-content/uploads/deliverables/METIS_D6.1_v1.pdf

## 3GPP, R1-140842, Ericsson, “Discussion on Wrapping Methodology,” Feb. 2014. http://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_76/Docs/R1-140842.zip

## Neill Tucker, “Array Calc V2.5 User Guide”, Oct. 2010. http://www.activefrance.com/Antennas/User_Guide%20v2_5.pdf