3GPP TSG-RAN WG1 Meeting #84 	R1-160978

# St Julian’s, Malta, 15-19 Feb, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT]CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR |  | rev | - | Current version: | 13.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Averages CRS/CSI-RS measurements from multiple subframes in LAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-core |  |  |  |  |  |  | Date: |  | 2016-2-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Clarification of UE averaging CRS/CSI-RS measurements from multiple subframes: e.g. two downlink transmission bursts with the second transmission burst starting with an initial partial subframe can be mistaken for one downlink transmission burst. For interference measurement based on CSI-IM: partial subframes are not used per agreements from RAN1 #83. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify text to capture the case with a partial subframe between two downlink transmission bursts |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Erroneous CQI calculation at UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR has isolated impact |  |  |  |  |  |  |  |  |

### <Unchanged parts are omitted>

### 7.2.3	Channel Quality Indicator (CQI) definition

### <Unchanged parts are omitted>

For a LAA Scell,

-	for channel measurements, if the UE averages CRS/CSI-RS measurements from multiple subframes

-	the UE should not average CSI-RS measurement in subframe n1 with CSI-RS measurement in subframe n2, if any OFDM symbol between subframe n1 and subframe n2 is not occupied.

-	the UE should not average CRS measurement in subframe n1 with CRS measurement in subframe n2, if any OFDM symbol of the second slot of subframe n1 or any OFDM symbol between subframe n1 and subframe n2 is not occupied.

-	for interference measurements based on CSI-IM, the UE shall derive the interference measurements for computing the CQI value based on only measurements in subframes  without any un-occupied OFDM symbol.

-     for interference measurements not based on CSI-IM, the UE shall derive the interference measurements for computing the CQI value based on only measurements in subframes  with occupied OFDM symbols.

### <Unchanged parts are omitted>