3GPP TSG RAN WG1 Meeting #84                                                                            R1-161016

St Julian’s, Malta, 15th - 19th February 2016

Agenda Item:	7.3.2.2.3

Source:	Lenovo

Title:	SA content for V2V

Document for:	Discussion and decision

# Introduction

During RAN#70 meeting, new V2V WI is approved [1]. This WI specifies LTE sidelink enhancements for V2V services, including the following objectives:

- To specify enhancement to sidelink physical layer structure necessary for V2V services [RAN1]

- To specify enhancement to sidelink synchronization procedure necessary for V2V services [RAN1, RAN2]

- Low priority is given to enhancements to Rel-12/13 SLSS-based synchronization.

- To identify what are necessary sidelink resource allocation enhancement option(s) among the ones captured in TR 36.885 for V2V services and specify the identified option(s) [RAN1, RAN2]

In the last RAN1#83 meeting, resource allocation enhancements are deemed necessary for PC5-based V2V. On scheduling assignment, the following principles are agreed:

  - Scheduling assignment

    - Each data transmission is scheduled by an SA. A UE knows at least time and frequency location of data transmission(s) after decoding the associated SA.

    - FFS the indication is implicit, explicit, or both

    - If SA and the associated data from a single transmitter is transmitted in different subframes:

    - FFS details

    - If SA and the associated data from a single transmitter are transmitted in the same subframe:

    - Q: Is it possible to support the case where data transmission in a subframe occurs without associated SA transmitted in the same subframe?

    - Alt 1: SA and Data are transmitted on separate physical channels (i.e., separated DFT precoding for SA and data):

      - RAN1 assumes that RAN4 will study the proper transmission characteristics (e.g, MPR) to support this.

      - FFS whether SA and data transmissions in the same subframe are always adjacent in the frequency domain.

      - In case of separate channels, study whether SA pool and data pool are orthogonal or can overlap.

    - Alt 2: A single DFT precoding applies to SA and data transmitted in the same subframe.

      - The whole bandwidth is divided into one or multiple sub-channels.

      - The transmission bandwidth of SA/data is fixed to the bandwidth of a single sub-channel.

    - Alt 3: SA and data are TDMed within one subframe.

      - The transmission bandwidth of SA is fixed.

    - Study the number of transmissions of a given TB

    - Study the number of transmissions of a given SA

    - FFS whether a single SA may schedule multiple TBs

    - FFS whether the time/frequency resources of a given SA is independent of the time/frequency resources of the associated data

In this contribution, we discuss the SA contents, corresponding to the different SA/Data association mechanism (TDM vs. FDM) in our companion contribution [2] and [3].

# Enhancements of existing SA contents for sidelink transmission

Existing SA is for the scheduling of PSSCH in Rel-12/13 for sidelink transmission. The following information is transmitted in the SA:

- Frequency hopping flag – 1 bit as defined in section 14.1.1 of  TS 36.213.

- Resource block assignment and hopping resource allocation –  bits

- For PSSCH hopping:

- NSL_hop MSB bits are used to obtain the value of  as indicated in section 8.4 of TS 36.213

-  bits provide the resource allocation in the subframe

- For non-hopping PSSCH:

-  bits provide the resource allocation in the subframe as defined in section 8.1.1 of TS 36.213

- Time resource pattern – 7 bits as defined in section 14.1.1 of TS 36.213.

- Modulation and coding scheme – 5 bits as defined in section 14.1.1 of TS 36.213

- Timing advance indication – 11 bits as defined in section 14.2.1 of TS 36.213

- Group destination ID – 8 bits as defined by higher layers

In order to indicate the transmission of PC5 based V2V communication more appropriately, it is necessary to re-design existing SA content with the following enhancements:

- Resource assignment: Legacy resource block assignment and hopping resource allocation field is used to indicate the frequency resource allocation in whole system bandwidth. For PC5 based V2V communication, due to the frequently generated message (e.g. 190 bytes or 300 bytes per 10Hz), FDM-based SA and data pool with the available bandwidth divided into multiple transmission resource units (TRU) can be used to meet the latency requirements of V2V message [2]. Here, the transmission resource unit can include a fixed number of PRBs in frequency domain. Hence, fewer bits for resource block assignment are needed. Especially, when the structure and size of TRU are (pre)configured and SA and associated data are always transmitted in same subframe and contiguous in frequency domain, the bit width of resource block assignment can be further reduced to 0. Meanwhile, the bit of frequency hopping flag can also be omitted.

-Time resource pattern: The time resource pattern is used to indicate the subframes for associated data transmission in Rel-12/13 PC5 communication. For PC5 based V2V communication, in order to meet more stringent service requirement and higher user density, additional randomization of T-RPT patterns or procedure should be designed if TDM based SA/ data association is adopted. For PC5 based V2V communication,  if only the FDM based SA/data resource pool is supported and SA and its associated data are always transmitted in same subframe , the indication bits of T-RPT can be removed.

- Modulation and coding scheme: There is no need to modify the field of MCS.

-Timing advance indication:  TA is used to indicate the transmission timing difference between SA and its associated data in Rel-12/13 PC5 communication when the Tx UE works in mode 1. If FDM based SA/data association is adopted for PC5 based V2V communication, there will be no timing difference between the SA and its associated data. Therefore, whether the TA field is needed in PC5 based V2V communication is dependent on the resource pool design of SA and data.

- Group destination ID: The group destination ID is included in SA for Rel-12/13 D2D, which allows UE to only decode the interested message. For Rel-12/13 D2D, this group ID is directly from RAN2. For PC5 based V2V communication, such a group destination ID may be useful.

Proposal 1: Existing SA contents should be enhanced by considering new SA/Data association mechanism.

# Additional contents in SA

In the above section, the existing contents in SA are analyzed based on possible enhancements for PC5 based V2V communication. Some new design for PC5 based V2V communication may introduce additional contents into SA, e.g.

-NDI/RV indication: In Rel-12/13 PC5 communication, the T-RPT can indicate the initial transmission and the subsequent retransmission in time domain for TDM based SA/data association within one SA period, and different RV versions for one TB are transmitted in a fixed order . Therefore, RV indication is not needed in the Rel-12/13 D2D SA. Similarly, NDI is also not needed in Rel-12/13 D2D SA when T-RPT works within one SA period. For PC5 based V2V communication, in order to meet the latency requirement of typical traffic (e.g.100ms) and event triggered traffic, existing mechanism of TDM-based SA/data resource pool needs to be improved. For example, if the FDM based SA/data association is adopted for PC5 based V2V communication, the definition of SA period and T-RPT may be not necessary. In this case, to ensure the mechanism of multiple transmissions of one TB with different RVs, NDI/RV indication is needed in SA.

Proposal 2: NDI/RV indication is needed in SA.

# Conclusions

In this contribution, we focus on SA contents corresponding to the different SA/Data association mechanism (TDM vs. FDM). The following proposals are provided:

Proposal 1: Existing SA contents should be enhanced by considering new SA/Data association mechanism.

Proposal 2: NDI/RV indication is needed in SA.

# References

- RP-152293, New WI proposal: Support for V2V services based on LTE sidelink

R1-161015, “Discussion on resource pool design for V2V,” Lenovo

R1-161014, “Mechanisms for V2V resource allocation,” Lenovo