3GPP TSG-RAN WG1 Meeting #84	R1-161089

St George's Bay, Malta, 15th – 19th November 2016

Agenda item:		7.1.5

Source:	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

Title:	Number of Repetitions for msg4 HARQ on PUCCH

Document for:		Discussion and Decision

# Introduction

In this document we discuss how the LC/CE UE determines how many repetitions to use to send the PUCCH HARQ feedback for message 4.

# Discussion

The table below summarises our understanding of how the UE/network determines the number of repetitions for each of the various messages that form the random access procedure. At present, we feel there is no clear description of how the UE determines the number of repetitions it should apply for PUCCH HARQ feedback in support of message 4.

| Message | Number of repetitions |
| --- | --- |
| Message 1: PRACH Preamble | SIB2 numRepetitionPerPreambleAttempt-r13 |
| Message 2: PRACH Response MPDCCH | Different repetitions upto a max. defined by SIB2 mpdcch-NumRepetition |
| Message 2: PRACH Response PDSCH | Msg2 MPDCCH DCI Repetition Number* *2/3 bit indicator to select which #transmissions for PDSCH from a subset defined by SIB2 “pdsch-maxNumRepetitionCEmodeA/B” |
| Message 3: RRC Connection Request PUSCH | RAR UL Grant Number of repetitions for Msg3* *2/3 bit indicator to select which #transmissions for PUSCH from a subset signaled in SIB2 by “pusch-maxNumRepetitionCEmodeA/B” |
| Message 3: RRC Connection Setup / HARQ Feedback MPDCCH | Same as Message 2 PRACH Response MPDCCH |
| Message 4: RRC Connection Setup PDSCH | Msg3 MPDCCH DCI Repetition Number* *2/3 bit indicator to select which #transmissions for PDSCH from a subset defined by SIB2 “pdsch-maxNumRepetitionCEmodeA/B” |
| Message 4: HARQ Feedback PUCCH | Not yet defined |

Table 1: #Transmissions for different Random Access Message

The simplest method to resolve this issue would be to reuse the number of repetitions used for message 3 (sent on the PUSCH) for the Message 4 HARQ feedback (sent on the PUCCH).  However given differences in the PUSCH and PUCCH link budgets, it is not clear whether an additional offset (e.g. fixed the next lowest level of repetition) is required to modify the number of repetitions for message 3 on PUSCH to obtain the number of repetitions to use for message 4 HARQ feedback on the PUCCH.

Proposal 1:  RAN1 is requested to clarify how the LC/CE UE determines how many transmissions to use to support the PUCCH HARQ feedback for message 4.  2 options are presented below:

Option A: 	The number of repetitions used by the LC/CE UE to send Message 4 HARQ feedback on the PUCCH, is equal to the number of repetitions used to send message 3 on the PUSCH.

Option B: 	The number of repetitions used by the LC/CE UE to send Message 4 HARQ feedback on the PUCCH, is offset by a fixed relationship (e.g. the next lowest repetition level) from the number of repetitions used to send message 3 on the PUSCH.

# Conclusions

In this document we discussed the need to clarify how the LC/CE UE determines how many repetitions to use to send the PUCCH HARQ feedback for message 4.   Below is our proposal for RAN1 to consider.

Proposal 1:  RAN1 is requested to clarify how the LC/CE UE determines how many transmissions to use to support the PUCCH HARQ feedback for message 4.  2 options are presented below:

Option A: 	The number of repetitions used by the LC/CE UE to send Message 4 HARQ feedback on the PUCCH, is equal to the number of repetitions used to send message 3 on the PUSCH.

Option B: 	The number of repetitions used by the LC/CE UE to send Message 4 HARQ feedback on the PUCCH, is offset by a fixed relationship (e.g. the next lowest repetition level) from the number of repetitions used to send message 3 on the PUSCH.

# Reference

- R1-157918,  TS36.211 CR0207 rev5, “Introduction of LC/CE MTC”

- R1-157916,  TS36.212 CR0181 rev1, “Introduction of Rel-13 features of eMTC in 36.212”

- R1-157926,  TS36.213 CR0540 rev4, “Introduction of further LTE Physical Layer Enhancements for MTC”

- R2-157138,  TS36.331 CR1944 rev3, “Addition of low complexity UEs and coverage enhancement features”

- R1-157733, 	RAN1 agreements for Rel-13 eMTC sorted and edited by topic, WI rapporteur (Ericsson)