3GPP TSG RAN WG1 Meeting #84                                                	R1-161167

Malta, 15th - 19nd February 2016

Source: 	Ericsson

Title:	System level evaluation results for TTI shortening techniques

Agenda Item:	7.3.4.1

Document for:	Discussion and Decision

# Introduction

The concept of TTI shortening [1] involves many aspects of the physical layer and MAC design and is therefore challenging to evaluate in a uniform way. In previous RAN2 and RAN1 meetings the system level gains of TTI shortening have been discussed with varying setup and results. During the last meeting a group of companies proposed a common WF for system level evaluation [2], and in this paper we present results for simulations according to the proposed setup.

# Discussion

## Simulator setup

### TBS selection

Based on the current TBS tables for UL and DL [3] the TBS for short TTI is found from the number of data symbols (REs), with the assumption of 144 REs / subband in the current tables, in a procedure described in [4] and [5]. The REs for CRS, DMRS, and SRS have been subtracted from the final number. The number of subbands in the current table with the closest number of data symbols is used to scale the TBS entries into the used number of data symbols, taking CRC into account and rounding off to closest octet.

### Scenario parameters

| Parameter | Value |
| --- | --- |
| Number of sites, sectors per site | 7, 3 |
| Inter-site distance | 500m |
| UE dropping | Random uniform, 80% indoor |
| UE speed | 0 (no mobility) |
| UE Multipath speed | 3 km/h |
| Frequency, duplex | 2 GHz, FDD |
| Bandwidth | 10 MHz |
| TX power | 46 dBm (eNB), 0.25 dBm (UE) |
| Antenna heights | 25m (eNB), 1.5m (UE) |
| N TX antennas x M RX antennas | 2x2 (eNB), 1x2 (UE) |
| Transmission mode, MIMO | TM4, 2x2 (DL), 1x2 (UL) |
| Antenna pattern | 3GPP TR36.819 |
| Noise figure | 5dB (eNB), 9dB (UE) |
| FTP download file size | 100Kbits, 100kB, 500KB, 1MB |
| FTP model | 1 |
| Fast Fading Model | ITU Uma TR36.819 |
| Pathloss Model | ITU Uma TR36.814 |
| TCP Configuration | Slow Start: Exponential default Congestion Avoidance: Reno Initial Window Size: 3 |

### System parameters

| Parameter | Value |
| --- | --- |
| Number of PDCCH symbols | 2 |
| CQI report delay | 6ms |
| Link adaptation | According to TBS selection from Section 2.1.1; Outer-loop correction (Target BLER 10%) |
| Core, transport, and internet delay | 0ms, 10ms |
| RLC AM max ReTX threshold | 32 |
| Scheduler algorithm | Proportional fair |
| UL access | SR-based |
| UL retransmissions | Non-adaptive |

| Parameter | 14 symbols TTI | 7 symbols TTI | 2 symbols TTI |
| --- | --- | --- | --- |
| SRS period | 10ms / 10 TTI | 10ms / 20 TTI | 10ms / 70 TTI |
| CQI period | 5ms / 5 TTI | 5ms / 10 TTI | 5ms / 35 TTI |
| SR period | 1ms / 1 TTI | 0.5ms / 1 TTI | 1/7ms / 1 TTI |
| sPDCCH size | 0 | 72 RE | 72 RE |
| PUCCH TTI | 14 symbols | 7 symbols | 2 symbols |
| PUCCH allocation | 1 PRB | 5 PRB | 5 PRB |
| UL DMRS symbols | 2 | 1 | 1 |
| TBS selection | PRB based | RE based | RE based |
| UL grant to data delay | 4ms / 4 TTI | 2ms / 4 TTI | 4/7ms / 4 TTI |
| SR to UL grant delay | 4ms / 4 TTI | 2ms / 4 TTI | 4/7ms / 4 TTI |
| UL HARQ delay | 4ms / 4 TTI | 2ms / 4 TTI | 4/7ms / 4 TTI |
| DL HARQ delay | 4ms / 4 TTI | 2ms / 4 TTI | 4/7ms / 4 TTI |
| DL HARQ to reTx delay | 4ms / 4 TTI | 2ms / 4 TTI | 4/7ms / 4 TTI |
| Maximum number of STTI bands in UL/DL | No limit | 4 | 4 |
| DL sTTI band minimum size | 1 PRB | 12 PRB | 12 PRB |
| UL sTTI band minimum size | 1 PRB | 12 PRB | 12 PRB |

The performance evaluation of short TTI is based on FTP downloads over TCP for file sizes between 100Kbits and 1MB, where the UEs arrive to the system at different rates per second. Each UE downloads a file and after the transfer is done, the UE disappears from the system (FTP model 1).

The user arrival rates generate different system loads related to the percentage of resource utilization (RU).

For the evaluation, different TTI lengths are considered: 14 symbols (legacy), 7 symbols and 2 symbols. The FTP object bit rate, download delay and absolute gains are shown for different loads (5%, 20%, 40% and 60% RU) comparing the short TTI techniques to the legacy.

Likewise, two different transport network delays are considered: 10ms and 0ms. In Section 2.2, the results for 1MB and 100KB file sizes with a 10ms transport network delay are presented. All the additional results with a transport network delay of 10ms, 0s and all the different file sizes are presented in the Annex.

## Results

In this section we present selected simulation results for 100kB and 1MB file size transmissions and 10ms transport network delay. More results for several other file sizes and different transport network delay are provided in the Annex

### FTP Transfer: 100kB file size (10ms transport network delay)

For small file size transmissions like the 100kB file download, the FTP transfer is done most of the time (or 100% of time) during the TCP slow start phase, i.e. the performance is not limited by the achievable throughput, but by TCP congestion window. Since the congestion window grows exponentially during the slowstart phase based on reception of TCP ACKs, the TCP RTT, i.e. the latency of the system, limits the overall performance. Therefore, as shown in Figure , short TTI provides significant gains by reducing the TCP RTT.

For the 100KB results the 2 symbols TTI provides the highest gains compared to the legacy performance. Assuming a 10ms transport network delay, the gain of reducing the radio network delay with 2 symbols TTI at low load for the median FTP object bit rate is about 70% (and ~50% for medium load), which corresponds to a download delay gain of 40%. Likewise, 7 symbols TTI provides gains compared to the legacy, those gains however are lower compared to 2 symbols TTI (about 40% less). When a 0ms transport network delay is considered, as shown in the Annex, the 2 symbols TTI may provide object bit rate gains up to 400%, which represents 80% gain in the download delay.

- Reducing TTI length down to 2 symbols provides the largest latency reduction for small file size transmissions

- Proposal 1:  A 2 OFDM symbol long TTI for DL PDSCH is recommended to be supported

Table 1	FTP bitrate and latency statistics for different loads and different TTI lengths (100kB file)

| Performance Indicator |  | Very low load |  |  | Low load |  |  |  | Medium load |  |  | High load |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s |  | 14s | 7s | 2s | 14s | 7s | 2s |
| FTP DL OBR CDF [Mbit/s] | Mean | 2.8 | 3.8 | 4.7 | 2.6 | 3.3 | 4.1 |  | 2.4 | 2.6 | 3.3 | 2.1 | 2.1 | 2.5 |
|  | 95% | 3.1 | 4.1 | 5.2 | 2.9 | 4.0 | 5.1 |  | 2.9 | 3.8 | 4.9 | 2.8 | 3.7 | 4.6 |
|  | 50% | 2.8 | 3.9 | 4.9 | 2.6 | 3.5 | 4.4 |  | 2.5 | 2.7 | 3.4 | 2.3 | 2.0 | 2.3 |
|  | 5% | 2.5 | 3.1 | 3.3 | 2.2 | 2.0 | 2.1 |  | 1.6 | 0.9 | 1.1 | 0.9 | 0.6 | 0.6 |
| Gain [%] | Mean | 0.0 | 34.2 | 67.4 | 0.0 | 27.9 | 57.2 |  | 0.0 | 7.1 | 34.3 | 0.0 | -4.1 | 15.6 |
|  | 95% | 0.0 | 31.9 | 67.7 | 0.0 | 36.6 | 74.1 |  | 0.0 | 32.8 | 69.9 | 0.0 | 29.5 | 64.3 |
|  | 50% | 0.0 | 37.7 | 75.1 | 0.0 | 33.5 | 67.6 |  | 0.0 | 7.9 | 36.2 | 0.0 | -14.4 | 1.3 |
|  | 5% | 0.0 | 25.5 | 34.6 | 0.0 | -8.9 | -6.2 |  | 0.0 | -44.5 | -28.3 | 0.0 | -38.9 | -35.9 |
| RU (%) |  | 5.0 | 7.8 | 6.0 | 20.0 | 30.4 | 24.5 |  | 40.0 | 57.2 | 45.9 | 60.0 | 74.8 | 63.7 |
| Performance Indicator |  | Very low load |  |  | Low load |  |  |  | Medium load |  |  | High load |  |  |
|  |  | 14s | 7s | 2s | 14s | 7s |  | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| DL Delay CDF [s] | Mean | 0.29 | 0.22 | 0.19 | 0.31 | 0.26 |  | 0.23 | 0.35 | 0.39 | 0.31 | 0.43 | 0.55 | 0.48 |
|  | 5% | 0.26 | 0.20 | 0.15 | 0.27 | 0.20 |  | 0.16 | 0.28 | 0.21 | 0.16 | 0.28 | 0.22 | 0.17 |
|  | 50% | 0.29 | 0.21 | 0.16 | 0.30 | 0.23 |  | 0.18 | 0.32 | 0.30 | 0.23 | 0.35 | 0.40 | 0.34 |
|  | 95% | 0.32 | 0.26 | 0.24 | 0.36 | 0.40 |  | 0.39 | 0.51 | 0.94 | 0.73 | 0.87 | 1.42 | 1.35 |
| Gain [%] | Mean | 0.0 | 22.7 | 32.4 | 0.0 | 16.6 |  | 27.2 | 0.0 | -12.6 | 9.2 | 0.0 | -28.6 | -12.6 |
|  | 5% | 0.0 | 24.2 | 40.3 | 0.0 | 26.8 |  | 42.6 | 0.0 | 24.7 | 41.1 | 0.0 | 22.7 | 39.2 |
|  | 50% | 0.0 | 27.4 | 42.9 | 0.0 | 25.1 |  | 40.3 | 0.0 | 6.3 | 26.1 | 0.0 | -16.8 | 1.3 |
|  | 95% | 0.0 | 20.3 | 25.4 | 0.0 | -10.2 |  | -7.7 | 0.0 | -81.6 | -39.9 | 0.0 | -66.0 | -58.6 |
| RU (%) |  | 5.0 | 7.8 | 6.0 | 20.0 | 30.4 |  | 24.5 | 40.0 | 57.2 | 45.9 | 60.0 | 74.8 | 63.7 |

Figure 1	 Median FTP Object Bitrate and Gain (100kB file size)

#### HARQ RTT and Processing Delay

The HARQ RTT defined for the results depicted in Figure 1 is equal to 8 TTI. Therefore, in order to further analyze the performance and gains of TTI shortening, different HARQ RTT (different processing delays) are considered and presented in Figure 2. The results show that the highest gain is obtained for all the cases with 2 symbols TTI, providing still a gain above 40% in the median FTP object bit rate for the highest considered HARQ RTT (22 HARQ RTT which comprises around 1.5ms of processing delay). In Table 2, the respective HARQ RTT and processing delays are described.

- The processing delay has a significant impact on the gains of TTI shortening, the higher the processing delay, the lower the gain

Figure 2	 Median FTP Object Bitrate and Gain (100kB file size)

Table 2	HARQ RTT – Processing Delay

| TTI Length | HARQ RTT [No. of TTIs] | Processing Delay [ms] |
| --- | --- | --- |
| 14 symbols (Legacy) | 8 | 3 |
| 7 symbols | 14 | 3 |
| 7 symbols | 8 | 1.5 |
| 2 symbols | 22 | 1.43 |
| 2 symbols | 12 | 0.71 |
| 2 symbols | 8 | 0.43 |

### FTP Transfer: 1MB file size (10ms transport network delay)

In this section we analyze the performance of shortened TTI with FTP downloads of 1MB file and a HARQ RTT of 8 TTI. For large file size transmissions, the FTP transfer is on average only ~10-20% of the transfer during the TCP slow start phase where the user performance is latency limited. Afterwards, the TCP congestion window is larger than the achievable throughput and the user performance is thus throughput-limited.

For users that can achieve a higher throughput the TCP slowstart phase naturally represents a larger percentage of the overall download time, while for users with lower throughput the TCP slowstart phase represents a small fraction of the overall download time. Therefore, users with higher achievable throughput will benefit more from reduced latency than users with lower throughput (compare 5th and 95th percentile results). For most users significant gains can be achieved at low load with short TTI, when there are enough available radio resources for the UEs. Larger gains are possible with 2 symbols TTI than with 7 symbols TTIs. As expected, the best users in the system (95th percentile) experience the largest improvement in FTP bitrate, around 60% with 2 symbols TTI, which represents an important reduction in the download time. However, the worst users whose performance is throughput-limited do not benefit from the latency reduction offered by short TTIs at all and see a degraded performance due to the larger overhead introduced by short TTIs. Therefore, these users should be served with legacy TTIs (however, in our simulation, we assumed all users to use the same TTI length). This shows the importance of designing short TTI operation so that it can coexist with legacy TTI operation.

- For large file size transmissions, 2 symbols TTI provides significant latency gains for the cell center users in the system

- Short TTI should be able to coexist with legacy TTI operation

- Proposal 2:  Shortened TTIs are FDM within the system bandwidth together with 1 ms TTI

- Table 3 	FTP bitrate and latency statistics for different loads and different TTI lengths (1MB file)

| Performance Indicator |  | Very low load |  |  | Low load |  |  | Medium load |  |  | High load |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| FTP DL OBR CDF [Mbit/s] | Mean | 12.7 | 14.4 | 16.7 | 10.4 | 10.5 | 10.3 | 8.0 | 7.0 | 7.7 | 5.9 | 5.0 | 5.2 |
|  | 95% | 15.1 | 18.2 | 24.2 | 14.6 | 17.9 | 22.7 | 13.7 | 15.9 | 19.7 | 12.6 | 13.1 | 15.7 |
|  | 50% | 13.1 | 15.3 | 17.3 | 10.9 | 10.4 | 9.4 | 8.1 | 6.0 | 6.1 | 5.0 | 3.7 | 3.6 |
|  | 5% | 8.4 | 7.3 | 6.0 | 4.3 | 3.3 | 1.9 | 2.1 | 1.3 | 1.3 | 1.1 | 0.9 | 0.8 |
| Gain [%] | Mean | 0.0 | 13.7 | 32.1 | 0.0 | 0.9 | -1.4 | 0.0 | -12.4 | -4.0 | 0.0 | -14.6 | -11.8 |
|  | 95% | 0.0 | 20.6 | 60.7 | 0.0 | 22.6 | 54.8 | 0.0 | 16.0 | 43.5 | 0.0 | 3.4 | 24.2 |
|  | 50% | 0.0 | 15.9 | 31.2 | 0.0 | -5.1 | -13.8 | 0.0 | -27.2 | -26.5 | 0.0 | -25.6 | -27.8 |
|  | 5% | 0.0 | -13.4 | -28.7 | 0.0 | -24.9 | -56.7 | 0.0 | -37.9 | -39.5 | 0.0 | -22.6 | -32.3 |
| RU (%) |  | 5.0 | 7.1 | 7.9 | 20.0 | 26.6 | 31.6 | 40.0 | 51.6 | 46.1 | 60.0 | 70.4 | 63.4 |
| Performance Indicator |  | Very low load |  |  | Low load |  |  | Medium load |  |  | High load |  |  |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| DL Delay CDF [s] | Mean | 0.66 | 0.63 | 0.64 | 0.93 | 1.06 | 1.47 | 1.53 | 2.21 | 2.19 | 2.48 | 3.12 | 3.33 |
|  | 5% | 0.53 | 0.44 | 0.33 | 0.55 | 0.45 | 0.35 | 0.58 | 0.50 | 0.41 | 0.63 | 0.61 | 0.51 |
|  | 50% | 0.61 | 0.53 | 0.47 | 0.74 | 0.78 | 0.86 | 1.02 | 1.45 | 1.44 | 1.63 | 2.18 | 2.25 |
|  | 95% | 0.96 | 1.13 | 1.38 | 1.91 | 2.54 | 4.39 | 4.12 | 6.56 | 6.68 | 7.12 | 9.16 | 10.48 |
| Gain [%] | Mean | 0.0 | 5.4 | 3.9 | 0.0 | -14.1 | -57.5 | 0.0 | -44.8 | -43.0 | 0.0 | -26.2 | -35.1 |
|  | 5% | 0.0 | 17.1 | 37.8 | 0.0 | 18.4 | 35.4 | 0.0 | 13.7 | 30.1 | 0.0 | 3.2 | 19.4 |
|  | 50% | 0.0 | 13.4 | 23.7 | 0.0 | -5.5 | -16.5 | 0.0 | -39.4 | -38.4 | 0.0 | -34.4 | -38.7 |
|  | 95% | 0.0 | -16.5 | -41.5 | 0.0 | -33.2 | -131.2 | 0.0 | -61.4 | -66.2 | 0.0 | -29.6 | -48.7 |
| RU (%) |  | 5.0 | 7.1 | 7.9 | 20.0 | 26.6 | 31.6 | 40.0 | 51.6 | 46.1 | 60.0 | 70.4 | 63.4 |

Figure 3	Median and 95th FTP Object Bitrate (1MB file)

### General trend

As seen in the previous sections, the largest gains can be observed at very low load. Figure 4 summarizes the trend of the gain in the mean user FTP bitrate at this load level for different sizes of the file to download and for different transport delays. The detailed tables with all metrics can be found in the Annex.

The technology potential of TTI shortening is substantial when 2 symbol TTI is supported and no transport delay exists. Figure 4 shows gains higher than 350% in that case with very small file size. The gains reduce as the file size increases above 500kB file size since part of the file download occurs after the slowstart where the latency matters less than the achievable throughput. Much smaller gains are achieved with 7 symbol TTI in this ideal scenario.

When a transport delay of 10ms is assumed, the gains in mean user FTP bitrate with 2 and 7 symbol TTI are reduced for all file sizes, but remain at a high level. Overall, the trend remains the same: gains are higher for smaller file sizes and with 2 symbol TTI rather than 7 symbol TTI.

Figure 4	Gain in mean user FTP bitrate at very low load [%]

# Summary

In this contribution we provided simulation results of FTP download with different file sizes and transport network delays. The above discussion is summarized with the following observations and proposals:

Observations:

- Reducing TTI length down to 2 symbols provides the largest latency reduction for small file size transmissions

- The processing delay has a significant impact on the gains of TTI shortening, the higher the processing delay, the lower the gain

- For large file size transmissions, 2 symbols TTI provides significant latency gains for the cell center users in the system

- Short TTI should be able to coexist with legacy TTI operation

- Proposal 1:  A 2 OFDM symbol long TTI for DL PDSCH is recommended to be supported

- Proposal 2:  Shortened TTIs are FDM within the system bandwidth together with 1 ms TTI

# References

R1-157146, Overview of TTI shortening, Ericsson, RAN1#83, November 2015.

R1-157806, WF on evaluation methodology for latency reduction, RAN1#83, November 2015.

3GPP 36.213 Table 7.1.7.2.1-1.

R1-160934, Physical layer aspects of short TTI for downlink transmissions, RAN1#84, February 2016.

R1-160939, Physical layer aspects of short TTI for uplink transmissions, RAN1#84, February 2016.

# Annex

## Further results: 10ms transport network delay

### FTP Transfer: 500 kB file size

| Performance Indicator |  | 14s Load: 5% RU |  |  | 14s Load: 20% RU |  |  | 14s Load: 40% RU |  |  | 14s Load: 60% RU |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| FTP DL OBR CDF [Mbit/s] | Mean | 8.0 | 10.1 | 12.2 | 7.0 | 7.9 | 9.1 | 6.0 | 5.8 | 6.4 | 4.6 | 4.0 | 4.3 |
|  | 95% | 9.1 | 11.9 | 15.6 | 8.7 | 11.6 | 15.1 | 8.4 | 11.0 | 13.9 | 8.1 | 10.0 | 11.9 |
|  | 50% | 8.2 | 10.7 | 13.3 | 7.4 | 8.4 | 9.2 | 6.4 | 5.7 | 5.8 | 4.5 | 3.2 | 3.0 |
|  | 5% | 6.5 | 6.4 | 5.1 | 4.0 | 2.8 | 2.4 | 2.1 | 1.4 | 1.3 | 1.1 | 0.8 | 0.7 |
| Gain [%] | Mean | 0.0 | 25.5 | 52.5 | 0.0 | 13.0 | 29.5 | 0.0 | -2.9 | 7.1 | 0.0 | -12.4 | -6.4 |
|  | 95% | 0.0 | 30.8 | 71.6 | 0.0 | 33.9 | 74.0 | 0.0 | 31.2 | 64.9 | 0.0 | 23.5 | 47.1 |
|  | 50% | 0.0 | 31.3 | 62.5 | 0.0 | 12.9 | 23.3 | 0.0 | -11.8 | -10.5 | 0.0 | -30.9 | -34.9 |
|  | 5% | 0.0 | -1.0 | -21.4 | 0.0 | -31.5 | -41.5 | 0.0 | -34.2 | -37.6 | 0.0 | -31.0 | -34.0 |
| RU (%) |  | 5.0 | 7.1 | 7.1 | 20.0 | 26.6 | 24.9 | 40.0 | 52.5 | 47.2 | 60.0 | 72.0 | 63.6 |
| Performance Indicator |  | 14s Load: 5% RU |  |  | 14s Load: 20% RU |  |  | 14s Load: 40% RU |  |  | 14s Load: 60% RU |  |  |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| DL Delay CDF [s] | Mean | 0.51 | 0.43 | 0.42 | 0.61 | 0.64 | 0.66 | 0.85 | 1.12 | 1.15 | 1.37 | 1.90 | 1.96 |
|  | 5% | 0.44 | 0.34 | 0.26 | 0.46 | 0.34 | 0.27 | 0.48 | 0.36 | 0.29 | 0.49 | 0.40 | 0.34 |
|  | 50% | 0.49 | 0.37 | 0.30 | 0.54 | 0.48 | 0.44 | 0.63 | 0.72 | 0.72 | 0.90 | 1.32 | 1.42 |
|  | 95% | 0.62 | 0.63 | 0.80 | 1.00 | 1.51 | 1.74 | 2.08 | 3.15 | 3.29 | 3.77 | 5.46 | 5.72 |
| Gain [%] | Mean | 0.0 | 16.1 | 17.9 | 0.0 | -4.7 | -7.5 | 0.0 | -31.0 | -33.9 | 0.0 | -38.7 | -42.5 |
|  | 5% | 0.0 | 23.5 | 41.7 | 0.0 | 25.3 | 42.5 | 0.0 | 23.8 | 39.3 | 0.0 | 19.0 | 31.9 |
|  | 50% | 0.0 | 23.7 | 38.3 | 0.0 | 11.3 | 18.7 | 0.0 | -14.4 | -13.8 | 0.0 | -45.6 | -55.2 |
|  | 95% | 0.0 | -1.4 | -28.1 | 0.0 | -48.3 | -72.2 | 0.0 | -52.1 | -61.3 | 0.0 | -45.1 | -51.5 |
| RU (%) |  | 5.0 | 7.1 | 7.1 | 20.0 | 26.6 | 24.9 | 40.0 | 52.5 | 47.2 | 60.0 | 72.0 | 63.6 |

### FTP Transfer: 100 kbit file size

| Performance Indicator |  | 14s Load: 5% RU |  |  | 14s Load: 20% RU |  |  | 14s Load: 40% RU |  |  | 14s Load: > 40% RU |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| FTP DL OBR CDF [Mbit/s] | Mean | 1.0 | 1.3 | 1.6 | 0.9 | 1.2 | 1.5 | 0.9 | 0.9 | 1.2 | 0.9 | 0.9 | 1.2 |
|  | 95% | 1.1 | 1.4 | 1.7 | 1.1 | 1.3 | 1.7 | 1.0 | 1.3 | 1.6 | 1.0 | 1.3 | 1.6 |
|  | 50% | 1.0 | 1.3 | 1.7 | 0.9 | 1.2 | 1.5 | 0.9 | 1.0 | 1.3 | 0.9 | 1.0 | 1.3 |
|  | 5% | 0.8 | 1.1 | 1.4 | 0.7 | 0.8 | 1.0 | 0.7 | 0.4 | 0.6 | 0.7 | 0.4 | 0.6 |
| Gain [%] | Mean | 0.0 | 32.3 | 65.4 | 0.0 | 24.3 | 58.5 | 0.0 | 8.2 | 42.7 | 0.0 | 7.8 | 42.2 |
|  | 95% | 0.0 | 28.6 | 59.9 | 0.0 | 25.4 | 57.3 | 0.0 | 24.6 | 57.7 | 0.0 | 24.7 | 57.7 |
|  | 50% | 0.0 | 32.9 | 67.5 | 0.0 | 27.6 | 63.8 | 0.0 | 13.7 | 50.9 | 0.0 | 13.4 | 50.6 |
|  | 5% | 0.0 | 41.1 | 74.8 | 0.0 | 11.3 | 40.1 | 0.0 | -35.9 | -9.8 | 0.0 | -36.6 | -11.2 |
| RU (%) |  | 5.0 | 8.4 | 5.9 | 20.0 | 37.2 | 26.6 | 40.0 | 62.6 | 47.6 | 40.5 | 63.0 | 48.1 |
| Performance Indicator |  | 14s Load: 5% RU |  |  | 14s Load: 20% RU |  |  | 14s Load: 40% RU |  |  | 14s Load: > 40% RU |  |  |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| DL Delay CDF [s] | Mean | 0.10 | 0.08 | 0.07 | 0.11 | 0.09 | 0.07 | 0.12 | 0.12 | 0.09 | 0.12 | 0.12 | 0.09 |
|  | 5% | 0.09 | 0.07 | 0.06 | 0.09 | 0.07 | 0.06 | 0.10 | 0.08 | 0.06 | 0.10 | 0.08 | 0.06 |
|  | 50% | 0.10 | 0.08 | 0.06 | 0.11 | 0.08 | 0.07 | 0.11 | 0.10 | 0.08 | 0.11 | 0.10 | 0.08 |
|  | 95% | 0.12 | 0.09 | 0.07 | 0.13 | 0.12 | 0.10 | 0.15 | 0.23 | 0.17 | 0.15 | 0.24 | 0.17 |
| Gain [%] | Mean | 0.0 | 23.4 | 35.2 | 0.0 | 17.8 | 34.6 | 0.0 | -2.8 | 23.2 | 0.0 | -3.3 | 22.9 |
|  | 5% | 0.0 | 22.2 | 37.5 | 0.0 | 20.3 | 36.4 | 0.0 | 19.8 | 36.6 | 0.0 | 19.8 | 36.6 |
|  | 50% | 0.0 | 24.8 | 40.3 | 0.0 | 21.6 | 38.9 | 0.0 | 12.1 | 33.7 | 0.0 | 11.8 | 33.6 |
|  | 95% | 0.0 | 29.1 | 42.8 | 0.0 | 8.5 | 28.2 | 0.0 | -56.3 | -11.3 | 0.0 | -57.7 | -12.6 |
| RU (%) |  | 5.0 | 8.4 | 5.9 | 20.0 | 37.2 | 26.6 | 40.0 | 62.6 | 47.6 | 40.5 | 63.0 | 48.1 |

## Results: 0s transport network delay

### FTP Transfer: 1MB file size

| Performance Indicator |  | 14s Load: 5% RU |  |  | 14s Load: 20% RU |  |  | 14s Load: 40% RU |  |  | 14s Load: 60% RU |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| FTP DL OBR CDF [Mbit/s] | Mean | 18.5 | 21.5 | 28.3 | 14.3 | 14.5 | 16.9 | 10.5 | 9.1 | 10.0 | 7.3 | 6.1 | 6.5 |
|  | 95% | 24.4 | 29.5 | 48.9 | 23.6 | 28.8 | 46.2 | 20.9 | 25.0 | 33.1 | 17.5 | 18.5 | 21.0 |
|  | 50% | 19.0 | 22.6 | 26.4 | 14.3 | 13.2 | 13.3 | 9.7 | 6.9 | 6.5 | 5.6 | 4.1 | 3.9 |
|  | 5% | 10.3 | 9.3 | 6.5 | 5.0 | 3.3 | 2.5 | 2.3 | 1.4 | 1.3 | 1.2 | 0.9 | 0.8 |
| Gain [%] | Mean | 0.0 | 16.3 | 52.8 | 0.0 | 1.1 | 17.6 | 0.0 | -13.9 | -5.6 | 0.0 | -15.9 | -10.4 |
|  | 95% | 0.0 | 21.0 | 100.6 | 0.0 | 22.1 | 96.3 | 0.0 | 19.3 | 57.8 | 0.0 | 5.3 | 19.5 |
|  | 50% | 0.0 | 18.1 | 37.9 | 0.0 | -8.2 | -7.4 | 0.0 | -31.2 | -34.5 | 0.0 | -26.4 | -30.6 |
|  | 5% | 0.0 | -10.2 | -37.8 | 0.0 | -33.9 | -49.2 | 0.0 | -38.0 | -43.9 | 0.0 | -26.6 | -31.9 |
| RU (%) |  | 5.0 | 7.0 | 7.8 | 20.0 | 26.4 | 26.1 | 40.0 | 51.8 | 48.6 | 60.0 | 70.7 | 63.9 |
| Performance Indicator |  | 14s Load: 5% RU |  |  | 14s Load: 20% RU |  |  | 14s Load: 40% RU |  |  | 14s Load: 60% RU |  |  |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| DL Delay CDF [s] | Mean | 0.48 | 0.45 | 0.48 | 0.74 | 0.89 | 1.13 | 1.34 | 1.99 | 2.14 | 2.25 | 2.96 | 3.19 |
|  | 5% | 0.33 | 0.27 | 0.16 | 0.34 | 0.28 | 0.17 | 0.38 | 0.32 | 0.25 | 0.46 | 0.44 | 0.39 |
|  | 50% | 0.42 | 0.36 | 0.31 | 0.56 | 0.61 | 0.61 | 0.86 | 1.29 | 1.36 | 1.51 | 2.02 | 2.16 |
|  | 95% | 0.79 | 0.88 | 1.28 | 1.64 | 2.49 | 3.22 | 3.82 | 6.11 | 6.74 | 6.92 | 9.30 | 10.02 |
| Gain [%] | Mean | 0.0 | 5.3 | -0.1 | 0.0 | -19.5 | -51.7 | 0.0 | -48.3 | -59.2 | 0.0 | -33.4 | -43.6 |
|  | 5% | 0.0 | 17.4 | 50.2 | 0.0 | 18.1 | 49.0 | 0.0 | 16.1 | 36.2 | 0.0 | 4.8 | 16.2 |
|  | 50% | 0.0 | 14.8 | 26.9 | 0.0 | -9.1 | -8.3 | 0.0 | -47.3 | -54.9 | 0.0 | -36.3 | -44.3 |
|  | 95% | 0.0 | -11.6 | -61.5 | 0.0 | -51.5 | -97.0 | 0.0 | -61.5 | -78.7 | 0.0 | -39.4 | -49.7 |
| RU (%) |  | 5.0 | 7.0 | 7.8 | 20.0 | 26.4 | 26.1 | 40.0 | 51.8 | 48.6 | 60.0 | 70.7 | 63.9 |

### FTP Transfer: 100 kB file size

| Performance Indicator |  | 14s Load: 5% RU |  |  | 14s Load: 20% RU |  |  | 14s Load: 40% RU |  |  | 14s Load: 60% RU |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| FTP DL OBR CDF [Mbit/s] | Mean | 4.6 | 8.9 | 19.6 | 4.2 | 7.1 | 13.8 | 3.8 | 4.8 | 8.4 | 3.2 | 3.4 | 5.4 |
|  | 95% | 5.3 | 10.9 | 28.7 | 5.1 | 10.3 | 27.1 | 5.0 | 9.4 | 21.7 | 4.8 | 8.4 | 16.3 |
|  | 50% | 4.6 | 9.3 | 20.7 | 4.3 | 7.5 | 12.8 | 4.0 | 4.5 | 6.5 | 3.5 | 2.7 | 3.6 |
|  | 5% | 3.8 | 5.5 | 6.7 | 3.1 | 2.9 | 3.4 | 2.1 | 1.2 | 1.5 | 1.1 | 0.6 | 0.7 |
| Gain [%] | Mean | 0.0 | 92.9 | 322.8 | 0.0 | 67.7 | 225.3 | 0.0 | 25.4 | 120.0 | 0.0 | 3.7 | 65.5 |
|  | 95% | 0.0 | 104.7 | 439.9 | 0.0 | 102.2 | 433.2 | 0.0 | 88.2 | 335.7 | 0.0 | 74.1 | 237.4 |
|  | 50% | 0.0 | 101.2 | 344.2 | 0.0 | 76.1 | 199.3 | 0.0 | 12.0 | 63.3 | 0.0 | -21.6 | 4.8 |
|  | 5% | 0.0 | 44.5 | 74.9 | 0.0 | -8.2 | 6.9 | 0.0 | -45.1 | -28.7 | 0.0 | -42.0 | -32.6 |
| RU (%) |  | 5.0 | 7.8 | 5.7 | 20.0 | 28.7 | 21.6 | 40.0 | 56.5 | 43.4 | 60.0 | 76.2 | 63.2 |
| Performance Indicator |  | 14s Load: 5% RU |  |  | 14s Load: 20% RU |  |  | 14s Load: 40% RU |  |  | 14s Load: 60% RU |  |  |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| DL Delay CDF [s] | Mean | 0.18 | 0.10 | 0.07 | 0.20 | 0.14 | 0.10 | 0.23 | 0.27 | 0.19 | 0.31 | 0.44 | 0.35 |
|  | 5% | 0.15 | 0.07 | 0.03 | 0.16 | 0.08 | 0.03 | 0.16 | 0.09 | 0.04 | 0.17 | 0.10 | 0.05 |
|  | 50% | 0.17 | 0.09 | 0.04 | 0.19 | 0.11 | 0.06 | 0.20 | 0.19 | 0.13 | 0.23 | 0.30 | 0.22 |
|  | 95% | 0.21 | 0.15 | 0.12 | 0.26 | 0.28 | 0.25 | 0.40 | 0.75 | 0.57 | 0.75 | 1.28 | 1.11 |
| Gain [%] | Mean | 0.0 | 44.5 | 62.3 | 0.0 | 28.7 | 49.3 | 0.0 | -15.4 | 17.0 | 0.0 | -40.2 | -10.6 |
|  | 5% | 0.0 | 51.1 | 81.5 | 0.0 | 50.5 | 81.2 | 0.0 | 46.8 | 76.8 | 0.0 | 42.6 | 70.2 |
|  | 50% | 0.0 | 50.3 | 77.3 | 0.0 | 43.1 | 66.3 | 0.0 | 8.2 | 36.7 | 0.0 | -27.6 | 4.6 |
|  | 95% | 0.0 | 30.0 | 41.8 | 0.0 | -9.7 | 4.3 | 0.0 | -83.6 | -40.3 | 0.0 | -72.8 | -52.0 |
| RU (%) |  | 5.0 | 7.8 | 5.7 | 20.0 | 28.7 | 21.6 | 40.0 | 56.5 | 43.4 | 60.0 | 76.2 | 63.2 |

### FTP Transfer: 500 kB file size

| Performance Indicator |  | 14s Load: 5% RU |  |  | 14s Load: 20% RU |  |  | 14s Load: 40% RU |  |  | 14s Load: 60% RU |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| FTP DL OBR CDF [Mbit/s] | Mean | 12.4 | 17.8 | 27.0 | 10.3 | 12.8 | 16.3 | 8.3 | 8.6 | 10.7 | 6.0 | 5.2 | 5.9 |
|  | 95% | 15.3 | 23.8 | 44.5 | 14.5 | 23.1 | 41.5 | 13.5 | 21.1 | 33.5 | 12.3 | 16.6 | 21.0 |
|  | 50% | 12.6 | 18.8 | 27.0 | 10.8 | 12.4 | 13.5 | 8.6 | 7.2 | 7.6 | 5.6 | 3.4 | 3.0 |
|  | 5% | 8.6 | 8.0 | 6.1 | 4.6 | 3.4 | 3.0 | 2.3 | 1.6 | 1.5 | 1.2 | 0.8 | 0.7 |
| Gain [%] | Mean | 0.0 | 43.6 | 117.3 | 0.0 | 24.0 | 58.5 | 0.0 | 2.5 | 27.6 | 0.0 | -13.4 | -1.7 |
|  | 95% | 0.0 | 56.2 | 191.8 | 0.0 | 59.5 | 185.8 | 0.0 | 55.9 | 148.0 | 0.0 | 34.4 | 70.2 |
|  | 50% | 0.0 | 48.5 | 112.6 | 0.0 | 15.1 | 24.9 | 0.0 | -16.6 | -12.7 | 0.0 | -39.9 | -46.6 |
|  | 5% | 0.0 | -7.5 | -30.5 | 0.0 | -26.9 | -36.0 | 0.0 | -29.7 | -36.5 | 0.0 | -36.6 | -39.9 |
| RU (%) |  | 5.0 | 6.9 | 7.5 | 20.0 | 25.9 | 25.2 | 40.0 | 51.6 | 45.9 | 60.0 | 72.8 | 65.0 |
| Performance Indicator |  | 14s Load: 5% RU |  |  | 14s Load: 20% RU |  |  | 14s Load: 40% RU |  |  | 14s Load: 60% RU |  |  |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| DL Delay CDF [s] | Mean | 0.34 | 0.28 | 0.29 | 0.44 | 0.45 | 0.50 | 0.68 | 0.91 | 0.97 | 1.17 | 1.71 | 1.87 |
|  | 5% | 0.26 | 0.17 | 0.09 | 0.28 | 0.17 | 0.10 | 0.30 | 0.19 | 0.12 | 0.32 | 0.24 | 0.19 |
|  | 50% | 0.32 | 0.22 | 0.15 | 0.37 | 0.32 | 0.30 | 0.47 | 0.57 | 0.56 | 0.72 | 1.21 | 1.37 |
|  | 95% | 0.47 | 0.51 | 0.72 | 0.87 | 1.19 | 1.36 | 1.87 | 2.64 | 2.96 | 3.34 | 5.28 | 5.55 |
| Gain [%] | Mean | 0.0 | 19.0 | 16.1 | 0.0 | -1.1 | -13.1 | 0.0 | -33.4 | -40.6 | 0.0 | -46.3 | -59.5 |
|  | 5% | 0.0 | 36.0 | 65.7 | 0.0 | 37.3 | 65.0 | 0.0 | 35.8 | 59.5 | 0.0 | 25.5 | 40.9 |
|  | 50% | 0.0 | 32.3 | 51.8 | 0.0 | 13.1 | 19.6 | 0.0 | -21.3 | -17.2 | 0.0 | -66.9 | -88.9 |
|  | 95% | 0.0 | -8.4 | -49.8 | 0.0 | -37.1 | -56.6 | 0.0 | -42.7 | -57.5 | 0.0 | -57.8 | -66.3 |
| RU (%) |  | 5.0 | 6.9 | 7.5 | 20.0 | 25.9 | 25.2 | 40.0 | 51.6 | 45.9 | 60.0 | 72.8 | 65.0 |

### FTP Transfer: 100 kbit file size

| Performance Indicator |  | 14s Load: 5% RU |  |  | 14s Load: 20% RU |  |  | 14s Load: 40% RU |  |  | 14s Load: > 40% RU |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| FTP DL OBR CDF [Mbit/s] | Mean | 1.9 | 3.7 | 8.7 | 1.7 | 3.0 | 6.7 | 1.5 | 2.1 | 4.4 | 1.5 | 1.9 | 4.1 |
|  | 95% | 2.3 | 4.4 | 11.3 | 2.3 | 4.2 | 10.7 | 2.2 | 3.8 | 9.6 | 2.1 | 3.8 | 9.3 |
|  | 50% | 1.9 | 3.8 | 9.1 | 1.7 | 3.0 | 6.7 | 1.5 | 2.0 | 3.9 | 1.5 | 1.8 | 3.5 |
|  | 5% | 1.4 | 2.7 | 4.9 | 1.2 | 1.5 | 2.4 | 0.9 | 0.6 | 0.9 | 0.9 | 0.5 | 0.8 |
| Gain [%] | Mean | 0.0 | 97.8 | 366.4 | 0.0 | 73.4 | 291.5 | 0.0 | 34.4 | 188.0 | 0.0 | 29.9 | 173.5 |
|  | 95% | 0.0 | 93.9 | 395.7 | 0.0 | 82.8 | 369.7 | 0.0 | 77.1 | 344.5 | 0.0 | 77.4 | 338.7 |
|  | 50% | 0.0 | 106.3 | 391.7 | 0.0 | 79.5 | 297.3 | 0.0 | 28.1 | 153.7 | 0.0 | 20.7 | 132.2 |
|  | 5% | 0.0 | 97.3 | 258.4 | 0.0 | 25.9 | 108.8 | 0.0 | -38.0 | -1.1 | 0.0 | -41.4 | -8.9 |
| RU (%) |  | 5.0 | 8.4 | 6.8 | 20.0 | 34.5 | 25.1 | 40.0 | 62.5 | 47.2 | 44.1 | 65.8 | 50.9 |
| Performance Indicator |  | 14s Load: 5% RU |  |  | 14s Load: 20% RU |  |  | 14s Load: 40% RU |  |  | 14s Load: > 40% RU |  |  |
|  |  | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s | 14s | 7s | 2s |
| DL Delay CDF [s] | Mean | 0.06 | 0.03 | 0.02 | 0.06 | 0.04 | 0.02 | 0.07 | 0.07 | 0.04 | 0.07 | 0.08 | 0.04 |
|  | 5% | 0.04 | 0.02 | 0.01 | 0.04 | 0.02 | 0.01 | 0.05 | 0.03 | 0.01 | 0.05 | 0.03 | 0.01 |
|  | 50% | 0.05 | 0.03 | 0.01 | 0.06 | 0.03 | 0.01 | 0.07 | 0.05 | 0.03 | 0.07 | 0.06 | 0.03 |
|  | 95% | 0.07 | 0.04 | 0.02 | 0.09 | 0.07 | 0.04 | 0.11 | 0.18 | 0.11 | 0.12 | 0.20 | 0.13 |
| Gain [%] | Mean | 0.0 | 48.0 | 71.8 | 0.0 | 36.6 | 66.5 | 0.0 | 0.0 | 44.0 | 0.0 | -6.2 | 38.9 |
|  | 5% | 0.0 | 48.4 | 79.8 | 0.0 | 45.3 | 78.7 | 0.0 | 43.5 | 77.5 | 0.0 | 43.6 | 77.2 |
|  | 50% | 0.0 | 51.5 | 79.7 | 0.0 | 44.2 | 74.7 | 0.0 | 21.4 | 60.0 | 0.0 | 17.2 | 56.9 |
|  | 95% | 0.0 | 49.2 | 71.9 | 0.0 | 16.3 | 50.1 | 0.0 | -62.2 | -2.3 | 0.0 | -70.5 | -9.8 |
| RU (%) |  | 5.0 | 8.4 | 6.8 | 20.0 | 34.5 | 25.1 | 40.0 | 62.5 | 47.2 | 44.1 | 65.8 | 50.9 |