3GPP TSG-RAN WG1 Meeting #84 	R1-161186

# St Julian’s, Malta, 15th – 19th February 2016

Agenda item:		6.2.3

Source:	Huawei, HiSilicon

Title:	TP on evaluation methodology

Document for:		Discussion/Decision

# Introduction

This contribution presents the scenarios for multi-carrier enhancements for UMTS for the technical report that it is proposed to include the findings for the study item in [1].

# Text proposal

[------------------------------------------------------------- TEXT START --------------------------------------------------------------]

# 6	Evaluation methodology

Regarding the evaluation methodology for multi-carrier enhancements, the following aspects are considered:

- The baseline for evaluation is 2+2 ms TTI configuration.

- Evaluations for the uplink throughput should be done for 2+10 ms TTI configuration and 10+10 ms TTI configuration.

- Carrier frequency: for uplink transmission in the dual band dual carrier scenario, the carrier frequencies are 900MHz and 2GHz.

- The total transmission power shared between the two carriers is fixed to 23dBm. In practice, how the transmit power is shared between the two carriers depends on the uplink scheduling used by the network. To take into account this fact, various configurations for power sharing between the two carriers are used in the evaluations.

- Several values for the distance between UE and NodeB are assumed in the evaluation in order to emulate the far field UE position. In the dual band dual carrier scenario, the path loss difference between two bands shall be reflected in the Attenuation Factor defined as below:

Attenuation Factor = Tx power – Effective noise power – received Ec/N0 (dB)				(x1)

where Effective noise power = Thermal noise density + Receiver noise figure + Interference margin + 10*log(Occupied channel bandwidth) + Uplink RoT

- For the E-DPDCHs, the data rate can be different for each of the carriers and a desired BLER level of 10% for the initial transmission is targeted for each of the carriers.

- ILPC and OLPC are turned off.

Simulation assumptions are shown in Table x1.

Table x1 	Simulation assumptions for Multi-Carrier Enhancements

| Parameter | Assumption |
| --- | --- |
| Node B Rx receiver type | RAKE |
| Number of UE antennas | 1 antenna |
| Number of Node B antennas | 2 antennas (uncorrelated) |
| Carrier frequency for dual band dual carrier scenario | Lower band: 900MHz Higher band: 2 GHz The formula of the Path Loss: 900MHz：PL(R) = 120.9 + 37.6log10(R) 2GHz: PL(R) = 128.1 + 37.6log10(R) R (km) is the distance between UE and Node B. |
| Total transmission power | The total transmit power shared between the two carriers is 23 dBm at antenna connector |
| Transmit power at the lower band | 13dBm, 16dBm, 19dBm, other values are optional. |
| Node B receiver noise figure | 5 dB |
| Thermal noise density (dBm/Hz) | -174 |
| Interference margin (dB) | 0 |
| Soft/softer handover | No soft/softer handover |
| Uplink rise-over-thermal (RoT) operation point | 10 dB |
| Channel model | PA3 |
| BLER target | 10% BLER for the initial transmission |
| Power control | ILPC: OFF OLPC: OFF |

[-------------------------------------------------------------TEXT OMITTED------------------------------------------------------------]

# Conclusion

It is proposed to agree to and capture the text proposal in this document in the Technical Report on Multi-Carrier enhancements for UMTS.

# References

- RP-152290, New SID: Study on Multi-Carrier Enhancements for UMTS