3GPP TSG RAN WG1 Meeting #84	R1-161358

St Julian’s, Malta, 15th-19th February 2016

Agenda Item:	7.2.1.2.2

Source:	Huawei, HiSilicon, Neul

Title:	NB-PRACH evaluation

Document for:	Discussion and decision

Introduction

In RAN1 adhoc meeting, a detailed design based on single-tone transmission was provided in [5] and the performance was also provided in [6], and a working assumption that NB-PRACH is based on single-tone transmission was approved. This contribution shows the performance of the single-tone hopping based NB-PRACH design that is updated in [4]. Improved performance is found for all formats, because resources are increased for format 0 and 1, and CFO estimation and compensation are performed for all formats at the receiver side.

Simulation setup

Configurations of NB-PRACH for 164/154/144dB MCL are shown in Table 1. In the contribution on NB-PRACH design [4], a set of 4 continuous groups is defined as a basic unit for NB-PRACH, where each group contains 6 symbols. For a given configuration, the NB-PRACH can be obtained by repeating the basic unit according to the number of repetitions.

Table 1: Configurations of NB-PRACH for 164/154/144dB MCL

| MCL | Number of symbols per group | Number of groups per repetition | Number of Repetitions | Duration (ms) |
| --- | --- | --- | --- | --- |
| 144 Opt1 | 6 | 4 | 2 | 12.8 |
| 144 Opt2 | 3 | 4 | 4 | 12.8 |
| 154 | 6 | 4 | 6 | 38.4 |
| 164 | 6 | 4 | 30 | 192 |

The simulation parameters are shown in Table 2.

Table 2: Simulation assumptions

| Parameter | Value |
| --- | --- |
| Channel model | TU |
| Doppler spread | 1 Hz, 100Hz |
| Antenna configuration | 1Tx, 2Rx |
| MS transmit power | 23 dBm |
| BS receiver noise figure | 3 dB |
| Frequency error | ±50 Hz |
| Frequency drift | ±22.5 Hz/s |
| Cell size | 35 km |
| DL sync. error | [-2.5, 2.5] us |

Coverage performance

The false alarm probabilities and miss detection probabilities of UEs with different coverage classes (144dB, 154dB and 164dB MCL) are shown in Table 3.

Table 3: False alarm and missed detection probabilities for TU-1Hz

| MCL (dB) | False Alarm Probability | Missed Detection Probability |
| --- | --- | --- |
| 144 opt1 | 0.05% | 0.5% |
| 144 opt2 | 0.05% | 0.3% |
| 154 | 0.1% | 0.6% |
| 164 | 0.1% | 0.8% |

Table 4: False alarm and missed detection probabilities for TU-100Hz

| MCL (dB) | False Alarm Probability | Missed Detection Probability |
| --- | --- | --- |
| 144 opt2 | 0.05% | 0.1% |
| 154 | 0.1% | 0.15% |
| 164 | 0.1% | 0.3% |

ToA estimation performance for TU-1Hz

The residual timing offset is shown in Figure 1 to Figure 4 for UEs with 144dB, 154dB and 164dB MCL, and the results are summarized in

Table 5.

Figure 1: Residual timing offset at 144dB MCL opt1

Figure 2: Residual timing offset at 144dB MCL opt2

Figure 3: Residual timing offset at 154dB MCL

Figure 4: Residual timing offset at 164dB MCL

Table 5: ToA estimation performance summary

| MCL | Probability of ToA Error in [-2.5~2.5]us |
| --- | --- |
| 144 dB opt1 | 98.1% |
| 144 dB opt2 | 98.1% |
| 154 dB | 97.0% |
| 164 dB | 96.1% |

ToA estimation performance for TU-100Hz

Figure 5: Residual timing offset at 144dB MCL opt2

Figure 6: Residual timing offset at 154dB MCL

Figure 7: Residual timing offset at 164dB MCL

Table 6: ToA estimation performance summary

| MCL | Probability of ToA Error in [-2.5~2.5]us |
| --- | --- |
| 144 dB opt2 | 99.9% |
| 154 dB | 95.3% |
| 164 dB | 95.8% |

Performance summary

The results shown in the previous sections can be summarized in the following observations:

Observation: The single-tone frequency-hopping NB-PRACH design proposed in [4] can provide sufficiently good ToA estimation accuracy and detection performance to support 15 kHz subcarrier spacing, with the normal cyclic prefix duration used in LTE (4.7-5.2) us.

We therefore propose to confirm the single-tone NB-PRACH working assumption from the ad-hoc, along with the common understanding that it uses frequency hopping.

Proposal: NB-PRACH scheme is based on single-tone frequency hopping, i.e. confirm the working assumption from the ad-hoc meeting.

Conclusions

This contribution presents performance simulation results for the NB-PRACH scheme proposed for NB-IoT in [4]. The following observations are made:

Observation: The single-tone frequency-hopping NB-PRACH design proposed in [4] can provide sufficiently good ToA estimation accuracy and detection performance to support 15 kHz subcarrier spacing, with the normal cyclic prefix duration used in LTE (4.7-5.2) us.

Proposal: NB-PRACH scheme is based on single-tone frequency hopping, i.e. confirm the working assumption from the ad-hoc meeting.

- References

RP-152234, “Revised Work Item: Narrowband IoT (NB-IoT)”, Huawei, HiSilicon, RAN#70, Sitges, Spain, December 2015.

R1-157783, “Way Forward on NB-IoT”, CMCC, Vodafone, Ericsson, Huawei, HiSilicon, Deutsche Telekom, Mediatek, Qualcomm, Nokia Networks, Samsung, Intel, Neul, CATR, AT&T, NTT DOCOMO, ZTE, Telecom Italia, IITH, CEWiT, Reliance-Jio, CATT, u-blox, China Unicom, LG Electronics, Panasonic, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, China Telecom, RAN1#83, Phoenix, USA, November 2015.

3GPP TR 45.820 v2.1.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”.

R1-160316, “NB-PRACH design”, Huawei, HiSilicon, Neul, RAN1#84 meeting, Malta, February 2016.

R1-160025, “NB-PRACH design”, Huawei, HiSilicon, Neul, RAN1 NB-IoT adhoc meeting, Budapest, January 2016.

R1-160026, “NB-PRACH evaluation”, Huawei, HiSilicon, Neul, RAN1 NB-IoT adhoc meeting, Budapest, January 2016.

- Appendix: ToA accuracy performance of NB-PRACH format 0, 1 under EPA1Hz channel

Figure 8: Residual timing offset at 144dB MCL

| 144dB MCL, Format0, EPA1 |  |  |
| --- | --- | --- |
| TOA error (1/1.92 us) | TOA error (us) | Probability |
| -5 ~ 5 | -2.60 ~ 2.60 | 99.85% |

Figure 9: Residual timing offset at 154dB MCL

| 154dB MCL, Format1, EPA1 |  |  |
| --- | --- | --- |
| TOA error (1/1.92 us) | TOA error (us) | Probability |
| -5 ~ 5 | -2.60 ~ 2.60 | 99.50% |