3GPP TSG RAN WG1 Meeting #84bis		R1-162803

Busan, Korea, 11th - 15th April 2016

Source:	NTT DOCOMO, INC.

Title:	Discussion on PRACH for eLAA UL

Agenda Item:	7.3.1.4

Document for: 	Discussion and Decision

Introduction

At the RAN1 #84 meeting, whether PRACH transmission is allowed and which random access scheme could be used on LAA SCells were discussed. Following agreements regarding PRACH were achieved:

Agreements:

- Contention based PRACH transmission on LAA Scell is not supported in Rel-14

- Non-contention based PRACH transmission on LAA Scell is supported in Rel-14 subject to LBT

- FFS: PRACH duration up to 1msec is supported

- FFS: A UL transmission burst containing PRACH without other UL channel immediately follows a single idle observation interval of at least 25 micro sec

- FFS: new PRACH waveform

Considering the above progress, in this contribution, we show our views on the PRACH format design, time-frequency resource allocation methods and possible solutions to increase the PRACH transmission opportunity.

PRACH design

In this section, we discuss the RA preamble format and PRACH time-frequency resources for LAA SCells.

Random access preamble format

There are five preamble formats in current LTE [1], the first four preamble formats e.g. preamble format 0, 1, 2, 3, with different guard period and preamble length are used to handle medium and large cells. These preamble formats are applicable to both FDD and TDD. For TDD, there is an additional preamble format, preamble format 4 which is transmitted in the UpPTS field of the special subframe. Since the UpPTS has at most two OFDM symbols, the preamble length and the guard time are much shorter than the other four preamble formats. Therefore, preamble format 4 is only applicable to very small cells. Considering that the radius of LAA Cells would not be large due to the transmission power limitation, PRACH duration up to 1msec would be sufficient for LAA SCells. Therefore, it is reasonable to re-use the time-domain structure of existing preamble formats 0 and 4 for eLAA PRACH preamble format. However, similar as PUSCH transmission on eLAA, new PRACH waveform should also be based on RB-level multi-cluster transmission to meet the occupied channel bandwidth and power spectral density requirements. Therefore, it is possible to multiplex the wideband PRACH and PUSCH on the same subframe. However, multiple guard bands may be needed to avoid the data interference at preamble edges for multi-cluster transmission. In this case PRACH format would need to be aligned with the format of multiplexed PUSCH. So, besides format 0 and 4, if additional new preamble formats are required for LAA carriers, the length for the guard period and the preamble sequence should take the supported radius of LAA SCells and detection performance into consideration.

Proposal 1: PRACH duration up to 1msec should be sufficient for eLAA SCells. Existing PRACH preamble format 0 and 4 should be the baseline for eLAA PRACH format.

PRACH time-frequency resource

- Time-domain

In the time domain, current LTE specifies the PRACH resource set and preamble format by the parameter prach-ConfigIndex which is broadcasted in the SIB2. Table 5.7.1-2 and 5.7.1-3 in [1] indicates the preamble format and the subframe where to transmit the RA preamble for FDD and TDD respectively. In licensed cells, the semi-static PRACH resource set configuration works well for both FDD and TDD, since the uplink subframe index for FDD and TDD is fixed within one radio frame. However, for LAA SCells, it is desirable to support flexible DL-UL configuration with variable length frame structure for dynamic DL-UL traffic adaptation, and hence the fixed uplink subframes cannot exist anymore. Therefore, current RRC-configured subframes for PRACH transmission are not suitable for LAA SCells and new indication of PRACH time-domain resources is needed. One potential way is to support flexible timing between the subframe where PDCCH order is received and subframe(s) for corresponding PRACH transmission with maintaining the minimum 6ms latency. As illustrated in Fig. 1, this single PDCCH order in a subframe can enable the UE to perform single PRACH transmission among one of the multiple subframes depending on UL LBT result. This method is beneficial to not only remove the fixed UL subframe restriction in time domain, but also increase the PRACH transmission opportunities for LAA SCells.

Figure. 1 PDCCH order indicates subframes for PRACH transmission

Proposal 2: For PRACH resources in the time domain, it is preferred to support flexible timing between the subframe where PDCCH order is received and subframe(s) for corresponding PRACH transmission.

- The single PDCCH order in a subframe can enable the UE to perform single PRACH transmission among one of the multiple subframes depending on UL LBT result.

- Frequency-domain

In the frequency domain, current LTE supports at most one PRACH resource on one subframe for FDD. While for TDD, multiple PRACH resources can be configured in a single subframe due to smaller number of uplink subframes per radio frame in TDD. For both FDD and TDD, each PRACH resource continuously occupies a bandwidth corresponding to six resource blocks (RBs) to match the smallest uplink cell bandwidth. However, for LAA SCells, to satisfy the occupied channel bandwidth and power spectral density requirements with achieving sufficient coverage performance [2], the new PRACH waveform should spread its transmission discontinuously over the entire bandwidth. Therefore RB-level multi-cluster transmission used for PUSCH transmission can be the baseline for PRACH transmission on LAA SCell. As in legacy LTE, it is desirable to multiplex PRACH and other UL channels e.g. PUSCH, PUCCH in the same subframe. However, guard bands between the PRACH clusters and other UL channel clusters may be needed to avoid the interference to preamble transmission. In addition, similar reason as in TDD, LAA SCell should also support multiplexing of multiple PRACH transmissions on the single subframe. One example is illustrated in Figure 2.

Figure. 2: Example of multiplexing PRACH and PUSCH transmission

Regarding the indication of frequency-domain resource for PRACH transmission, since LAA SCell only supports non-contention based PRACH transmission which is triggered by PDCCH order, it is preferred to take current mechanism for TDD with multiple PRACH resources in the single subframe (e.g. for TDD UL/DL configuration 5, PRACH configuration index = 18) as starting point to minimize specification effort. That is eNB should indicate one specific PRACH resource among preconfigured PRACH resources in the frequency domain by PRACH Mask Index in DCI format 1A for the UE to avoid the collision. The only difference between TDD is the preconfigured PRACH resources in frequency domain should fulfill the multi-cluster transmission.

Proposal 3: For PRACH resources in the frequency domain,

- PRACH resource based on interleaved RBs can be considered to meet the bandwidth regulation and to coexist with LAA PUSCH signals based on multi-cluster transmission in the same subframe.

- It is necessary to support frequency-domain multiplexing of multiple UEs’ PRACH transmission in one subframe;

- The indication of frequency-domain resource allocation should be based on current mechanism for TDD with multiple PRACH resources in the single subframe.

  - The preconfigured PRACH resources in frequency domain should fulfil the multi-cluster transmission.

Channel access framework for PRACH

Although short PRACH format for eLAA is sufficient, LBT is still required before PRACH transmission considering some regional regulation such as in Japan. Timely acquisition of uplink timing is very important since other uplink transmission can be performed only after uplink timing is established. Therefore, LBT mechanism for PRACH should be designed so as to achieve good balance between successful transmission probability and fair coexistence with other systems/transmissions. Fast channel access procedure based on Cat.2 UL LBT can be considered. For example, a single idle observation interval of at least x us e.g. x=25 allows the start of a PRACH transmission which facilitates multiplexing multiple UEs’ PRACH transmission. Besides, energy detection threshold should be set considering that whether the absence of any other technology sharing the carrier can be guaranteed on a long term basis or not.

Proposal 4: Fast channel access mechanism based on Cat.2 UL LBT is preferred for PRACH transmission.

Enhancements for Random Access Procedure

Even if fast channel access for PRACH transmission is specified, LBT busy may still occur and prevent PRACH transmission on unlicensed carrier. In this case, it is recommended in [1] that the handling of preamble transmission dropping from Rel-12 Dual Connectivity can be used as baseline: PHY informs MAC of RA preamble dropping and accordingly, MAC does not increase preamble transmission counter and preamble transmission power. However, the UE MAC may keep transmitting RA preamble on one “always busy” unlicensed carrier even if it is possible to obtain the timing advance by transmitting PRACH on another available unlicensed carrier within the same Timing Advance Group (TAG). Therefore, some enhancements besides designing the proper LBT parameters to increase PRACH transmission opportunities are needed. Following two options can be considered.

Option 1: Per-unlicensed carrier transmission of random access preamble.

This option is similar to current random access procedure in LTE with the difference that multiple random access procedures can be performed on multiple unlicensed carriers within the same TAG. Since the availability of the unlicensed carriers cannot be ensured, in order to increase PRACH transmission opportunities, it is preferable to trigger multiple RA preamble transmissions on PRACH resources which are staggered in the time domain on these unlicensed carriers. Once one successful random access response (RAR) is detected by UE, the UE uses the TA indicated by the RAR for all unlicensed carriers within the TAG and stops transmitting preambles on other unlicensed carriers. One example is illustrated in Figure 5-1.

Figure 3-1: Example of per-unlicensed carrier transmission of RA preamble

Option 2: Cross-unlicensed carrier transmission of random access preamble.

Different from option 1 that separate random access procedure is performed on separate unlicensed carrier, option 2 allows one random access procedure shared across unlicensed carriers within the same TAG as shown in Figure 5-2. PRACH resources are also staggered in the time domain on the unlicensed carriers. UE performs LBT on the unlicensed carrier which has the earliest PRACH resource. If LBT busy occurs, UE performs LBT on next unlicensed carrier until finding one LBT-idle carrier to transmit the RA preamble. After the successful transmission of the preamble, UE tries to decode RAR during the RAR window and stops transmitting preambles on other carriers. If the RAR is successfully received by UE, the UE will use the TA indicated by the RAR for all unlicensed carriers within the TAG; otherwise UE resumes the cross-unlicensed carrier transmission of RA preamble, e.g. performs LBT on the next unlicensed carrier having the earliest PRACH resource.

Figure 3-2: Example of cross-unlicensed carrier transmission of RA preamble

Proposal 5: Some enhancements to increase the PRACH transmission opportunities can be considered.

Conclusion

In this contribution, we have discussed on PRACH design for eLAA UL. We made the following proposals.

Proposal 1: PRACH duration up to 1msec should be sufficient for eLAA SCells. Existing PRACH preamble format 0 and 4 should be the baseline for eLAA PRACH format.

Proposal 2: For PRACH resources in the time domain, it is preferred to support flexible timing between the subframe where PDCCH order is received and subframe(s) for corresponding PRACH transmission.

- The single PDCCH order in a subframe can enable the UE to perform single PRACH transmission among one of the multiple subframes depending on UL LBT result.

Proposal 3: For PRACH resources in the frequency domain,

- PRACH resource based on interleaved RBs can be considered to meet the bandwidth regulation and to coexist with LAA PUSCH signals based on multi-cluster transmission in the same subframe.

- It is necessary to support frequency-domain multiplexing of multiple UEs’ PRACH transmission in one subframe;

- The indication of frequency-domain resource allocation should be based on current mechanism for TDD with multiple PRACH resources in the single subframe.

  - The preconfigured PRACH resources in frequency domain should fulfil the multi-cluster transmission.

Proposal 4: Fast channel access mechanism based on Cat.2 UL LBT is preferred for PRACH transmission.

Proposal 5: Some enhancements to increase the PRACH transmission opportunities can be considered.

- References

- 3GPP, TS 36.211 v13.0.0 (2015-12), “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation (Release 13),” Dec. 2015

- ETSI EN 301 893 V1.8.1, Harmonized European Standard, “Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN”