3GPP TSG RAN1 Meeting #84bis 	R1-163689

# Busan, South Korea, April 11-15, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0202 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identify MPDCCH order in CE mode B in TS 36.212 (Solution 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,HiSilicon,CATT,MediaTek,Sierra Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1 |  |  |  |  |  |  | Date: |  | 2016-04-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To capture agreements about MPDCCH order identification in CE mode B. Agreements When an M-PDCCH is used for M-PDCCH order, the two spare bits corresponding to the location of two MSB bits in the MCS field can be fixed to 11 (in addition to resource block assignment field set to all ‘1’s) based on option 2b. Some editorial corrections for 36.212 |  |  |  |  |  |  |  |  |
| Summary of change: |  | For DCI format 6-1B used for MPDCCH order, two bits corresponding to the location of two MSB bits in the MCS field are fixed to 11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPDCCH order significantly restricts PDSCH operation. Specification is more difficult to read. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.3.3.1.13, 5.3.3.1.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in [1].

BL/CE:  A Bandwidth-reduced Low-complexity or Coverage Enhanced (BL/CE) UE is capable of coverage enhancement mode A support and intends to access a cell in a coverage enhancement mode or is configured in a coverage enhancement mode.

< Unchanged parts are omitted >

5.3.3.1.13	Format 6-1B

DCI format 6-1B is used for the scheduling of one PDSCH codeword in one cell.

The following information is transmitted by means of the DCI format 6-1B:

- Flag for format 6-0B/format 6-1B differentiation – 1 bit, where value 0 indicates format 6-0B and value 1 indicates format 6-1B

Format 6-1B is used for random access procedure initiated by a PDCCH order only if format 6-1B CRC is scrambled with C-RNTI and all the remaining fields are set as follows:

- Reserved bits – +2 bits, where all bits shall be set to 1

- Preamble Index – 6 bits

- PRACH Mask Index – 4 bits [5]

- Starting CE level – 2 bits provide the PRACH starting CE level as defined in [5]

- All the remaining bits in format 6-1B for compact scheduling assignment of one PDSCH codeword are set to zero

Otherwise,

- Modulation and coding scheme – 4 bits as defined in section 7.1.7 of [3]

- Resource block assignment – +1 bits for PDSCH as defined in [3]:

-  MSB bits provide the narrowband index as defined in section 6.2.7 of [2]

- 1 bit provides the resource allocation within the indicated narrowband, where value 0 indicates RBs with PRB index {0, 1, 2, 3} and value 1 indicates that all 6 PRBs are used.

- Repetition number – 3 bits as defined in section 7.1.11 of [3]

- HARQ process number – 1 bit

- New data indicator – 1 bit

- HARQ-ACK resource offset – 2 bits as defined in section 10.1 of [3]

- DCI subframe repetition number –2 bits as defined in section 9.1.5 of [3]

When the format 6-1B CRC is scrambled with a RA-RNTI then the following fields among the fields above are reserved:

- HARQ process number

- New data indicator

- HARQ-ACK resource offset

< Unchanged parts are omitted >

5.3.3.1.14	Format 6-2

DCI format 6-2 is used for paging and direct indication.

< Unchanged parts are omitted >