3GPP TSG RAN WG1 Meeting #84bis	 		R1-163683

Busan, Korea, 11th – 15th April 2016

Title:	LS on PUSCH transmission for eLAA

Release:		Rel-14

Work Item:		eLAA_LTE

Source:		TSG RAN WG1

To:		TSG RAN WG4

Cc:

Contact Person:

#### Name:	Jeongho Jeon

E-mail Address:	jeongho.jeon@intel.com

Attachments:		-

1. Overall Description:

In RAN1 #84bis, RAN1 made the following agreement and working assumption for eLAA PUSCH transmission:

Agreement:

- For eLAA PUSCH transmission, one interlace is the basic unit of resource allocation, which is composed of 10RBs for 20MHz

  - Working assumption: the 10RBs are spaced equally in frequency domain for 20MHz

    - Ex for 20MHz eLAA SCell: interlace 0 is composed of RBs 0,10,20,...,90

    - Send an LS to RAN4 asking whether or not RAN4 sees issues with the working assumption. RAN1 also discussed the possibility of having unequal spacing in frequency domain for the 10-RB interlace based resource allocation

  - FFS the case of other system bandwidth(s)

The working assumption is that the 10RBs are spaced equally in frequency domain for 20MHz system bandwidth. It is noted that, together with the working assumption, RAN1 also discussed the possibility of having unequal spacing in frequency domain for the 10-RB interlace based resource allocation in the context of inter-modulation distortion.

2. Actions:

RAN1 would like respectfully to ask RAN4 if they have any significant concerns about RAN1’s working assumption for equally-spaced RBs that might be mitigated by means of unequally-spaced RBs (e.g. based on consideration of power backoff required), and, if so, whether RAN4 would have any significant concerns about unequally-spaced RBs.

3. Date of Next TSG RAN WG1 Meetings:

TSG-RAN WG1 Meeting #85	23 - 27 May 2016	Nanjing, China

TSG-RAN WG1 Meeting #86	22 – 26 August 2016	Gothenburg, Sweden