3GPP TSG RAN WG1 Meeting #84bis		R1-163906

Busan, Korea, 11th - 15th April 2016

Title:	LS on resource allocation for V2V

Response to:	-

Release:	Release 14

Work Item:	LTE_SL_V2V

Source:	RAN WG1

To:	RAN WG2

Contact Person:

Name:	Hanbyul Seo

E-mail Address:	hanbyul.seo@lge.com

1. Overall Description:

RAN1 would like to inform the following RAN1 agreement on the resource allocation for V2V

Agreement:

- Both SA decoding and energy measurement are supported for sensing in UE autonomous resource selection.

  - FFS how each of SA decoding and energy measurement is used.

  - Note: strive for a design with managable complexity especially in terms of SA blind decodes, UE buffer, etc.

Agreement:

- If at TTI n resource selection/reselection is triggered in UE autonomous resource selection mode,

  - The UE at least senses between TTI n-a and TTI n-b (FFS a and b with a>b>0), where a and b are integers

    - Working assumption: The values a and b are common for V2V UEs.

  - UE selects time-frequency resource(s) for PSSCH

  - UE transmits SA in TTI n+c where c is an integer

    - FFS whether c is a fixed value (>= 0) or variable.

Agreement:

- In UE autonomous resource selection mode, SA can be transmitted for every TB.

  - FFS whether to support transmitting and/or receiving TB without SA

  - FFS whether every data (re)transmission for the same TB has the associated SA transmission.

Agreement:

- In UE autonomous resource selection mode,

  - UE transmits SA at TTI n+c indicating the associated data which is transmitted at TTI n+d (FFS d with d>=c), where c and d are integers

  - UE indicates whether it intends to reuse the frequency resource signaled for transmission at TTI n+d for potential transmission at TTI n+e for another TB (FFS e with d<e), where e is an integer

    - FFS whether this indication is implicit or explicit.

    - FFS if and how to signal the value for e

    - FFS how the UE determines the value for e

    - FFS whether e is a single value or can be multiple values

    - FFS whether, and if so how, a UE can notify later that it no longer intends to use the resource at TTI n+e.

    - FFS how the UE decides to indicate this

    - Other details FFS

Agreement:

- Proposal 1:

  - Priority handling is supported over PC5 for eNB-scheduled and UE- autonomous V2V communication

- Proposal 2:

  - The priority information is taken into account in the resource (re)selection for UE-autonomous mode

    - Further details are FFS

- Proposal 3:

  - Down-select between two alternatives at RAN1#85 meeting

    - Alt.1 Priority information is signaled in SCI

    - Alt.2 Priority information is not signaled in SCI

Conclusion for Energy measurement for sensing:

- Continue discussion until next meeting for the following options for the purpose of resource selection for PSSCH

  - Measurement of reception power of the decoded SA

  - Measurement of energy in SA resource

  - Measurement of energy in data resource

Agreement:

- Reselection is triggered if

  - A counter meets an expiration condition, or

    - The counter is reset to a value when reselection is triggered.

      - FFS whether this value is random, fixed, and/or the same across UEs

  - UE identifies that the current resource allocation cannot fulfil the requirement, e.g., latency, reliability, priority, fairness, or QoS requirement (details FFS), or

    - FFS how to impose limitation in the resource size selected by a UE

  - Working assumption: UE detects resource allocation (including the indication of the intention of reusing the frequency resource as per the existing agreement) of another UE which potentially overlaps with its current resource allocation (including the indication of the intention of reusing the frequency resource as per the existing agreement)  and the detected situation meets a condition, or

    - The condition is FFS, not precluding applying different conditions for different priorities (if any relevant case happens in priority handling from RAN1 point of view).

  - eNB triggers reselection, or

    - FFS whether some information needs to reported from UE to eNB

  - FFS additional condition(s), e.g.,

    - If it is supported that a UE can notify later that it no longer intends to use the resource at a future TTI, reselection triggering condition related to this notification.

    - Reselection triggered by notification on resource collision from another UE

    - Reselection triggered by sensing of resources utilization above a threshold

    - Reselection triggered by higher layer

    - Other conditions are not precluded

Agreement:

- For SPS of V2V traffic for mode-1 SPS on PC5:

  - The eNB may configure multiple SPS configurations for a given UE

    - At least SPS-configuration-specific MCS (if MCS is part of the SPS-configuration) and SPS-configuration-specific periodicity can be configured

      - FFS if/which other SPS parameters can differ across the SPS-configurations

    - The eNB can dynamically trigger/release the different SPS-configurations by use of (E)PDCCH

      - Details of the trigger/release are FFS

  - Working assumption: The UE can indicate to the eNB that it does not intend to transmit data before a transmission associated to an SPS configuration

    - FFS any details of the signaling protocol

    - FFS whether eNB acknowledgment of the UE indication is needed

Agreement:

- When SA and the associated data are transmitted in the same TTI, they can be transmitted in non-adjacent RBs.

Working assumption:

- In V2V, SA resource and data resource are always FDMed from system perspective

  - If significant issues are found, can consider further supporting TDM

Agreement:

- A data pool is always associated with an SA pool.

  - An RB of an SA pool in a TTI cannot be included in the associated data pool.

  - An RB of an SA pool in a TTI cannot be included in another SA pool (if exists).

  - Working assumption: At least an RB of a data pool in a TTI can be included in another data pool (if exists).

  - An RB of an SA pool in a TTI cannot be included an un-associated data pool (if exists)

Agreement:

- The following two cases are supported:

  - SA and the associated data are transmitted in the same TTI,

  - SA and the associated data are transmitted in different TTIs

    - The scheduling timing between SA and associated data is variable

      - In UE-autonomous resource selection mode, the timing is chosen by the transmitting UE from a configurable range

      - In eNB-scheduling mode, the timing is determined by eNB

      - SA includes information about the scheduling timing

  - Note: the association does not necessarily include the case of intention of using the resources for a different TB, if any (which is FFS)

2. Actions:

To RAN WG2

ACTION: 	RAN1 respectfully asks RAN2 to take into consideration the above RAN1 conclusion in its work.

3. Date of Next RAN1 Meetings:

RAN1#85	23rd  – 27th May 2016	Nanjing, China

RAN1#86	22nd – 26th August 2016	Gothenburg, Sweden