3GPP TSG-RAN1 Meeting #86	R1-167814

# Gothenburg, Sweden, 22nd Aug 2016 - 26th Aug 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.212 | CR | 0343 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of HS-SCCH Order for HS-SCCH DRx feature in enhanced Cell Fach |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Europe Inc.(France) |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FACH_DTXDRX-Core |  |  |  |  |  | Date: |  |  | 2016-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To introduce HS-SCCH order for HS-SCCH DRX operation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of HS-SCCH Order for HS-SCCH DRx feature in enhanced Cell Fach |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mechanism by which a UE is paged in CELL FACH with this feature will be unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | 25.214, 25.331 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

****************************************** TEXT START *****************************************

## 4.6C	Coding for HS-SCCH orders

### 4.6C.1	Overview

HS-SCCH orders are commands sent to the UE using HS-SCCH. No HS-PDSCH is associated with HS-SCCH orders.

The following information is transmitted by means of the HS-SCCH order physical channel.

- 	Extended order type (2bits)							xeodt,1, xeodt,2

-	Order type (3 bits):									xodt,1, xodt,2, xodt,3

-	Order (3 bits):										xord,1, xord,2, xord,3

-	UE identity (16 bits):								xue,1, xue,2, …, xue,16

The coding for HS-SCCH orders is specified in subclause 4.6.1 for a cell where the UE is not configured in MIMO mode and not configured in MIMO mode with four transmit antennas or if the UE is in CELL_FACH state and in subclause 4.6B.1 for a cell where the UE is configured in MIMO mode, in subclause 4.6D.1 for a cell where the UE is configured in MIMO mode with four transmit antennas with the exception of HS-DSCH serving cell change order, which is always transmitted using HS-SCCH type 1 specified in subclause 4.6.1.

### 4.6C.2	HS-SCCH Order information field mapping in the CELL_DCH state

#### 4.6C.2.1	Order type mapping

If the extended Order type xeodt,1, xeodt,2 = ’01’ and the Order type xodt,1, xodt,2, xodt,3 = ''000’, then the mapping for xord,1, xord,2, xord,3 is according to subclause 4.6C.2.2.1.

If the extended Order type xeodt,1, xeodt,2 = ’01’ and the Order type xodt,1, xodt,2, xodt,3 = ''011’, then the mapping for xord,1, xord,2, xord,3 is according to subclause 4.6C.2.2.3.

If the extended Order type xeodt,1, xeodt,2 = ’11’ and the Order type xodt,1, xodt,2, xodt,3 = ''010’, then the mapping for xord,1, xord,2, xord,3 is according to subclause 4.6C.2.2.4.

If the extended Order type xeodt,1, xeodt,2 = "11" and the Order type xodt,1, xodt,2, xodt,3 = ''011", then the mapping for xord,1,xord,2, xord,3 is according to subclause 4.6C.2.2.5.

Otherwise the mapping for xeodt,1 , xeodt,2, xodt,1, xodt,2, xodt,3, xord,1, xord,2, xord,3 is according to subclause 4.6C.2.2.2.

#### 4.6C.2.2	Order mapping

##### 4.6C.2.2.1	Orders for activation and deactivation of DTX, DRX and HS-SCCH-less operation and for HS-DSCH serving cell change

If the order is transmitted from the serving HS-DSCH cell or a secondary serving HS-DSCH cell, for this Order type, xord,1, xord,2, xord,3 is comprised of:

-	DRX activation (1 bit):							xord,1 = xdrx,1

-	DTX activation (1 bit):							xord,2 = xdtx,1

-	HS-SCCH-less operation activation (1 bit):	xord,3 = xhs-scch-less,1

If xdrx,1= ''0’, then the HS-SCCH order is a DRX De-activation order.

If xdrx,1= ''1’, then the HS-SCCH order is a DRX Activation order.

If xdtx,1= ''0’, then the HS-SCCH order is a DTX De-activation order.

If xdtx,1= ''1’, then the HS-SCCH order is a DTX Activation order.

If xhs-scch-less,1= ''0’, then the HS-SCCH order is a HS-SCCH-less operation De-activation order.

If xhs-scch-less,1= ''1’, then the HS-SCCH order is a HS-SCCH-less operation Activation order.

If the order is transmitted from a non-serving cell using HS-SCCH type 1 specified in subclause 4.6.1 and xord,1, xord,2, xord,3 = ''000’, then it is an HS-DSCH serving cell change order .

xdrx,1, xdtx,1 = ''10’ is a reserved combination (not used in this release).

**************************************** TEXT OMITTED ***************************************

### 4.6C.3	HS-SCCH Order information field mapping in the CELL_FACH and CELL_PCH states

#### 4.6C.3.1	Order type mapping

If the extended Order type xeodt,1, xeodt,2 = ’01’ and the Order type xodt,1, xodt,2, xodt,3 = ''000’, then the mapping for xord,1, xord,2, xord,3 is according to subclause 4.6C.3.2.

#### 4.6C.3.2	Order mapping

For this Order type, the Order mapping xord,1, xord,2, xord,3 and the associated meaning is as follows:

##### 4.6C.3.2.1	Orders for Network Triggered HS-DPCCH Transmission

For this Order type, the Order mapping xord,1, xord,2, xord,3 and the associated meaning is as follows:

If xord,1, xord,2, xord,3 = ''001’, then the HS-SCCH order is an order for triggering the random access procedure in CELL_FACH as specified in subclause  6.1A in [4].

##### 4.6C.3.2.2	Orders for HS-DSCH Reception in HS-SCCH DRX

If xord,1, xord,2, xord,3 = ''110” then the HS-SCCH order is an order for stopping the HS-SCCH DRX procedure and starting continuous reception of HS-PDSCH as specified in [4] subclause 6A.1.3.2

******************************************* TEXT END ******************************************