3GPP TSG RAN WG1 #87	R1-1612503

# Reno, USA 14th - 18th November 2016

Agenda item:	7.1.3.3

Source: 	Samsung

Title: 	Simulation results for explicit CSI feedback for NR MIMO

Document for:	Discussion and Decision

# Introduction

In RAN1#86b [1], the following agreement about CSI acquisition was made.

Agreements:

      - NR supports CSI reporting with two types of spatial information feedback

        - Type I feedback: Normal

  - Codebook-based PMI feedback with normal spatial resolution

        - Type II feedback: Enhanced

  - “Explicit” feedback and/or codebook-based feedback with higher spatial resolution

      - For Type I and II, CSI feedback per subband as well as wideband feedback are supported

      - For Type I and II, beam-related feedback can be included

Samsung’s high-level view on CSI acquisition for DL and UL NR MIMO is presented in companion contributions [2-3]. The companion contribution [4] proposes explicit feedback as Type II CSI reporting. This contribution provides simulation results comparing Type I (PMI codebook-based) and Type II (explicit) CSI types and shows that performance close to ideal CSI can be achieved with explicit CSI reporting, and hence makes the proposal that explicit CSI reporting should be supported as Type II CSI reporting in NR MIMO.

# Linear Combination Based Type II (Explicit) CSI Reporting

Figure 1: LC based explicit feedback framework

In Figure 1, a linear combination (LC) based reduced dimensional explicit CSI reporting framework is proposed in which a form of the DL channel is represented as an LC of basis vectors: l=0L-1clbl, where

- {bl:l=0,1,…,L-1} are basis vectors that are selected from a basis set. This selection is WB. LTE Rel. 13/14 Class A W1 codebook and LTE Rel. 14 Advanced CSI W1 codebook are two alternatives for the basis set.

- {cl:l=0,1,…,L-1} are LC coefficients that are selected from a channel quantization codebook. This selection is SB. The quantization codebook can be a scalar codebook (magnitude and phase or real and imaginary) or a vector codebook.

A few examples of the forms of DL channel and their LC representations are as follows:

- Channel: For a subcarrier k in SB f, the DL channel is represented as Hk≈l=0L-1clbl;

- Covariance matrix: For SB f, the covariance matrix is represented as K=1|f|k∈fHkHkH≈l=0L-1clblblH; and

- Eigenvector: For SB f, the dominant eigenvector of the covariance matrix K is represented as e≈l=0L-1clbl.

Proposal 1: For Type II (explicit) CSI reporting, a form of DL channel using a LC framework is reported, where

- the form of DL channel is at least one of channel, covariance matrix, and eigenvectors; and

- the LC framework to report explicit CSI as l=0L-1clbl comprises of the following:

  - Basis set: for WB selection of L basis vectors from a basis set

    - LTE Rel. 13/14 Class A W1 codebook and/or Rel. 14 Advanced CSI W1 codebook can be a good starting point for designing the basis set for NR.

  - Channel quantization codebook: for SB selection of LC coefficients for the selected L basis vectors.

    - The quantization codebook can be a scalar codebook (magnitude and phase or real and imaginary) or a vector codebook.

# Simulation Results

To illustrate the potential performance gains with explicit CSI reporting, simulation results are provided for the following CSI reporting schemes:

- Class A: CSI reporting is based on Rel. 13 Class A codebook (and its extension to > 16 antenna ports) with Codebook-Config = 1.

- LC Implicit: CSI reporting is based on linear combination (LC) codebook proposed in companion contributions [5-7], where Rel. 13 Class A W1 codebook corresponding to Codebook-Config = 3 is considered as the W1 codebook for LC codebook.

- LC Explicit: CSI reporting is based on proposed LC based reduced dimensional explicit CSI reporting. In particular, for each SB, the dominant eigenvector is represented as e≈B00Bc, where B=[b0, b1,…,b7] are L = 8 DFT beams forming (4,2) beam group as shown in Figure 2 and c is the LC coefficient vector. The un-quantized LC coefficients are assumed in the simulation which are obtained as the least-squared solution: c=B*B-1B*e.

Figure 2: LC explicit framework used in simulation

- Ideal: the dominant DL eigenvector(s) is (are) known at the gNB/TRP (performance upper bound).

The non-full-buffer system-level evaluation is carried out for UMa-200m and UMi-2GHz channel models in medium (50% target RU) traffic loading scenarios, and dynamic switching between SU-MIMO and MU-MIMO is considered in the simulation. For each scenario, three sets of results are provided for 16, 32, and 64 antenna ports with (N1,N2) = (2,4), (4,4), and (8,4), respectively, where we assume that the first dimension is horizontal and the second dimension is vertical. The detailed results are provided in Table 3 in the Appendix. The downtilt angles in the elevation domain are chosen according to [8]. In these simulations, partial-port non-precoded CSI-RS (proposed in [4]) associated with 16 antenna ports is used for CSI estimation, and the corresponding CSI-RS overhead is taken into account in the final throughput calculation. Note that the CSI-RS overhead is the same for 16, 32, and 64 antenna ports. An illustration of partial port CSI-RS is shown in Figure 3 in which the partial antenna ports carrying CSI-RS are shown in black. Cell association antenna pattern is approximated by one-TXRU pattern, and proportional fair scheduling (max 4 layers per time-frequency resource) have been used. For MU-MIMO, SLNR precoding is considered. The relevant simulation parameters are enlisted in Table 2. The rest of the simulation assumption is according to [8].

Figure 3: Partial port CSI-RS

The performance gains for 16 antenna ports with “Class A” as reference are shown in Figure 4 and Figure 5 for UMa-200m and UMi-2GHz, respectively. Similar results for 32 and 64 antenna ports are shown in Figure 6 – Figure 9. From these results, we can make the following observation.

Observation 1:

- Large performance gap between Rel. 13 Class A codebook based implicit and ideal CSI schemes; the gap reduces with LC codebook [5-7] based implicit CSI scheme; however, the performance gap is still large.

- LC based explicit CSI scheme, on the other hand, can reduce this gap significantly.

Figure 4: Performance gain: 16 ports, UMa-200m

Figure 5: Performance gain: 16 ports, UMi-2GHz

Figure 6: Performance gain: 32 ports, UMa-200m

Figure 7: Performance gain: 32 ports, UMi-2GHz

Figure 8: Performance gain: 64 ports, UMa-200m

Figure 9: Performance gain: 64 ports, UMi-2GHz

We also compare the performance of increasing number of antenna ports from 16 to 64 with Class A for 16 ports as reference. The comparison is shown in Figure 10 and Figure 11. We can make the following observation.

Observation 2: The performance gain with LC based explicit CSI scheme is maintained as number of antenna ports increases from 16 to 64.

Figure 10: Performance gain: UMa-200m

Figure 11: Performance gain: UMi-2GHz

# Conclusions

In this contribution, simulation results comparing implicit and explicit CSI reporting schemes for NR MIMO are presented. The observations and proposals made are summarized as follows.

Observation:

- Large performance gap between Rel. 13 Class A codebook based implicit and ideal CSI schemes; the gap reduces with LC codebook [5-7] based implicit CSI scheme; however, the performance gap is still large.

- LC based explicit CSI scheme, on the other hand, can reduce this gap significantly.

- The performance gain with LC based explicit CSI scheme is maintained as number of antenna ports increases from 16 to 64.

Proposal 1: For Type II (explicit) CSI reporting, a form of DL channel using a LC framework is reported, where

- the form of DL channel is at least one of channel, covariance matrix, and eigenvectors; and

- the LC framework to report explicit CSI as l=0L-1clbl comprises of the following:

  - Basis set: for WB selection of L basis vectors from a basis set

    - LTE Rel. 13/14 Class A W1 codebook and/or Rel. 14 Advanced CSI W1 codebook can be a good starting point for designing the basis set for NR.

  - Channel quantization codebook: for SB selection of LC coefficients for the selected L basis vectors.

    - The quantization codebook can be a scalar codebook (magnitude and phase or real and imaginary) or a vector codebook.

# References

- RAN1#86b, Chairman’s notes

- R1-1612500, “CSI acquisition for DL NR MIMO,” Samsung.

- R1-1612501, “CSI acquisition for DL NR MIMO,” Samsung.

- R1-1612502, “Advanced CSI feedback for NR,” Samsung.

- R1-1612415, “Linear combination codebook design framework,” Samsung.

- R1-1612416, “Rank 1 linear combination codebook and simulation results,” Samsung.

- R1-1612417, “Rank 2 linear combination codebook and simulation results,” Samsung.

- 3GPP, TR36.897v1.0.1, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”

# Appendix: Simulation Assumptions

Table 2: Simulation Parameters

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer (Medium load 50% Target RU, Lambda = 4) |
| Channel model | UMi-2GHz, UMa-200m |
| Number of BS (H,V) antenna elements | (8,8), x-polarized, subarray partition |
| (N1,N2, P) | 16, 32, 64 ports: (4,2,2), (4,4,2), (8,4,2), |
| (O1,O2) | (8,8) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2 |
| SU/MU pre-coding | SLNR |
| Scheduling | MU, Proportional fair, up to 4 layers |
| Channel estimation | Non-ideal |
| Transmission rank | 1,2 |
| Receiver | MMSE-IRC |
| CSI feedback schemes | Reference: Rel. 13 Class A Codebook-Config = 1 LC codebook: proposed in [5-7], Codebook-Config = 3 LC Explicit: 8 beams, (4,2) beam group Co-phase: QPSK for each beam Coefficient: Unquantized Ideal: dominant eigenvectors are known to the gNB/TRP |

Table 3: Summary of non-full-buffer simulation results

| Channel | #ports | Scheme | Avg. UPT | 50% UPT | 5% UPT | Avg. UPT gain | 50% UPT gain | 5% UPT gain | RU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UMa-200m | 16 | Class A | 18.84 | 16.00 | 5.56 | 100.0% | 100.0% | 100.0% | 59.6% |
|  |  | LC Implicit | 20.80 | 18.23 | 6.82 | 110.4% | 113.9% | 122.7% | 55.7% |
|  |  | LC Explicit | 25.87 | 22.73 | 9.47 | 137.3% | 142.0% | 170.3% | 48.9% |
|  |  | Ideal | 26.97 | 23.15 | 10.16 | 143.1% | 144.7% | 182.9% | 47.6% |
|  | 32 | Class A | 20.58 | 17.70 | 7.07 | 109.2% | 110.6% | 127.2% | 56.0% |
|  |  | LC Implicit | 22.30 | 20.10 | 8.27 | 118.3% | 125.6% | 148.7% | 52.8% |
|  |  | LC Explicit | 26.93 | 24.10 | 10.19 | 142.9% | 150.6% | 183.4% | 46.8% |
|  |  | Ideal | 29.56 | 27.03 | 11.45 | 156.8% | 168.9% | 206.0% | 43.9% |
|  | 64 | Class A | 20.83 | 17.89 | 6.76 | 110.5% | 111.8% | 121.7% | 55.4% |
|  |  | LC Implicit | 22.44 | 20.16 | 8.12 | 119.1% | 126.0% | 146.2% | 52.5% |
|  |  | LC Explicit | 26.79 | 24.26 | 10.15 | 142.2% | 151.6% | 182.6% | 47.2% |
|  |  | Ideal | 30.69 | 28.19 | 11.77 | 162.8% | 176.2% | 211.7% | 43.1% |
| UMi-2GHz | 16 | Class A | 18.69 | 15.81 | 5.91 | 100.0% | 100.0% | 100.0% | 60.6% |
|  |  | LC Implicit | 20.87 | 18.27 | 7.09 | 111.7% | 115.5% | 120.0% | 56.6% |
|  |  | LC Explicit | 27.41 | 24.67 | 9.46 | 146.7% | 156.1% | 160.1% | 48.6% |
|  |  | Ideal | 28.88 | 26.14 | 10.13 | 154.6% | 165.4% | 171.4% | 47.2% |
|  | 32 | Class A | 21.51 | 18.38 | 7.29 | 115.1% | 116.2% | 123.3% | 55.3% |
|  |  | LC Implicit | 23.33 | 21.02 | 8.54 | 124.8% | 132.9% | 144.5% | 52.6% |
|  |  | LC Explicit | 29.45 | 27.21 | 10.69 | 157.6% | 172.1% | 180.9% | 45.6% |
|  |  | Ideal | 32.67 | 32.13 | 12.08 | 174.8% | 203.2% | 204.3% | 42.6% |
|  | 64 | Class A | 21.74 | 18.86 | 7.20 | 116.3% | 119.3% | 121.9% | 54.7% |
|  |  | LC Implicit | 23.37 | 21.38 | 8.48 | 125.0% | 135.2% | 143.6% | 52.0% |
|  |  | LC Explicit | 29.23 | 27.21 | 10.80 | 156.4% | 172.1% | 182.8% | 45.2% |
|  |  | Ideal | 34.24 | 34.78 | 13.21 | 183.2% | 220.0% | 223.5% | 40.2% |