3GPP TSG RAN WG1 Meeting #88                                                                           R1-1707117

Hangzhou, P.R. China, 15th –19th May 2017

Source:               ZTE

Title:                    Multi-TRP Transmission and interference coordination

Agenda Item:      7.1.2.1.7

Document for:    Discussion and Decision

Introduction

In RAN1#88bis meeting, the following agreements on the DL transmission with multiple TRPs are also achieved [1]:

- Support NR reception of at least one but no more than two of the following

  - Single NR-PDCCH corresponding to the same NR-PDSCH data layers from multiple TRPs within the same carrier

    - Note that: this is intended to have spec impact

  - Single NR-PDCCH corresponding to different NR-PDSCH data layers from multiple TRPs within the same carrier

  - Multiple NR-PDCCH corresponding to different NR-PDSCH data layers from multiple TRPs within the same carrier

- In case of multiple NR-PDCCH, consider the following for the reduction of UE PDCCH detection complexity.

  - Note the following may or may not have RAN1 specification impact.

  - Note that different NR-PDSCH data layers from single TRP are special case.

  - The alignment of PDCCH generation rules among TRPs, e.g. one identical control resource set across TRPs

  - Signalling the maximum number of multiple NR-PDCCH reception via L1 and/or high layer signalling

  - Other techniques can be considered.

In this contribution, the detailed interference coordination schemes, e.g., interference randomization and multi-TRP transmission are elaborated. Initial evaluation results for non-coherent JT among TRPs are also provided.

Joint Transmission

As demonstrated by the observations of SLS results listed in [2], the coordination among TRPs would enhance the stability and data rate of the link, especially for improving the coverage of cell-edge UEs. Moreover, in the application of the above 6GHz with high probability of LoS situation, it is expected spatial multiplexing would be better supported via the joint transmission from multi-TRPs to achieve higher degree of freedom. According to the current WI in Rel-15, the non-coherent JT which is classified based on mapping of codewords to TPs as follows [3]:

Mapping of codewords (CWs) to TPs:

Case 1: Different CWs are transmitted from different TPs. Each TP perform adaptive precoding independently

Case 2a: The same CW is transmitted from different TPs with spatial diversity (e.g. SFBC) / spatial multiplexing

Case 2b: The same CW is transmitted from different TPs using SFN

In general, Case 1 requires less coordination among different TRPs compared to cases 2a and 2b in which the coordinated TRPs need to coordinate to transmit the same codeword.

In addition, three schemes of resource allocation on different TPs are considered:

Scheme 1: The resource allocations from different TPs for a UE are fully overlapped

Scheme 2: The resource allocations from different TPs for a UE are partially overlapped

Scheme 3: The resource allocations from different TPs for a UE are not overlapped

It is observed that scheme 1 often requires coordination to make sure it is fully overlapped and scheme 3 allows the case without much coordination.

We can consider these three cases of codewords to TP mapping and resource allocation schemes as the starting point of study on joint transmission for NR MIMO.  Considering this starting point, we list out some of the aspects for consideration on the standardization support for multi-TRP joint transmission.

- Beam management

In RAN1#86, it was agreed to strive for the same procedure design for intra-TRP and inter-TRP beam management [4]. And it was also agreed to study the coherent/non-coherent MIMO transmission based on uniform/non-uniform array structure at TRP or UE in RAN1#86bis [5].  With this agreement, MIMO transmission based on multiple panels at the same TRP or at different TPRs should share the same beam management procedure. Unified beam management framework should be considered for beam management from one or multiple TRPs.  In [6], the concept of beam grouping is introduced.  Beam grouping can be used for multiple TRP transmission so that beams are grouped based on channel properties.   Group based beam reporting and QCL based beam indication have been discussed in [7].  Same framework of beam reporting and indication should be supported for both multi-panel and multi-TRP systems.

- CW/CBG-to-layer mapping

In the case of multiple TRP transmission, different channel properties can be expected between each TRP to UE, which will lead to various numbers of ranks per channel. According to this phenomena, the dynamic CW to layer mapping should be supported in case of multiple layer transmission with 2CW, e.g., two CWs for the transmission with rank 5~8 as agreed. Moreover, for the case only one CW is supported, multiple CBGs within one CW can be transmitted from different TRP according to the layer mapping schemes that: each CBG can be mapped to one layer set which corresponds to one TRP.  In this way, it can be expected to minimize the re-transmission of the CBGs within one CW since the independent error probability can be achieved in per CBG manner based on the different channel condition.

- Control signaling on resource allocation

In LTE, resource allocation for one component carrier can be only signaled by one DCI.  In order to support three resource allocation schemes in NR, more flexible control signaling (e.g., single or multiple NR-PDCCH cases) should be considered. However, in order to minimize the type and size of DCI, and reduce the complexity for UE decoding, the transmission of the single or multiple DCI can be configured according to adopted RA schemes among NR-PDSCH data layers from multiple TRPs. For examples, in case 2b, it can be easily supported by the agreed single NR-PDCCH with one RA filed since the same NR-PDSCH with fully overlapped resource allocation is transmitted from multiple TRPs .

For the transmission with different NR-PDSCH data layers from different TRP, the same method (single DCI with one RA field) can be adopted for the implementation of case 2a and case 1 with fully overlapped RAs. Moreover, the multiple PDCCHs will be considered once the partial overlapping or non-overlapping scheme with discontinuous RA per CW is adopted among the different NR-PDSCH layers in case 1.

- MCS assignment

In LTE, MCS assignment is done per codeword i.e. both modulation and coding are the same across the same codeword. If partial overlapping scheme of resource allocation is adopted, different interference is experienced for the same codeword and hence using one MCS across the same codeword is not appropriate.   MCS assignment should be considered such case to adapt different interference situation in the same codeword.  E.g. different modulation schemes can be allowed.

- CSI feedback

In LTE, multiple CSI processes are used to support CSI feedback for multi-TRP transmission.  CSIs are derived mostly in independent manner for each TRP.  In NR, it is beneficial to consider aggregation of multiple CSI-RS for aggregated CSI feedback.  Design of aperiodic CSI can be more flexible so that multiple CSI-RS resources can be selected for channel measurement considering JT. CSI feedback considering different codeword to layer mapping should also be supported.

- Advanced receiver

In LTE, CWIC is considered as advanced receiver for non-coherent JT.  Advanced receiver such as successive interference cancellation (SIC) has the ability to eliminate the inter-codeword interference between JT TPs by subtracting correctly detected signal from received signal.  To deduce the actual CSI seen by SIC receiver, different interference assumptions can be applied for different codewords. Whether different interference assumptions are necessary and how to signal it needs to be studied.  One way of doing it is to put channel and interference measurement resource in a resource pool [9].

Following the discussion, we have the following proposals:

Proposal 1: Group based beam management is considered to support MIMO transmission based on multiple panels at the same TRP or at different TPRs should share the same beam management procedure.

Proposal 2: In case of multi-panel/TRP scenarios for ideal backhaul, the resource mapping scheme for the CBGs within one CW should be supported in per layer manner.

Proposal 3:  Aspects of flexible codeword-to-layer mapping, flexible resource allocation from multiple TRPs, MCS assignment, CSI feedback and advanced receiver should be studied to support multi-TRP transmission in NR.

Interference randomization and coordination

In addition, interference randomization can be considered in order to mitigate serious interference among TRPs. Specifically, for applications of beam-based RS, such as CSI-RS for P1 beam management and L3 mobility which may be periodically transmitted in some fixed subframes even for different cells.

As shown in Figure 1, once beam #0 of cell#0 and beam#0 of cell#1 collides, serious interference will always exist between these two beams if the beam transmission order is not changed at both cells. To avoid this situation, randomization of beam transmission order which is called beam randomization, could be considered as following cases:

- Multiple beams sweep across multiple slots:

It is a common situation at high frequency. After implementation of the proposed methods, beam order varied from slot#0 to slot#N.  As shown in Figure 2, the interference in the first OFDM symbol is changed to be that between beam#0 of cell#0 and beam#13 of cell#1.

- Only a few beams are transmitted in one slot:

It is a common situation at lower frequency. As illustrated in Figure 3, in one subframe, if only two beams are in each adjacent cell, different beam positions for adjacent cells can be implemented by beam randomization in order to avoid serious interference.

Figure 1 Illustration of beam index mapping in slot#0

Figure 2 Illustration of beam index mapping in slot#N

Figure 3 Illustration of beam randomization with fewer beams

Proposal 4: To randomize inter-cell interference, beam randomization should be considered in NR, especially for periodically CSI-RS.

Besides the beamforming techniques in Tx side, Rx beamforming may also play an important role for both improving Rx power and limiting interference, especially when the number of UE antenna elements are large in higher frequency bands. After selecting the best one from multiple receive beams, interference corresponding to other Rx directions can be restricted. Actually, Rx beamforming scheme is belonged to interference mitigation in spatial domain. Especially for uplink transmission, e.g. PUCCH feedback, eNB may receive multiple PUCCHs from different UEs in one resource via different receive beams. It is noted that if orthogonality cannot be achieved by different receive beams, orthogonal resources in other domain must be allocated to UEs, e.g. orthogonal resources in time domain, in frequency domain, or in code domain.

Proposal 5: Rx beamforming should be considered to mitigate interference in NR.

Performance evaluation

Figure 4 Indoor Hotspot

As shown in Figure 4, system level simulation is done to evaluate the performance benefits of non-coherent JT under the scenario of indoor hotspot with two different antenna configurations of UE, i.e., two and four Rx antennas.  Simulation assumptions can be found in the Appendix.  It can be observed that NCJT can provide performance gain comparing with the baseline with DPS/DPB.  In low load scenarios, NCJT provided tremendous gain in all regions of the UPT metrics.   High percentile UEs tend to have better gain.  It can reach as high as 50% gain for low to medium load scenarios.  For these UEs, the major benefit is coming from supporting more transmission layers with joint transmission, even with small number of maximum supported layer (up to 2).  NCJT provided consistent performance benefit for lower percentile UEs in different loading situations.  It has gain of 16-20% even for medium to high loads.  Interference mitigation by NCJT has the major effect on those interference-limiting UEs.

Table 1 Simulation results for 4Rx UEs

| FTP with pre-defined RU | TP coordination scheme | Mean UPT | 5% UPT | 50% UPT | 95% UPT | Real RU |
| --- | --- | --- | --- | --- | --- | --- |
| 5% | DPS/DPB | 52.09 | 28.55 | 56.34 | 56.34 | 6.64% |
|  | NCJT | 70.31 | 36.62 | 74.4 | 88.89 | 6.69% |
|  | Gain | 34.98% | 28.27% | 32.06% | 57.77% |  |
| 20% | DPS/DPB | 43.88 | 13.23 | 53.2 | 56.34 | 19.40% |
|  | NCJT | 53.56 | 15.68 | 56.14 | 84.26 | 18.46% |
|  | Gain | 22.06% | 18.52% | 5.53% | 49.56% |  |
| 40% | DPS/DPB | 32.66 | 3.07 | 31.01 | 56.34 | 37.99% |
|  | NCJT | 40.62 | 3.68 | 38.1 | 85.11 | 33.27% |
|  | Gain | 24.37% | 19.87% | 22.86% | 51.06% |  |
| 70% | DPS/DPB | 19.02 | 1.63 | 12.62 | 55.95 | 67.29% |
|  | NCJT | 21.73 | 1.89 | 14.37 | 68.29 | 66.45% |
|  | Gain | 14.25% | 15.95% | 13.87% | 22.06% |  |

Table 2 Simulation results for 2Rx UEs (RU ~80%)

| TP coordination scheme | Mean UPT | 5% UPT | 50% UPT | 95% UPT | RU |
| --- | --- | --- | --- | --- | --- |
| DPS/DPB | 5.638 | 0.6676 | 3.2051 | 18.6047 | 0.872 |
| NCJT | 6.7479 | 0.7027 | 3.413 | 21.978 | 0.854 |
| Gain | 19.69% | 5.26% | 6.49% | 18.13% |  |

Observation 1:  High potential gain can be observed from initial evaluation results of non-coherent JT with SIC receiver

Conclusions

This contribution provides initial evaluation results on multi-TRP transmission using non-coherent JT.  High potential gain can be observed from initial evaluation results of non-coherent JT.  It appears to have performance benefit to support multi-TRP schemes in NR.  To support multi-TRP transmission, we have the following observation and proposals.

Observation 1:  High potential gain can be observed from initial evaluation results of non-coherent JT with SIC receiver

Proposal 1: Group based beam management is considered to support MIMO transmission based on multiple panels at the same TRP or at different TPRs should share the same beam management procedure.

Proposal 2: In case of multi-panel/TRP scenarios for ideal backhaul,, the resource mapping scheme for the CBGs within one CW should be supported in per layer manner.

Proposal 3:  Aspects of flexible codeword-to-layer mapping, flexible resource allocation from multiple TRPs, MCS assignment, CSI feedback and advanced receiver should be studied to support multi-TRP transmission in NR.

Proposal 4: To randomize inter-cell interference, beam randomization should be considered in NR, especially for periodically CSI-RS.

Proposal 5: Rx beamforming should be considered to mitigate interference in NR.

- References

- 3GPP RAN1#88bis Chairman Notes

- R1-1703848, “WF on observations of SLS results for network coordination and advanced receivers”, Samsung, Huawei, Nokia, ZTE

- TR 36.741, “Study on Further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE”,

- 3GPP RAN1 #86 Chairman Notes

- 3GPP RAN1 #86bis Chairman Notes

- R1-1610891, “WF on Group-based Beam Management”, ZTE, ZTE Microelectronics, ASTRI, Nokia, ASB, CATT

- R1-1704398, “DL beam management for NR MIMO”, ZTE, ZTE Microelectronics

- 3GPP RAN1 #88 Chairman Notes

- R1-1704404, “On CSI framework details”, ZTE, ZTE Microelectronics

- R1-1611409, “Discussion on interference issues in NR”, ZTE, ZTE Microelectronics

- Appendix

Table A-1 System simulation assumptions for non-coherent JT evaluation

| Parameter | Values used for evaluation |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Duplex Mode | FDD |
| System Band | 10 MHz |
| Carrier Number | 1 |
| Network Synchronization | Synchronized |
| Antenna Configuration | Transmitter: 2Tx cross-polarized antenna Receiver: 2Rx and 4Rx at UE(cross-polarized antenna with 0.5λ antenna spacing) |
| Tx Power | 24 dBm |
| eNB antenna height | 6m |
| UE antenna height | 1.5m |
| Number of small cells | 8 |
| Minimum distance (2D distance) | Small cell-small cell: 20m |
|  | Small cell-UE: 3m |
| Traffic model | FTP Model 1 with packet size 0.5 Mbytes (low load ~5%, 20% RU,40% RU,70% RU) |
| Handover margin | 3dB |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Metrics | Mean, 5%, 50%,95% UPT |
| UE Speed | 3km/h |
| CQI/PMI reporting interval and frequency granularity | 5ms for CQI/PMI, 6RB |
| Feedback scheme | Rel-12 enhanced CSI feedback, PUSCH mode 3-2 2Tx codebook, 16Tx FD-MIMO codebook CQI, PMI and RI reporting triggered per 5ms Feedback delay is 5 ms |
| Transmission scheme | TM10, Two CSI processes, DPS/DPB/non-coherent JT with rank adaptation |
| Scheduler | Proportional Fair |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 REs per PRB |
| Backhual | Ideal backhaul |
| Receiver | MMSE-IRC and SIC |
| HARQ Scheme | Chase Combining |
| Maximum number of retransmissions | 3 |
| Channel estimation | Non-ideal modeling of channel estimation error modeling is used, based on CSI-RS for channel measurements, based on DMRS for data demodulation, based on IMR for interference measurement |