3GPP TSG RAN WG1 #89         																		R1-1707374

Hangzhou, P.R. China 15th – 19th May 2017

Source: 	Intel Corporation

Title:	Calibration results for Phase 2 NR MIMO link level calibration

Agenda item:	    7.1.2.5

Document for:	Discussion

# Introduction

Three calibration phases has been agreed:

-	Phase 1: Calibration is to check the link and system level channel models with basic beamforming behavior in terms of SNR/SINR distribution. Link level evaluation assumptions for this item can be found in R1-1701823. System level evaluation assumptions for this item can be found in R1-1703534.

-	Phase 2: Calibration is to check the link/system level performance through observing the metrics of BLER, spectrum efficiency and outage. Link level evaluation assumptions for this item can be found in R1-1703535. System level evaluation assumptions for this item can be found in R1-1703536.

-	Phase 3: Calibration is to check the additional link (if necessary)/system-level features including UE movement, UE rotation and channel blockage in terms of coupling loss, ASA, SINR and spectral efficiency. System level evaluation assumptions for this item can be found in R1-1701828.

In this contribution, we provide phase 2 link level calibration results.

# Calibration Results

Results from figure 1-2 are for 30GHz results.

Figure 1 BLER w/ beamforming @ CDL-A, 30GHz

Figure 2 BLER @ CDL-B, 30GHz

Results from figure 3-4 are for 4GHz results.

Figure 3 BLER w/ beamforming @ CDL-A, 4GHz

Figure 4 BLER @ CDL-B, 4GHz

# Conclusion

In this contribution, we provided phase 2 link level calibration results.

# References

- R1-1703535, Evaluation assumptions for Phase 2 NR MIMO link level calibration, ZTE, ZTE Microelectronics

# Appendix

Table 1 Simulation assumptions for Phase 2 link-level calibration

|  | Below 6GHz | Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 4 GHz | 30 GHz |
| Subcarrier Spacing | 15kHz | 60kHz |
| Data allocation | 8 RBs - Note: Error free PDCCH decoding is assumed. - First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel - One-port DMRS (12 REs) |  |
| Channel Model | CDL-A and CDL-B - delay spread =100ns - UE speed=3km/h. - The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in Section 7.7.5.1 in TR 38.900 accordingly. |  |
| TXRU mapping to antenna elements | One TXRU per vertical dimension per polarization | One TXRU per panel per polarization |
| TXRU mapping weights | TXRU virtualization only in the vertical dimension (i.e. 1D virtualization) using DFT, i.e., 1D sub-array partition model defined in TR36.897 | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming1). |  |
| BS antenna configurations | (M,N,P,Mg,Ng) = (4,4,2,1,1). (dV,dH) = (0.5, 0.5)λ | (M,N,P,Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (2.0, 4.0)λ |
| BS antenna element radiation pattern | According to TR36.873 | See Table A.2.1-6 in TR 38.802 |
| UE antenna configurations | (M,N,P,Mg,Ng) = (1,1,2,1,1) | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. 2) Θmg,ng=90; Ω0,1=Ω0,0+180; |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |  |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |  |
| UE antenna element radiation pattern | Omnidirectional | See Table A.2.1-8 in TR 38.802 |
| BF scheme | Analog BF based on beam selection + Digital BF based on ideal SVD |  |
| MIMO mode | SU-MIMO with rank=1 |  |
| UE receiver type | MMSE-IRC |  |
| MCS | LTE MCS = 1 |  |
| DMRS channel estimation | 2DMMSE Based on LTE antenna port 7 |  |
| Metrics | BLER w/ beamforming as a function of transmission SNR ranging from -20 dB to 0dB |  |

- Note1: the DFT beam candidate in this beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows:

, for i=1,…,rN

where r =1 (which is analogous to oversampling factor of 1) , N denotes the number of vertical/horizontal antennas.

- Note2: The polarization angles are 0 and 90 and the panel with the best receive SNR is chosen for output metric. i.e. no combining is done between panels for above 6GHz. (Ωmg,ng, Θmg,ng)  is for orientation of the panel (mg, ng), 0≤mg<Mg, 0≤ng<Ng,  where the orientation of the first panel (Ω0,0, Θ0,0) is the same as UE orientation, Ωmg,ng is the array bearing angle and Θmg,ng is the array downtilt angle defined in [TR 36.873].