3GPP TSG RAN WG1 Meeting #89                                                                         	  R1-1707529

Hangzhou, China 15th  – 19th  May 2017

# Source:              CATT

Title:                   PUSCH Power Control In Multi-Beam Configuration

Agenda Item:     7.1.9

Document for:   Discussion and Decision

Introduction

In RAN1#88bis, the beam-specific power control was agreed as follows,

Agreements:

- For beam specific power control, NR defines beam specific open & closed loop parameters.

  - FFS: details on beam common parameter(s)

  - Note: Agreed on RAN1 #88 FFS details on “beam specific”, especially regarding handling layer/layer-group/panel specific/beam group specific/beam pair link specific power control

- gNB is aware of the power headroom differences for different waveforms, if the UE can be configured for both waveforms.

  - FFS: offset configured/specified, reported,

  - FFS on the details of power control parameters for example, P_c, Max or other open/closed loop parameter

In this contribution, we discuss the general framework of UL power control for dynamic beam switching in multi-beam configuration.

Power Control Framework for Multi-beam Configuration

UL power control is designed to set the transmit power to meet the desired QoS at the reception point(s) but minimize the interference to the neighboring reception points.  The UL transmit power is also adjusted with the Tx bandwidth, service types and closed-loop command.   For multi-beam configuration, the number of reception points could be more than one.   The UL transmit power would be adapted to the target reception point(s) for link adaptation gain.

Consideration of NR PUSCH Power Control In Multi-Beam Configuration

The principle of fractional power control is to set the transmit power in compensation of fractional path loss of specific link in achieving the target SINR and minimizing the interference to the neighboring receiving points.  In LTE, the power control formula is as follows.

The transmit power is set to achieve the target SINR (PO_PUSCH) with fractional compensation of path loss (α).

- is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks valid for subframe i.

- The target power consists of a cell specific nominal component and provided from higher layers for j=0 and 1 and a UE specific component  provided by higher layers for j=0 and 1.

- is the scaling factor of fractional power control

- is the downlink path loss estimate calculated in the UE  in dB.

- for and 0 for where  is a UE-specific parameter for different type of traffic, such as UCI on PUSCH,  provided by higher layers

if accumulation is enable and if accumulation is not enabled.    is a UE specific correction value through close-loop power control indicated by DCI.    is the offset with respect to the DL subframe where the UL grant was received

The general principle of power control in LTE takes into account all factors, such as the target SINR for the specific services, UL transmit bandwidth, the path loss to the serving cell, fractional power control with higher layer configured parameters, adjustment for different transport format (UCI), and close-loop power correction.    The design principle of power control in NR should also consider these factors with additional variables in consideration as follows,

- Different Numerologies – NR support different numerologies and operation of mixed numerologies in FDM/TDM.   It was agreed that one PRB contains 12 REs in frequency regardless what the numerology is.   When UL transmission is configured with different subcarrier spacing to the default subcarrier spacing, the bandwidth of PUSCH resource would be increased/decreased in proportion to the increase/decrease of subcarrier spacing.  UL power control should contain the adjustment factor  for subcarrier spacing different to the default subcarrier spacing.   The adjustment factor for different numerology is defined as follows,

where is the configured subcarrier spacing for UL transmission and is the default subcarrier spacing used for PRACH in the initial access.

Proposal 1: NR power control should include adjustment factor  when the numerology configured for UL transmission is different from the default numerology

- Different reliability requirements –  NR supports services with different reliability requirements, such as URLLC and eMBB.  Thus, SIR target would be set differently for services with different reliability requirements.   Target power is set based on target SINR and consists of cell specific nominal component and UE specific component.  The cell specific nominal component is specified for power setting of cell specific operation, such as RACH Msg3 power.  The UE specific component is used to set different target based on the UE-specific operation, e.g., SPS retransmission.   In order to support services with different reliability requirements, the UE specific component should be set to different value.

Proposal 2:   In order to support services with different reliability requirements, such as eMBB and URLLC, the UE specific component should be set to different value.

- Multiple Access Scheme – NR would support different UL multiple access schemes, such as scheduled based orthogonal multiple access (SB-OMA), grant free access, and non-orthogonal multiple access (NOMA).  The interference level of different multiple access schemes would be different.  Thus, the setting of target power needs to adapt to the interference level of each multiple access scheme in order to meet the target SINR.   Since multiple access scheme is a cell specific operation, the nominal component  of target power should be set to different value to adapt for different interference level of each multiple access scheme.

Proposal 3:  NR would support different UL multiple access schemes, such as scheduled based orthogonal multiple access (SB-OMA), grant free access, and non-orthogonal multiple access (NOMA).  The nominal component of target power should be set to different value to adapt for different interference level of each multiple access scheme.

- CP-OFDM and DFT-S-OFDM based waveforms - It was agreed in RAN1#86bis that both CP-OFDM and DFT-S-OFDM based waveforms are supported, at least for eMBB uplink for up to 40GHz.   CP-OFDM and DFT-S-OFDM based waveforms have different PAPR values.   To operate UE linear amplifier (LNA) for both waveforms without signal distortion, the maximum power for CP-OFDM waveform with high PAPR should be back off with factorbased on the measurement.  The value of the back off factor is determined by the difference of average PAPR between the measured PAPR of configured waveform and the average of reference DFT-S-OFDM waveforms.

Proposal 4:   The maximum power should include the back-off factor for UL transmission with configured waveform based on measured PAPR with respect to the average PAPR of DFT-S-OFDM waveform

- Multi-beam configuration – In multi-beam configuration, UE would detect one or more DL beams above the detection threshold.  For IDLE mode UE, UE would select a DL beam for initial access.  For CONNECTED mode UE, UE could dynamically adapt to one of multiple detected beams with best channel quality for DL data services.   Similarly, UE could dynamically associate with a selected DL beam for UL PUSCH transmission with DL/UL beam correspondence as shown in Figure 1.   For beam specific power control, it was agreed in RAN1#88bis that NR defines beam specific open & closed loop parameters.   Power control would take UL link adaptation into account in multi-beam configuration.   Power controls for two types of dynamic UL link adaptation are discussed in the following,

Figure 1: Illustration of UL link adaptation in multi-beam configuraiton

  - Dynamic UL link adaptation among beams -  For multi-beam configuration, the path loss function  is associated with multiple measured path losses derived from individual, which is measured from beam m of M detected beams of the serving cell c, where  .  The path loss function would be set to individual path loss  if the close-loop power correction factoris not accumulated.  If the close-loop power correction factoris accumulated from previous one, the close-loop power control is operated to adapt the single radio channel for link quality adaptation.  The path loss function needs to follow a single path loss of a selected beam as the reference link.  The path loss function of UL dynamic link adaptation of any DL beam m is calculated based on the reference path loss and additional path loss deviation of  path loss of beam m () comparing to the path loss of reference link ( )

where is the correlation factor between beams configured by higher layer and .   If the selected beam m is in the same TRP with the reference beam, the correlation factorcould be set close to 1.

Proposal 5:   Path loss function should be based on single reference link with additional deviation function when multiple path losses are measured from multiple detected beams in multi-beam configuration.

  - UL Joint reception – When multiple beams are configured with little or no DL/UL beam correspondence, e.g., each beam transmitted from each TRP as shown in Figure 1, UL transmission could be received at multiple reception points/beams.  The path loss function  would be defined as joint optimization of multiple measured path losses  Power control for joint reception of multiple reception points/beams in the cooperative set is to achieve the target SINR but minimize the interference to neighboring beams not in the cooperative set.    The optimization of the path loss function could be defined as follows,

where the path loss compensation is based on the minimum of all path loss measurement.   This optimization of joint reception is to ensure at least one reception point/beam getting desired power for the path loss compensation and minimize the interference to other received points/beams not in the cooperative set.  The received signals at other reception points/beams would be the enhancement after maximum ratio combining.     The other power control optimization of joint reception is based on the maximum path loss of multiple measured path losses to ensure each reception points/beams getting proper power compensation to achieve target SINR as follows,

Another power control optimization for joint reception is a weighted combination of the path losses as follows,

The weighting factor could be equal or configured by higher layer for bias compensation toward some reception points/beams.

Proposal 6:   Path loss function for joint reception at multiple reception points/beam could be optimized based minimum, maximum or weighting average of multiple path losses measured from multiple detected points/beams in the cooperative set in multi-beam configuration.

PUSCH Power Control Formula

After taking all system analysis of NR power control, NR PUSCH power control formula is summarized in the following,

The transmit power is set to achieve the desired power (PO_PUSCH) set by the target SINR with fractional compensation of path loss (α).

- is the  back off factor of operating waveforms , such as  CP-OFDM and DFT-S-OFDM waveforms, of serving cell c at slot i.

  - j=0,  is the back off factor of measured PAPR of configured DFT-S-OFDM waveform

  - j=1,  is the back off factor of measured PAPR of configured CP-OFDM waveform

- is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks valid for slot i.

- is the numerology adjustment factor when the configured numerology of UL PUSCH transmission in slot i  is different to the default numerology.

- The target power consists of a cell specific nominal component and provided from higher layers and a UE specific component  provided by higher layers for j=0 and 1.

  - For cell specific nominal component  configured by higher layer,

    - j=0,  is the power setting based on the SINR target associated with scheduled-based orthogonal multiple access (SB-OMA)

    - j=1,  is the power setting based on the SINR target associated with grant free multiple access (GRMA)

    - j=2,  is the power setting based on the SINR target associated with non-orthogonal multiple access (NOMA)

  - For UE specific component configured by higher layer,

    - j=0, is the power setting based on the SINR target associated with QCI index based on eMBB service reliability requirements

    - j=1, is the power setting based on the SINR target associated with QCI index based on URLLC service reliability requirements

- is the scaling factor of fractional power control configured by higher layer

- is the path loss function.  For multiple path loss measurements from multiple detected beams in multi-beam configuration, path loss function is to support dynamic link adaptation or joint reception.  Path loss function of serving cell C could be defined as follows,

  - For dynamic link adaptation,

    - j=0, is theof the target beam m of detected beams {1, 2, …., M} when the close-loop power correction factoris not accumulated

    - j=1,  is based on the path loss of reference link and the deviation of the desired link when the close-loop power correction factoris accumulated

and is the correlation factor between beams configured by higher layer and .

  - For joint reception at multiple reception points/beams,

    - j=2,

    - j=3,

    - j=4,  where the weighting factor could be equal or configured by higher layer for bias toward selected reception points/beams

- is a UE-specific parameters for different type of traffic, such as UCI on PUSCH,  provided by higher layers

- is the power correction by close-loop power control.  if accumulation is enable and if accumulation is not enabled.    is a UE specific correction value through close-loop power control indicated by DCI.    is the offset with respect to the DL slot where the UL grant was received.

Conclusion

In this contribution, we discuss PUSCH power control for NR based on LTE fractional power control with additional considerations of mixed numerologies, services with different reliability requirements, different multiple access schemes, CP-OFDM and DFT-S-OFDM waveforms, and link adaptation/joint reception in multi-beam configuration.  Based on the discussion, we have the following proposals:

- Proposal 1: NR power control should include adjustment factor  when the numerology configured for UL transmission is different from the default numerology

- Proposal 2:   In order to support services with different reliability requirements, such as eMBB and URLLC, the UE specific component should be set to different value.

- Proposal 3:  NR would support different UL multiple access schemes, such as scheduled based orthogonal multiple access (SB-OMA), grant free access, and non-orthogonal multiple access (NOMA).  The nominal component of target power should be set to different value to adapt for different interference level of each multiple access scheme

- Proposal 4:   The maximum power should include the back-off factor for UL transmission with configured waveform based on measured PAPR with respect to the average PAPR of DFT-S-OFDM waveform

- Proposal 5:   Path loss function should be based on single reference link with additional deviation function when multiple path losses are measured from multiple detected beams in multi-beam configuration.

- Proposal 6:   Path loss function for joint reception at multiple reception points/beam could be optimized based minimum, maximum or weighting average of multiple path losses measured from multiple detected points/beams in the cooperative set in multi-beam configuration.

We proposed NR PUSCH power control formula for the serving cell c as follows,

References

- R1-1706720	, “Summary of Issues on Power Control”, 	ZTE

- R1-1706841, “WF on uplink power control”, Nokia, ASB, InterDigital, OPPO, NTT DOCOMO, Intel, ZTE,Huawei, Ericsson, LGE