3GPP TSG RAN WG1 Meeting #89		                      R1-1707791

Hangzhou, China 15th - 19th May 2017

Agenda Item:  7.1.3.1.1

Source:	Spreadtrum Communications

Title:	Discussion on NR-PDCCH structure

Document for:	Discussion and decision

# Introduction

Following agreements were made in WG1 meeting #88bis [1]:

Agreement:

NR-PDCCH can be mapped contiguously or non-contiguously in frequency with localized or distributed mapping of REGs to a CCE (in the physical domain)

- Note: The number of contiguous REGs in the CCE needs further discussion.

- Note: Localized/distributed mapping can be achieved without/with interleaving.

Agreements:

- A CCE may be mapped to REGs with interleaved or non-interleaved REG indices within a CORESET

- Definition of a REG bundle: The UE may assume that the same precoder is used for the REGs in a REG bundle and that the REGs in a REG bundle are contiguous in frequency and/or time

- REG bundling per CCE is supported for NR-PDCCH

  - FFS: Whether this applies to common search space

  - FFS: Whether all REGs have DMRS or not

  - FFS: Whether wideband precoding is supported and the definition of a REG bundle if it is supported

  - FFS: whether REG bundle size is different for mapping of NR-PDCCH with or without interleaved mapping of CCE to REGs

  - FFS on REG bundle size

  - FFS whether REG bundle size is configurable

Working assumption:

  - A NR-CCE is defined as 6 REGs

    - Candidate bundle sizes for distributed REG-to-CCE mapping: 2 or 3 REGs if NR-CCE is defined as 6 REGs

  - FFS: impact of the NR-CCE definition on CORESET size, CCE aggregation levels, data resource allocation granularity, etc.

# NR-PDCCH structure

Based on the current agreements, the NR-PDCCH “logical” structure is shown in the following figures assuming that the NR-CCE consists of 6 NR-REGs with 2 and 3 NR-REG bundling. Logical structure means that the figures do not take into account the physical position of NR-REG bundles either in time or in frequency.

Figure 1 Localized NR-REG to NR-CCE mapping

Figure 2 Distributed NR-REG to NR-CCE mapping

It was agreed that NR-PDCCH can be mapped contiguously or non-contiguously in frequency with localized or distributed mapping of NR-REGs to a NR-CCE (in the physical domain). Contiguous or non-contiguous mapping can be implemented by mapping logical NR-REG bundles to physical NR-REG bundle positions in NR-resource grid.

In both localized and distributed NR-REG to NR-CCE mappings there are three basic parameters to be implicitly or explicitly signaled to UE:

- Bundling size

- Frequency or time-first NR-REG to NR-CCE mapping direction

- Physical NR-REG bundle positions in NR-resource grid

Figure 1 describes three different logical to physical resource mappings (CASE A, CASE B and CASE C). In CASE A, NR-REG bundles can be mapped to any available PRBs in control region. This means that those resources must also be pointed out for terminals blind decoding NR-PDCCH (e.g. in system information). In CASE B and CASE C, NR-REG bundles are spanned symmetrically to the available system bandwidth and it would enough point out the used allocation pattern.

If NR-REG bundle positions for NR-REG to NR-CCE mapping to NR-resource grid are not restricted, i.e. symmetrical mapping to physical recourses is not used, the distributed mapping will complicate the configuration of the used physical recourses. Symmetrical mapping of NR-REGs to physical recourses would also help NR-PDCCH reuse for possible different NR RBG sizes. Allocation patterns could be fitted to NR-RBG size, so that there is no need to separately assign PDSCH reused in control region.

Figure 3 Examples of “symmetrical” physical mapping supporting different PDSCH NR-RBG sizes in control region reuse.

Observation: Distributed NR-REG to NR-CCE mapping will determine the upper limit for the signaling of the used physical recourses (physical NR-REG bundle positions in resource grid).

Proposal: Symmetrical NR-REG bundle to physical recourses mapping should be used in NR.

One of the RAN WG1 meeting #88bis agreement was that NR-CCE may be mapped to NR-REGs with interleaved or non-interleaved NR-REG indices within a CORESET. It was questioned whether NR-REG bundle size is different for mapping of NR-PDCCH with or without interleaved mapping of NR-CCE to NR-REGs.

In LTE interleaving for PDCCH is done REG wise. In LTE used resources for PDCCH among adjacent cells are more probably interfering each other than what will be in NR due to EPDCCH like CORESET definition. If NR-REG bundling is used, one possible solution for interleaving could be NR-REG bundle wise interleaving.

Observation: Interleaving could also be done in NR-REG bundle wise.

# Control resource set definition

Control resource set (CORESET) is similar to the EPDCCH-PRB-Set in LTE (LTE PDCCH has not “set definition”). The CORESET size in time and/or frequency domain is determined  at least according to required maximum aggregation level, the size of NR-REG in resource elements, NR-CCE size in NR-REGs and DCI payload: as a reference the maximum aggregation level for LTE PDCCH is 8, REG size 4 REs, CCE size 9 REGs and maximum payload size around 70 bits for 100 PRB bandwidth.

Table 1 below compares the available number of CCEs/NR-CCEs and resource elements available for different bandwidths in case of LTE PDCCH and NR. 2 OFDM and 3 OFDM symbols allocations correspond to time first allocation of NR-REGs to NR-CCE depicted in Figure 1 and Figure 2. In NR case 33 % DMRS overhead in PRB allocation and 6 NR-REGs per NR-CCE are assumed. LTE PDCCH columns in Table 1 include in addition to the reference symbol overhead also the allocation of PCFICH and PHICH.

Multiple CORESET definition has been discussed as a means to adapt static or rapid channel variations as well as it can be utilized in COMP scenarios. Exemplary CORESET #1 can be configured for localized transmission and CORESET #2 as a fall-back for distributed transmission. Legacy LTE defines 2 EPDCCH-PRB-Sets for enhanced control channel but does not have such a PRB-Set definition for PDCCH.

In order to estimate applicability for existence of multiple CORESETs, the number of available NR-CCEs has to be calculated. Those are listed for different system bandwidths in Table 1. After the maximum NR-CCE number is known, it has to be divided between two CORESETs. Table 2 - Table 4 list all possible CORESET maximum aggregation levels for the lowest system bandwidths thought to be supported for NR.

Table 1 Maximum number of available CCEs/NR-CCEs

Table 2 Possible CORESET sizes for 25 PRB bandwidth

Table 3 Possible CORESET sizes for 50 PRB bandwidth

Table 4 Possible CORESET sizes for 100 PRB bandwidth

For NR-PDCCH aggregation levels up to 8 will be used but also ALs 16 and 32 have been mentioned. From the resource allocation point of view combining the PDCCH control region and EPDCCH like PRB-Set concept does not allow same degrees of freedom than in LTE. In order to support 2 CORESETs with AL 8 at least 50 PRB and 2 OFDM symbols (with time first or frequency first NR-REG to NR-CCE mapping) are required.

Observation: Two CORESET configuration cannot be supported and used as a backup method with all system bandwidths and control region sizes to time dimension.

Common search space (CSS) candidates reserve candidates from two biggest aggregation levels in LTE. If group common search space candidates are transmitted together UE specific candidates within the USS resources they will most probably be transmitted also with the biggest ALs available in analog beam. This increases blocking probability or prevents sending UE specific PDCCHs especially when hierarchical search spaces are used. Thus in challenging channel conditions, when e.g. gNodeB wants to transmit simultaneously downlink assignment, uplink grant and UE group common control, aggregation levels up to 16 are needed.

Observation: Aggregation level 16 should be supported in NR.

# Conclusions

In this paper, following observations and proposal were presented:

Observation: Distributed NR-REG to NR-CCE mapping will determine the upper limit for the signaling of the used physical recourses (physical NR-REG bundle positions in resource grid).

Proposal: Symmetrical NR-REG bundle to physical recourses mapping should be used in NR.

Observation: Interleaving could also be done in NR-REG bundle wise.

Observation: Two CORESET configuration cannot be supported and used as a backup method with all system bandwidths and control region sizes to time dimension.

Observation: Aggregation level 16 should be supported in NR.

# Reference

- 3GPP TSG RAN WG1 Meeting #88bis, “RAN1 Chairman’s Notes”