3GPP TSG RAN WG1 Meeting #89	R1-1707964

Hangzhou, China 15th – 19th May 2017

Agenda item:	7.1.2.3.5

Source: 	Samsung

Title: 	Differential reporting for Type II CSI

Document for:	Discussion and Decision

# Introduction

In RAN #75, a new WID related to new radio (NR) has been approved [1]. The objective of the NR WID as captured in [1] includes the following (copied from Section 4.1 of [1]).

According to the outcome of the study item, the NR should specify the followings [TR38.912]: - Physical layer aspects including [RAN1]: - … - Downlink and uplink functionality related to multi-antenna transmission/reception enabling closed loop and open/semi-open loop transmissions, beam management, interference measurement, Type I codebook-based CSI acquisition and Type II CSI acquisition as well as CSI acquisition for reciprocity-based operation, the associated reference signal designs, and related quasi-colocation assumptions.

In RAN1-NR#1 [2] and RAN1#88 [3], the following agreements about Type II CSI acquisition (highlighted text above) were made.

Agreements:

- Support at least one scheme taken from Category 1, 2, and/or 3 for Type II CSI

  - Possible down selection can be performed throughout Phase I WI

    - If more than one schemes is supported, these schemes should be complementary

    - This includes further refinement within each category

    - Note: other schemes within each category are not precluded

  - Descriptions for Category 1 and 2 are given in the following slides

    - For the purpose of summary in TR38.802

- Category 1: precoder feedback based on linear combination codebook

- Dual-stage W = W1W2 codebook

  - W1 consists of a set of L orthogonal beams, e.g. 2D DFT beams

    - The set of L beams is selected out of a basis, e.g. oversampled 2D DFT beams

    - Beam selection is wideband

  - W2: L beams are combined in W2 with common W1

    - Subband reporting of phase quantization of beam combining coefficients

  - Beam amplitude scaling quantization can be configured for wideband or subband reporting

    - …

Agreements:

- Refine the description in 38.802 for Type II CSI Category I as follows

  - Dual-stage W = W1W2 codebook

    - W1 consists of a set of L orthogonal beams taken from 2D DFT beams

      - The set of L beams is selected out of a basis composed of oversampled 2D DFT beams

        - L {2, 3, 4, FFS 6} (L is configurable)

      - Beam selection is wideband

    - W2: L beams are combined in W2 with common W1,

      - Subband reporting of phase quantization of beam combining coefficients

        - Configurable between QPSK and 8-PSK phase related information quantization

- …..

The codebook for Type II CSI reporting (highlighted text above) is proposed in companion contribution [4]. This contribution addresses the large reporting payload of Type II CSI and proposes a differential Type II CSI reporting mechanism in which payload is reduced in each CSI reporting instance. Simulation results are provided to show that the proposed differential Type II CSI reporting mechanism maintains good performance with lower CSI reporting payload in each reporting instance.

# Differential Type II CSI Reporting Mechanism

Table 1: W2 phase quantization payload (8PSK phase quantization)

| Number of beams (L) | Rank 1 (#bits/SB) | Rank 2 (#bits/SB) |
| --- | --- | --- |
| 2 | 9 | 9 × 2 = 18 |
| 4 | 21 | 21 × 2 = 42 |
| 6 | 33 | 33 × 2 = 66 |
| 8 | 45 | 45 × 2 = 90 |

One-shot reporting of Type II CSI can amount to significantly large reporting payload. For the Type II CSI codebook proposed in [4] (Category 1, i.e., pre-coder quantization), a summary of W2 phase quantization payload bits for 8PSK phase quantization codebook and varying number of beams (L values) is shown in Table 1. The payload for amplitude quantization is in addition to what is shown in Table 1. The motivation of supporting Type II CSI in NR is to provide high-resolution CSI to gNB, which can only be achieved with high rank (e.g. rank 2) transmission with sufficiently large number of beams for combination. However, as shown, the phase reporting payload increases significantly for rank 2 CSI reporting as the number of beams increases. Such large phase reporting payload sizes may not be supported in a single reporting instance because phase reporting is per SB.

To reduce payload, a differential CSI reporting mechanism is proposed in which the W2 payload is reduced by breaking it into T > 1 CSI reporting instances using differential CSI approach in which T > 1 coarse or lower-resolution CSIs are reported in T CSI reporting instances, where

- each CSI is self-decodable (independent of other CSIs); and

- an aggregate (linear sum) of T lower-resolution CSIs results in a higher-resolution CSI.

An example is shown in Figure 1 in which, the CSI corresponding to Lt = 2 < L beams (a subset of L W1 beams) is reported in the t-th CSI reporting instance of T = L/2 reporting instances. Two examples (L = 4 and 8) are shown in Figure 1. Note that the CSI reporting overhead is approximately the same in each CSI reporting instance. Note also that the CSI reported in each reporting instance in itself is ‘coarse’ or ‘lower-resolution’ Type II CSI (corresponding to Lt = 2 beams) and the aggregate of CSIs reported in T reporting instances is ‘refined’ or ‘higher-resolution’ Type II CSI (corresponding to L = 4 or 8 beams). Each CSI reporting instance is self-decodable and comprises a valid (lower-resolution) pre-coder which can be used by the gNB without other CSI reporting instances.

Such a differential approach can also be used for phase quantization.

Figure 1: Differential CSI reporting

Beam pairs for differential CSI

Figure 2: Beam pairs for T = L/2 differential CSI reports

To derive each of T = L/2 differential CSIs, two beams are selected from the basis matrix Bl for layer l, the construction of which is explained in the companion contribution [4].

- Bl is a N1N2×L basis matrix bkl,10,kl,20,..,bkl,1L-1,kl,2L-1 common to both polarizations, where

  - bkl,1i,kl,2i is one of the L orthogonal DFT beams selected from the selected (L1,L2) basis set,

  - {kl,1i,kl,2i:i=0,1…,L-1} are corresponding indices of L beams where kl,10,kl,20 is the strongest beam for layer l.

For brevity, L columns of the basis matrix Bl=bkl,10,kl,20,..,bkl,1L-1,kl,2L-1 are denoted as b0,..,bL-1. In particular, the pairs of selected beams correspond to 2 adjacent beams or columns of the basis matrix starting from the left or beam b0, i.e., beam pairs (b0, b1), (b2, b3), and so on. An illustration of the beam pairs are shown in Figure 2 for L = 4, 6, and 8.

Configuration and reporting of differential CSI

A UE is configured with a WB CSI report comprising of

- orthogonal basis set, L selected beams, and the strongest beam (per layer) using W1 codebook (these three together determines the basis matrix Bl for layer l), and

- RI;

and T = L/2 differential CSI reports (T SB reports) comprising of the following components.

- 1st CSI report (t = 0) comprises of a SB report to indicate amplitude and phase of coefficients using W2 codebook for L0 = 2 beams which are the first two columns (from left) of the basis matrix Bl for layer l (Figure 2). The number of coefficients (amplitude and phase) to be reported in the 1st CSI report is 2Lt - 1 = 3.

  - Since the 1st column of the basis matrix Bl corresponds to the strongest beam, which is used to derive to 1st CSI report, the coefficient corresponding to the strongest beam is not reported (it is fixed to 1).

- Remaining CSI reports (t = 1, 2, …,T-1) comprise of SB reports to indicate amplitude and phase of coefficients using W2 codebook for Lt = 2 beams (where t > 1) which are the (2t + 1)-th and (2t + 2)-th columns (from left) of the basis matrix Bl for layer l (Figure 2). The number of coefficients (amplitude and phase) to be reported in each of the remaining CSI reports is 2Lt = 4.

The UE assumes the last reported WB CSI report (which includes Bl) to select 2 beams in T CSI reports. If amplitude reporting is WB or WB + SB, then T WB amplitude components are reported in their respective reporting instances.

Proposal: To reduce Type II CSI reporting payload, a differential CSI reporting mechanism is supported wherein

- T = L/2 coarse yet self-decodable CSIs are reported in T CSI reporting instances, where the CSI reported in each reporting instance is derived using two beams that correspond to adjacent beam or column pairs, (b0, b1), (b2, b3),…, of the basis matrix Bl proposed in [4] are considered (cf. Figure 2).

- A UE is configured with a WB CSI report to report (orthogonal basis set, L selected beams, and per layer strongest beam), and T = L/2 SB differential CSI reports to report phase and amplitude of coefficients corresponding to 2 beams in each of T CSI reports.

# Simulation results

For performance evaluation of the proposed differential Type II CSI reporting, the non-full-buffer system-level evaluation is carried out for UMi channel model in medium (50% target RU) traffic loading scenario, and dynamic switching between SU-MIMO and MU-MIMO is considered in the simulation. The results are provided for 16 and 32 antenna ports with (N1, N2) = (4, 2) and (4, 4), respectively, where we assume that the first dimension is horizontal and the second dimension is vertical. The relevant simulation parameters are enlisted in Table 3.

For comparison, the Rel. 14 advanced CSI codebook is considered as reference, and ideal CSI in which dominant eigenvectors are known at gNB and Type II CSI codebook with unquantized combining coefficients (unquantized amplitude and phase) are considered as performance upper bounds.

The following two Type II CSI reporting mechanisms are compared:

- One-shot: the Type II CSI for L beams is reported in a single reporting instance , as proposed in [4].

- Differential: T = L/2 differential CSIs are reported in T consecutive reporting instances, i.e., in each reporting instance, Type II CSI corresponding to 2 beams is reported (cf. Section 2).

Both phase and amplitude quantization codebooks are 3 bits, where phase quantization is SB, and amplitude quantization is WB. Two L values, L = 4 and 8, are considered in this evaluation. The summary of W2 reporting payload per reporting instance is tabulated in Table 2.

Table 2: W2 reporting payload (per reporting instance)

| L | One-shot (#bits/SB) | Differential (per reporting), (#bits/SB) |  |
| --- | --- | --- | --- |
|  |  | t = 0 | t > 0 |
| 4 | 42 | 18 | 24 |
| 8 | 90 | 18 | 24 |

The performance comparison for the abovementioned schemes are shown in Figure 3 – Figure 6. The performance loss of differential CSI reporting when compared with one-shot reporting is shown in Figure 7 and Figure 8. We make observe the following.

Observation:

- Differential CSI reporting mechanism maintains attractive (with small loss) Type II CSI gain (Figure 3 – Figure 6) with W2 reporting payload of 2 beams per reporting instance

- When compared with one-shot reporting, differential CSI reporting incurs less than 6% and 8% losses in avg. and 5% UPT respectively. This loss is small considering the reduction in W2 reporting payload.

Figure 3: Performance comparison, 16 ports, L = 4 beams

Figure 4: Performance comparison, 16 ports, L = 8 beams

Figure 5: Performance comparison, 32 ports, L = 4 beams

Figure 6: Performance comparison, 32 ports, L = 8 beams

Figure 7: Performance of differential over one-shot reporting for 16 ports

Figure 8: Performance of differential over one-shot reporting for 32 ports

# Conclusions

In this contribution, to reduce Type II CSI payload, a differential CSI based Type II CSI reporting mechanism is presented. The observations and proposals made are summarized as follows.

Proposal: To reduce Type II CSI reporting payload, a differential CSI reporting mechanism is supported wherein

- T = L/2 coarse yet self-decodable CSIs are reported in T CSI reporting instances, where the CSI reported in each reporting instance is derived using two beams that correspond to adjacent beam or column pairs, (b0, b1), (b2, b3),…, of the basis matrix Bl proposed in [4] are considered (cf. Figure 2).

- A UE is configured with a WB CSI report to report (orthogonal basis set, L selected beams, and per layer strongest beam), and T = L/2 SB differential CSI reports to report phase and amplitude of coefficients corresponding to 2 beams in each of T CSI reports.

Observation:

- Differential CSI reporting mechanism maintains attractive (with small loss) Type II CSI gain (Figure 3 – Figure 6) with W2 reporting payload of 2 beams per reporting instance

- When compared with one-shot reporting, differential CSI reporting incurs less than 6% and 8% losses in avg. and 5% UPT respectively. This loss is small considering the reduction in W2 reporting payload.

# References

- RP-170855, “New WID on New Radio Access Technology,” NTT DOCOMO Inc.

- 3GPP, RAN1-NR#1, Chairman’s Notes

- 3GPP, RAN1#88, Chairman’s Notes

- R1-1707962, “Type II CSI reporting,” Samsung.

# Appendix: Simulation Assumptions

Table 3: Simulation Parameters

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer (Medium load 50% Target RU, Lambda = 4) |
| Channel model | UMi-2GHz |
| Number of BS (H,V) antenna elements | (8,8), x-polarized, subarray partition |
| (N1,N2, P) | 16, 32 ports: (4,2,2), (4,4,2) |
| (O1,O2) | (8,8) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2 |
| SU/MU pre-coding | SLNR |
| Scheduling | MU, Proportional fair, up to 4 layers |
| Channel estimation | Non-ideal |
| Transmission rank | 1,2 |
| Receiver | MMSE-IRC |
| CSI feedback schemes | Reference: Rel. 14 advanced CSI codebook Type II CSI codebook: proposed in [4] Ideal: dominant eigenvectors are known to the gNB/TRP |