3GPP TSG RAN WG1 Meeting #88bis                                            	R1-1708442

Spokane, USA 3rd - 7th April 2017

Source:	NTT DOCOMO, INC.

Title:	Discussion and evaluation on NR PRACH design

Agenda Item:	7.1.1.4.1

Document for: 	Discussion and Decision

- Introduction

At the RAN1#88bis meeting, random access channel and procedure for multi-beam operation were discussed. Regarding the RACH preamble, the following conclusion and agreements were reached [1].

Agreements: - NR RACH capacity shall be at least as high as in LTE - Such capacity is achieved by time/code/frequency multiplexing for a given total amount of time/frequency resources - Zadoff-Chu sequence is adopted in NR - FFS other sequence type and / or other methods in addition to Zadoff-Chu sequence for the scenario, e.g., high speed and large cells - FFS definition of large cell and high speed - FFS other sequence type and / or other methods for capacity enhancements, e.g.: - At least in multi-beam and low speed scenario, regarding multiple/repeated PRACH preamble formats, option 2 with OCC across preambles - FFS: Option 2 with OCC across multiple/repeated preambles in high speed scenarios - PRACH preamble design composed with multiple different ZC sequences - Sinusoidal modulation on top of option 1 Agreements: - For Zadoff-Chu sequence type, the RAN1 specifications will support two NR-PRACH sequence lengths (L) - L = 839: SCS = {1.25, 2.5, 5} KHz - Select one of - L = 63/71: SCS = {15, 30, 60, 120, 240} KHz - L = 127/139: SCS = {7.5, 15, 30, 60, 120} KHz - FFS: Supported subcarrier spacings for each sequence length - FFS for other sequence types Conclusions: - For Random Access, - following remaining issues need to be finalized in the next meeting - RACH preamble sequence length and subcarrier spacing - Selection of L between L = 63/71 and L = 127/139 - Further down-selection of subcarrier spacing if needed - RACH format design - Supported preamble formats of option 1 and detailed design of each - Whether/how to support option 2 and option 4 - CP/GT length - Followings remaining issues need to be finalized by Nov. meeting - RACH configuration - RACH procedure - Power control/power ramping - Note that all RRC related aspects need to be finalized by Oct. meeting

In this contribution, we provide our views on PRACH sequence design based on evaluation results. Finally we give proposal on list of preamble formats.

- PRACH preamble Design

  - Sequence type and length

- Sequence type

RAN1 agreed Zadoff-Chu sequence is adopted to NR PRACH, while other sequence types for specific scenario are FFS. However, supporting multiple sequence types would cause complexity of preamble format definition, impact on test spec and implementation/operation complexity. Also, Zadoff-Chu sequence can cover general use cases by different parameters for each of use cases. Therefore, we should initially focus on PRACH format design based on Zadoff-Chu sequence for general use cases unless significant benefit from other sequence type is confirmed.

Proposal 1: PRACH format design based on Zadoff-Chu sequence should be focused unless significant benefit from other sequence type is confirmed.

- Sequence length of Zadoff-Chu sequence

RAN1 agreed to support NR-PRACH sequence length of 839 and additional one sequence length, and it will be selected from 63, 71, 127 and 139. NR-PRACH with sequence length of 839 and SCS of 1.25*N kHz has transmission bandwidth about 1.08*N MHz. Similar to this, NR-PRACH with sequence length of 71 or 139 and SCS of 7.5*N or 15*N kHz has transmission bandwidth about 1.08*N MHz. On the other hand, NR-PRACH with sequence length of 63 or 127 has slightly different bandwidth. It may cause some complex design for RACH resource/occasion. In addition, there should be no significant performance difference between sequence length 63 and 71 and between 127 and 139 if pure Zadoff-Chu sequence is used for PRACH. Therefore, we can focus on 71 and 139 for further down selection.

In NR, multiple RACH resources can compensate the lack of capacity even when short sequence length is applied [2]. Especially for above 6GHz frequency bands, large amount of frequency resources would be available thanks to wide bandwidth, and required PRACH capacity would be low because of small cell radius, while for below 6GHz frequency bands, sequence length of 839 can be used for high capacity case. In addition, assuming certain subcarrier spacing, shorter sequence length leads narrower transmission bandwidth which is beneficial in terms of coverage. Therefore, we propose to support sequence length of 71 for Zadoff-Chu sequence in addition to 839.

Proposal 2: NR should support sequence length of 71.

  - Subcarrier spacing

Subcarrier spacing for PRACH can be different from subcarrier spacing for data/control on the same carrier. Narrower subcarrier spacing for PRACH is beneficial in terms of coverage thanks to higher PSD, while PRACH symbol length becomes longer. Due to half duplex constraint, long PRACH symbol length prevents any DL transmission including DL control channel. Therefore, we think NR PRACH format should support some variety of subcarrier spacing for PRACH for each frequency band/range so that NW can choose it based on subcarrier spacing used for data/control, coverage requirements and so on.

Subcarrier spacing for PRACH is also related to robustness against frequency offset e.g., Doppler effect in high speed scenario. In LTE, Ncs restricted set is available for high speed scenario, but it has drawback such as decreased capacity. In NR, wider subcarrier spacing can be used for PRACH to address Doppler effect in high speed scenario instead of restricted set. Therefore, it is beneficial to support relatively wide subcarrier spacing values for PRACH so that NR does not need to support Ncs restricted set. Based on our evaluation results for high speed scenario shown in Figure 1, subcarrier spacing of e.g., 2.5 kHz and 5 kHz seems work well in very high speed scenario.

Figure 1 – Misdetection probability at carrier frequency of 4 GHz.

SCS = left: 1.25 kHz, middle: 2.5 kHz, right: 5.0 kHz

Observation 1: For below 6GHz bands, subcarrier spacing of 1.25 kHz for PRACH shows performance degradation in case of over 120km/h, while subcarrier spacing of 2.5 kHz and 5 kHz for PRACH shows good performance in case of up to 500km/h.

Proposal 3: For below 6GHz bands, relatively wider subcarrier spacing values such as 2.5 kHz or 5 kHz should be considered instead of Ncs restricted set for high speed scenario.

  - Consecutive multiple/repeated PRACH preamble options

    - Support of other options for PRACH capacity enhancement

RAN1 agreed to support PRACH format based on multiple/repeated RACH OFDM symbols with single CP at the beginning (Option 1). For capacity enhancement, following methods have been proposed [1].

  - FFS other sequence type and / or other methods for capacity enhancements, e.g.:

    - At least in multi-beam and low speed scenario, regarding multiple/repeated PRACH preamble formats, option 2 with OCC across preambles

      - FFS: Option 2 with OCC across multiple/repeated preambles in high speed scenarios

    - PRACH preamble design composed with multiple different ZC sequences

    - Sinusoidal modulation on top of option 1

Option 1 can be used for general usage scenarios RAN1 has discussed so far, such as combining reception of multiple/repeated RACH OFDM symbols for coverage expansion and TRxP Rx beam sweeping during multiple/repeated RACH OFDM symbols. Remaining above possible options such as Option 2 with OCC and Option 4 may be beneficial for PRACH capacity enhancement, but multiple RACH resources can also enhance the PRACH capacity with more flexibility [2]. In addition, Option 2 without OCC would be unnecessary since its usage scenario is unclear. Therefore, we should focus on PRACH format design based on Option 1 only unless clear necessity of other options is clarified. Considering the tight schedule of Rel-15 NR standardization, we propose to focus on Option 1 based design in Rel-15.

Proposal 4: PRACH preamble format design should be based on Option 1 (one or multiple/repeated RACH OFDM symbol(s) with single CP at the beginning) at least for Rel-15 NR.

    - Number of symbol repetitions for Option 1

In Option 1, symbol repetition can be used for coverage extension by combining reception and for TRxP Rx beam sweeping.

- Coverage extension

For some cases such as large cell coverage, symbol repetition for combining reception would be needed. We think that at least one of preamble formats should satisfy the same level of MCL with LTE format 2, i.e. about 142dB  for both below 6GHz bands and above 6GHz bands. These formats are shown in Section 2.4, i.e. NR format 4 for below 6GHz bands and NR format 2 for above 6GHz bands.

- TRxP Rx beam sweeping

For above 6GHz, we should consider TRxP Rx beam sweeping for scenario without TRxP beam correspondence. In such case, the number of TRxP Rx beams can be different depending on TRxP implementation/capability, and hence we can consider multiple formats with some variety of number of symbols. However, supporting large number of different formats is not appropriate in terms of test effort and format indication overhead. Thus, the number of formats for TRxP Rx beam sweeping should be minimized.

The number of TRxP Rx beams may depend on the number of SS blocks. At the last meeting, following agreements were reached [3].

Agreements: - The considered maximum number of SS-blocks, L, within SS burst set for different frequency ranges are - For frequency range up to 3 GHz, the maximum number of SS-blocks, L, within SS burst set is [1, 2, 4] - For frequency range from 3GHz to 6 GHz, the maximum number of SS-blocks, L, within SS burst set is [4, 8] - For frequency range from 6 GHz to 52.6 GHz, the maximum number of SS-blocks, L, within SS burst set is [64] - The way the value of L is reflected in specification is FFS - Aforementioned values are to be used to facilitate the NR initial access design and evaluate the specification impact - Possibility of having unified frequency agnostic signaling design is not precluded

However, we think that actual number of TRxP Rx beams would not be the same with the number of SS blocks even without TRxP beam correspondence. For example, TRxP Rx beams for certain RACH resources would be generated based on DL Tx beam, i.e. associated SS block beam. Hence, we propose to support at most 14 repetition symbols within one PRACH format for TRxP Rx beam sweeping case. 14 repetition symbols would be reasonable considering slot length for data/control.

Proposal 5: At most 14 repetition symbols within a PRACH format are supported for TRxP Rx beam sweeping scenario.

  - PRACH preamble formats and use cases

PRACH preamble formats/configurations should be classified in different groups at least for below 6GHz bands and for above 6GHz bands since some of formats/configurations would clearly target specific frequency range. In other words, PRACH preamble format/configuration index in RACH configuration for different frequency range should correspond to different preamble format design so that format indication overhead can be reduced. For example, separate tables as shown in Table I and Table II can be defined in specification.

Proposal 6: PRACH preamble formats/configurations should be classified in different groups at least for below 6GHz bands and for above 6GHz bands.

Our NR preamble format designs with example of use case descriptions are shown below.

- Below 6GHz bands

Table I – PRACH preamble formats for below 6 GHz bands

For below 6 GHz bands, base subcarrier spacing is 2.5 kHz which can cover high speed scenario of 500km/h. NR format 2 allocates long time resources including long CP/GT for very large cell with large delay spread. In such case, wide subcarrier spacing is not preferred since it causes large CP/GP overhead. Thus, we use subcarrier spacing of 1.25 kHz in some formats. NR format 5 can be used for LTE-NR coexistence case.

- Above 6GHz bands

Table II – PRACH preamble formats for above 6 GHz bands

For above 6 GHz band, base subcarrier spacing is 60 kHz or 120 kHz.  Since long PRACH format prevents any DL transmission including DL control channel during RACH resource, such wide subcarrier spacing even for PRACH would be beneficial. Furthermore, usage for UL data of the other UEs would be limited depending on TRxP Rx beam direction. Also, drawback of wide subcarrier spacing i.e., low PSD due to wide bandwidth would be moderate because of small cell coverage compared with that for below 6GHz bands. Note that two formats, NR format 3 and 4 with different PRACH format duration are supported for beam sweeping scenarios with consideration for different data/control slot lengths.

- Conclusion

In this contribution, we discussed on PRACH sequence design for NR and provided our proposal on the list of preamble formats. We made the following observations and proposals.

Proposal 1: PRACH format design based on Zadoff-Chu sequence should be focused unless significant benefit from other sequence type is confirmed.

Proposal 2: NR should support sequence length of 71.

Observation 1: For below 6GHz bands, subcarrier spacing of 1.25 kHz for PRACH shows performance degradation in case of over 120km/h, while subcarrier spacing of 2.5 kHz and 5 kHz for PRACH shows good performance in case of up to 500km/h.

Proposal 3: For below 6GHz bands, relatively wider subcarrier spacing values such as 2.5 kHz or 5 kHz should be considered instead of Ncs restricted set for high speed scenario.

Proposal 4: PRACH preamble format design should be based on Option 1 (one or multiple/repeated RACH OFDM symbol(s) with single CP at the beginning) at least for Rel-15 NR.

Proposal 5: At most 14 repetition symbols within a PRACH format are supported for TRxP Rx beam sweeping scenario.

Proposal 6: PRACH preamble formats/configurations should be classified in different groups at least for below 6GHz bands and for above 6GHz bands.

References

- 3GPP RAN1 #88bis, Chairman’s note, April 2017.

- 3GPP, R1-1708443, NTT DOCOMO, INC., “Discussion on 4-step RA procedure for NR,” May 2017.

- 3GPP, R1-1706812, Nokia, “Outcome of offline session on number of SS blocks,” April 2017.

[Annex]  Simulation assumptions

Table A1 – Simulation assumptions.

|  | Below 6GHz | Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 4 GHz | 30 GHz |
| Channel Model | CDL-C, AWGN |  |
| Delay Scaling | 100 ns | 30 ns |
| Circular Angle Spread at BS after angle scaling (including subrays) | ASD: values from sec. 7.7.5.1 in 38.900 ZSD: 1° | ASD: values from sec. 7.7.5.1 in 38.900 ZSD: 1° |
| Circular Angle Spread at UE after angle scaling (including subrays) | ASA: values from sec 7.7.5.1 in 38.900 ZSA: 5° | ASA: values from sec. 7.7.5.1 in 38.900 ZSA: 5° |
| Circular Mean Angle at BS after angle scaling (including subrays) | Uniformly distributed AoD: [-30°,30°] |  |
| Circular Mean Angle at UE after angle scaling (including subrays) | Uniformly distributed AoA: [-30°,30°] |  |
| Antenna Configuration at BS | (M,N,P) = (1,1,2) with omni-directional antenna element | (4,8,2), with directional antenna element (HPBW = 65° in elevation and azimuth, directivity gain 8dB), (dV,dH)=(0.5,0.5) λ |
| Mechanical downtilt at BS | 0° |  |
| Antenna Configuration at the UE | (1,1,2) with omni-directional antenna element | (1,1,2) with omni-directional antenna element |
| Antenna port virtualization | No beamforming and no beam selection | One 2D beam generated by the Kronecker product of 2 weights, consisting of 4 beams in vertical plane and 8 beams in horizontal plane |
| Frequency Offset | 0.05 ppm at TRP , 0.1 ppm at UE |  |
| UE speed | 3 km/h, 120 km/h | 3 km/h |
| Initial timing Offset | Preamble format 0: Uniformly distributed [0,100s] i.e. assuming a maximum cell radius of 14.4 km. Preamble format 4: Uniformly distributed [0,10s] i.e. assuming a maximum cell radius of 1.4 km. For other formats both values should be simulated, [0 100]us and [0 10]us | Two different values to be used: Uniformly distributed in [0 5]us and [0 2.5]us |
| Preamble Detector | Each company should provide details on used algorithm |  |
| FFT size | 2048 |  |
| Sampling Frequency | 30.72 MHz |  |
| T_SEQ | 24576 |  |
| T_CP | 3158 |  |