3GPP TSG-RAN WG1 Meeting #89	R1-1708561

Hangzhou, P.R.China, 15th – 19th May 2017

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Evaluation results for scenario 1

Agenda item:	6.2.9.1

Document for:	Discussion and Decision

# 1	Introduction

In RAN#71, the SI named “Further Enhancements to LTE Device to Device, UE to Network Relays for IoT and Wearables” was agreed with the objectives of enhancement for the UE-to-NW relaying functionalities and LTE sidelink to enable low complexity/cost/energy IoT and wearables with QoS. From RAN1 perspective, the following objectives are identified [1]:

Identify mechanisms to enable QoS, reliable, and/or low complexity/cost & low energy sidelink [RAN1, RAN2, RAN4].

In RAN1#88, the evaluation methodology for the further enhancements to LTE sidelink and UE-to-Network relaying was discussed and the baseline evaluation methodology and simulation conditions were agreed [2].  In RAN1#88bis, the following additional channel modelling assumptions were agreed [4]:

- Channel modeling

  - Extend applicability of UE-UE channel models from TR 36.843 to distances between 1 and 3 meters

  - For Scenario 1, at distances below 1 m apply pathloss calculated at 1 m

  - For Scenario 2, keep the 3 meter UE-UE minimum distance

- The eNB – Remote UE channel model is defined as outdoor LOS path loss for 700 MHz and ISD = 1732 m deployment options from TR 36.843 plus a building penetration loss calculated according to Scenario 2 from TR 45.820.

- For UE – UE channel modeling, replace each value of 20 dB penetration loss by the statistical BPL value calculated according to Scenario 2 from TR 45.820.

  - No spatial correlation of building penetration loss for UE – UE links.

This paper provides preliminary evaluation results for scenario 1 based on the agreed evaluation conditions. The results for scenario 2 are presented in a companion paper [5].

# 2	Evaluation conditions

In scenario 1, N relay UEs, together with M remote UE per relay UE are dropped in the cell with 80% UEs are dropped in indoor hotspot building. It is assumed that the traffic between eNB and the M*N remote UEs are relayed by the N relay UEs in both directions (i.e., each relay UE only serves its associated M remote UEs). The evaluations mainly focus on the sidelink part, that is, the sidelink DL from relay UE to remote UE and sidelink UL from remote UE to relay UE.

The system evaluations were made based on the agreed evaluation conditions in last RAN1 meetings. The details of the evaluation conditions are listed in appendix. In particular, the traffic model of VoIP was used in the simulations with packet size of 328 bits, frame duration of 20ms and activity factor of 75%. The performance metrics include outage ratio and average packet throughput for sidelink UL (from remote UE to relay UE) and sidelink DL (from relay UE to remote UE), and power consumptions of transmit/receive/idle operations for remote UEs and relay UE.

In the simulations, sidelink communication resource pool is specifically assumed such that sidelink DL and sidelink UL is separated in different sidelink subframes for both SA and data. In particular, the sidelink communication resource pool is assumed to have bandwidth of 40 PRBs and period of 40ms with the following parts:

- Sidelink DL SA: 4 SFs

- Sidelink UL SA: 4 SFs

- Sidelink DL data: 16 SFs

- Sidelink UL data: 16 SFs

Note that for simplicity for data relaying between remote UE and eNB via relay UE, only sidelink DL/UL data transmissions are actually simulated in the simulations and the sidelink DL/UL SA are only considered in the power consumption computation and Uu part is assumed to be ideal and not included.

In the simulations, each VoIP packet of 328 bits occupies 2 adjacent PRBs over 2 (re)transmissions with QPSK and Turbo coding. At the receiver side, packet detection is based on either of the 2 (re)transmissions (i.e. soft combining is not used). Regarding resource allocation, it is initially assumed that in sidelink UL, remote UE(s) randomly select the time frequency resource in the associated data resource pool, while in sidelink DL, relay UE randomly selects orthogonal time frequency resources for its linked remote UEs.

Regarding power setting/control, the transmit power of 23dBm is assumed for relay UE which will be evenly distributed if multiple remote UE packets are frequency multiplexed in sidelink downlink. For remote UEs, both power control and non-power control are simulated. The power control is initially assumed to be based on sidelink pathloss as follows , where Pmax=0dBm or 23dBm, M=2, =1， PO,SL= -95dBm。

# 3	Evaluation results

The initial evaluation results for scenario 1 are described in this section. In the simulations, two cases are simulated, one with (relay UE number per cell) N=10, (remote UE number per relay UE) M=2 and the other with N=10, M= 8. In Figure 1, the pathloss between the relay UEs and their associated remote UEs is illustrated. We can observe that the two cases have similar distributions and the pathloss difference of the most remote UE-relay UE pairs are not large (e.g., about 90% pathloss within range of 49dB to 62dB).

Observation 1: In scenario 1, the pathloss between relay UE and their remote UEs is rather concentrated with relatively small pathloss value.

The simulated average outage, average per remote UE throughput and average power consumption per sidelink subframe are listed in Table 1 and 2. From the results, we can get the following observations.

Observation 2: In scenario 1 for VoIP traffic in sidelink UL, the outage/throughput performances with maximum transmit power of 0dBm are similar to those with 23dBm, no matter with or without power control. The power control has limited improvement on outage/throughput, but may be beneficial for power efficiency.

Observation 3:  In scenario 1, with increase of number of remote UEs per relay UE, the outage/throughput performance and power consumption of relay UE degrade.

Note that different from the significant benefits of power control for remote UE in scenario 2, it was observed in scenario 1 that the power control for sidelink UL has limited performance gains in outage/throughput. The reasons may be that the multiple remote UEs served by the relay UE has similar pathloss in sidelink UL in scenario 1 as stated in observation 1, which alleviates the impact of in-band emission interference on the performance.

Figure 1: CDF of pathloss between relay UE and remote UEs

Table 1: Results of avg. outage, avg. per remote UE throughputs and avg. power consumption per subframe

with M=2, N=10

|  | Sidelink UL |  |  |  | Sidelink DL |
| --- | --- | --- | --- | --- | --- |
|  | Pmax, remote UE = 0dBm |  | Pmax, remote UE = 23dBm |  | Pmax,relay UE= 23dBm |
|  | w/o PC | w/ PC | w/o PC | w/ PC |  |
| Outage rate | 0.0211 | 0.0167 | 0.0211 | 0.0167 | 0.0149 |
| Throughput, kbps | 12.274 | 12.276 | 12.274 | 12.276 | 12.3 |
| Power consumption per TTI | 0.2508 | 0.2508 | 0.6160 | 0.2508 | 0.9863 |
| Power efficiency | 6.098e-6 | 6.098e-6 | 2.439e-5 | 6.098e-6 | 7.013e-5 |

Note: * For power consumption metric, columns of Sidelink UL/DL correspond to remote UE/relay UE, respectively. The same for Table 2.

Table 2: Results of avg. outage, avg. per remote UE throughputs and avg. power consumption per subframe

with M=8, N=10

|  | Sidelink UL |  |  |  | Sidelink DL |
| --- | --- | --- | --- | --- | --- |
|  | Pmax, remote UE = 0dBm |  | Pmax, remote UE = 23dBm |  | Pmax,relay UE= 23dBm |
|  | w/o PC | w/ PC | w/o PC | w/ PC |  |
| Outage rate | 0.2397 | 0.2412 | 0.2397 | 0.2412 | 0.2458 |
| Throughput, kbps | 12.125 | 12.151 | 12.125 | 12.151 | 12.1 |
| Power consumption per TTI | 0.2509 | 0.2509 | 0.6163 | 0.2509 | 1.9388 |
| Power efficiency | 6.098e-6 | 6.098e-6 | 2.439e-5 | 6.098e-6 | 8.954e-5 |

# 3	Conclusion

In this paper, the preliminary evaluation results for the further enhancements to LTE sidelink and UE-to-Network relaying in scenario 1 are presented. Based on the discussions, the following observations are provided.

Observation 1: In scenario 1, the pathloss between relay UE and their remote UEs is rather concentrated with relatively small pathloss value.

Observation 2: In scenario 1 for VoIP traffic in sidelink UL, the outage/throughput performances with maximum transmit power of 0dBm are similar to those with 23dBm, no matter with or without power control. The power control has limited improvement on outage/throughput, but may be beneficial for power efficiency.

Observation 3: In scenario 1, with increase of number of remote UEs per relay UE, the outage/throughput performance and power consumption of relay UE degrade.

# References

- RP-161839, “Further Enhancements to LTE Device to Device, UE to Network Relays for IoT and Wearables”, RAN#73, New Orleans, September 2016.

- RAN1#88 chairman notes

- 3GPP TR 36.843 V12.0.1, “Study on LTE Device to Device Proximity Services – Radio aspects”.

- RAN1#88bis chairman notes

- R1-1708562

# Appendix

| Parameters | Scenario 1 |
| --- | --- |
| Network layout | Network Layout Option 1 from [3]: - Urban macro (500m ISD) + 1 RRH/Indoor Hotzone per cell Generation of Pico eNBs in Indoor Hotzones is disabled |
| Carrier frequency | 2 GHz for all links, FDD paired spectrum with 10 MHz per UL and DL |
| Channel model | Apply channel models from [3]. |
| Remote UE dropping | The dropping procedure ensures each Relay UE has M remote UEs in proximity of [D1,D2] - D1 and D2 are defined in distance range. D1 is the minimum distance between relay UE and UE, D2 is maximum distance D1=0m and D2=15m assumed in simulations. M = 2, 8 per relay UE |
| Relay UE dropping | 10 per cell |
| Remote UE parameters | - Number of Antennas: 1 TX, 1 RX - Antenna gain: 0 dB - Noise figure: 9 dB - Maximum TX power: 0, 23 dBm - Duplex: Remote UE cannot simultaneously receive in DL and SL and receive in DL if transmits in SL/UL (Half Duplex UE) |
| Relay UE parameters | - Number of Antennas: 1 TX, 2 RX - Antenna gain: 0 dB - Noise figure: 9 dB - Maximum TX power: 23 dBm - Duplex: Relay UE can simultaneously receive in DL and SL as well as transmit in UL/SL while receive in DL. Note that this encompassed both in band and out of band relaying |
| Traffic model | VoIP model from [3] - Encoder frame length: 20ms - Voice activity factor: 75% - Voice payload per frame: with header compression 41 Bytes (328 bits) - Outage definition: 2% error rate |
| Power consumption model | As defined in R1-1704106 |
| In-band emission model | IBE model from [3] with W,X,Y, Z = [0,0,0,0] |
| Performance metrics | Outage, throughput, power consumption and power efficiency |