3GPP TSG RAN WG1 #89 	R1-1708604

May 15th - 19th, 2017

Hangzhou, China

Agenda item:	7.1.2.5

Source: 	Qualcomm Incorporated

Title: 	MIMO Calibration Results for Sub6GHz

Document for:	Discussion/Decision

# 1	Introduction

After RAN1 86, the calibration assumptions for MIMO-NR study were discussed over email [1] for sub-6 GHz and mmW scenarios. In this contribution, we provide initial DL results for Phase-1 calibration for link and system study, for the sub-6 GHz scenario (dense-urban).  We also discuss simulation assumptions for Phase 2 calibration.

# 2	Link Simulations: Phase 1 calibration

Following assumptions are made in reference to [1]:

#1. Channel Model:

CDL-A, CDL-B, delay spread = 100ns, and UE speed=3km/h.  The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in Section 7.7.5.1 in TR 38.900 accordingly.

#2. Carrier frequency, bandwidth, subcarrier spacing

Carrier frequency = 4GHz, bandwidth = 20MHz, subcarrier spacing = 15kHz.

#3. TXRU mapping to antenna elements & TXRU mapping weights

TXRU virtualization only in the vertical dimension (i.e., 1D virtualization) using DFT with oversampling factor=1. Number of TXRUs = Mg x Ng x N x 2

#4. Beam sweeping approach

Method 1 is used, i.e., the DFT beam directly is pointing to the strongest cluster.

#5. Criteria for beam selection

DFT beam towards strongest cluster is used at both BS and UE.

#6. Metrics

Adopt the metric of CDF of receive SNR w/ beamforming at SNR=0dB for Phase 1 calibration.

#7. Other parameters:

Table 1. Other parameters

| Parameter | Values |
| --- | --- |
| BS antenna configurations | (M,N,P,Mg,Ng) = (4,4,2,1,1). (dV,dH) = (0.5, 0.5)λ |
| BS antenna pattern | As in 36.873 (Table 7.1-1) |
| UE antenna configurations | (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1) |
| BS array orientation | azimuth 0 degree; downtilt: 110 degree |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree |
| UE antenna pattern | omnidirectional |
| BF scheme | Analog BF based on beam selection + Digital BF based on ideal SVD |
| MIMO mode | SU-MIMO with rank=1 |

Calibration results for CDL-A and CDL-B channel are presented below in Fig.1 and Fig.2.

Figure 1: CDFs of Received SNR in CDL-A Channel

Figure 2: CDFs of Received SNR in CDL-B Channel

# 3	System Simulations: Phase 1 calibration

### 3.1 Simulation assumptions

In this contribution, we focus on a single layer NR network with sub-6GHz carrier frequency.  Following simulation assumptions have been used based on [1], [2].

Table 1: System Simulation Assumptions

| Parameters | Value |
| --- | --- |
| Layout | Single Layer (Macro) |
| Inter-BS Distance | 200m |
| Carrier Frequency | 4 GHz |
| Simulation BW | 20 MHz (30 kHz/RE, 480 kHz/RB) |
| Channel Model | 3D-UMA (as in 36.873) |
| Tx Power (BS) | 38 dBm over 20 MHz |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,4,2,1,1), (dV, dH) = (0.5, 0.5)λ, with per-elevation-element-complex-gain given by: , where m=1,…,K. =15 deg (36.873, Table 7.1-1) |
| BS Antenna Configuration | As in 36.873 (Table 7.1-1) |
| BS Antenna Height | 25 m |
| BS Antenna Element gain | 8 dBi |
| UE Receiver Noise Figure | 9 dB |
| UE Antenna Configuration | (M,N,P,Mg,Ng) = (1,1,2,1,1) Co-Pol, as in 36.873 |
| UE Antenna Height | As in 3D-UMA modelling in 36.873 |
| UE Antenna Gain | 0 dBi |
| Traffic Model | Full-buffer |
| UE Distribution | 80% indoor (3 kmph) and 20% outdoor (30 kmph) Uniform UE dropping: 10 UEs/TRP on the floor as in TR 36.873 |
| UE Receiver | MMSE-IRC |
| Rank of Transmission | 1 |
| Interference Reporting | None |
| Channel Estimation on UL | Ideal |
| Calibration Error at BS for reciprocity | None |
| Channel/Interference estimation on DL | Ideal |
| Scheduler | RR |

### 3.2	Simulation results

Per agreements in [1], we present the CDF of DL SINR without and with beam-forming below. Figure 1 shows the CDF of DL SINR without beam-forming (geometry).

Figure 1: CDF of DL SINR without beam-forming (Geometry) (Dense Urban @ 4 GHz)

Figure 2 shows the CDF of DL SINR (averaged across resource blocks and time in linear domain) with beam-forming.

Figure 2: CDF of DL SINR (single stream) with beam-forming (Dense Urban @ 4 GHz)

# 4 System Simulation: Phase 2 calibration assumptions

As proposed in [2], the simulation assumptions for Phase 2 calibration were agreed upon. We reproduce the system simulation assumptions in the tables below.

Table 2 Simulation assumptions for Phase 2 dense-urban (macro-layer) system-level calibration*

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Mode | DL only |
| Bandwidth | 20MHz |
| Subcarrier Spacing | 15kHz |
| Channel Model | 3D-UMa in TR36.873 |
| TXRU mapping to antenna elements | One TXRU per vertical dimension per polarization |
| TXRU mapping weights | TXRU virtualization only in the vertical dimension (i.e. 1D virtualization) using DFT, i.e., 1D sub-array partition model defined in TR36.897 |
| Criteria for selection for serving TRP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming**. |
| Criteria for Beam Selection for interfering TRP | Considering the real traffic in adjacent cells, the actual beam or SVD precoder that is used by the non-serving TRPs in its data transmission is used as interfering beams. |
| Constraints for the range of selective beams per TRP sector | [-60, 60] in azimuth domain and [100, 160] in zenith domain |
| Scheduling algorithm | Round robin |
| Traffic Model | Full buffer |
| BS antenna element radiation pattern | According to TR36.873 |
| UE antenna element radiation pattern | Omnidirectional |
| ISD | 200m |
| BS Tx power | 44dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.8)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | (M,N,P,Mg,Ng) = (1,1,2,1,1) Notes: the polarization angles are 0 and 90 |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| Noise figure for BS | 5dB |
| UE receiver noise figure | 9dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |
| Metric | spectral efficiency |

*Notes: Independent calibration for macro layer and micro layer for dense urban scenario.

- Macro layer: macro-only 10 users per TRP, all UEs are connected to macro layer. Users randomly and uniformly dropped throughout the macro geographical area

Table 3 Other simulation assumptions for all above system-level calibration scenarios

| Parameter | Values |
| --- | --- |
| Inter-panel calibration | Ideal |
| Control overhead | Zero |
| UE receiver type | MMSE-IRC |
| BF scheme | Analog BF based on beam selection + Digital BF based on ideal SVD* |
| MIMO mode | SU-MIMO with rank=1 |
| MCS | Use LTE MCS |

Figure 3 below shows the Spectral Efficiency CDF for Phase 2 calibration. Note that overheads have not been considered for spectral efficiency computation.

Figure 3: Spectral Efficiency for Phase 2 Calibration. Assmuptions as in Tables 2, 3 above.

# 5	Conclusions

In this contribution, we have presented the CDFs of DL SINR without and with beam-forming, per Phase-1 and Phase 2 Calibration requirements.

# References

- R1-1608661,	“Summary of [86-20] email discussion on MIMO calibration for NR”, ZTE et al.

- 3GPP TR 36.873, “Study of 3D Channel Model for LTE”.