3GPP TSG-RAN WG1 #89	R1-1708677

Hangzhou, China, 15th – 19th May, 2017

Source:	Ericsson

Title:	UL beam management details

Agenda Item:	7.1.2.2.1

Document for:	Discussion and Decision

# Introduction

In RAN1#87ah, the following working assumptions were made:

Working assumption:

- NR supports at least one NW-controlled mechanism for beam management for UL transmission(s)

  - Details are FFS, including at least the following study:

    - Signal(s) for the mechanism(s) if necessary

      - E.g., SRS, PRACH preamble, UL DMRS

      - Additional contents can also be included, e.g., beam reporting

    - Method(s) and content for TRP to indicate selected UE Tx beam and configure UE sweeping

    - Impact of beam correspondence Status

      - E.g., When to use the mechanism(s)

      - E.g., Procedures such as U-1, U-2, U-3, and beam correspondence based procedure

    - UE capability reporting

      - E.g., capability of analog beamforming

    - Consider the cases when UL and DL are from the same TRP and from different TRPs

    - Conditions when the mechanism is particularly useful

In RAN1#88b, the following agreements were made:

Agreements:

Codebook based transmission for UL is supported at least by following signaling in UL grant:

- SRI+TPMI+TRI, where

  - The TPMI is used to indicate preferred precoder over the SRS ports in the selected SRS resource by the SRI.

  - No SRI when a single SRS resource is configured

    - The TPMI is used to indicate preferred precoder over the SRS ports in the configured single SRS resource.

- Support indication on selection of multiple SRS resources

  - FFS details

In this contribution we discuss the need of UL beam management, which reference signals to use for this purpose and how to handle UL beam management for UEs with multiple panels.

# Discussion

Beam management using downlink (DL) reference signals (a.k.a DL beam management) has been agreed in 3GPP and at least for the sake of discussion the processes P1, P2 and P3 has been defined [1]. It is expected that for most cases beam correspondence will be fulfilled both at the TRP and UE, which means that DL beam management could be used to find suitable TRP TX/RX beams as well as UE TX/RX beams and in this way establish BPL(s) that could be used for both DL and UL.

It has also been agreed to establish multiple BPLs to provide diversity for e.g. PDCCH transmissions. For example, a first set of PDCCH candidates are transmitted over a first BPL while remaining second set of candidates over a second BPL. When beam correspondence is fulfilled at both TRP and UE, these multiple BPLs can also be used for uplink transmissions such as PUCCH, containing HARQ-ACK and/or CSI feedback, and/or PUSCH transmissions.

In case analog beamformer is used at both the TRP and UE, it is important that the TRP knows which UE TX beam is used for uplink transmission in order for the TRP to use the correct TRP RX beam. One way to solve this is to specify that the UE should use the same TX beam for transmitting an uplink response to the network as the RX beam the UE used to receive the downlink message from the TRP that scheduled the uplink response. In this way, there is no explicit signalling needed to indicate which beam the UE shall use for transmitting the uplink message.

When beam correspondence is fulfilled, the UE should use the same TX beam for transmitting an uplink response to the network as the RX beam the UE used to receive the downlink message from the TRP that scheduled the uplink response.

If beam correspondence is not fulfilled at the TRP and/or the UE, the TRP RX beam and/or UE TX beam cannot be determined based on DL beam management. In this case (and potentially in other cases) UL beam management is needed, i.e. beam management based on measurements on UL reference signals.

UL beam management will be needed in case beam correspondence is not fulfilled at the TRP and/or UE.

UL beam management should be initiated and controlled by the TRP (network). A suitable reference signal is the Sounding reference signal (SRS), which is used in UL for LTE to attain channel state information (CSI). One advantage with using SRS for UL beam management is that the CSI could be attained simultaneously as suitable TRP RX and/or UE TX beams are attained, which will reduce the delay between UL beam management and UL data transmission.

SRS should be used as reference signal for UL beam management.

UL beam management should be controlled by the network.

It is expected that at least two different types of UL beam management process will be needed, which are illustrated in Figure 1. In UL procedure 1 the TRP RX beam is updated by transmitting a burst of SRSs in one UE TX beam, allowing the TRP to sweep its RX beam. In UL Procedure 2 the UE transmits SRSs in a UE TX beam sweep to update the UE TX beam. For UL procedure 2, signaling is needed from the TRP to the UE to indicate which UE TX beam is preferred. One preferred way is to transmit one SRS resource per UE TX beam, which means that for a UE TX beam sweep, multiple SRS resources will be used. The SRS resources can be multiplexed by using TDM, FDM and/or CDM. The TRP can then feedback the preferred UE TX beam(s) to the UE by using an SRS resource indicator (SRI).

UL beam management process U1 and U3 requires DL signalling to inform the UE about the best UE TX beam.

Figure 1

In case multiple panels is used at the UE, it would be preferred if the UL beam management process (e.g. UL procedure 1 or UL procedure 2) for respective panel could be performed in parallel, in order to overhead. One way to solve this is to let the UE signal to the TRP (in e.g. UE capabilities) the number of simultaneous parallel UL beam management processes that the UE can handle. Then, whenever UL beam management is needed, the TRP allocates one set of SRS resources for each of the multiple parallel UL beam management procedures. In case of UL procedure 2, where feedback needs to be signalled to the UE, the TRP should feedback one SRI for each of the multiple parallel procedures.

NR should support multiple parallel UL beam management procedures to handle multi panel UEs in an efficient way.

# Conclusions

- UL beam management will be needed in case beam correspondence is not fulfilled at the TRP and/or UE.

UL beam management process U1 and U3 requires DL signalling to inform the UE about the best UE TX beam.

- When beam correspondence is fulfilled, the UE should use the same TX beam for transmitting an uplink response to the network as the RX beam the UE used to receive the downlink message from the TRP that scheduled the uplink response.

SRS should be used as reference signal for UL beam management.

UL beam management should be controlled by the network.

NR should support multiple parallel UL beam management procedures to handle multi panel UEs in an efficient way.