3GPP TSG RAN WG1 #89	R1-1708764

May 15th – 19th, 2017

Hangzhou, China

Agenda item:	6.2.1.2.1.2

Source: 	Qualcomm Incorporated

Title: 	Flexible Uplink DMRS Position

Document for:	Discussion and Decision

# 1	Introduction

As agreed in RAN1 #87, in a 1-slot sTTI, the UL DMRS location is identical to that of the legacy LTE, i.e., symbol 3 of each slot is reserved for UL DMRS transmission. In RAN1 #88b, the following agreement for a 2-symbol sTTI was reached:

Agreement:

- For 2/3-symbol sPUSCH transmission, at least the following configurations are supported and one of the configurations for sTTI#n+x is indicated by the UL grant at sTTI#n, if the scheduled sTTI#n+x includes both data and the associated RS, where x is processing time

  - sTTI#n+x = sTTI#0: {R D D}, FFS:{D D R}, FFS: {D R D}

  - sTTI#n+x = sTTI#1,2,3,4: {R D}, FFS: {D R}

  - sTTI#n+x = sTTI#5: {R D D}, FFS: {D R D}

    - FFS if data in the last symbol can be transmitted in cell specific SRS subframes

  - Other configurations are not precluded

- FFS for the case where data and associated RS are in different sTTIs

In a 2-symbol sTTI operation, transmitting UL DMRS in every sTTI could be costly. In this contribution paper, we provide the means to reduce the overhead of UL DMRS transmission.

# 2	DMRS Configuration in UL sTTI

Clearly, for the case of 2-symbol sTTI operation, a static allocation of a DMRS signal results in an excessive DMRS overhead, thereby reducing the users’ data throughput.

For the 2-symbol sTTI, the eNB can dynamically schedule the uplink DMRS position within the sPDCCH grant, and can multiplex multiple users by configuring each user to transmit with a different cyclic shift value. To signal the transmission of an uplink DMRS position, we can specify a set of different patterns that contain different data and DMRS placements within the sTTI as shown in Figure 1. As an example, to consider 4 different patterns, 2 bits are required to be added in the UL grant.

This set of patterns can create any variety of PUSCH + DMRS sequences via multiple sequential sPDCCH grants.  Using pattern #3, a user can configure a DMRS transmission without sending a PUSCH allocation, thereby allowing for pre-allocating a DMRS reference signal for a future PUSCH allocation in a future sTTI. Also, note that two sets of patterns are given for the 2 symbol and 3 symbol sTTI cases. As an another example, during sTTI#0, one user may be configured with the {D,D,R} pattern, while the other user is configured with {null,null,R}. For the second user, this DMRS symbol may be used for the demodulation of sPUSCH sent during sTTI#1 with the {D,D} pattern. The final set of patterns should allow for DMRS sharing across different sTTIs.

Figure 1: Combinations of Dynamically Scheduled Uplink sPUSCH and DMRS allocations.

Proposal 1: A dynamic UL DMRS structure should be implemented via selection of one of multiple defined sPUSCH, DMRS, and null patterns.

In addition to employing pattern #3 described before, to further reduce the overhead, the previously sent DMRSs can also be used for the decoding of the current sTTI. As an example, if in sTTI n, pattern #1 is triggered, in sTTI n+1, pattern #0 can be used where its channel estimation is based on the preceding DMRS. More generally, a time window with a configurable length can be defined during which the previous DMRS is still valid for the decoding of the current DMRS. Within this time window, the phase continuity across the DMRS and data symbols should be maintained. Also, it is reasonable to assume that DMRS sharing is only allowed across sTTIs contained within one subframe.

Proposal 2: Consider defining a time window during which the previously sent UL DMRS is valid for decoding of the current sTTI.

Proposal 3: DMRS sharing is only allowed across sTTIs contained within one subframe.

# 3	         Conclusions

Proposal 1: A dynamic UL DMRS structure should be implemented via selection of one of multiple defined sPUSCH, DMRS, and null patterns.

Proposal 2: Consider defining a time window during which the previously sent UL DMRS is valid for decoding of the current sTTI.

Proposal 3: DMRS sharing is only allowed across sTTIs contained within one subframe.