3GPP TSG-RAN WG1 Meeting #89	R1-1709096

Hangzhou, China, May 15 – 19, 2017

Agenda Item:	7.1.3.3.2

Source:	Ericsson

Title:	On MCS/transport Block Size Determination for PUSCH

Document for:	Discussion and Decision

# Introduction

We discuss MCS and transport block size (TBS) determination for NR PUSCH in this contribution.

# Discussion

## MCS/TBS determination in LTE PUSCH

For an initial transmission of an LTE PUSCH, an MCS index, IMCS, is signalled in the DCI to the UE.  UE uses the received IMCS as the row key to read off modulation order, Qm, and TBS index, ITBS, from an MCS table specified in TS 36.213. To determine the TBS, the UE first calculates the total number of allocated PRBs, NPRB. The UE then determines the TBS from a TBS table specified in TS 36.213 using ITBS as row key and NPRB as column key.

In case of retransmission, the eNB can elect to signal a modulation order to the UE and the UE shall assume the TBS determined from DCI transported in the latest PDCCH for the same transport block using 0≤IMCS≤27 or 28.

The PUSCH TBSs are designed assuming 144 REs per PRB are available for carrying the PUSCH. The MCS table is designed assuming DFT-S-OFDM waveform for PUSCH.

Rel-8 LTE PUSCH was designed to operate a synchronous HARQ protocol. For initial transmissions, the redundancy version is restricted to rvidx=0.

## MCS table design for NR PUSCH

NR supports asynchronous HARQ protocol for the PUSCH. It is hence proposed that the redundancy version for the PUSCH to be signalled via a RV index field in the DCI separate from the MCS index field.

Furthermore, NR PUSCH supports both OFDM and DFT-S-OFDM waveforms. One alternative to support both is to have two MCS tables for NR PUSCH. For the case of OFDM waveform, the same MCS table designed for the PDSCH can be reused for the PUSCH. For the case of DFT-S-OFDM waveform, a new MCS table should be designed to optimize the link performance. This two-table solution as shown in Table 1 requires slow RRC reconfiguration to switch between the two waveforms. Considering the uncertainty period after confirmation, this can be too slow. For instance, the differences between channel dispersiveness, link quality and number of layers from different TRPs may require different waveforms to be used.

Table 1 Two different MCS tables for NR PUSCH with OFDM or DFT-S-OFDM waveform (The target code rates are those agreed for LTE MCS table for 256QAM support [1] and those from Rel-8 LTE PUSCH).

| MCS Index | OFDM PUSCH |  | DFT-S-OFDM PUSCH |  |
| --- | --- | --- | --- | --- |
|  | Modulation order | Code Rate R ×1024 | Modulation order | Code Rate R ×1024 |
| 0 | 2 | 120 | 2 | 100 |
| 1 | 2 | 193 | 2 | 130.83 |
| 2 | 2 | 308 | 2 | 160.83 |
| 3 | 2 | 449 | 2 | 209.17 |
| 4 | 2 | 602 | 2 | 256.67 |
| 5 | 4 | 378 | 2 | 315.83 |
| 6 | 4 | 434 | 2 | 374.17 |
| 7 | 4 | 490 | 2 | 438.33 |
| 8 | 4 | 553 | 2 | 501.67 |
| 9 | 4 | 616 | 2 | 565.83 |
| 10 | 4 | 658 | 2 | 630 |
| 11 | 6 | 466 | 4 | 315 |
| 12 | 6 | 517 | 4 | 361.67 |
| 13 | 6 | 567 | 4 | 408.33 |
| 14 | 6 | 616 | 4 | 460.83 |
| 15 | 6 | 666 | 4 | 513.33 |
| 16 | 6 | 719 | 4 | 548.33 |
| 17 | 6 | 772 | 4 | 582.50 |
| 18 | 6 | 822 | 4 | 646.25 |
| 19 | 6 | 873 | 4 | 708.75 |
| 20 | 8 | 682.5 | 4 | 770 |
| 21 | 8 | 711 | 6 | 513.33 |
| 22 | 8 | 754 | 6 | 555 |
| 23 | 8 | 797 | 6 | 599.17 |
| 24 | 8 | 841 | 6 | 643.33 |
| 25 | 8 | 885 | 6 | 685 |
| 26 | 8 | 916.5 | 6 | 727.50 |
| 27 | 8 | 948 | 6 | 758.33 |
| 28 | 2 | reserved | 6 | 895.33 |
| 29 | 4 |  | 2 | reserved |
| 30 | 6 |  | 4 |  |
| 31 | 8 |  | 6 |  |

To support fast switching between OFDM and DFT-S-OFDM waveforms, it is beneficial for NR to adopt an MCS table that can address waveform options in addition to modulation orders and code rates. One such example is shown in Table 2.

For 0≤IMCS≤58, the UE reads the modulation order, Qm, target code rate, R, and the waveform for the PUSCH from MCS table using the received MCS index, IMCS.

For 59≤IMCS≤63, the UE reads the modulation order, Qm, from the MCS table using the received MCS index, IMCS. The UE applies the waveform determined from the DCI in the most recently received PDCCH for the same TB using 0≤IMCS≤58. Alternatively, it can be further discussed whether the reserved entries can also specific the waveform.

Proposal 1 For NR PUSCH, redundancy version index is not specified in the MCS table but signalled separately in the DCI.

Proposal 2 To support fast switching between OFDM and DFT-S-OFDM waveforms, adopt an MCS table that can address waveform options in addition to modulation orders and code rates for NR PUSCH.

Table 2 Example MCS table for NR PUSCH with OFDM waveform (The target code rates other than those for π/2 BPSK are those from Table 1).

| MCS Index | Modulation order | Code Rate R ×1024 | Waveform |
| --- | --- | --- | --- |
| 0 | 1 | 100 | DFT-S-OFDM |
| 1 | 1 | 150 | DFT-S-OFDM |
| 2 | 2 | 100.00 | DFT-S-OFDM |
| 3 | 2 | 120 | OFDM |
| 4 | 2 | 130.83 | DFT-S-OFDM |
| 5 | 2 | 160.83 | DFT-S-OFDM |
| 6 | 2 | 193 | OFDM |
| 7 | 2 | 209.17 | DFT-S-OFDM |
| 8 | 2 | 256.67 | DFT-S-OFDM |
| 9 | 2 | 308 | OFDM |
| 10 | 2 | 315.83 | DFT-S-OFDM |
| 11 | 2 | 374.17 | DFT-S-OFDM |
| 12 | 2 | 438.33 | DFT-S-OFDM |
| 13 | 2 | 449 | OFDM |
| 14 | 2 | 501.67 | DFT-S-OFDM |
| 15 | 2 | 565.83 | DFT-S-OFDM |
| 16 | 2 | 602 | OFDM |
| 17 | 2 | 630.00 | DFT-S-OFDM |
| 18 | 4 | 315.00 | DFT-S-OFDM |
| 19 | 4 | 361.67 | DFT-S-OFDM |
| 20 | 4 | 378 | OFDM |
| 21 | 4 | 408.33 | DFT-S-OFDM |
| 22 | 4 | 434 | OFDM |
| 23 | 4 | 460.83 | DFT-S-OFDM |
| 24 | 4 | 490 | OFDM |
| 25 | 4 | 513.33 | DFT-S-OFDM |
| 26 | 4 | 548.33 | DFT-S-OFDM |
| 27 | 4 | 553 | OFDM |
| 28 | 4 | 582.50 | DFT-S-OFDM |
| 29 | 4 | 616 | OFDM |
| 30 | 4 | 646.25 | DFT-S-OFDM |
| 31 | 4 | 658 | OFDM |
| 32 | 6 | 466 | OFDM |
| 33 | 4 | 708.75 | DFT-S-OFDM |
| 34 | 4 | 770.00 | DFT-S-OFDM |
| 35 | 6 | 513.33 | DFT-S-OFDM |
| 36 | 6 | 517 | OFDM |
| 37 | 6 | 555.00 | DFT-S-OFDM |
| 38 | 6 | 567 | OFDM |
| 39 | 6 | 599.17 | DFT-S-OFDM |
| 40 | 6 | 616 | OFDM |
| 41 | 6 | 643.33 | DFT-S-OFDM |
| 42 | 6 | 666 | OFDM |
| 43 | 6 | 685.00 | DFT-S-OFDM |
| 44 | 6 | 719 | OFDM |
| 45 | 6 | 727.50 | DFT-S-OFDM |
| 46 | 6 | 758.33 | DFT-S-OFDM |
| 47 | 6 | 772 | OFDM |
| 48 | 6 | 822 | OFDM |
| 49 | 6 | 873 | OFDM |
| 50 | 6 | 895.33 | DFT-S-OFDM |
| 51 | 8 | 682.5 | OFDM |
| 52 | 8 | 711 | OFDM |
| 53 | 8 | 754 | OFDM |
| 54 | 8 | 797 | OFDM |
| 55 | 8 | 841 | OFDM |
| 56 | 8 | 885 | OFDM |
| 57 | 8 | 916.5 | OFDM |
| 58 | 8 | 948 | OFDM |

| 59 | 1 | Reserved | reserved |
| --- | --- | --- | --- |
| 60 | 2 |  |  |
| 61 | 4 |  |  |
| 62 | 6 |  |  |
| 63 | 8 |  |  |

## TBS determination for NR PUSCH

NR is designed to be forward compatible and thus offer substantial more flexibility than LTE. A NR slot can be configured to contain 7 or 14 OS. For a NR system operating 14-OS slot configuration, the actual length of the PUSCH can be of several lengths to accommodate various system operation targets. Some different PUSCH transmission length examples are shown in Figure 1.

In addition, NR supports mini-slots of 2—13 OS in length.

Figure 1 Different PUSCH transmission lengths.

Observation 1 Given the flexibility offered by NR, MCS and TBS determination needs to support flexible amount of PUSCH resources efficiently in a unified, flexible and extensible approach.

Observation 2 Pre-calculated TBS table assuming fixed PUSCH resources as in LTE cannot support all use cases and is not forward compatible.

For NR PUSCH, we propose that the MCS table provides modulation order, Qm, and target code rate, R, instead of the TBS index ITBS. One such example for PUSCH with OFDM waveform is provided in Table 2. The target code rates are those agreed for LTE MCS table for 256QAM support [1].

For 0≤IMCS≤58, the UE determines the TBS based on, for example,

TBS=8×NPRB∙NREDL,PRB∙υ∙Qm∙R8

where

- ν is number of layers the codeword is mapped onto

- NREDL,PRB is the number of REs per PRB per slot/mini-slot available for carrying the PUSCH.

For 59≤IMCS≤63, the TBS is assumed to be determined from the DCI in the most recently received PDCCH for the same TB using 0≤IMCS≤58.

As discussed in the above, NREDL,PRB is assumed to be 144 in LTE. For NR to support various slot/mini-slot/PUSCH lengths, a more flexible framework for NREDL,PRB is to be used. One or more of the following components can be considered:

- A default NREDL,PRB value may be defined in specs and can be applicable to, e.g., system information, paging and random access reply transmissions. For this purpose, NREDL,PRB=144 can be considered for 14-OS slot and NREDL,PRB=72 can be considered for 7-OS slot.

- Some default NREDL,PRB values for different PUSCH transmission lengths can be specified in the specification.

- The network can configure the UE to apply a specific NREDL,PRB to the PUSCH.

- The network can configure a set of NREDL,PRB values (e.g., four values) to the UE. The DCI then contains an index to instruct the UE to apply one of the pre-configured NREDL,PRB value for the current PUSCH.

- If code rates lower than those available in the MCS table are found to be necessary for a specific use case (e.g., URLLC), the network can configure/select a NREDL,PRB value that is substantially lower such that the allocated resources are used to carry a substantially smaller TB.

Proposal 3 To support flexible PUSCH transmission duration and use cases, TBS determination in NR is calculated using configured and/or signalled parameters including one parameter that provides the number of REs per PRB per slot/mini-slot available for carrying the PUSCH.

Proposal 4 A candidate TBS determination is

TBS=8×NPRB∙NREDL,PRB∙υ∙Qm∙R8

where

- ν is number of layers the codeword is mapped onto

- NREDL,PRB is the number of REs per PRB per slot/mini-slot available for carrying the PUSCH.

- NPRB is the number of allocated PRBs

- modulation order, Qm, and target code rate, R, are read from a MCS table based on IMCS signalled in the DCI.

## TBS determination for specific service packet sizes

Certain important services are expected to generate large number of packages in the system. For instance, voice over IP (VoIP) service is expected to be one of the main application cases also in NR. Possible sizes from the VoIP service include, for instance, 144, 176, 208, 224, 256 and 328 bits. It is beneficial for the system to handle these specific packet sizes specifically to optimize performance of the system and the specific services.

The TBSs calculated based on the above proposal can be close to these sizes and match them in certain cases. However, an explicit optimization can be considered. For instance, it may be worthwhile to over-write some combinations of  IMCS and NPRB to these specific sizes.

Proposal 5 For important services expected to generate large number of packages in the system, explicit optimization of the TBS to handle these specific sizes can be considered.

# Conclusion

In this contribution, we discuss MCS and TBS determination for NR PUSCH.

We have the following observations:

Observation 1 Given the flexibility offered by NR, MCS and TBS determination needs to support flexible amount of PUSCH resources efficiently in a unified, flexible and extensible approach.

Observation 2 Pre-calculated TBS table assuming fixed PUSCH resources as in LTE cannot support all use cases and is not forward compatible.

We propose

Proposal 1 For NR PUSCH, redundancy version index is not specified in the MCS table but signalled separately in the DCI.

Proposal 2 To support fast switching between OFDM and DFT-S-OFDM waveforms, adopt an MCS table that can address waveform options in addition to modulation orders and code rates for NR PUSCH.

Proposal 3 To support flexible PUSCH transmission duration and use cases, TBS determination in NR is calculated using configured and/or signalled parameters including one parameter that provides the number of REs per PRB per slot/mini-slot available for carrying the PUSCH.

Proposal 4 A candidate TBS determination is

TBS=8×NPRB∙NREDL,PRB∙υ∙Qm∙R8

where

- ν is number of layers the codeword is mapped onto

- NREDL,PRB is the number of REs per PRB per slot/mini-slot available for carrying the PUSCH.

- NPRB is the number of allocated PRBs

- modulation order, Qm, and target code rate, R, are read from a MCS table based on IMCS signalled in the DCI.

Proposal 5 For important services expected to generate large number of packages in the system, explicit optimization of the TBS to handle these specific sizes can be considered.

# References

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting#77