TSG-RAN WG1 Meeting#89	R1-1709100

Hangzhou, China, May 15 – 19, 2017

Source:	Ericsson

Title:	On Polled Hybrid-ARQ Acknowledgement

Agenda Item:	7.1.3.3.3

Document for:	Discussion and Decision

# Introduction

In this contribution we present some views related to polled hybrid-ARQ acknowledgements.

# Discussion

Hybrid-ARQ acknowledgement ‘immediately’ after reception of downlink data is clearly beneficial from a latency perspective, as well as for operation in unlicensed spectrum. Hence this was agreed at RAN1 #87.

Indicating multiple timing possibilities in the DCI may require multiple bits. Either these bits can indicate predefined timing relations (same slot, next slot, next-next slot, etc), point to a set of RRC configured relations (same slot, RRC conf 1, RRC conf 2, etc), or “polling” can be used. In the latter case, the DCI could contain a single bit indicating whether the UE should transmit the hybrid-ARQ acknowledgement (in the same slot) or not. If the UE was instructed not to transmit a hybrid-ARQ acknowledgement, the network could at a later time instant “poll” the UE for the status of the previous receptions. Polling could have the benefit of avoiding complicated HARQ-ACK codebooks as used in LTE. They can also be useful for operation in unlicensed spectrum for devices not capable of ‘immediate’ ACK.

Proposal: Introduce polled HARQ-acknowledgement for PDSCH

Figure 1: ACK timing.

# Conclusion

- Introduce polled HARQ-acknowledgement for PDSCH