3GPP TSG RAN WG1 Meeting #89	R1-1709456

Hangzhou, China, 15-19 May 2017

Agenda Item:	7.1.2.4.1

Source:	Huawei, HiSilicon

Title:	Summary of offline discussion on CSI-RS

Document for:	Discussion and decision

Introduction

There are seven proposals in the summary document on companies views on CSI-RS [1], and six WFs [2] -[7] related to CSI-RS design for CSI acquisition and beam management.

Summary

Following proposals have been agreed during offline discussion from Monday to Wednesday morning.

Offline agreements:

- Support at least OCC as one type of CDM sequence

  - FFS: CDM of CSI-RS antenna ports on an RE-level comb based on phase rotation (cyclic shift) sequences

- Support at least CDM 8 for an X-port CSI-RS resource at least when X=32

  - FFS:  Support for other pairings of CDM value and number of ports X

Offline agreements:

- For an X-port CSI-RS resource, at least for X = 1, support density D >= 1 RE/port/PRB

  - FFS: Support for D > 1 for other values of X

  - Note: For X = 1, CDM code value(s) assumed to be 1

- References

- Huawei, HiSilicon, Samsung, “Summary of remaining issues and views on NR CSI-RS”, R1-1709204, Hangzhou, China, May 15-19, 2017.

- Huawei, HiSilicon, Intel, MediaTek, Samsung, “WF on CDM design”, R1-1709455, Hangzhou, China, May 15-19, 2017.

- Samsung, “WF on location of CSI-RS OFDM symbols”, R1-1709397, Hangzhou, China, May 15-19, 2017.

- Huawei, HiSilicon, LTE, “WF on CSI-RS pattern design”, R1-1709572, Hangzhou, China, May 15-19, 2017.

- ZTE, Huawei, HiSilicon, “WF on CDM pattern of CSI-RS for CSI acquisition”, R1-1709599, Hangzhou, China, May 15-19, 2017.

- Intel, “Way Forward on CSI-RS for Beam Management”, R1-1709303, Hangzhou, China, May 15-19, 2017.

- ZTE, “WF on CSI-RS for beam management”, R1-1709317, Hangzhou, China, May 15-19, 2017.