3GPP TSG-RAN WG1 Meeting #89			R1-1709243

Hangzhou, P.R. China 15th – 19th May 2017

Title:	LS on UL HARQ processes and collision handling for short processing time with 1ms TTI

Release:	Rel-15

Work Item:	LTE_sTTIandPT

Source:	RAN1

To:	RAN2

Contact Person:

#### Name:	Toshizo Nogami

E-mail Address:	nogamit@sharplabs.com

Attachments: 	None

1. Overall Description:

In RAN1#88bis meeting, the following agreements for 1ms TTI with shortened processing time were made for collision handling of grants with n+4 timing and n+3 timing.

Agreement 1: If the UE receives conflicting PHICH with n+4 timing and UL grant with n+3 timing scheduling PUSCH for the same UL subframe of a carrier, only the PUSCH scheduled by UL grant with n+3 timing is transmitted. Note: This might not have specification impact

Furthermore, in RAN1#89 meeting, the following agreements for UL HARQ-processes with n+4 timing were made .

Agreement 2: - For a HARQ-process with n+4 timing, layer 1 delivers HARQ-ACK to MAC layer regardless of a later received n+3 UL grant detection. - If there is a collision with an n+3 UL grant, an explicit DCI is required for the retransmission, i.e. autonomous non-adaptive PUSCH retransmission is not adopted

The latest bullet of this agreement implies that in order to resume the HARQ process with the n+4 timing, a new DCI reception for this process is required.

For asynchronous UL HARQ, RAN1 also achieved the following agreements in RAN1#89 meeting.

Agreement 3: For a UE configured with n+3 1ms TTI, synchronous UL HARQ is supported for UL transmissions with legacy processing time (n+4) Agreement 4: The maximum number of UL HARQ processes for n+3 1ms TTI is the same as for n+4 1ms TTI - FFS: In case the UE is configured also with sTTI Agreement 5: The PUSCH UL HARQ processes of n+3 1ms TTI and n+4 1ms TTI are not shared

2. Actions:

RAN1 respectfully asks RAN2 to take the above agreements into consideration in their work, and cover relevant agreements in the RAN2 specification(s).

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN1 Meeting #90	      21st – 25th Aug 2017	Prague		CZ

TSG-RAN1 Meeting #90bis     9th – 13th Oct 2017	Prague		CZ