3GPP TSG RAN WG1 Meeting #89		R1-1709716

# Hangzhou, China, May 15-19, 2017

Title:	Monitoring of partial NPDCCH search spaces

# Response to:

# Release:	Rel-13

Work Item:	NB_IOT

Source:	RAN1

To:	RAN2

Copy:

# Contact Person:

# Name:	Alberto Rico

E-mail Address:	albertor@qti.qualcomm.com

Attachments:	None

# 1. Overall Description:

RAN1 discussed whether the UE is required to monitor partial candidates and partial search spaces when  there is a change in the NPDCCH search space and/or configuration.

RAN1 made the following conclusion:

- It is RAN1’s understanding that the UE is not required to monitor candidates that start before m+45 belonging to a new configuration (believed to be already captured in 36.331 s.11.2).

There are different views in RAN1 as to whether the current specifications require a UE to monitor candidates which start at or after m+45 belonging to a new configuration in a search space that starts before m+45.

# 2. Actions to RAN2:

RAN1 respectfully requests RAN2 to provide confirmation of the required UE behaviour according to the RAN2 specifications.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #90	Aug. 21-25, 2017		      Prague (CZ)

TSG RAN WG1 Meeting #90-Bis	Oct. 9-13,  2017 		      Prague (CZ)

# 4. Appendix

# The time references in the description part of this LS are made with respect to Figure 1, where:

  - m is the last subframe carrying the RRC Connection Reconfiguration message

  - s is the first subframe of an NPDCCH search space (this is termed k0 in TS 3 6.213)

  - c is the first subframe of an NPDCCH candidate belonging to the NPDCCH search space starting in subframe s. (This is termed kb in TS 36.213)

Figure 1 Example of search space/candidate starting before applying RRC reconfiguration.