3GPP TSG RAN WG1 Meeting 91	R1-1720169

RENO, USA, NOV 27th  –  DEC 1st, 2017

Source:          	CATT

Title:  	Summary of Offline Discussion on Remaining Minimum System Information

Agenda Item:     	7.1.2.2

Document for:   	Discussion and Decision

# Introduction

This purpose of the document is to provide a summary of offline discussion on the remaining key issues and the proposed solutions on AI 7.1.2.2 “Remaining details on remaining minimum system information” for facilitating the continue discussion in RAN1#91. The following is a list of remaining RMSI key issues:

- DL bandwidth combining the SS/PBCH block and initial active DL BWP

- Association between SS blocks and the corresponding RMSI(s) in wideband operation

- Different Numerologies of SS/PBCH block and RMSI CORESET

- RMSI CORESET(s) Configuration

- RMSI PDCCH monitoring window configuration

- FDM configuration between SS/PBCH block and RMSI(CORESET/NR-PDSCH)

- Down-selection of RMSI TTI

- RMSI Transmission Block

- RMSI PDSCH transmission

# Key Issues and Proposed Solutions

## DL bandwidth combining the SS/PBCH block and initial active DL BWP

In RAN1#90bis, the following agreements were made for the UE minimum bandwidth and the initial active DL bandwidth part in RAN1#90bis [1].

Agreements: - UE minimum bandwidth in the context of confinement of RMSI and CORESET containing PDCCH scheduling RMSI is defined as the largest bandwidth that all UEs must support regardless of UE capability, which is at least no less than the SS/PBCH bandwidth. - Bandwidth for RMSI and CORESET containing PDCCH scheduling RMSI supports at least the same bandwidth as SS/PBCH (e.g.,[24 PRBs]). - Note: The UE minimum bandwidth will be finally determined by RAN4. Agreements: - The initial active DL BWP is defined as frequency location and bandwidth of RMSI CORESET and numerology of RMSI. - PDSCH delivering RMSI are confined within the initial active DL BWP

However, the bandwidth multiplexing the SS/PBCH block and the initial active DL BWP is undecided. Based on the contributions submitted to RAN1#90bis [2-19], we may consider the following options for the bandwidth combining the SS/PBCH block and the initial active DL BWP.

- The bandwidth, X, multiplexing both the SS/PBCH block and the initial active DL BWP is confined within

  - Alt.1: initial active DL BWP

  - Alt.2: UE minimum bandwidth

  - Alt.3: the minimum carrier bandwidth for a given band

The intention of Table 2-2.1 is to collect the views from interested companies on the bandwidth, X, multiplexing both the SS/PBCH block and the initial active DL BWP.

Table 2.1-1 Comments for the Bandwidth combining the SSB and initial active DL BWP

| Company Name | Comments |
| --- | --- |
| CATT | It was agreed that initial DL BWP was defined as “the frequency location and bandwidth of RMSI CORESET and numerology of RMSI”, and “NR supports FDM transmission of QCLed SS/PBCH block and RMSI (CORESET/NR-PDSCH)”. It is then clear that Alt. 1 does not apply. Our opinion is to support Alt. 2 since the supported bandwidth combining RMSI and SSB should depend on UE’s minimum capability. |
| vivo | The motivation of this constraint is not clear. The total bandwidth of SS block and RMSI CORESET may exceed UE minimum bandwidth in some cases to facilitate flexible network deployment. UE with low capability can perform RF re-tuning for RMSI decoding purpose, so it is unnecessary to support simultaneously receiving SSB and RMSI CORESET. |
| Spreadtrum | If several FDMed SS blocks can be associated with one RMSI, the limitation of X shall not be introduced. If only one SS block(in frequency domain) can be associated with one RMSI, our opinion is to support Alt. 2. The reason is the same as CATT. |
| Samsung | The limitation of X should be mainly for finding the best set of RMSI CORESET locations in the frequency domain, relative to the SSB BW, given the limited bits available in PBCH for the CORESET BW/offset indication. UE minimum BW, min CH BW, RMSI PDCCH coverage (number of CCEs), etc., should be jointly considered to determine X. Our proposal is to discuss frequency-domain CORESET mapping principles, including the resulting value of X. |
| Ericsson | Our preference is alternative 3 |
| Nokia | For FDM operation perspective our preference would be alt 3. |
| Huawei | Based on the agreements from last meeting, the cell-defining SS/PBCH block can be located outside the initial active DL BWP at least in case of FDM between cell-defining SS/PBCH block and corresponding RMSI. However, it may be beneficial that the combined the bandwidth of the two can still be confined within a limited bandwidth, e.g. UE minimum bandwidth. The fundamental reason is that the frequency location of initial active DL BWP should be signaled in PBCH. Considering that SS/PBCH block is transmitted on a sync raster, if there is no restriction on the combined bandwidth of the cell-defining SS/PBCH block and RMSI, the number of bits required to indicate frequency offset may become too large. Alt.2 is preferred. This restriction should be also be considered when frequency location and bandwidth for RMSI CORESET are discussed. |
| Intel | Our preference is Alt 2 |
| ZTE | There is no need to limit X to UE minimum BW or the minimum carrier BW, in general, since RF re-tuning for RMSI reception is acceptable. PBCH already configures if RMSI SCS is the same as SSB SCS (except for 240 kHz SSB). A larger Y is motivated when SSB and RMSI use different SCS, i.e. RMSI is in a different BWP than SSB. In other words, the configuration of Y may depend on if RMSI SCS is the same as SSB SCS, in order to save configuration bits in PBCH. |
| InterDigital | Our preference is Alternative 2 |
| Qualcomm | It is highly desirable to confine SSB/RMSI BW within the initial active DL BWP. If this confinement is not possible in some cases, the SSB/RMSI BW is confined within the UE minimum BW. Whether such confinement is within the initial active DL BWP or the UE minimum BW could depend on RMSI CORESET BW, SSB SCS and SSB/RMSI multiplexing. To make progress in RMSI CORESET design, RAN1 should make assumptions on UE minimum BW and send an LS to RAN4 for informing the UE minimum BW assumptions. |
| CMCC | As mentioned by CATT, Alt 1 does not apply for the FDM case. For Alt 3, we think it is too restrictive since the minimum carrier BW for below 6GHz is only 5MHz/10MHz. For Alt 2, since the UE minimum bandwidth is to be determined by RAN4, it is unclear for RAN 1 to design indication of RMSI CORESET without knowing the exact value. We tend to agree with VIVO’s comment that there is no need for the constraint. Instead, what is more important is to discuss about is the maximum value of Y that can be supported by the limited payload size in PBCH to indicated the PRB offset of RMSI CORESET. For UEs that have a high capability that can simultaneously receive PBCH, otherwise, it retunes RF for RMSI reception, for either of these UEs, the RMSI is confined within the initial active DL BWP which is always no greater than the UE minimum bandwidth. |
| OPPO | We suggest to determine X after we decide whether many SS/PBCH blocks to one RMSI would be suppoted. If many SS/PBCH blocks to one RMSI is supported, there shall not be limitation of X. Otherwise, alt 2 can be adopted. |

Summary:

About the bandwidth multiplexing the SS/PBCH block and the initial active DL BWP, the opinions may be summarised as follows:

- Opinion 1: confined within initial active DL BWP

  - Supported by: MediaTek, LGE(TDM), Lenovo, Motorola Mobility, Qualcomm (TDM of SSB and RMSI CORESET)

- Opinion 2: confined within UE minimum bandwidth

  - Supported by: CATT, Huawei, LGET(FDM), Spreadtrum (assume one-to-one mapping in wideband), Intel, InterDigital, Qualcomm (FDM of SSB and RMSI CORESET)

- Opinion 3: confined within the minimum carrier bandwidth for a given band

  - Supported by: Ericsson, Nokia

- Opinion 4: is not limited by initial active DL BWP or UE minimum bandwidth

  - Supported by: vivo, ZTE,OPPO(when many SS/PBCH blocks to one RMSI is supported)

- Opinion .5: change definition on initial active DL BWP to include RMSI BW + SSB BW:

  - Supported by: NTT DOCOMO

- Opinion .7: jointly considered with UE minimum BW, min CH BW, RMSI PDCCH coverage (number of CCEs), etc.,  :

  - Supported by: Samsung

- Opinion  8 : confined as RMSI BW + SSB BW for one-to-one mapping

  - Supported by: Xiaomi

From the design of the RMSI CORESET configuration point of view, we will need to decide whether the RMSI CORESET configuration that supports the following scenarios including FDM and TDM cases:

The bandwidth combining the SS/PBCH blocks and RMSI CORESET is confined within minimum carrier bandwidth;

both the SS/PBCH blocks and RMSI CORESET is confined within UE minimum bandwidth;

the bandwidth including the SS/PBCH blocks and RMSI CORESET is not confined within UE minimum bandwidth

Suggested Offline Proposals:

- RMSI CORESET configuration should support the scenarios when the bandwidth multiplexing the SS/PBCH blocks and initial active DL BWP

  - is confined within minimum carrier bandwidth in TDM mode;

  - is confined within minimum carrier bandwidth in FDM mode;

  - is confined within UE minimum bandwidth for TDM mode;

  - is confined within UE minimum bandwidth for FDM mode;

- FFS: send an LS to RAN4 for informing the UE minimum BW assumptions

- FFS: RMSI CORESET configuration supports the scenarios when bandwidth multiplexing the SS/PBCH blocks and initial active DL BWP is NOT confined within UE minimum bandwidth for TDM and/or FDM case;

## Association between SS blocks and the corresponding RMSI(s) in wideband operation

For wideband operation, it was agreed that SS/PBCH blocks may be transmitted in one or more BWPs. However, the association between SS Block(s) and RMSI(s) is undecided. From the proposals submitted in RAN1#90bis, we may have the following options:

Option 1: One-to-one association between one SS/PBCH block and one RMSI

Option 2: Many-to-one association between multiple SS/PBCH blocks and one RMSI

Option 3: One-to-many association between one SS/PBCH block and multiple RMSIs

Figure 2.2 Association between SS blocks and the corresponding RMSI(s) in wideband operation

From UE perspective, both the options shown in Figures 2.2(a) and 2.2(b) can be seen as one-to-one association between an SS/PBCH block and an RMSI. (Note: The Options 2 and 3 in Figure 2.2(b) and (c) may not be valid options if DL bandwidth combining the SS/PBCH block and initial active DL BWP is confined with the initial active DL BWP (Alt.1 in the Section 2.1)).

Note: It is unclear whether Figure 2.2(c) is a viable option since it requires the CORESET configuration in MIB to point to two CORESET locations in different frequency locations.

In Table 2-2, interested companies are encouraged to provide their views on the options for the association between SS blocks and the corresponding RMSI(s) in wideband operation.

Table 2.2-1 Comments for the Bandwidth combining the SSB and initial active DL BWP

| Company Name | Comments |
| --- | --- |
| CATT | For both one-to-one mapping and many-to-one mapping between SSBs and RMSI CORESET in wideband operation, it can be seen as one-to-one mapping from UE’s perspective. For one-to-many mapping, the RMSI CORSET configuration could be much more complicated since it is not one-to-one mapping from UE’s perspective. Also, the benefit of one-to-many mappings in wideband operation is unclear. |
| vivo | Option 1 can be considered as a baseline. With regard to option 2 and option 3, we notice each of them has two cases: TDM based and FDM based. For TDM based case, SSB and CORESET are located at the same frequency carrier, as shown in the following new Figures 2.2(d) and 2.2(e). FDM based case are given by CATT in Figures 2.2(b) and 2.2(c). We support TDM based option 2 and option 3 for the following reasons: - For TDM based option2, it introduces some additional benefits, e.g. support SS block repetition. It can be considered as a further enhancement on top of Alt.1. - For TDM based option3, it provides higher flexibility for RMSI scheduling and it is up to the NW to transmit RMSI PDCCH in any one of the associated CORESET instances. This mapping relation is exactly the RMSI timing configuration we discussed in 2.5. For FDM based option 2 and option 3, we have the following observations: - For FDM based option2, it seems unclear that how to associate multiple SSBs in different BWPs with the same CORESET. Additional information on CORESET position, e.g. frequency offset from a reference point, may be needed for the other SSB. Use case of FDM based option2 is not clear so far. - For FDM based option3 shown in figure(d), UE with high capability has to monitor multiple CORESETs at the same time; and for the minimum capability UE, the receiver has to retune to various BWPs one by one and to blind detect each associated CORESET. Also, use case of this mapping relation is not clear. |
| Spreadtrum | Our opinion is to support option 2 - If some of the SS blocks can be detected during the cell-search process, and they cannot be used to access the corresponding cell, these SS blocks may cause some unnecessary works on PSS/SSS/PBCH detecting and decoding during the cell-search process. This scenario will bring extra complexity and latency to the cell-search procedure. - If each of the FDMed SS blocks in a wideband CC is associated with a corresponding RMSI, it ensures that every SS block can be used to find and access the corresponding cell, but it may bring multiple FDMed RMSIs in a wideband CC which results in high overheads. - If several FDMed SS blocks in a wideband CC can be associated with one RMSI. The overhead of RMSI is low and a UE can use any of the FDMed SS blocks in a wideband CC to find a cell, obtain the corresponding RMSI and access the cell. |
| Samsung | Option 1 is baseline and it is proposed to focus on option 1 in this meeting. Option 2 may naturally be supported at least in limited cases if spec is written to support option 1. Additional support of option 3 does not seen to be essential. |
| Ericsson | Our understanding is that nothing additional needs to be done to support option 1. Support for option 2 depends on the range on the resource allocation, but we do not see this as critical for Rel-15. |
| Nokia | Our preference is to focus on option 1. From IDLE mode operation perspective, if multiple SSBs are needed to distribute the load, UE can be informed the inter-frequency carriers to measure and it would be possible to support them with option 1 at least in Rel-15. In case of initial cell selection, UEs should search until it finds a SSB that has an associated RMSI as per option 1 (and indication of RMSI presence in PBCH). |
| Huawei | Option 1 is supported and a UE can be indicated that a certain SS/PBCH block is not associated with a corresponding RMSI in PBCH. |
| Intel | Option 1 as baseline. Discuss further on additional options, if they can be supported without any additional bits in PBCH. |
| ZTE | Option 1 needs to be supported. Option 2 can also be supported for some combinations of SSB frequency separation without additional configuration bits, also depending on the details of FDMed CORESET configuration. From the UE perspective, Option 2 looks like Option 1 since the UE uses the cell-defining SSB to obtain RMSI CORESET configuration. |
| InterDigital | Option 1 is considered as baseline. Other options can be considered if there is a need. |
| Qualcomm | In wideband operation, we could have two types of SSB: (1) cell-defining SSBs having associated RMSI and (2) SSBs for measurement purposes (aka measurement SSBs) which don’t have associated RMSI. We assume that this question is applied to cell-defining SSBs only. In our view, only one-to-one mapping between cell-defining SSB to RMSI CORESET (Option 1) is specified due to its efficiency in signalling RMSI CORESET frequency location. - The option of having one RMSI CORESET associated with multiple cell-defining SSBs (Option 2) leads to high overhead in signalling RMSI CORESET frequency location since SSBs can be on any sync raster within the wideband CC. - The option of having multiple RMSI CORESET associated with one cell-defining SSB (Option 3) is not necessary. |
| CMCC | Option 1 can be considered as a baseline. Regarding the one-to-many case, we see it important to allow an SSB to be associated with one RMSI CORESET within the same slot and also other RMSI CORESET in different slot(s) from SSB transmission. We see this scenario beneficial for both gNB and UE. For gNB, this provides more flexibility in transmitting RMSI. For UE, this provides better RMSI reception performance in several scenarios: 1. when UE fails to decode PDCCH in the same slot of SSB; 2. When RMSI and SSB are FDMed and UE does not support mixed numerology on the same symbol. In these scenarios, if one-to-many mapping is supported, the UE does not need to wait until the next RMSI periodicity to receive. |
| OPPO | Option 2 shall be supported to reduce the initial access latency and system overhead. For well support of many SS/PBCH blocks to one RMSI, the indication granularity of the frequency offset of RMSI CORESET relative to the SS/PBCH frequency location shall be large enough. We don’t see the clear benefits of option 3. |

Summary:

Option 1: One-to-one association between one SS/PBCH block and one RMSI CORESET in wideband operation (i.e., in frequency domain)

  - Supported by: Huawei, NTT DOCOMO, Qualcomm, vivo, Lenovo, Motorola Mobility, Intel, InterDigital, etc.

Option 2: Also support many-to-one association between multiple SS/PBCH blocks and one RMSI CORESET from network perspective

  - Supported by: Ericsson, Spreadtrum Communications, Samsung, CATT, ZTE,OPPO

Option 3: One-to-many association between one SS/PBCH block and multiple RMSI CORESETss

  - Supported by:

Suggested Offline Proposals:

- RMSI CORESET configuration should support one-to-one association between one SS/PBCH block and one RMSI in wideband operation

- FFS: RMSI CORESET configuration supports many-to-one association between multiple SS/PBCH blocks and one RMSI in wideband operation

## Different Numerologies of SS/PBCH block and RMSI CORESET

NR supports different numerologies between SS/PBCH block and RMSI (PDCCH/PDSCH) by the use of the ‘1’ bit indication in PBCH.

### Different numerologies for FDM

When the SSBs and the RMSI CORESETs are FDMed, the UE will need supporting simultaneous reception of SSBs and the RMSI PDCCH with mixed numerologies in order to take the advantage of FDM. However, according to the LS from RAN4 (R4-1708864), “Support of simultaneous reception of Data/SS with mixed numerologies is optional from UE implementation perspective.” Then, the question is whether and how to support FDM of SSBs and the RMSI CORESETs with mixed numerologies.

Note: For SSBs with SCS 240kHz, the corresponding RMSI SCS is either 60 kHz or 120kHz. Thus, when SSBs with 240 kHz SCS are FDMEd with RMSI, the RMSI SCS is always different from the SSB SCS.

Table 2.3-1 Comments on the support of different numerologies for FDM

| Company Name |  | Comments |
| --- | --- | --- |
| CATT |  | - Supporting different numerologies for SS/PBCH blocks and the RMSI CORESETs for FDM cases. - Support all possible combination of the different numerologies for SS/PBCH blocks and the RMSI CORESETs, namely {SSB SCS, RMSI SCS} = {{15, 30}, {30, 15}, {120, 60}, {240, 60}, {240, 120}} kHz in addition to the cases where the SSB and RMSI have the same numerology. - Define the RMSI CORSET configuration for each combination of the different numerologies in separate tables. - Note 1: The RMSI CORSET configurations for some combination may be same. - Note 2: For UEs that do not supporting simultaneous reception of SS/PBCH blocks and RMSI PDCCH with mixed numerologies may require longer time for initial access. It will be up to RAN4 to define the performance requirements. |
| vivo |  | We suggest only supporting the same numerology for FDM cases except for SSBs with SCS 240kHz since 240kHz is not applicable to data. The main motivation for supporting FDM operation is to achieve fast beam sweeping for RMSI and potentially to reduce the delay for initial access procedure. Comparing two schemes: FDM with same numerology and FDM with different numerology, FDM with different numerology will have no extra benefit compared with FDM with same numerology. Furthermore, FDM with different numerology will introduce the following shortages: - a guard band is needed in order for avoiding the interference at edges of SSB and RMSI CORESET - additional UE complexity as UE has to switch from one sampling rate to another in turns All potential combinations of SSB and CORESET for TDM case or FDM case are listed here: - Combination 1(FDM+TDM): SSB SCS 15kHz, CORESET SCS 15kHz - Combination 2(FDM+TDM): SSB SCS 30kHz (pattern2), CORESET SCS 30kHz - Combination 3(FDM+TDM): SSB SCS 30kHz (pattern1), CORESET SCS 30kHz - Combination 4(FDM+TDM): SSB SCS 120kHz, CORESET SCS 120kHz - Combination 5(TDM only): SSB SCS 15kHz, CORESET SCS 30kHz - Combination 6(TDM only): SSB SCS 30kHz (pattern1), CORESET SCS 15kHz - Combination 7(TDM only): SSB SCS 120kHz, CORESET SCS 60kHz - Combination 8(FDM+TDM): SSB SCS 240kHz, CORESET SCS 60kHz - Combination 9(FDM+TDM): SSB SCS 240kHz, CORESET SCS 120kHz In our opinion, there are two ways to design the lookup table for RMSI CORESET configuration: - alt1: as mentioned by CATT, define separate tables for each combination - alt2: define two tables for same numerology case and mixed numerology case respectively There are 9 different combinations of SSB and CORESET. It is noted that some combinations with the same RMSI CORESET config-index have the same RMSI CORESET configurations, e.g. combination 6&7, combination 1&2, combination 3&4. However, considering that these combinations may have different CORESET BW limitations, Alt 1 still requires 9 tables in specification for all combinations but with a large amount of duplicated content. Alt 2 will only require 2 tables. Moreover, there seems no differences on the lookup table size between Alt 1 and Alt 2. Thus we simply prefer alt.2. The following form could be considered as a start point of the RMSI CORESET configuration table: Bit field in NR-PBCH; Deployment Scenario; CORESET Configuration. 0000; Combination x; Description of the RMSI CORESET T/F resources pattern e.g., AL=8, FDM’ed with SSB, CORESET BW=12 PRBs). Combination y. etc. 0001; ……; ……. Each codepoint (e.g. RMSI CORESET config-index) in the table contains multiple combinations of SSB SCS/RMSI CORESET SCS, which have their own corresponding RMSI CORESET configurations. RMSI CORESET configuration can be determined once the RMSI CORESET config-index and SSB/RMSI CORESET combinations are specified through MIB. Thus, only 2 tables are required, one for same numerology case and the other for the mixed numerology case. |
| Samsung | Among the different SSB and RMSI SCS combinations of {{15, 30}, {30, 15}, {120, 60}, {240, 60}, {240, 120}} kHz, other than {240, 60} combination should be supported. There is a challenge to support {240,60} combination for the FDM case, as the number of OFDM symbols available for CORESET mapping in RMSI numerology is quite limited (see below figure). Hence, it is proposed to focus on the support of {{15, 30}, {30, 15}, {120, 60}, {240, 120}} kHz in this meeting, and discuss further next year about the support of {240,60}. |  |
| Ericsson |  | We do not see that limiting the numerology configurations leads to any reduction in specification and implementation effort. The current agreement that RMSI can use SCS 15 and 30 kHz for FR1 and 60 and 120 kHz for FR2 are sufficient. |
| Nokia |  | SSB numerology(ies) are frequency band dependent and the PBCH includes 1 bit numerology indication for RMSI. Hence we do not see need to restrict further. |
| Huawei |  | For FDM case, only support same numerologies for SS/PBCH blocks and the RMSI CORESETs. For 240kHz, only TDM case is supported. In general a table-based approach can be considered including the indication of RMSI timing configuration and CORESET resource configuration. We suggest discussing all related aspects for different combinations of SCS for SS/PBCH and RMSI. |
| Intel |  | Baseline of having have numerology when RMSI is FDM multiplexed. Additional modes (with different numerology) can be discussed and supported if they can be supported without additional increase in number of bits in PBCH. |
| ZTE |  | Same view as Ericsson and Nokia. |
| InterDigital |  | In our view we don’t see the need to restrict numerology configuration. |
| Qualcomm |  | RMSI numerology can be different from SSB numerology. This does not mean that UE needs to receive both SSB and RMSI at the same time. NR does not support FDM of SSB and RMSI CORESET for below-6GHz. |
| CMCC |  | Since the numerology is already indicated in PBCH, it is natural to support both same and different numerologies. However, we would like further understand the scenario where 240KHz SSB and 60kHz RMSI would be used. |
| OPPO |  | The LS from RAN4 has clear statement that “Support of simultaneous reception of Data/SS with mixed numerologies is optional from UE implementation perspective.”. For SSB SCS 240k, it is impossible to have same SCS as RMSI. For all other cases, we think it should configure same SCS of SSB and RMSI |

Summary:

Different numerologies for SS/PBCH blocks and the RMSI CORESETs in FDM mode:

  - Option 1: Support all possible combinations:

    - Supported by: CATT, InterDigital, Ericsson, Nokia, ZTE, Qualcomm (need further discussion on whether to support 240kHz SSB and 60kHz RMSI or not)

  - Option 2: Support all possible combinations except {240,60}:

    - Supported by: Samsung

  - Option 3: Support only for the combinations when SS/PBCH SCS is 240kHz

    - Supported by: vivo,OPPO

  - Option 4: Do not support any combination

    - Supported by: Intel

  - Option 5: FFS:

    - Supported by: MediaTek

NR does not support FDM of SSB and RMSI CORESET for below-6GHz

  - Supported by: Qualcomm

Suggested Offline Proposals:

For different numerologies for SS/PBCH blocks and the RMSI CORESETs in FDM mode, down-select one of the following options:

  - Option 1: Support all possible combinations

  - Option 2: Support some of possible combinations ,

    - E.g, When RMSI SCS is half of the SS/PBCH block SCS {SSB SCS, RMSI SCS} = {30, 15}, {120, 60}, {240, 120} kHz

  - Option 3: Support only for the combinations when SS/PBCH SCS is 240kHz

    - E.g., {SSB SCS, RMSI SCS} = {240, 60}, {240, 120} kHz

  - Option 4: Do not support any combination

  - Option 5: Support all possible combinations only for above-6GHz

### Different numerologies for TDM

Similar with FDM cases, we will need to decide whether and how to support the RMSI CORESET configuration for different numerologies between the SS/PBCH blocks and the RMSI CORESETs.

Table 2.3-2 Comments on the support of different numerologies for TDM

| Company Name | Comments |
| --- | --- |
| CATT | - Supporting different numerologies for SS/PBCH blocks and the RMSI CORESETs for TDM cases. - Support all possible combination of the different numerologies for SS/PBCH blocks and the RMSI CORESETs, namely {SSB SCS, RMSI SCS} = {{15, 30}, {30, 15}, {120, 60}, {240, 60}, {240, 120}} kHz in addition to the cases where the SSB and RMSI have the same numerology. - Define separately the RMSI CORSET configuration for each combination of the different numerologies - Note: The RMSI CORSET configurations for some combination may be same. |
| vivo | We share CATT’s view that mixed numerologies for SSB SCS /RMSI CORESET SCS for TDM cases should be supported but only one table is needed for the mixed numerology case, as discussed in Table 2.3-1. UE can get the exact combination after receiving DL numerology indicator and will determine the configuration of its corresponding CORESET from above table. |
| Samsung | Similarly to the TDM case, it is proposed to focus on the support of {{15, 30}, {30, 15}, {120, 60}, {240, 120}} kHz in this meeting, and discuss further next year about the support of {240,60}. |
| Ericsson | Again, the current agreements are all that is needed(15 and 30 kHz for FR1 and 60 and 120 kHz for FR2) |
| Nokia | Same as for FDM. |
| Huawei | For TDM case, all possible combinations of numerologies for SS/PBCH block and RMSI should be supported and the configuration table for RMSI CORESET can include all combinations. |
| Intel | In the TDM case, all combination of numerology should be supported. |
| ZTE | Same as for FDM. |
| InterDigital | All combinations of numerologies should be supported for TDM. |
| Qualcomm | RMSI numerology can be different from SSB numerology. For below-6GHz, NR supports TDM of SSB and RMSI CORESET only. |
| CMCC | Since the numerology is already indicated in PBCH, it is natural to support both same and different numerologies. However, we would like further understand the scenario where 240KHz SSB and 60kHz RMSI would be used. |

Summary:

Different numerologies for SS/PBCH blocks and the RMSI CORESETs in TDM mode:

  - Option 1: Support all possible combinations:

    - Supported by: InterDigital, vivo, CATT, Ericsson, Nokia, Intel, ZTE, Qualcomm

  - Option 2: FFS:

    - Supported by: MediaTek

For below-6GHz, NR supports TDM of SSB and RMSI CORESET only.

  - Supported by: Qualcomm

Suggested Offline Proposals:

For different numerologies for SS/PBCH blocks and the RMSI CORESETs in TDM mode, down-select one of the following options:

  - Option 1: Support all possible combinations

  - Option 2: Support some of the combinations

    - E.g, {SSB SCS, RMSI SCS} = {30, 15}, {120, 60}, {240, 120} kHz

  - Option 3: Support all possible combinations only for below-6GHz

## RMSI CORESET(s) Configuration

The following agreements were made on RMSI CORESET(s) in RAN1#90bis.

Agreements: - UE assumes the DMRS of NR-PDCCH transmitted in the CORESET for RMSI and the DMRS of NR-PDSCH for RMSI/broadcast OSI is QCLed with the corresponding SS/PBCH block - FFS: On the details on the associations between SS blocks and monitoring windows for RMSI CORESETs/broadcast OSI.

Agreements: ([90b-NR-04] CORSET and RMSI Configuration)  (Working assumption) PBCH contents, except the SSB index, should be the same for all SS/PBCH blocks within an SSB burst set for the same center frequency  The maximum number of bits for configuration of RMSI CORESET(s) and RMSI timing in PBCH is X bits excluding the subcarrier spacing.  X is TBD, and can be chosen to be up to [8] bits.  Note: RMSI CORESET(s) means the CORESET(s) configured by PBCH  Configuration of RMSI CORESET(s) should consider at least the following properties:  bandwidth (PRBs)  frequency position (frequency offset relative to SS/PBCH block)  A set of consecutive OFDM symbol indices in a slot corresponding to a single CORESET  FFS: signaling details including what is captured in specifications and what is signaled in the MIB

Agreements: ([90b-NR-23] Email discussion on association among CORESET/search space/RNTI/DCI format size) Proposal 1 - For PDCCH monitoring occasion of 1-slot, 2-slot, [5-slot], [10-slot], and [20-slot], - Slot-level offset value for PDCCH monitoring occasion is also supported. - For N-slot monitoring occasion, the offset is one from [0, N-1]. - Note: symbol-level bit-map of monitoring occasion within a slot agreed at RAN1#90bis is still available. Proposal 2 - For the DMRS of NR-PDCCH in a CORESET, - The QCL configuration/indication is on a per CORESET basis (Alt.1). Proposal 3 - The value(s) of TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, and/or TPC-SRS-RNTI, are provided by RRC signaling. - The association between at least each of the following RNTIs and a DCI format is specified in the specification. - C-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI. FFS: other RNTI(s). - The value of C-RNTI is obtained as part of random access procedure. - The association between a DCI format and a type of search space (UE-common search space and UE-specific search space) is specified in the specification. - UE-common search space contains a DCI format of C-RNTI, RNTI(s) for SPS/grant-free, TPC-PUSCH-RNTI, TPC-PUCCH,RNTI, TPC-SRS-RNTI, and INT-RNTI. - UE-specific search space contains a DCI format of C-RNTI and RNTI(s) for SPS/grant-free. Proposal 4 - By PBCH, a UE obtains at least one CORESET configuration at least for PDCCH scheduling RMSI associated with a given SS block. - The set of aggregation levels and candidates per aggregation level for PDCCH scheduling RMSI is specified in the specification. - FFS the indication of the support of aggregation level 16 in the cell - FFS: Set of search spaces for OSI, random access, and paging. - By RMSI, the UE can be configured with at least one CORESET configuration at least for PDCCH for random access. - If not configured by RMSI, the CORESET configuration(s) for random access is/are the one(s) configured by PBCH. - FFS: whether the CORESET configuration can be configured outside of the initial active DL BWP. - By UE-specific RRC signalling, the UE can be configured with one or more CORESET configuration(s) at least for PDCCH scheduling UE-specific data. - Each CORESET configuration is associated with one or more sets of search spaces. - Note: each set of search spaces is associated with one CORESET configuration.

The RMSI CORESET configuration indicates the resource of the PDCCH that schedules the PDSCH carrying RMSI. While some of the details of the RMSI CORESET (e.g., The set of aggregation levels and candidates per aggregation level for PDCCH scheduling RMSI ) may be decided in control channel session, the Initial Session needs, at least, to determine the configuration properties of RMSI CORESET(s) agreed in the email discussion of [90b-NR-04].

Based on the contributions submitted to RAN1#90bis, the RMSI CORESET may be configured with following options:

Option 1: Using a table with a list of configuration indices. Each index represents a set of pre-defined configuration for the  CORESET properties (e.g., bandwidth (PRBs), frequency position, CORESET duration/OFDM symbols)

Option 2: Configure each CORESET property separately (e.g., frequency location is defined with a different parameter from the one used for time location, etc.)

Option 3: Some properties are configured together using a table, while others are configured separately.

In addition, regardless which of the above options is adopted, some of the properties/parameters may be explicitly indicated by the RMSI CORESET configuration carried in the MIB, while some others could be derived from the indicated properties/parameters once the relationship between them are defined in the specifications.

The intention of Table 2-3.1 is to collect the considerations from interested companies on the preferred option for the configuration of RMSI CORESET(s).

Table 2.4-1: Comments on the Configuration Methods of RMSI CORESET(s)

| Company Name | Option 1 | Option 2 | Option 3 | Comments |
| --- | --- | --- | --- | --- |
| CATT | Yes |  | [Yes] | Our opinion is to only use Option 1 if possible. Option 3 can be considered if some RMSI CORESET properties are identified to be totally independent of others. In this case, the configuration of RMSI CORESET may be simpler with separate smaller tables. |
| vivo | Yes | No | No | We prefer option1 to potentially save NR-PBCH payload. In addition, the following extra two aspects need to be considered when designing RMSI CORESET configuration: - Aggregation level and/or search space should be jointly considered in addition to combinations of parameters (such as bandwidth, duration, start timing and frequency position of CORESET etc.); - RMSI CORESET configurations should consider using DL numerology indicator as a prior information, which can potentially save PBCH payload overhead or provide much more flexible room for RMSI CORESET configuration. |
| Spreadtrum | [yes] | No | Yes | Our opinion is to support Option 3. Some RMSI CORESET properties are totally independent of others |
| - Samsung |  |  |  | - As we mentioned during the e-mail discussion, whether the spec will capture a big table or multiple small tables should be up to the spec editor. - We suggest focusing on technical aspects, i.e., (1) identify detailed parameters need to be indicated for the time/frequency domain mapping of CORESETs; (2) options/values should be supported for each of the mapping parameters considering the limited payload of the PBCH, and (3) whether some/all group of parameters can/should be jointly indicated. - As a side note, we feel that the discussion is likely to be more efficient if the parameters can be separately discussed; and could get alignment for the parameters one by one. |
| - Ericsson | Yes |  |  | - Agree to focus on what to signal rather than format, but believe that a table is likely to make it easier to get agreements. |
| - Nokia | Yes |  |  | - If, after we have concluded the set of configurations needed, we can partition them in meaningful manner it can be considered, but currently we feel that table format is the preferable way. |
| - Huawei | [yes] | - No | - Yes | - Our view is that this may not be essential for discussion. It will be much more important to focus on figuring out the configuration details for RMSI. We share similar view with Samsung that we could list all the possible aspects and discuss them separately. |
| - Intel | [yes] |  | - Yes | - Once we have figured out all the essential combinations of configurations that is needed to be supported, we should be able to converge on a unified solution. If the number of bits is not enough, we may be able to look towards option 1. However, from our analysis, the number of required combinations that CORESET configuration should support is fairly large, and we do not have much bits in PBCH, therefore option 3 is most likely outcome. |
| - ZTE |  |  | - Yes | - Focus on the technical aspects and configurations rather than signalling format (i.e. table(s) and/or explicit parameters) |
| - InterDigital | Yes |  | - [Yes] | - Our view is to use Option 1. Option 3 can also be considered. |
| - Qualcomm | Yes |  |  | - RMSI CORESET configuration table includes combinations of SSB SCS, RMSI SCS, SSB/RMSI CORESET multiplexing (TDM vs. FDM), RMSI CORESET BW, OFDM symbol location for RMSI CORESET, RMSI CORESET frequency position. |
| - CMCC | [Yes] |  | - Yes | - Some of the parameters may be dependent |
| - OPPO | [Yes] | - No | - Yes | Our opinion is to support Option 3. - We think some RMSI CORESET properties can be summarized in a table, but others such as frequency offset may be independent to others |

The intention of Table 2-2.2 is to collect the opinions on the configuration of each RMSI CORESET properties, such as the values [ranges], the granularity etc. of the properties. The discussion may need to consider both TDM/FDM of SSB and RMSI CORESET cases.

Table 2.4-2: Comments on the individual RMSI CORESET(s) Properties

| Company Name | CORESET Properties | Comments |
| --- | --- | --- |
| CATT | bandwidth (PRBs) | RMSI CORESET bandwidth and the number of OFDM symbol indices of a single CORESET may be related, since it is important to ensure that the total PRBs is larger enough to support reliable RMSI CORESET delivery. For example, assume we need minimum total 48 PRBs for RMSI CORESET, the bandwidth needs to be 48 PRBs if there is only one OFDM symbol in the RMSI CORESET. The bandwidth may be reduced to 24 PRBs if using two OFDM symbols in the RMSI CORESET. |
|  | frequency position | For NR, SSB may be placed anywhere within the carrier bandwidth. Thus, the configuration of the RMSI CORESET frequency position needs to be flexible enough in one hand. However, due to the limitation of the number of configurations, we may need to limit the possible number of configurations. In our view, the relative frequency positions between RMSI CORESET and the SSB can be limited up to a few, e.g.,[5] possible configurations. |
|  | OFDM symbol indices of a single CORESET | See the comments for bandwidth (PRBs) |
|  | Others(?) |  |
| Spreadtrum | bandwidth (PRBs) | The configuration of “bandwidth” needs to consider a various combinations of SSB SCS and CORESET SCS |
|  | frequency position | The configuration of the RMSI CORESET frequency position needs to be flexible enough to support “Many-to-one association between multiple SS/PBCH blocks and one RMSI in frequency domain”. |
|  | OFDM symbol indices of a single CORESET | The configuration of “OFDM symbol indices of a single CORESET” needs to consider a various combinations of SSB SCS and CORESET SCS |
|  | Others(?) |  |
| Samsung | bandwidth (PRBs) | We propose to jointly consider the number of PRBs and the number of CORESET OFDM symbols, as the number of PRBs and number of CORESET OFDM symbols jointly determine the number of CCEs for the CORESET. Given the coverage challenges especially for over6GHz case, our preference is to support 48 and 96 total number of REGs so that maximum 8 and 16 CCE aggregations can be supported for RMSI PDCCH transmissions. The number of PRBs and the number of OFDM symbols should be indicated in terms of RMSI numerology, and hence the candidate values for the indication should take into account the SSB BW, min CH BW, UE min BW, channel & sync raster, along with the number of CCEs to support. For this feature, up to 4 options seem to be sufficient. |
|  | frequency position | The frequency position of CORESET should be defined as the relative frequency position comparing to SS/PBCH block, in terms of CORESET numerology. Due the possible misalignment of SS/PBCH PRB grid and CORESET PRB/RE grid, as well as potentially different BW of SS/PBCH block and CORESET, both PRB level and RE level (e.g. floating sync) frequency offsets (or a combination of the two to save bit-width) should be indicated in MIB. The configuration should also take into account the relationship between SS/PBCH SCS and CORESET SCS, multiplexing pattern (e.g. TDM or FDM), and CORESET BW. The exact number of bits for indicating the frequency position depends on the flexibility NR wants to support (e.g. combination of SCS ratio, CORESET BW, and multiplexing pattern), and a baseline we propose for discussion can be 8 bits (including floating sync), or 4 bits (excluding floating sync). |
|  | OFDM symbol indices of a single CORESET | OFDM symbol indices can be broken down into the number of consecutive OFDM symbols and the starting OFDM symbol index. The number of symbols has been discussed above; and it should consider the coverage (i.e., # of REGs) The starting OFDM symbol index should be determined dependent upon whether TDM vs. FDM is used, and also whether mini or full slot is used. For the starting OFDM symbol index, up to 1 bit (2 options) indication can be supported, and for multi-beam support the SSB index should jointly considered for the final mapping. |
|  | Others(?) | The following CORESET parameters should be hard coded in the spec: - REG bundle size (6 PRBs) - Transmission type (interleaved only) - RS type, i.e., WB vs. SB (WB-RS only) - QCL’ed RS (one-to-one mapping, i.e., SS block i is QCL’ed with CORESET i) - The number of PDCCH candidates for each aggregation level - TBD by the control session |
| Ericsson | bandwidth (PRBs) | TDM: 18, 24, 48 and 96 FDM: 24, 48 and 96 For TDM, 18 is introduced to avoid having to signal frequency offset with fine granularity in narrow bandwidths. |
|  | frequency position | Alternatives TDM: - Centered around SS/PBCH block - Aligned with lower frequency edge of SS/PBCH block - Aligned with higher frequency edge of SS/PBCH block Alternatives FDM: - Below SS/PBCH block in frequency with possible offset - Above SS/PBCH block in frequency with possible offset |  |
|  | OFDM symbol indices of a single CORESET | Depends on bandwidth of CORESET: - 3 OS for 18 and 24 PRB - 2 OS for 24, 48 and 96 PRB - 1 OS for 48 and 96 PRB |  |
|  | Others(?) |  |  |
| Nokia | bandwidth (PRBs) | Option of supporting at least 24 PRB and 48 PRB bandwidth for FDM and TDM |
|  | frequency position | For TDM, PRB offset {-3,-2,-1,0,1,2,3} where the offset reference numerology is the min(SSB_scs,RMSI_scs) to enable alignment to data PRB grid and placement in the system bandwidth. For FDM, placement to above and below of the SSB, with PRB offset options {0,1} to align with the data PRB grid. |
|  | OFDM symbol indices of a single CORESET | Depending on the bandwidth {24 PRB, 2 symbols}, {48 PRBs, 1 symbol} to reach AL=8 |
|  | Others(?) | REG bundle size: 6PRBs Interleaved PDCCH transmission |
| Huawei | bandwidth (PRBs) | To save the signaling overhead, the OFDM symbols indices and the bandwidth of the RMSI CORESET can be determined jointly. In addition, the frequency pattern of the RMSI CORESET can also be indicated in a joint manner. For the bandwidth, it can be signaled in the number of PRBs, REG bundle or CCEs. For example, it can be a multiple of 6 PRBs (corresponding to 1 CCE in 1 OFDM symbol). Considering the possible aggregation levels {4, 8, 16}, the candidate values of the size of the CORESET can be {24, 48, 96} PRBs. For the OFDM symbol indices, both the starting OFDM symbol and the duration should be indicated. The starting OFDM symbol can be predefined hence no signaling is needed. The duration can be dependent on the bandwidth of the RMSI CORESET. For 24PRBs, only 2 or 3 OFDM symbols are supported. For 48 PRBs, only 1 and 2 OFDM symbols are supported. For 96 PRBs, only 1 OFDM symbols is supported. For the frequency pattern of the CORESET, a distributed resource mapping spanning over the initial active bandwidth part is preferable to maximize the frequency diversity gain. The possible frequency location of the CORESET(s) can be limited to a few predefined frequency patterns. The predefined frequency patterns include the comb number, comb interval etc. within size of the COERSET. Furthermore, a scalable design based on the bandwidth of the initial active bandwidth part should be supported. In summary, a joint indication can be considered. |
|  | frequency position | For the frequency position, we propose to consider either FDM or TDM where the cell-defining SS/PBCH block and initial active DL BWP are next to each other (above or below) or the center frequency of cell-defining SS/PBCH block and initial active DL BWP are approximately aligned. In this case, the frequency location can indicated together with TDM or FDM. |
|  | OFDM symbol indices of a single CORESET | Details are provided for bandwidth indication. |
|  | Others (?) | The REG bundle size, CCE-to-REG mapping and transmission type are predefined - REG bundle size: 6 - CCE-to-REG mapping: time-first or frequency-first Transmission type: interleaving |
| Intel | bandwidth (PRBs) | Support at least two BWs for CORESET, somewhere near 24 PRB, and 42 PRB when CORESET uses the same SCS as SS/PBCH. |
|  | frequency position | For the larger PRB case, only support pre-determined location with respect to SS/PBCH blocks. However, to deal to variable SS/PBCH block position within the system bandwidth, some signaling is needed to support few PRB shift between CORESET and SS/PBCH blocks. |
|  | OFDM symbol indices of a single CORESET | In general, this would always be OFDM symbols 1~3. Except for a special mode that supports LTE-NR coexistence, where the CORESET OFDM symbols would be places in the edge of the 2nd SS/PBCH block position, which cannot be used due to CRS collision. In the FDM case, the COREST symbols should be fixed (with an offset) with respect to the associated SS/PBCH block position. |
|  | Others (?) | The CORESET should support 1 ~ 3 OFDM symbol configuration at least in majority of the CORESET configuration cases. Additionally, in general the CORESET should have enough CCEs to support at least 1 AL 8 PDCCH when 2 or 3 OFDM symbol configuration is used. |
| ZTE | bandwidth (PRBs) | Two different CORESET BWs can be supported (using 1 bit): - CORESET BW ≤ Minimum carrier BW (for frequency range) (roughly equal to) - CORESET BW ≤ Minimum UE BW (for frequency range) (roughly equal to) Alt a is needed to support the minimum carrier BW. Alt b is useful for carrier BWs greater than or equal to UE minimum BW. The number of PRBs that this corresponds to depends on RMSI SCS. |
|  | frequency position | Four different frequency positions are defined and selected by 1 bit + whether RMSI SCS uses same or different SCS as SSB: - Aligned with lower frequency edge of SS/PBCH block with possible offset - Aligned with higher frequency edge of SS/PBCH block with possible offset - Below SS/PBCH block in frequency with possible offset Above SS/PBCH block in frequency with possible offset |
|  | OFDM symbol indices of a single CORESET | 1-2 symbols for TDM and up to 4 symbols for FDM |
|  | Others (?) |  |
| Qualcomm | bandwidth (PRBs) | Sub-6GHz: 24 and 48; above-6GHz: 24, 48 and 108. |
|  | # of OFDM symbols per CORESET | - 2 OFDM symbols when CORESET BW = 24 PRBs (for both below-6GHz and above-6GHz) - 1 OFDM symbols when CORESET BW = 48 PRBs (for both below-6GHz and above-6GHz) - 1 OFDM symbol when CORESET BW = 108 PRBs (for above-6GHz only) |
|  | frequency position | - Depending on SSB SCS, RMSI SCS and sync raster - The offset is in RB step size - Separately designed from PRB grid offset (e.g., 4 bits in PBCH) |
|  | Others | - SSB/RMSI CORESET multiplexing (FDM vs. TDM) |

| CMCC | bandwidth (PRBs) | The bandwidth for RMSI should consider the coverage performance and the number of candidates for RMSI PDCCH monitoring. Aggregation level 16 should be supported in RMSI CORESET considering the macro coverage performance and the demodulation performance gap between PBCH and PDCCH. At least one candidate of 16CCE, that is 96 RRB for one symbol or 48 PRB for 2symbols should be supported. In summary, the CORESET bandwidth of 24 PRB, 48PRB and 96 PRB should be supported. |
| --- | --- | --- |
|  | frequency position |  |
|  | OFDM symbol indices of a single CORESET | 1, 2, 4 symbols 1 OS for {24,48,96} PRBs 2 OS for {24,48,96} PRBs 4 OS for {24, 48} PRBs For TDM case, the stating symbol could be any DL symbol in the slot that can be configured for PDCCH transmission For FDM case, the starting symbol is the first symbol of the SSB |
|  | Others (?) | Support AL=16 to ensure the coverage of NR PDCCH |
| OPPO | bandwidth (PRBs) | We share the same view with CATT. |
|  | frequency position | Due to the limitation of the number of the PBCH CORESET indication bits, only few candidate frequency positions can be supported. But for well support of many SS/PBCH blocks to one RMSI, the indication granularity of the frequency offset of RMSI CORESET relative to the SS/PBCH frequency location shall be large enough, e.g., with 20MHz granularity. |
|  | OFDM symbol indices of a single CORESET | The starting OFDM symbol index should be determined dependent upon whether TDM vs. FDM is used and the QCL relationship with the SS/PBCH blcoks. |

In the following, interested companies may provide more details on the suggestion of the RMSI CORESET configurations with the details if available. Each company may use a separate table.

Table 2.4-3: Configuration of the RMSI CORESET(s) Properties

| CATT | Details for the RMSI CORESET Configuration |
| --- | --- |
| RMSI CORESET configurations need considering the FDM/TDM of SSBs and RMSI CORESETs with the same or different numerologies. Details is presented in the tdoc R1-1720170. |  |
| Samsung | Details for the RMSI CORESET Configuration |
| Proposal: RMSI CORESET parameters are configured according to: - Frequency domain parameters, the number of CORESET OFDM symbols and a set of aggregation levels are indicated via the common MIB contents - A set of aggregation levels: - 4, 8, [16] CCE aggregation levels should be supported (for ensuring the PDCCH coverage) - 48 and 96 total number of REGs should be supported (depending on FDM vs. TDM and available operator BW), where the number is determined implicitly by the indication of CORESET BW and number of OFDM symbols - The following parameters are CORESET specific, and determined by the SSB index conveyed in the SSB and the common PBCH contents - Starting position of the CORESET OFDM symbols - RMSI PDCCH monitoring windows. - OSI (SIBx, x>1) PDCCH monitoring windows are determined CORESET specifically, by the SSB index conveyed in the SSB, the common PBCH contents and the RMSI contents. - The PDCCH DMRS of the CORESET i (for which some of those parameters are determined according to SSB index i) can be assumed to be QCL’ed with SSB i Proposal: The RMSI CORESET indication should include the following set of information: - Information 1: RMSI CORESET BW & number of OFDM symbols – 2 bits - Information 2: Frequency offset of CORESET –8 bits (including floating sync), or 4 bits (excluding floating sync) - Information 3: RMSI CORESET slot offset oRMSI – 1 bit - Information 4: Number of CORESETs per slot, starting OFDM symbol number, RMSI CORESET window periodicity – 1 bit For the details, please refer to our tdoc R1-1720274. |  |
| Huawei | Details for the RMSI CORESET Configuration |
| As suggested at the beginning of this section, some of the CORESET properties can be jointly configured. Details can be found in our tdoc R1-1719372. |  |

| CMCC | Details for the RMSI CORESET Configuration |
| --- | --- |
| Aggregation level 16 should be supported in RMSI CORESET considering the coverage gap between SSB and PDCCH. Different aggregation level sets can be configured in RMSI CORESET according to the RMSI CORESET size. For the detail, please refer to our tdoc R1-1720591 [26]. |  |

Summary:

RMSI CORESET working assumption

- It was proposed to confirm the following WA by a number of companies:

  - (Working assumption) PBCH contents, except the SSB index, should be the same for all SS/PBCH blocks within an SSB burst set for the same center frequency

To limit the candidates of the RMSI CORESET configurations, we may need to decide whether RMSI CORESETs associated the SS/PBCH blocks within an SSB burst set have the same  CORESET properties, such as the bandwidth, the frequency position and the same number of consecutive OFDM symbols regardless whether RMSI CORSETs are multiplexed with SS/PBCH blocks in FDM or TDM.

RMSI CORESET configuration approach

- Option 1: Using a table with a list of up to 2^x (x is up to 8) configuration indices. Each index represents a set of pre-defined configuration for the  CORESET properties (e.g., bandwidth (PRBs), frequency position, CORESET duration/OFDM symbols)

  - Supported by: AT&T, Qualcomm, vivo, [Spreadtrum], CATT, Ericsson, Nokia

- Option 2: Discuss each CORESET property separately (e.g., frequency location is defined with a different parameter from the one used for time location, etc.)

  - Supported by: LGE, Samsung, Xiaomi

- Option 3: Some of the properties are configured together using a table, while others are configured separately.

  - Supported by: Spreadtrum, [CATT], Intel, ZTE

Possible RMSI CORESET duration

- 1 symbol: most companies

- 2 symbols: most companies

- 3 symbols: LGE, NTT DOCOMO, Intel

- 4 symbols: vivo, CATT, CMCC

RMSI Bandwidth

- Depending on SCS and minimum carrier bandwidth: LGE

- Either minimum carrier bandwidth or UE minimum bandwidth: ZTE (1 bit)

- Two types of RMSI CORESET bandwidth supported: Intel

RMSI {Bandwidth(RBs), duration(symbols)}

- {{24, 2},{48,1}: Nokia, Samsung, Qualcomm

- {{48,72,96}/a, a={1,2,3,4}} : CATT

- {{24, 48, 96},1}, {{24, 48}, 2}, {24,3}}: NTT DOCOMO

- {108,1}: Qualcomm (only for above-6GHz)

- {48, 96}/a，a={1,2,4}: vivo

- {18, 3},{24,2 or 3}, {48, 1, 2 or 3} and {96, 2 or 3}: Ericsson

- {{24,2}, {24,3}, {42,1}, {42,2}, {42,3}} for the case when same SCS is used between SS/PBCH & RMSI, for other cases scales the number of PRBs based on SCS difference: Intel

RMSI CORESET Starting OFDM symbols for the CORESET

  - Alt1.

    - TDM: the first OFDM symbol in the slot containing corresponding SSB

    - FDM: the first OFDM symbol of the SSB

      - supported by: Huawei, NTT DOCOMO

  - Alt.2

    - Only in downlink control region, i.e., symbols {0, 1, 2}, of downlink slots for Sub-6.

      - supported by: Qualcomm

  - Alt.3

    - Depend on the SSB index and RMSI CORESET configuration indexes

      - supported by: Samsung, CATT, vivo, ZTE

  - Alt.4

    - TDM

      - For 15kHz and 30kHz; first symbol of the slot(s)

      - For 60khz and 120kHz; Symbol #0 and #6

    - FDM; first symbol of the corresponding SSB

  - Alt 5

    - TDM: the first 1 ~ 3 OFDM symbol(s) in the other half radio frame slots that contain SSB(s)

    - FDM: in OFDM symbols with fixed time domain offset relations with SSB

      - Supported by: Intel

  - Alt 6

    - TDM: the first 1 ~ 3 OFDM symbol(s) of the  correponding slot for slot based RMSI scheduling and the first OFDM symbol of the correpoding RMSI candidate time locations which reuse the nominal SSB time locations for 2/4 symbols non-slot transmission

    - FDM: in OFDM symbols with fixed time domain offset relations with SSB

      - Supported by: OPPO

RMSI frequency location

  - Offset relative to SSB:

    - PRB offset depends on the RMSI CORESET configuration indexes for both FDM and TDM  (SCS offset due to floating synch is indicated by 4bits in PBCH)

      - supported by: CATT

    - Defined by the subcarrier spacing of SS/PBCH block

      - Supported by: LGE

    - the lowest PRB of RMSI to the lowest PRB of SSB after the PRB grid offset is already applied:

      - Supported by: Qualcomm

    - 1 bit is used to indicate the frequency position, also based on if RMSI SCS is same or different than SSB SCS.

      - ZTE

    - PRB offset for the CORESET relative to the SSB center/edge, where the offset reference numerology is the min(SSB_scs,RMSI_scs)

      - Nokia

    - PRB offset for the CORESET relative to the SSB center/edge, where the offset reference numerology is the SSB

      - Intel

  - Offset granularity

    - Multiple PRBs

      - Qualcomm, CATT,OPPO (large enough granularity, e.g.,100 PRBs)

    - Unit of 1 PRB for 24 PRB CORESET

      - Intel

    - Unit of 2 PRB for 42 PRB CORESET

      - Intel

RMSI scheduling

- Both slot and non-slot based: Ericsson, Nokia, CATT, vivo,Xiaomi, ZTE

RMSI PDCCH Properties

- REG bundle size of 6 PRBs:

  - Supported by: Huawei, Nokia, NTT DOCOMO, Samsung, CATT, vivo, Intel, Qualcomm

- Interleaved Transmission:

  - Supported by:  Huawei, Nokia, Samsung, CATT, vivo, Qualcomm

- Aggregation Level:

  - 4 CCEs: CMCC, Samsung, CATT, vivo, Intel, Qualcomm,OPPO

  - 8 CCEs: CMCC, Samsung, CATT, vivo, Intel, Qualcomm,OPPO

  - 16 CCEs: NTT DOCOMO, Samsung, CATT, vivo, CMCC

- RS type:

  - WB-RS only: Samsung, Qualcomm

Suggested Offline Proposals:

- Confirm the following working assumptions

  - (Working assumption) PBCH contents, except the SSB index, should be the same for all SS/PBCH blocks within an SSB burst set for the same centre frequency

- All of the RMSI CORESETs associated the SS/PBCH blocks within an SSB burst set for the same centre frequency should have the same settings for the following properties regardless whether RMSI CORSETs are multiplexed with SS/PBCH blocks in FDM or TDM, namely,

  - the same  bandwidth

  - the same frequency position

  - the same number of consecutive OFDM symbols

- RMSI CORESET configuration is are defined by a table with a list of up to 2^x configuration indices. Each index represents a set of pre-defined configuration for the dependent properties (e.g., bandwidth (PRBs), frequency position, CORESET duration/OFDM symbols)

  - Note: If any of the RMSI CORESET properties is identified totally independent on others, it can be defined separately from the table.

- RMSI CORESET configuration include at least the following RMSI CORESET durations

  - Alt 1) 1 and 2 symbols, and FFS [3], [4] symbol

  - Alt 2) 1, 2, a dn 3 symbols

- RMSI CORESET configuration supports at least the following configuration of the RMSI bandwidths

  - Limited within the minimum carrier bandwidth of 5MHz for sub-6 GHz and 50 MHz for above-6GHz

  - With the granularity of 6 REGs

- RMSI CORESET configuration supports at least the following combination of {Bandwidth(RBs), duration(symbols)}

  - Further discuss among { {48, 1}, {96, 1}, {24, 2}, {48, 2}, {42,1}, {42,2}, {24,3}, }

  - Further discuss whether and how to scale {RB,symbol} when different numerology between CORESET and RMSI is applied.

  - FFS: other combinations within {{48,,96}/a, a={3,4}}

- The starting OFDM symbols for RMSI CORESET are defined in the RMSI CORESET configuration, which can, at least, be:

  - TDM:

    - the first OFDM symbol in the slot containing corresponding SSB

  - FDM:

    - the first OFDM symbol of the SSB

- RMSI frequency location

  - FFS

- RMSI CORESET configuration supports both slot-based and non-slot based scheduling

- RMSI PDCCH Properties

  - REG bundle size of 6 PRBs:

  - Interleaved Transmission:

  - Aggregation Level: 4 CCEs, 8 CCEs

    - FFS: 16 CCEs

  - RS type: WB-RS only: Samsung

## RMSI PDCCH monitoring window configuration

The agreements were reached for RMSI PDCCH monitoring window [90b-NR-04]:

Agreements (RAN1#90bis) - There is an RMSI PDCCH monitoring window associated with an SS/PBCH block, which recurs periodically. - Each window has duration of x consecutive slot(s). - FFS: x is e.g., 1/2/4 - FFS: whether x is frequency band dependent - FFS: whether x is configured in PBCH. - The period, y, of the monitoring window can be the same as or different from the period of the SS/PBCH block burst set. - FFS: y is e.g., 10/20/40/80/160 ms - FFS: whether y is frequency band dependent - FFS: whether y is configured in PBCH - FFS: whether y is dependent on RMSI TTI - FFS: whether there is a dependency between the period of the monitoring window and the period of the SS/PBCH block burst set. - FFS: whether it is allowed to configure the overlapping monitoring windows associated with different SS/PBCH blocks - FFS: Monitoring window time offset - From RAN1’s perspective, the considered values of the RMSI TTI for down-selection are 80ms and 160ms. Agreements: ([90b-NR-04] CORSET and RMSI Configuration)  RMSI timing configuration should consider at least the following properties:  RMSI PDCCH monitoring window periodicity y  RMSI PDCCH monitoring window duration x  FFS: RMSI PDCCH monitoring window offset  FFS: The number of RMSI PDCCH monitoring occasions per SSB within the RMSI PDCCH monitoring window periodicity  FFS: signaling details including what is captured in specifications and what is signaled in the MIB

Figure 2-3 Illustration of RMSI for PDCCH monitoring window (TDM)

In the following tables, interested companies are encouraged to provide their views on the RMSI with the PDCCH monitoring window. The discussion may need to consider both TDM/FDM of SSB and RMSI CORESET cases.

Table 2.5-1: Comments on the RMSI Timing Window Duration

| Company Name | Window duration, x (slots) | Is x Frequency band dependent? | Is x Configured in PBCH? | Additional Comments |
| --- | --- | --- | --- | --- |
| CATT | No need to configure. | No | No | With the WA that RMSI CORESET configuration is the same for all SSBs in the same burst set, a UE will have the information on all RMSI CORESET locations associated with any of the SSBs in the same burst set. It can be up to UE’s implementation whether to measure the RMSI CORESETs associated with other SSBs. Thus, there is no need to configure the RMSI timing window duration. |
| vivo | 1/5/10 | No | Yes | It’s not clear that what the “consecutive slot(s)” refers to. In our opinion, it should refer to consecutive slots with CORESET SCS. A configurable monitoring window can be beneficial for giving some scheduling flexibility in some cases where multiple CORESET instances exist within the monitoring window and NW is free to transmit the RMSI PDCCH in any of the instances. We think slot level window duration should be supported at least. It can be one slot or more than one slots depending on the NW configuration: - When the frequency resource is sufficient to support FDM operation between CORESET and SSB, 1 slot window duration can be configured for shortening access delay. - When signaling overhead is a concern, a sparser RMSI CORESET/PDSCH resource allocation may be deployed, then the window duration can be longer (e.g. 10 slots). - If LTE-NR share the same carrier, it is normal to align their UL-DL configs to avoid additional inter-system interference. According to the already defined UL-DL configs in LTE, a 5-slot monitoring window can be acceptable for UE to capture CORESET in DL subframes. |
| Spreadtrum | No need to configure. | No | No | No need to make a long window where a UE must perform several blind detections. As CATT said, it can be up to UE’s implementation whether to measure the RMSI CORESETs associated with other SSBs. |
| Samsung | No | No | No | It would be preferred to limit the window duration to be 1 slot (same as LTE) to limit the UE complexity. Also, a single CORESET QCL’ed with the detected SSB should be mapped in the window to minimize UE power consumption. It would be preferred to maximize the commonality between RMSI timing configuration and OSI timing configuration |
| Ericsson | 2 and 5 ms | No | Yes | To avoid large overhead from signaling time position of CORESET, a larger window is used. |
| Nokia | 2 CORESET occasions | Yes | No |  |
| Huawei | Yes | Yes | Yes | To allow some flexibility for RMSI delivery at the gNB, it was proposed that the RMSI PDCCH monitoring occasions within a RMSI PDCCH monitoring window can be larger than one. Considering the signaling overhead and UE power consumption, the duration of the monitoring window x can be configured with only 1 bit in PBCH. The candidate values for 1 bit x can be dependent on carrier frequency. For example, x can be configured to be 1 or 2 slots for sub 6GHz and 2 or 4 slots for above 6GHz. It should be noted that in case of FDM between RMSI and SS/PBCH block, although the duration of RMSI PDCCH monitoring window is one slot, from network perspective the number of PDCCH monitoring occasions can be same as the number of SS/PBCH blocks within a slot, which is in line with the motivation to support FDM. From UE point of view, one PDCCH monitoring occasion can be selected without specification impact. The duration of RMSI PDCCH monitoring window x is indicated with 1 bit. The candidate values of x is 1 or 2 for sub 6GHz and 2 or 4 for above 6GHz. |
| Intel | 4 ms for FR1, 2.5ms for FR2 | Yes | No | Flexibility in placement of RMSI is need to support various SS/PBCH block periodicity. For example, the specification should somehow support the case when all SS/PBCH is being utilized by the network and also the same when 5msec SS/PBCH block perioidicity is being utilized. Having a long monitoring window allows the network to deal with all the cases mentioned above. For further details, please refer to our contribution in R1-1720059 |
| ZTE | 1, 2, 4 CORESET occasions/slots | No | Yes | The monitoring window duration can be configured together with whether CORESET is in slots with SSB or not, with 2 bits. If CORESET is in SSB slot, then a single slot monitoring window is sufficient, i.e. the slot with associated SSB. If CORESET is in non-SSB slots, then the monitoring window can be configured to 1, 2 or 4 slots. - '00': RMSI CORESET in slot with SS/PBCH block, monitoring window duration= 1 slot - '01': RMSI CORESET in slot without SS/PBCH block, monitoring window duration = 1 slot - '10': RMSI CORESET in slot without SS/PBCH block, monitoring window duration = 2 slot '11': RMSI CORESET in slot without SS/PBCH block, monitoring window duration = 4 slot |
| Qualcomm | No | No | No | RMSI CORESET timing is relative to the associated SSB timing. The SSB/RMSI CORESET timing offset can be fixed in specification. The window duration is fixed to be 1 slot. |
| CMCC |  |  |  | If RMSI CORESET can also be used for OSI or other common PDCCH transmission, there can be multiple RMSI CORESETs QCL’ed with each SSB. For example, in DL slots following the SSB slots during each SS burst set period, the RMSI CORESETs QCL’ed with different SSBs are also present to transmit other common PDCCHs. |
| OPPO | 1 | No | No | We don’t the see the clear benefits of support one RMSI PDCCH monitoring window while it bring additional complexity for the UE to blindly detect RMSI PDCCH. Further, it may be harmful for the UE to perform PDCCH combination within one RMSI TTI. Therefore we propose x is fiexd to 1. |

Table 2.5-2: Comments on the RMSI Timing Window Period(s)

| Company Name | Properties | Comments |
| --- | --- | --- |
| CATT | Window period(s), y (ms) | If only one-to-one association between the SSB and RMSI CORESET is supported, the monitoring window period should be the same as the SSB burst set period. If one-to-many association between the SSB and RMSI CORESET instances is supported, monitoring window period can be configured to be the transmission period of the candidate RMSI CORESET instances, which should be configured to be the same or {½, ¼, 1/8} of the SSB burst set period. |
|  | Frequency band dependent? | It should be simpler to configure the window period depending on SSB burst period instead of frequency band. |
|  | Configured in PBCH? | Yes, if one-to-many association between the SSB and RMSI CORESET instances is supported. |
|  | Dependent on RMSI TTI? | No. |
|  | Is y dependent on SSB burst periodicity? | Yes, if one-to-many association between the SSB and RMSI CORESET instances is supported. |
|  | Window period(s), y (ms) | 10/20/80ms |
|  | Band dependent? | no |
|  | Configured in PBCH? | Yes RMSI PDCCH window periodicity and window duration should be jointly encoded and some unreasonable combinations (e.g. 80ms periodicity + 1ms window duration) could be excluded. Window periodicity (ms); Window duration (slot with CORESET SCS). 10; 1. 10; 5. 20; 1. 20; 5. 80; 5. 80; 10. |
|  | Dependent on RMSI TTI? | Yes. The largest monitoring cycle length can be the set as RMSI TTI while other candidate values may depend on usage scenarios. |
|  | Is y dependent on SSB burst periodicity? | Yes. There may be some association between y and SSB period, one candidate value of y is the same as the default SSB period (20ms). However, RMSI PDCCH monitoring periodicity is not always expected to be configured the same with SSB periodicity. For instance, it is beneficial for RRM measurement purpose by using an SS burst set periodicity of 5 ms, but RMSI CORESET is not mandated to be transmitted so frequently, so that RMSI PDCCH monitoring window periodicity could be longer. |

| Samsung | Window period(s), y (ms) | Assuming that the RMSI TTI is fixed in the spec (e.g., 80 or 160 msec), the window periodicity should be chosen to be able to provide RMSI PDSCH coverage in the worst case, i.e., the same worst coverage as PBCH. Given that the number of RMSI PDSCH symbols can vary from 2 to 12-14 OFDM symbols, it seems necessary to support window periodicity indication, i.e., the periodicity should be shorter if min-slots are used; and the periodicity can be longer if full slots are used. 1 bit periodicity indication may be sufficient both for TDM and FDM cases. |
| --- | --- | --- |
|  | Frequency band dependent? | The periodicity should be mainly determined dependent upon desired RMSI coverage. Not sure if it is necessary to design the indication table differently for different frequency band |
|  | Configured in PBCH? | Yes, to support desired RMSI coverage. |
|  | Dependent on RMSI TTI? | RMSI TTI should be fixed in the spec. |
|  | Is y dependent on SSB burst periodicity? | The initial cell-selection UEs only knows 20msec periodicity; and hence the spec design should be able to support this case. Additional optimization for different periodicity values for handover case can be less prioritized. |
| Ericsson | Window period(s), y (ms) | 20 ms. For FDM, same periodicity as for SS/PBCH block can be considered- |
|  | Frequency band dependent? | No |
|  | Configured in PBCH? | No |
|  | Dependent on RMSI TTI? | No |
|  | Is y dependent on SSB burst periodicity? | No, since the UE does not know it yet. |
| Nokia | Window period(s), y (ms) | {40ms, 80ms, 160ms} |
|  | Frequency band dependent? | No |
|  | Configured in PBCH? | Yes |
|  | Dependent on RMSI TTI? | No |
|  | Is y dependent on SSB burst periodicity? | No |
| Huawei | Window period(s), y (ms) | For power saving, the SS/PBCH block burst set periodicity can be configured to be larger than 20ms, e.g. 40/80/160ms. The same motivation could be applied to RMSI transmission. Furthermore, the access delay can be reduced if the RMSI PDCCH monitoring window periodicity is smaller than 20ms. Therefore, there is some benefit if the RMSI periodicity can be the same as SS/PBCH burst set periodicity. On the other hand, SS/PBCH block can be used for measurement with small periodicity, where RMSI may not need a small periodicity due to overhead. Moreover, if the SS/PBCH block burst set periodicity is large, the RMSI with large periodicity and small number of repetitions may not provide sufficient coverage. Therefore, it is also necessary to support an RMSI periodicity different from the SS/PBCH block burst set periodicity. The periodicity of the RMSI PDCCH monitoring window can be configured to either 20ms or the same as the SS/PBCH burst set periodicity. |
|  | Frequency band dependent? | We don’t see the need to couple the RMSI PDCCH window periodicity with frequency since it essentially only impact the coverage and signaling overhead. |
|  | Configured in PBCH? | Yes. The periodicity of the RMSI PDCCH monitoring window can be configured to either 20ms or the same as the SS/PBCH burst set periodicity. |
|  | Dependent on RMSI TTI? | No |
|  | Is y dependent on SSB burst periodicity? | As suggested above, y can either be a fixed value or same as the SS/PBCH burst set periodicity as signalled in PBCH. |
| Intel | Window period(s), y (ms) | 40 ms This assumes many-to-many is possible between SS/PBCH and CORESET resources. |
|  | Frequency band dependent? | No |
|  | Configured in PBCH? | No |
|  | Dependent on RMSI TTI? | No |
|  | Is y dependent on SSB burst periodicity? | No |
| ZTE | Window period(s), y (ms) | - A quarter of RMSI TTI |
|  | Frequency band dependent? | No |
|  | Configured in PBCH? | No |
|  | Dependent on RMSI TTI? | Yes, but RMSI TTI can be fixed in spec. |
|  | Is y dependent on SSB burst periodicity? | No |
| Qualcomm | Window period(s), y (ms) | The RMSI PDCCH monitoring window periodicity is not signaled in PBCH. The same design principles as SSB periodicity is applied. In particular, a default periodicity is defined in the specification. - The default periodicity is [20]ms for below-6GHz and [20]ms for above-6GHz. - UE assumes a default periodicity when UE performs initial access - NW can configure a periodicity when UE is in Idle and Connected state if multiple periodicity values are supported in NR. UE assumes a default periodicity if no periodicity configuration is provided to UE. |
|  | Frequency band dependent? | [Yes] |
|  | Configured in PBCH? | No |
|  | Dependent on RMSI TTI? | No |
|  | Is y dependent on SSB burst periodicity? | No |
| OPPO | Window period(s), y (ms) | We think the window period may depend on the actural TBS of the RMSI since there would be optinal IEs within RMSI. For large TBS, small y shall be applied. Otherwise, for small TBS, large y shall be applied. |
|  | Frequency band dependent? | NO |
|  | Configured in PBCH? | May be. |
|  | Dependent on RMSI TTI? |  |
|  | Is y dependent on SSB burst periodicity? | NO, We do not think the y should relate to SSB burst periodicity |

Table 2.5-3: Comments on the additional RMSI Timing Window properties

| Company Name | Can monitoring windows be overlapped with associated with different SS/PBCH blocks? | Is monitoring window time offset needed? | What is the number of RMSI PDCCH monitoring occasions per SSB within the RMSI PDCCH monitoring window periodicity? |
| --- | --- | --- | --- |
| CATT | No. | No. | 1 |
| vivo | Yes. CORESET in the overlapped region could be shared by multiple SS blocks (e.g. SSB repetition cases). UE will try to decode PDCCH with the QCL assumption of between the PDCCH DMRS and its detected SS block. | Yes. We would like to define window offset as the time gap between the start of the window and the start of the slot containing the detected SS block. It consists of a single slot or a set of consecutive slots with CORESET SCS. With multiple configurable monitoring window durations, additional monitoring window offset information would be useful to provide some scheduling flexibility to the network. The following advantages would be brought in with window offset: - Window offsets should be introduced to shift the monitoring window to next MBSFN subframes if NR reuse MBSFN subframes in LTE-NR DL shared scenarios. - It allows urgent DL transmissions and enables the network to configure uplink transmission occasions in the 5ms time window of SS burst set. - It helps UE to avoid some unnecessary RMSI PDCCH monitoring, especially when sparser RMSI CORESET/PDSCH resource allocation may be deployed. The considered values of window offset are 0/1/2/3/4 slot with CORESET SCS. The following 4-bit lookup table should be considered for RMSI timing configuration. Bits in NR-PBCH; Window periodicity (ms); Window duration (slot with CORESET SCS); Window offset (slot with CORESET SCS). 0000-0100; 10; 1; 0/1/2/3/4. 0101; 10; 5; 0. 0110-1010; 20; 1; 0/1/2/3/4. 1011; 20; 5; 0. 1100-1110; 80; 5; 0/1/2. 1111; 80; 10; 0. | It may depend on the RMSI PDCCH monitoring duration and assoications between SSB/CORESET. |
| Spreadtrum | Yes | Yes | 1 |
| Samsung | One to one mapping between SSBs and QCLed CORESETs can be a baseline. Can discuss necessity of many to one mapping. | CORESET monitoring timing offset needs to be specified at least for the TDM case (see the below picture), e.g., based on the SSB index and mini vs. full slot PDSCH transmissions. For the FDM case, the timing offset can be hard coded in the spec. The timing offset reference can be the frame timing and/or detected SSB slot timing in RMSI numerology. | 1, to minimize UE power consumption |
| Ericsson | Yes | Yes | 1 per slot for slot based PDCCH Up to 4 per slot for non-slot based PDCCH, but UE knows where based on SS/PBCH block |
| Nokia | Yes | Yes | Two |
| Huawei | No | Yes. In principle, it may not be proper to fix the RMSI PDCCH monitoring windows locations since this poses some restrictions on network operations. Therefore, the RMSI PDCCH monitoring window time offset needs to be indicated to the UE and the following two options can be considered: - Option 1: An absolute time offset O to a fixed reference time locations - Option 2: A relative time offset O to a reference SS/PBCH block In both options, the RMSI PDCCH monitoring window time offset is indicated to the UE with a granularity of half radio frame. | It depends on the RMSI PDCCH window duration x. For sub 6GHz, it can be 1 or 2; for above 6GHz it can be 2 or 4 for above 6GHz. |
| Intel | YES | No UE simply tries to find and perform blind decode of the RMSI PDCCH within the configured CORESET per slot. | 1 per slot for all slots configured for CORESET |
| ZTE | Yes | Yes. PBCH indicates non-overlapping, partially overlapping or fully overlapping monitoring windows, and it depends on the monitoring window duration. | 1 |
| Qualcomm |  | It is not needed when SSB and associated RMSI CORESET are multiplexed within a same DL slot However, when SSB and associated RMSI CORESET are not multiplexed within a same DL slot, some timing offset between RMSI CORESET timing and SSB timing is needed. This timing might be fixed in the specification. | 1 |
| CMCC | Yes | Yes | 1 when the association between SSB and CORESET is one-to-one, and more than 1 when the association between SSB and CORESET is one-to-many. |
| OPPO | YES | YES | 1 |

Summary:

Association between the SSB and its corresponding RMSI CORESET(s) in time-domain

  - Alt.1 one-to-one (NULL) association between an SSB in a burst set and its corresponding RMSI CORESET

    - FFS: the absence of RMSI CORESETs for some SSBs

    - Supported by: Huawei, NTT DOCOMO, OPPO, Potevio, Qualcomm, Samsung, Sony, CATT, Xiaomi, InterDigital

  - Alt.2 one-to-many association between the SSB and its corresponding RMSI CORESETs

    - Supported by: Nokia, LGE, Samsung, vivo, ZTE, Lenovo, Motorola Mobility, Intel, CMCC

  - Alt.3: both Alt.1 and Alt. 2 depending on the SSB periodicity

    - Supported by: ?

RMSI Timing Window Periodicity:

  - Configurable:

    - 10/20/40/80/160 ms: AT&T

    - 20ms or SSB burst set periodicity: Huawei

    - 40/80/160ms: Nokia

    - 10/20/80ms: vivo

    - 10/20 for RMSI TTI of 80ms or 10/20/40ms for RMSI TTI of 160ms: Lenovo, Motorola Mobility

    - 40ms: Intel

  - Specified:

    - Max{20, SSB burst set periodicity}: Ericsson

    - Fixed 20ms: LGE, NTT DOCOMO (SIB1), Qualcomm (default)

    - Fixed value: ZTE, Intel

    - The same as the SSB burst set period: CATT

  - Dependency on SS/PBCH burst set Periodicity

    - No: AT&T, Intel

    - Yes: Ericsson, Huawei, OPPO, Potevio, CATT,Xiaomi

RMSI Timing Window Duration:

  - 1/2/3/4 slots: AT&T, ZTE

  - 1/2 slots: Huawei

  - 2/4 slots: LGE, Lenovo, Motorola Mobility

  - 2 consecutive CORESETs: Nokia

  - {1, 5, 10} slots: vivo

  - 1 slot only: NTT DOCOMO, OPPO, Potevio, Qualcomm, Samsung, Sony, CATT

  - 2/5 slots: Ericsson

  - 4ms for FR1, 2.5ms for FR2: Intel

RMSI Timing Window Offset:

  - A granularity of half radio frame: Huawei, Xiaomi

  - Configurable with three values: 0 ms, 2 ms (if SS/PBCH SCS == 30 or 240 kHz) or 4 ms (if SS/PBCH SCS == 15 or 120 kHz), 10 ms: Nokia

  - Define in the specification: Qualcomm, CATT

  - {0, 1 ,2 ,3 ,4} slots: vivo

  - Define in the specification for FDM, but configured for TDM: Samsung

  - 0, 2, 4, 6 and 8 slots for 2 slot window and 0 and 5 slots for 5 slot window

  - +5ms or -5ms (in the other half radio frame of SSB): Intel

Suggested Offline Proposals:

- Down-selection of the following alternatives on the association between the SSB and its corresponding RMSI CORESET(s) in time-domain:

  - Alt.1 one-to-one (NULL) association between an SSB in a burst set and its corresponding RMSI CORESET

  - Alt.2 one-to-many association between the SSB and its corresponding RMSI CORESETs

  - Alt.3: both Alt.1 and Alt. 2 depending on the SSB periodicity

- RMSI Timing Window Periodicity:

  - TBD

- RMSI Timing Window Duration:

  - TBD

- RMSI Timing Window Offset:

  - TBD

## FDM between SS/PBCH block and RMSI(CORESET/NR-PDSCH)

In RAN1#90bis, the following agreement was reached on FDM between SS/PBCH block and CORESET/NR-PDSCH:

Agreement: - NR supports FDM transmission of QCLed SS/PBCH block and RMSI (CORESET/NR-PDSCH), when - there is no latency requirement for UE to acquire RMSI if the combined bandwidth for SS/PBCH block and RMSI (CORESET/NR-PDSCH) exceeds the UE capability, and - the number of RMSI CORESETs to monitor within a slot is 1, and - the number of slots of a CORESET corresponding to an SS/PBCH block is 1 within a monitoring window for RMSI CORESET (NR-PDCCH) - FFS: whether the number of slots of a CORESET corresponding to an SS/PBCH block can be larger than 1

For the remaining FFS, i.e., “whether the number of slots of a CORESET corresponding to an SS/PBCH block can be larger than 1”, we may have the following options:

For FDM transmission of QCLed SS/PBCH block and RMSI (CORESET/NR-PDSCH), the number of slots of a CORESET corresponding to an SS/PBCH block can be:

- Option 1: only 1

- Option 2: larger than 1

  - N=?

- Note:

  - We will also need to decide the CORESET configuration to be discussed in the later in the document

Table 2.6-1: Comments for the number of slots of a CORESET when FDM SSB/CORESET

| Company Name | Comments |
| --- | --- |
| CATT | The main benefit of FDM transmission is obtained by transmitting SSB and associated CORESET in the same OFDM symbols. Thus, no special benefit to support N > 1, which requires the transmission of RMSI CORESET in the non-SSB slots. |
| vivo | We prefer option2. As discussed in 2.5, there may exist multiple instances of the PBCH-configured CORESETs within the monitoring window. These individual CORESET instances may locate in different slots and can be considered as the same CORESET. It seems there is no problem that UE can assume they are all QCL’ed with one SS block. UE with high capability is able to receive CORESET1 and SSB simultaneously, whereas minimum capability UE has to perform RF returning if the total bandwidth of FDM’ed SSB/CORESET has exceeded UE minimum bandwidth. Considering the time required by RF link adaption and MIB decoding, minimum capability UE is able to receive the CORESET instance in slot 2. Therefore, there are still benefits for fast access to support N>1. |
| Spreadtrum | We prefer to support N=1 when FDM SSB/CORESET. The reason is the same as CATT. |
| Samsung | Single CORESET mapping within a monitoring window seems to be sufficient and straightforward. |
| Ericsson | Our proposal is that pure FDM is a separate configuration where the monitoring window is the same as the SSB duration. |
| Nokia | As per last meeting agreement the number of monitoring occasions should be 1 corresponding to the SSB. |
| Huawei | Option 1 is preferred. |
| Intel | Option 1 for FDM case. |
| ZTE | Option 1 is preferred. |
| Qualcomm | The number of slots of a CORESET corresponding to an SS/PBCH block is 1 only e.g., having SSB and associated RMSI transmitted in the same slot. |
| CMCC | We support option 2. |
| OPPO | We support option 1 |

Summary:

The issue is associated with the discussion on RMSI PDCCH monitoring window configuration.

Suggested Offline Proposals:

- For FDM transmission of QCLed SS/PBCH block and RMSI (CORESET/NR-PDSCH), the number of slots of a CORESET corresponding to an SS/PBCH block is down-selected from the following options:

  - Option 1: only 1

  - Option 2: N > 1

    - FFS: N

## RMSI TTI for down-selection are 80ms and 160ms

In RAN1#90bis, we made the following agreement on the RMSI TTI

Agreement: - From RAN1’s perspective, the considered values of the RMSI TTI for down-selection are 80ms and 160ms.

So, we may either make the down-selection or let RAN2 to make the decision. In the following table, please provide your preference.

Table 2.7-1 Comments for the RMSI TTI

| Company Name | 80ms | 160ms | Let RAN2 decide | Other Comments |
| --- | --- | --- | --- | --- |
| CATT | 80 ms TTI is used for SSB periodicity < 160ms | 160ms TTI is used only if SSB periodicity = 160ms |  |  |
| vivo |  |  | yes | Leave this to RAN2, RAN1 may focus on CORESET monitoring. |
| Samsung | Less preferred | Preferred | No | Quite high time-domain density is spent for the SSB set in 80 msec PBCH TTI. Hence, we propose to use 160 msec TTI to give more room for mapping RMSI. The PHY resource related discussion should happen in RAN1 rather than RAN2 |
| Ericsson |  | Yes | Ideally yes, but that would hold up RAN1 work |  |
| Nokia |  | Yes |  |  |
| Huawei | First preference |  | No | Working assumption can be made in RAN1 and send RAN2 an LS together with other agreement to see whether there is any issue. |
| Intel |  | Yes |  | 160msec based on 40msec RMSI CORESET periodicity. Which allows 4 potential soft-combining of RMSI. |
| ZTE |  |  | Yes |  |
| Qualcomm |  |  |  | In LTE, SIB1 has periodicity of 80ms and 4 repetitions within 80ms. For NR, RAN1 shall determine RMSI TTI and RMSI periodicity. - RMSI within RMSI TTI is repeated multiple times depending on RMSI periodicity and TTI - The RMSI repetitions correspond to different redundancy versions of RMSI coding. |

Summary:

- RMSI TTI:

  - 80ms: Huawei, NTT DOCOMO (SIB1), CATT

  - 160ms: NTT DOCOMO (SIB2), Samsung, CATT, Ericsson, Nokia, Intel

  - Decided by RAN2: vivo, ZTE

Suggested Offline Proposals:

- Down-selection of RMSI TTI between 80ms and 160ms as a working assumption,

- Send an LS with the selected RMSI TTI  to RAN2 (together with other agreements) to see whether there is any issue.

## RMSI Transmission Block

During email discussion [90b-NR-04], a question was asked on whether the RMSI is transmitted as one TB or can be multiple TBs. We may need to address this issue in next RAN1#91. In the following table, please provide your inputs (Note: This issue may be associated with the final total size of RMSI, which may depend on the discussion results from other agenda, e.g., the size of RACH configuration).

Table 2.8-1 Comments on whether RMSI can be spitted into multiple TBs

| Company Name | Comments |
| --- | --- |
| CATT | This issue partly depends on the size of minimum system information provided by RAN2. |
| Samsung | Take the information in Table 15 in R1-1720274 as starting point, and collect further decisions during RAN1#91 to determine final number of bits for the RMSI; then make a decision during RAN1#91 whether to split the transmissions of RMSI into two, i.e., SIB1 and SIB2. - If a decision is made to split the RMSI into to chunks, discuss further which parameters should be included in the SIB1 and SIB2. |
| Ericsson | Leave the decision whether to split the RMSI to RAN2. RAN1 provides transmission opportunities within the TTI and RAN2 can decide which information is scheduled when. |
| Nokia | Preferably keep the RMSI in one TB. If due to payload size, division is needed we would need to determine which mechanism will be used to deliver the TB#2. |
| Huawei | Single TB is preferred to simplify the design for RMSI delivery scheme. |
| Intel | Single TB is preferred. May require further analysis once the CORESET design is finalized. |
| ZTE | Single TB is preferred, if possible. |

Summary:

Proposals on whether RMSI need to be spitted into multiple TBs:

- NR RMSI is transmitted in two TBs (i.e., NR-SIB1 and NR-SIB2).

  - Supported by: NTT DOCOMO

- Make the decision based on the final number of bits for the RMSI

  - Supported by: Samsung, CATT

- Leave the decision to RAN2

  - Supported by Ericsson

- Baseline of 1 TB, revisit once CORESET design is complete

  - Supported by Intel

Suggested Offline Proposals:

- Select one of the following options on the number of TBs for RMSI

  - Alt.1 one TB only

  - Alt.2 two TBs

  - Alt.3 leave the decision to RAN2

- Send an LS to RAN2 on RAN1’s decision

## RMSI PDSCH

Summary:

- NR-PDSCH carrying the RMSI is periodically transmitted with different redundancy versions within the TTI of the RMSI (NR-SIB1)

  - Supported by: Ericsson, Huawei, Nokia, Intel

Suggested Offline Proposals:

- NR-PDSCH carrying the RMSI is periodically transmitted with different redundancy versions within the TTI of the RMSI

  - FFS: more details

## Other Remaining Issues

Table 2.10-1: Comments other remaining RMSI issues

| Company Name | Comments |
| --- | --- |
| Spreadtrum | Considering the load balance, semi-static updating of the frequency position of RMSI CORESET, i.e. semi-static updating of the frequency position of initial active DL BWP, shall be supported in NR. |

# Conclusion

TBD.

# References

- R1-17xxxxx, “RAN1 Chairman’s Note for 3GPP TSG RAN WG1 Meeting 90bis”.

- R1-1719342	Remaining details of RMSI	ZTE, Sanechips

- R1-1719372	RMSI Delivery	Huawei, HiSilicon

- R1-1719558	Further discussion on RMSI transmission	MediaTek Inc.

- R1-1719622	Remaining details on Remaining minimum system information	AT&T

- R1-1719742	Discussion on remaining issues of RMSI delivery	Lenovo, Motorola Mobility

- R1-1719758	Discussion on Remaining Minimum System Information	vivo

- R1-1719827	Association between SS blocks and the corresponding RMSI(s) in wideband operation	Spreadtrum Communications

- R1-1719894	RMSI delivery and CORESET configuration	LG Electronics

- R1-1720059	Remaining details of RMSI	Intel Corporation

- R1-1720170	On Remaining details on RMSI	CATT

- R1-1720274	Remaining details on RMSI	Samsung

- R1-1720376	Remaining details on remaining minimum system information delivery	Potevio

- R1-1720453	Remaining details on remaining minimum system information	Sony

- R1-1720582	Discussion on FDM based RMSI CORESET Design	CMCC

- R1-1720600	Discussion on remaining details for RMSI delivery in PBCH	Xiaomi Technology

- R1-1720623	On Remaining Details of System Information Delivery 	InterDigital, Inc.

- R1-1720649	Remaining system information delivery consideration	Qualcomm Incorporated

- R1-1720791	Remaining details on Remaining minimum system information delivery	NTT DOCOMO, INC.

- R1-1720836	Discussion on the RMSI delivery	OPPO

- R1-1720863	Remaining details on remaining minimum system information	FiberHome

- R1-1721362	On Remaining System Information Delivery		Nokia, Nokia Shanghai Bell

- R1-1721364	Remaining details on Remaining minimum system information	Ericsson

- R1-1719198 	Offline summary for Al 7.1.2.2 Remaining details on Remaining Minimum System Information  CATT

- R1-1701519 	Response LS on minimum system information	 RAN2

- R1-1720591	 Discussion on aggregation level 16 for NR PDCCH 	CMCC