3GPP TSG RAN WG1 Meeting #92	R1-1801377

Athens, Greece, 26 Frebuary-2 March, 2018

Agenda Item:	6.2.4.1

Source:	Huawei, HiSilicon

Title:	Remaining issues on 1024QAM

Document for:	Discussion/Decision

- Introduction

In Email discussion after RAN1#91, the following were agreed on support of DL 1024QAM.

Agreement:

- 5 new MCS entries are introduced (4 + 1 for retransmission).

Agreement:

- The removed set of entries from the MCS table are approximately every other entry starting from MCS 5 or 6.

Agreement:

- Two new entries are added in the CQI table.

- The new two CQI entries are:

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 14 | 1024QAM | 853 | 8.3321 |
| 15 | 1024QAM | 948 | 9.2578 |

Agreement:

- The removed CQI entries are entries 5 and 7 from 256QAM table.

Agreement:

- The new TBS entries for 100 PRBs, 1 layer are {105528, 110136, 115040, 119816/125808}.

Agreement:

- For each of the maximum TBS entries above, the complete TBS table is agreed as in the attached .xls in Email Discussion (also copied in Appendix A).

In this contribution, we mainly discuss the remaining issue on 1024QAM, i.e. MCS table configuration of 1024QAM and UE categories to support DL 1024QAM.

- MCS table for DL 1024QAM

MCS table for 1024QAM

In RAN#90bis, adding M entries with approximate uniformly spaced SE for 1024QAM, removing M entries with uniformly spaced SE between remaining entries while keeping the lowest MCS were agreed for MCS table of 1024QAM.

According to endorsed agreements in email discussion, we propose to remove a set of entries approximately every other entry starting from MCS #6 to achieve more uniformly spaced SE, which generates the MCS table for 1024QAM as in Table 1. Therefore, we have the following proposal:

Proposal 1: The removed set of entries from the MCS table of 256QAM is {#6, #8, #10, #12, #14}.

Table 1. Proposed MCS Table for 1024QAM

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 2 |
| 2 | 2 | 4 |
| 3 | 2 | 6 |
| 4 | 2 | 8 |
| 5 | 4 | 10 |
| 6 | 4 | 12 |
| 7 | 4 | 14 |
| 8 | 6 | 16 |
| 9 | 6 | 18 |
| 10 | 6 | 20 |
| 11 | 6 | 21 |
| 12 | 6 | 22 |
| 13 | 6 | 23 |
| 14 | 6 | 24 |
| 15 | 8 | 25 |
| 16 | 8 | 27 |
| 17 | 8 | 28 |
| 18 | 8 | 29 |
| 19 | 8 | 30 |
| 20 | 8 | 31 |
| 21 | 8 | 32 |
| 22 | 8 | 33/33A/33B |
| 23 | 10 | 35 |
| 24 | 10 | 36 |
| 25 | 10 | 37 |
| 26 | 10 | 38/38A |
| 27 | 2 | Reserved |
| 28 | 4 |  |
| 29 | 6 |  |
| 30 | 8 |  |
| 31 | 10 |  |

- DL UE category

In order to support 1024QAM for larger peak data rate, the existing Rel-14 DL UE categories need to be extended. For extension of DL UE categories, only those that currently support 256QAM should be the bases for 1024QAM UE capability as agreed in RAN1#90bis, including DL Category 11~20.

Current DL UE Category table involves 3 main fields corresponding to TBS value, ‘Maximum number of DL-SCH transport block bits received within a TTI’, ‘Maximum number of bits of a DL-SCH transport block received within a TTI’, ‘Total number of soft channel bits’. Straightforward solution to extend the base DL Categories is to define new DL UE categories requiring UE to increase memory hardware to store the received bits accordingly. Possible extension of current DL Category 11 to support at most 3 CC are illustrated in the following table as DL Category 21. Additionally, aiming to fully explore the peak data rate of 1024QAM in LTE system, current DL UE Category 17 to support 32CC can also be considered to be extended as DL Category 22 shown in the following table.

# Table 2: Downlink physical layer parameter values set by the field ue-CategoryDL

| UE DL Category | Maximum number of DL-SCH transport block bits received within a TTI (Note 1) | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits | Maximum number of supported layers for spatial multiplexing in DL |
| --- | --- | --- | --- | --- |
| DL Category 11 | 603008 | 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM) | 7308288 | 2 or 4 |
| DL Category 17 | 25065984 | 391656 (8 layers, 256QAM) | 303562752 | 8 |
| DL Category 21 | 753760 | 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM) 236160(4 layers, 1024QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM) 119816(2 layers, 1024QAM) | 9135360 | 2 or 4 |
| DL Category 22 | 31332480 | 489570 (8 layers, 1024QAM) | 379453440 | 8 |

According to the discussion above, we have the following proposal:

Proposal 2: At least introduce new DL UE Categories 21, 22 based on DL UE categories 11 and 17 as presented in Table 2.

- Conclusion

In this contribution, we discussed the remaining issue on 1024QAM, with the following proposal:

Proposal 1: The removed set of entries from the MCS table of 256QAM is {#6, #8, #10, #12, #14}.

Proposal 2: At least introduce new DL UE Categories 21, 22 based on DL UE categories 11 and 17 as presented in Table 2.

# Table 2: Downlink physical layer parameter values set by the field ue-CategoryDL

| UE DL Category | Maximum number of DL-SCH transport block bits received within a TTI (Note 1) | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits | Maximum number of supported layers for spatial multiplexing in DL |
| --- | --- | --- | --- | --- |
| DL Category 11 | 603008 | 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM) | 7308288 | 2 or 4 |
| DL Category 17 | 25065984 | 391656 (8 layers, 256QAM) | 303562752 | 8 |
| DL Category 21 | 753760 | 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM) 236160(4 layers, 1024QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM) 119816(2 layers, 1024QAM) | 9135360 | 2 or 4 |
| DL Category 22 | 31332480 | 489570 (8 layers, 1024QAM) | 379453440 | 8 |

Reference

- “RAN1 Chairman’s Notes”, RAN1#91, Reno, November, 2017.

Appendix A

| NRB | 105528 | 110136 | 115040 | 119816 | 125808 |
| --- | --- | --- | --- | --- | --- |
| 1 | 1064 | 1096 | 1160 | 1192 | 1224 |
| 2 | 2088 | 2216 | 2280 | 2408 | 2472 |
| 3 | 3112 | 3240 | 3496 | 3624 | 3752 |
| 4 | 4264 | 4392 | 4584 | 4776 | 4968 |
| 5 | 5352 | 5544 | 5736 | 5992 | 6200 |
| 6 | 6456 | 6456 | 6968 | 7224 | 7480 |
| 7 | 7480 | 7736 | 7992 | 8504 | 8760 |
| 8 | 8504 | 8760 | 9144 | 9528 | 9912 |
| 9 | 9528 | 9912 | 10296 | 10680 | 11064 |
| 10 | 10680 | 11064 | 11448 | 11832 | 12384 |
| 11 | 11448 | 12216 | 12576 | 12960 | 13536 |
| 12 | 12576 | 12960 | 13536 | 14112 | 14688 |
| 13 | 13536 | 14112 | 14688 | 15840 | 15840 |
| 14 | 14688 | 15264 | 15840 | 16992 | 17568 |
| 15 | 15840 | 16416 | 16992 | 18336 | 18336 |
| 16 | 16992 | 17568 | 18336 | 19080 | 19848 |
| 17 | 17568 | 18336 | 19848 | 20616 | 21384 |
| 18 | 19080 | 19848 | 20616 | 21384 | 22152 |
| 19 | 19848 | 20616 | 22152 | 22920 | 23688 |
| 20 | 21384 | 22152 | 22920 | 23688 | 24496 |
| 21 | 22152 | 22920 | 24496 | 25456 | 26416 |
| 22 | 22920 | 24496 | 25456 | 26416 | 27376 |
| 23 | 24496 | 25456 | 26416 | 27376 | 28336 |
| 24 | 25456 | 26416 | 27376 | 28336 | 29296 |
| 25 | 26416 | 27376 | 28336 | 30576 | 30576 |
| 26 | 27376 | 28336 | 29296 | 31704 | 31704 |
| 27 | 28336 | 29296 | 30576 | 32856 | 32856 |
| 28 | 29296 | 30576 | 31704 | 34008 | 35160 |
| 29 | 30576 | 31704 | 32856 | 35160 | 35160 |
| 30 | 31704 | 32856 | 34008 | 36696 | 36696 |
| 31 | 32856 | 34008 | 35160 | 36696 | 37888 |
| 32 | 34008 | 35160 | 36696 | 37888 | 39232 |
| 33 | 35160 | 36696 | 37888 | 39232 | 40576 |
| 34 | 35160 | 37888 | 39232 | 40576 | 42368 |
| 35 | 36696 | 37888 | 40576 | 42368 | 43816 |
| 36 | 37888 | 39232 | 40576 | 43816 | 43816 |
| 37 | 39232 | 40576 | 42368 | 43816 | 45352 |
| 38 | 40576 | 42368 | 43816 | 45352 | 46888 |
| 39 | 40576 | 42368 | 45352 | 46888 | 48936 |
| 40 | 42368 | 43816 | 45352 | 48936 | 48936 |
| 41 | 43816 | 45352 | 46888 | 48936 | 51024 |
| 42 | 43816 | 46888 | 48936 | 51024 | 52752 |
| 43 | 45352 | 46888 | 48936 | 51024 | 52752 |
| 44 | 46888 | 48936 | 51024 | 52752 | 55056 |
| 45 | 46888 | 48936 | 51024 | 55056 | 55056 |
| 46 | 48936 | 51024 | 52752 | 55056 | 57336 |
| 47 | 48936 | 51024 | 55056 | 57336 | 57336 |
| 48 | 51024 | 52752 | 55056 | 57336 | 59256 |
| 49 | 51024 | 52752 | 57336 | 59256 | 61664 |
| 50 | 52752 | 55056 | 57336 | 59256 | 61664 |
| 51 | 52752 | 55056 | 59256 | 61664 | 63776 |
| 52 | 55056 | 57336 | 59256 | 61664 | 63776 |
| 53 | 55056 | 57336 | 61664 | 63776 | 66592 |
| 54 | 57336 | 59256 | 61664 | 63776 | 66592 |
| 55 | 57336 | 59256 | 63776 | 66592 | 68808 |
| 56 | 59256 | 61664 | 63776 | 66592 | 68808 |
| 57 | 59256 | 61664 | 66592 | 68808 | 71112 |
| 58 | 61664 | 63776 | 66592 | 68808 | 71112 |
| 59 | 61664 | 63776 | 68808 | 71112 | 73712 |
| 60 | 63776 | 66592 | 68808 | 71112 | 75376 |
| 61 | 63776 | 66592 | 71112 | 73712 | 76208 |
| 62 | 66592 | 68808 | 71112 | 73712 | 76208 |
| 63 | 66592 | 68808 | 73712 | 75376 | 78704 |
| 64 | 66592 | 71112 | 73712 | 76208 | 78704 |
| 65 | 68808 | 71112 | 75376 | 78704 | 81176 |
| 66 | 68808 | 73712 | 76208 | 78704 | 81176 |
| 67 | 71112 | 73712 | 76208 | 81176 | 81176 |
| 68 | 71112 | 75376 | 78704 | 81176 | 84760 |
| 69 | 73712 | 76208 | 78704 | 81176 | 84760 |
| 70 | 73712 | 76208 | 81176 | 84760 | 87936 |
| 71 | 75376 | 78704 | 81176 | 84760 | 87936 |
| 72 | 76208 | 78704 | 81176 | 84760 | 87936 |
| 73 | 76208 | 81176 | 84760 | 87936 | 90816 |
| 74 | 78704 | 81176 | 84760 | 87936 | 90816 |
| 75 | 78704 | 81176 | 84760 | 90816 | 93800 |
| 76 | 81176 | 84760 | 87936 | 90816 | 93800 |
| 77 | 81176 | 84760 | 87936 | 90816 | 93800 |
| 78 | 81176 | 84760 | 90816 | 93800 | 97896 |
| 79 | 84760 | 87936 | 90816 | 93800 | 97896 |
| 80 | 84760 | 87936 | 90816 | 97896 | 97896 |
| 81 | 84760 | 87936 | 93800 | 97896 | 101840 |
| 82 | 87936 | 90816 | 93800 | 97896 | 101840 |
| 83 | 87936 | 90816 | 93800 | 97896 | 101840 |
| 84 | 87936 | 93800 | 97896 | 101840 | 105528 |
| 85 | 90816 | 93800 | 97896 | 101840 | 105528 |
| 86 | 90816 | 93800 | 97896 | 101840 | 107832 |
| 87 | 90816 | 93800 | 101840 | 105528 | 107832 |
| 88 | 93800 | 97896 | 101840 | 105528 | 110136 |
| 89 | 93800 | 97896 | 101840 | 105528 | 110136 |
| 90 | 93800 | 97896 | 101840 | 107832 | 112608 |
| 91 | 93800 | 97896 | 105528 | 110136 | 112608 |
| 92 | 97896 | 101840 | 105528 | 110136 | 115040 |
| 93 | 97896 | 101840 | 107832 | 112608 | 115040 |
| 94 | 97896 | 101840 | 107832 | 112608 | 115040 |
| 95 | 101840 | 105528 | 110136 | 115040 | 117256 |
| 96 | 101840 | 105528 | 110136 | 115040 | 119816 |
| 97 | 101840 | 105528 | 112608 | 117256 | 119816 |
| 98 | 101840 | 107832 | 112608 | 117256 | 119816 |
| 99 | 105528 | 110136 | 115040 | 119816 | 124464 |
| 100 | 105528 | 110136 | 115040 | 119816 | 125808 |
| 101 | 105528 | 110136 | 117256 | 119816 | 125808 |
| 102 | 107832 | 110136 | 117256 | 124464 | 125808 |
| 103 | 107832 | 112608 | 117256 | 124464 | 128496 |
| 104 | 110136 | 115040 | 119816 | 124464 | 128496 |
| 105 | 110136 | 115040 | 119816 | 125808 | 128496 |
| 106 | 112608 | 115040 | 119816 | 125808 | 133208 |
| 107 | 112608 | 117256 | 124464 | 128496 | 133208 |
| 108 | 115040 | 119816 | 124464 | 128496 | 133208 |
| 109 | 115040 | 119816 | 125808 | 130392 | 133208 |
| 110 | 115040 | 119816 | 125808 | 130392 | 137792 |