3GPP TSG RAN WG1 Meeting #92	R1-1801385

Athens, Greece, February 26 – March 2, 2018

Source:	Huawei, HiSilicon

Title:	Summary of email discussion on the link level evaluation for LTE URLLC

Agenda Item:	6.2.7.1

Document for:	Discussion and Decision

Introduction

The objective of this email approval is to collect the views of companies and get the agreements on the remaining link level evaluation assumptions for URLLC in LTE.

In this contribution, the discussions are summarized based on companies’ inputs, which are attached in this document [1].

Agreements in RAN1#91

The following agreements on link level simulation assumptions have been reached at RAN1#91:

Agreement: Use 700MHz as baseline for the carrier frequency in link level evaluations for the macro deployment scenario Agreement: Use 2GHz as the baseline carrier frequency in link level evaluations for the indoor hotspot deployment scenario Agreement: Use TDL-C and TDL-E as the baseline channel model for link level evaluations in TR 38.901 for the macro deployment scenario Agreement: Use the following in link level simulations. Packet size; 32 bytes at Layer 2 PDU as a baseline. FFS an optional larger packet size..

Endorsed agreements in Email approval

The followings are endorsed in email discussion 91-LTE-07:

Agreement:: Adopt following for link level simulation assumptions for LTE URLLC Urban Macro–URLLC; Indoor Hotspot-eMBB. Channel model; NLOS: TDL-C in TR 38.901 LOS: TDL-E in TR 38.901; NLOS: TDL-A in TR 38.901 LOS: TDL-D in TR 38.901. Delay spread scaling parameter DSdesired; LOS: 93ns NLOS: 363ns; LOS: 20ns NLOS: 39ns. UE speed; 3km/h, 30km/h; 3km/h. Transmission mode for PDSCH; TM2 as baseline.. DL control payload in simulation for PDCCH/SPDCCH; Companies report their assumptions.. UL control payload in simulation for PUCCH/SPUCCH; A single carrier (using a single TTI length in each direction), single codeword for PDSCH is assumed as the baseline. Processing time line; Companies report their assumptions.. SINR range; A range including 5th percentile downlink/uplink SINR in system level simulation. Latency bound; 1ms, 10ms Companies report delay assumptions according to table X. Sub-carrier spacing; 15kHz. TTI length; Subslot (2 or 3 symbols per TTI), slot (7 symbols per TTI, 0.5ms), 1ms TTI (14 symbols per TTI, 1ms) Other values are not precluded (companies report if other value is used). Number of UEs; 1 UE (other UE numbers are not precluded). Channel estimation; Practical. Receiver type; MMSE. Table X Latency analysis for URLCC Step; Description; Value. 1.0; Scheduling request and scheduling for uplink transmission. 1.1; Transmitter Processing Delay (eNB for DL; UE for UL). 1.2; Frame Alignment. 1.3; Data channel transmission duration note. 1.4; Receiver Processing Delay. 1.5; HARQ Retransmission. Total one way delay [ms]. Note: This includes the potential blind/HARQ-less repetitions. Agreement:: Use the following in link level simulations BS TX antenna configuration; 2 TX ports. Agreement:: Use the following in link level simulations BS RX antenna configuration; 2/4 Rx ports. UE TX antenna configuration; FFS: - 1TX port as baseline, 2 TX ports as optional or - 2 TX ports. UE RX antenna configuration; 2RX ports as baseline, 4RX as optional for 700 Mhz. FFS for 2 GHz - 2 RX ports as baseline, 4 RX ports as optional or - 4 RX ports. Agreement:: Use the following in link level simulations Modulation and coding rate; A subset of existing LTE MCS set in Table 7.1.7.1-1 used as baseline. FFS the entries of the subset. The use of other MCSs with lower code rate is not precluded. Agreement:: Use the following in link level simulations Link adaptation for PDSCH; Disabled as baseline. Companies report if link adaptation is used.. Agreement:: The derivation of overall data reliability is down-selected between following options: Option 1: The reliability of each channel are evaluated independently by link level simulation. The overall reliability is computed analytically based on the reliability obtained in link level simulation (companies report their details in analysis). Option 2: The reliability of the (S)PPDCCH and PDSCH are evaluated jointly by link level simulation, and independently for other channels. The overall reliability is computed analytically based on the reliability obtained in link level simulation (companies report their details in analysis). Agreement:: Use the following in link level simulations System bandwidth; 20 MHz. Resource allocation; Companies report Up to 20 MHz.

Summary of discussions

## Packet sizes

On packet sizes for LLS, one open issue is whether

Agreement: Use the following in link level simulations.

| Packet size | 32 bytes at Layer 2 PDU as a baseline. FFS an optional larger packet size. |
| --- | --- |

There are two options on the optional larger packet sizes:

- Option 1: Replace the FFS part with “Other values are not precluded”.

- Option 2: Also cover additional larger packet sizes than 32 bytes in simulation assumptions.

8 companies (Huawei, HiSilicon, Ericsson, Nokia, NSB, ZTE, Sanechips, LGE) prefer option 1, 1 company (Intel) prefers option 2 to include {50, 200, 256} bytes, and 1 company (Qualcomm) prefers option 2 to include {50, 100, 200} bytes.

The reasons to support option 1 are that the current agreed requirements and ITU requirements do not require larger than 32 bytes packet sizes, interested companies can still provide results for larger packet sizes. The reasons to support option 2 are that larger payload sizes can be considered for 10ms latency and also SA1 thoughts on packet sizes.

Based on majority view, the following is proposed:

Proposal 1: Use the following in link level simulations

| Packet size | 32 bytes at Layer 2 PDU as a baseline. Other values are not precluded. |
| --- | --- |

## UE antenna configurations

In the endorsed agreements, the UE TX antenna configuration and RX antenna configuration for 2GHz are still FFS.

For UE TX antenna configuration:

- Option 1: 1TX port as baseline, 2 TX ports as optional

- Option 2: 2 TX ports

8 companies (Huawei, HiSilicon, Nokia, ZTE, Sanechips, Qualcomm, LGE) prefer option 1. 1 company (Ericsson) prefers option 2.

The reasons to support option 1 are most of UEs on the market are 1-TX-antenna UEs, it’s a more common LTE configuration. The reasons to support option 2 are that UEs with URLLC services are high-end UEs, more antennas are beneficial for URLLC performance.

Based on the majority view, the following is proposed:

Proposal 2: Use the following in link level simulations

| UE TX antenna configuration | 1TX port as baseline, 2 TX ports as optional |
| --- | --- |

For UE RX antenna configuration for 2GHz:

- Option 1: 2 RX ports as baseline, 4 RX ports as optional

- Option 2: 4 RX ports

5 companies (ZTE, Sanechips, Huawei, HiSilicon, LGE) prefer option 1, 3 companies (Ericsson, Intel, Qualcomm) prefer option 2, 2 companies (Nokia, NSB) prefer to consider both 2/4 RX ports.

The reasons to support option 1 are that 1 TX antenna is a more common configuration. The reasons to support option 2 are that the URLLC UEs are high end UEs, the NR considers 4-RX UEs for bands > 1.8GHz.

Based on the inputs, and consider to study the impacts of RX antenna configuration to URLLC services, the following is proposed for compromise:

Proposal 3: Use the following in link level simulations

| UE RX antenna configuration | 2/4 RX ports |
| --- | --- |

## MCS set

On the modulation and coding rate, the common view is that the MCS is also a part of the study, while some specific MCS can be used as a starting point for the study and comparison of results.

On the subset of the MCS considered as the starting point, 4 companies (Ericsson, ZTE, Sanechips, LGE) agree to consider MCS index 0, 3, 6 in Table 7.1.7.1-1 and Table 8.6.1-1 in 36.213, while 2 companies (Huawei, HiSilicon) think MCS index 5, 10 or 17 can also be considered to investigate URLLC performance. 2 companies (ZTE, Sanechips) also mentioned that if only a subset of existing MCS is considered, then not only MCS with lower coding rate, the higher coding rate/modulation MCS should also be considered to investigate more potential enhancements.

Based on the inputs, the following is proposed:

Proposal 4: Use the following in link level simulations

| Modulation and coding rate | IMCS={0,3,6} (see 3GPP TS 36.213, table 7.1.7.1-1 and table 8.6.1-1) The use of other MCSs is not precluded |
| --- | --- |

Conclusion

The document provides a summary of email discussion [91-LTE-07] on remaining issues of link level evaluation assumptions for LTE URLLC. Based on the summarization, the following proposals and observation are given below:

Proposal 1: Use the following in link level simulations

| Packet size | 32 bytes at Layer 2 PDU as a baseline. Other values are not precluded. |
| --- | --- |

Proposal 2: Use the following in link level simulations

| UE TX antenna configuration | 1TX port as baseline, 2 TX ports as optional |
| --- | --- |

Proposal 3: Use the following in link level simulations

| UE RX antenna configuration | 2/4 RX ports |
| --- | --- |

Proposal 4: Use the following in link level simulations

| Modulation and coding rate | IMCS={0,3,6} (see 3GPP TS 36.213, table 7.1.7.1-1 and table 8.6.1-1) The use of other MCSs is not precluded |
| --- | --- |

Reference