3GPP TSG RAN WG1 Meeting #92	R1-1801400

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	7.7

Source:	Huawei, HiSilicon

Title:	Consideration on evaluation features and parameters for IMT-2020 self evaluation

Document for:	Discussion and Decision

Introduction

In this contribution, consideration on NR evaluation features and parameters for IMT-2020 self-evaluation for eMBB technical performance requirements is provided.

General feature consideration

As discussed in [1], the general features of eMBB evaluation are shown in Table 1.

TABLE 1

General technical features for eMBB

| General features | FR1 (below 6 GHz) | FR2 (above 24 GHz) |
| --- | --- | --- |
| NR on single-band | Un-paired spectrum: TDD Paired spectrum: FDD, flexible duplex, etc. | Un-paired spectrum: TDD |
| LTE on single-band | Un-paired spectrum: TDD Paired spectrum: FDD | - |
| NR on multi-band | NR with SUL+TDD band using macro layer (at least for UL user experienced data rate) |  |
| NR/LTE inter-working on multi-band | NR/LTE dual connectivity with two frequency bands, with or without SUL |  |

To further progress the work such that one could set up the simulator for eMBB evaluation, the detailed technical features for NR are proposed in Section 3. Specifically, the technical features that could contribute to eMBB spectral efficiency and user experienced data rate are considered.

Evaluation technical features for eMBB

The following technical features and parameters can be considered as starting point for self evaluation for eMBB. Other features and parameters that are supported by Rel-15 are not precluded.

DL FDD (FR1)

The features for DL FDD evaluation are provided in Table 1.

Table 1 NR Features for DL FDD evaluation (FR1)

| DL FDD – FR1 |  |  | NR |
| --- | --- | --- | --- |
| Waveform |  |  | OFDM based |
| Multiple access |  |  | OFDMA |
| Modulation |  |  | Up to 256QAM |
| Coding on PDSCH |  |  | LDPC Max code-block size=8448bit with BP decoding |
| Numerology | Slot/non-slot |  | 14 OS slot |
|  | SCS |  | 15kHz |
| RB number |  |  | 52 |
|  |  |  | Larger RB number can be considered to reflect the benefit of reduced guard band ratio for larger BW. A scalar can be applied to spectral efficiency results, SE’=SE×(1-gb(NRB))/(1-gb(NRB0)); where gb(N) is the guard band ratio at given number of RB, N. |
| Simulation bandwidth |  |  | 10 MHz |
| Frame structure |  |  | Slot Format 0 (all downlink) for all slots |
| MIMO |  | Transmission mode | Closed loop MU-MIMO |
|  |  | Codebook | - NR Type II codebook, with Class A CSI-RS - Up to 4 beam basis, WB+SB and 8PSK (overhead impact on UL) Detailed configuration to be reported |
|  |  | Precoder bundling | 4PRB, or allocated PRB; (It impacts DMRS channel estimation, but does not limit the scheduling unit.) |
|  |  | MU dimension | Up to 24 layer (considering 24 semi-orthogonal DMRS ports) |
|  |  | SU dimension | Rank adaptation |
|  |  | Codeword (CW)-to-layer mapping | For 1~4 layers, CW1; For 5 layers or more, two CWs |
|  |  | RS consideration | DMRS, CSI-RS, TRS |
|  |  | DMRS transmission | NR DMRS pattern according to TR38.211 for MU and SU DMRS pattern 2 |
|  |  | CSI-RS transmission | Class A/Class B according to CSI measurement (with codebook selection) and interference measurement |
|  |  | CSI feedback | For slot/non-slot: PMI, CQI: every [5] slot; RI: every [5] slot, CRI: every [5] slot Subband based (same as PUSCH 3-2) |
|  |  | Interference measurement | Option 1: SU-CQI Option 2: SU-CQI and MU-CQI for intra-cell interference measurement based on NZP CSI-RS (non-overlapped); For intra-cell interference measurement, Class B CSI-RS is used |
| HARQ |  | CBG retransmission | Max CBG=1 in one TB |
|  |  | ACK/NACK delay | N+4 N+2/N+1 |
| PDCCH resource sharing |  |  | Resource sharing with PDSCH might be considered in overhead calculation |
| Overhead |  |  | SS/PBCH block (1 SS/PBCH block in every 20ms) CSI-RS, DMRS and TRS PDCCH |

DL TDD (FR1)

The features for DL TDD evaluation are provided in Table 2. It is noted that the parameters that share the same value as in Table 1 (NR FDD table) is not shown.

Table 2 NR Features for DL TDD evaluation (FR1)

| DL TDD – FR1 |  |  | NR |  |
| --- | --- | --- | --- | --- |
| Numerology | Slot/non-slot |  | 14 OS slot |  |
|  | SCS |  | 15kHz | 30kHz |
| RB number |  |  | 106 | 51 |
|  |  |  | Larger RB number can be considered to reflect the benefit of reduced guard band ratio for larger BW. A scalar can be applied to spectral efficiency results, SE’=SE×(1-gb(NRB))/(1-gb(NRB0)); where gb(N) is the guard band ratio at given number of RB, N. |  |
| Simulation bandwidth |  |  | 20 MHz |  |
| Frame structure |  |  | DSUDD ‘S’ = Slot format 31 (d..dxuu) x=gap ‘D’ = Slot format 0 (all downlink) ‘U’ = Slot format 1 (all uplink) |  |
| MIMO |  | Transmission mode | Closed loop MU-MIMO |  |
|  |  | Downlink CSI measurement | - Option A: BS transmits Class B CSI-RS precoded by N “eigen-vector”, UE measures on N CSI-RS port(s) to derive equivalent channel, and then derive rank L (L≤N) and corresponding CQI - Option B: BS transmits Class A/B CSI-RS and UE measures on CSI-RS port(s) to derive channel, and use type I/II codebook to derive precoder, and use the precoder to derive rank and CQI. |  |
|  |  | SRS transmission | - Non-precoded SRS - 1 slot is configured to up to 6 OFDM symbols for SRS transmissions. - For NTNR UE (N=1/2/4), one SRS resource for N port SRS. - For 1T4R/2T4R UE (SRS switch) - 1T4R: 1 SRS resource with 1 port SRS, totally 4 SRS resources for SRS switch; different SRS resources occupy different OFDM symbols - 2T4R: 1 SRS resource with 2 port SRS, totally 2 SRS resources for SRS switch; different SRS resources occupy different OFDM symbols - For 1T2R UE (SRS switch) - 1 SRS resource with 1 port SRS, totally 2 SRS resources for SRS switch; different SRS resources occupy different OFDM symbols |  |

DL flexible duplex (FR1)

In this scheme, NR configures a DL dominated slot in every radio frame on DL part of the paired spectrum. Several OFDM symbols in the DL dominant slot are used to transmit UL SRS to enable channel reciprocity on DL carrier (see Figure 1). Multiple BSs share the same configuration on DL dominant slot such that there is no cross-link interference.

Figure 1 Illustration of frame structure configuration on DL part for flexible duplex

The features for DL flexible duplex evaluation are provided in Table 3. It is noted that the parameters that share the same value as in Table 1 (NR FDD table) is not shown.

Table 3 NR Features for DL flexible duplex evaluation (FR1)

| DL Flexible duplex – FR1 |  | NR |
| --- | --- | --- |
| Frame structure on DL part of the spectrum |  | [DDSDD] S = d…dgu…u where ‘g’=gap, e.g., Slot format 45 (d..dxxu…u with the first ‘x’=d, and the second ‘x’=g) D = Slot format 0 (all downlink) |
| MIMO | Transmission mode | Closed loop MU-MIMO |
|  | Downlink CSI measurement | Taken from NR TDD (Table 2) |
|  | SRS transmission | Taken from NR TDD (Table 2) |
| Overhead |  | SS/PBCH block, CSI-RS, DMRS and TRS, and PDCCH: taken from NR FDD Additional overhead: OFDM symbols for UL SRS and DL/UL switch (gap symbol). |

UL FDD (FR1)

The features for UL FDD evaluation are provided in Table 4.

Table 4 NR Features for UL FDD evaluation (FR1)

| UL FDD – FR1 |  |  | NR |
| --- | --- | --- | --- |
| Waveform |  |  | OFDM based |
| Multiple access |  |  | OFDMA |
| Modulation |  |  | Up to 256QAM |
| Coding on PDSCH |  |  | LDPC Max code-block size=8448bit |
| Numerology |  | Slot/non-slot | 14 OS slot |
|  |  | SCS | 15kHz |
| RB number |  |  | 52 |
|  |  |  | Larger RB number can be considered to reflect the benefit of reduced guard band ratio for larger BW. A scalar can be applied to spectral efficiency results, SE’=SE×(1-gb(NRB))/(1-gb(NRB0)); where gb(N) is the guard band ratio at given number of RB, N. |
| Simulation bandwidth |  |  | 10 MHz |
| Frame structure |  |  | Slot Format 1 (all uplink) for all slots / non-slots |
| MIMO | Transmission mode |  | UL SU-MIMO / MU-MIMO |
|  | UL Codebook |  | - NR 2Tx/4Tx codebook |
|  | MU dimension |  | Up to [4] layers |
|  | SU dimension |  | Rank adaptation |
|  | Codeword (CW)-to-layer mapping |  | For 1~4 layers, CW1 |
|  | RS consideration |  | DMRS, SRS |
|  | DMRS transmission |  | DMRS pattern according to OFDM |
|  | SRS transmission |  | Non-precoded 1 slot is configured to up to 6 OFDM symbols for SRS transmissions. |
| HARQ | CBG retransmission |  | Max CBG=1 in one TB |
|  | ACK/NACK delay |  | N+4 N+2/N+1 |
| Overhead |  |  | DMRS and SRS PUCCH DL feedback |

UL TDD (FR1)

The features for UL TDD evaluation are provided in Table 5. It is noted that the parameters that share the same value as in Table 4 (UL FDD table) is not shown.

Table 5 NR Features for UL TDD evaluation (FR1)

| UL TDD – FR1 |  | NR |  |
| --- | --- | --- | --- |
| Numerology | Slot/non-slot | 14 OS slot |  |
|  | SCS | 15kHz | 30kHz |
| RB number |  | 106 | 51 |
|  |  | Larger RB number can be considered to reflect the benefit of reduced guard band ratio for larger BW. A scalar can be applied to spectral efficiency results, SE’=SE×(1-gb(NRB))/(1-gb(NRB0)); where gb(N) is the guard band ratio at given number of RB, N. |  |
| Simulation bandwidth |  | 20 MHz |  |
| Frame structure |  | DSUDD ‘S’ = Slot format 31 (d..dxuu) x=gap ‘D’ = Slot format 0 (all downlink) ‘U’ = Slot format 1 (all uplink) |  |
| MIMO | UL Codebook | - Non-codebook based |  |
|  | SRS transmission | Precoded 1 slot is configured to up to 6 OFDM symbols for SRS transmissions. |  |

NR multi-band operation with SUL and TDD band

This can be used for UL user experienced data rate evaluation. The features for SUL + TDD evaluation are provided in Table 6.

Table 6 Features for UL SUL+TDD evaluation

| UL SUL+TDD | NR |
| --- | --- |
| Test environment | Dense Urban - eMBB |
| Evaluation configuration | Configuration C, Macro layer only, with 30 GHz (TDD band) and 4 GHz (SUL band) |
| Cell association and band selection | UEs select one band for UL data transmission (i.e., either 30 GHz or 4 GHz). UE selects the band and serving TRxP based on pre-defined critiera |
| Assumptions for UEs on 30 GHz | Follow the assumption for NR UL TDD for FR2 |
| Assumptions for UEs on 4 GHz | Follow NR UL FDD in Table 4. |

Conclusion

In this contribution, the evaluation features and parameters for eMBB spectral efficiency and user experienced data rate evaluation are proposed. It is proposed that,

Proposal: Use the technical features and parameters as listed through Table 1 to Table 6 in Section 3 as starting point for self evaluation of NR for eMBB technical performance requirements.

- Other features and parameters that are supported by Rel-15 are not precluded.

- References

- R1-1801789, “General consideration on IMT-2020 self evaluation”, Huawei, HiSilicon, Feb 2018.