3GPP TSG RAN WG1 Meeting #92	R1-1801457

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.1.2.3.2

Source:	Huawei, HiSilicon

Title:	Remaining issues on CSI-RS design

Document for:	Discussion and decision

Introduction

On CSI-RS sequence design, it has been agreed as WA that [1]

Working Assumption Agree to the following text for TS.38.211 with the understanding that the 1 port rate=1/2 case needs to be checked. If there is an issue with the working assumption for 1 port rate=1/2, technical details will be modified. 7.4.1.5.3 Mapping to physical resources For each CSI-RS component configured, the UE shall assume the sequence being mapped to physical resources according to under the condition that the resource elements indexed by are within the resource blocks occupied by the CSI-RS for which the UE is configured. The reference point for is subcarrier 0 in common resource block 0. The value of ρ is given by the higher-layer parameter CSI-RS-Density.

On CSI-RS periodicity, it has been agreed that [1]:

Agreement: - Introduce additional periodicities of {4,8,16,32,64} slots and the corresponding slot offsets to at least the following periodic/semi-persistent RS: - CSI-RS (includes ZP-CSI-RS and NZP-CSI-RS) - SRS - FFS on whether and how to align to additional periodicities as agreed in the frame structure agenda - “The two periodicities form X ms + Y ms total periodicity, where X, and Y are from {0.5, 0.625, 1, 1.25, 2, 2.5, 5, 10} ms” - FFS on CSI-IM periodicity of 1 slot

In this paper, we discuss the remaining issues on CSI-RS.

Sequence mapping

It was agreed as a working assumption in [1] that for all CSI-RS ports in one CSI-RS resource, same sequence is generated and mapped to the REs occupied by each CSI-RS port across the scheduled resource blocks, i.e., CSI-RS sequence is generated based on the number of REs occupied by one CSI-RS port and mapped to the REs with one-to-one mapping. We did not find any critical issue and hence propose to confirm the WA.

Proposal 1: Confirm the WA on CSI-RS sequence mapping.

Pattern indication

In the existing indication framework, the component patterns can be identified by number of ports, density, CDM type and frequency/time indications. Even with these RRC parameters, the CSI-RS component configurations in row 6/7 of Table 7.4.1.5.2-1 in 38.211 still cannot be distinguished, as these two rows have same number of ports, same density, same CDM type, and same frequency/time indication, as listed in Table 1.

Table 1  Characteristics of CSI-RS patterns in Row 6/7 of Table 7.4.1.5.2-1 in 38.211

| Indication | Row 6 | Row 7 |
| --- | --- | --- |
| # of ports | 8 | 8 |
| Density | 1 | 1 |
| CDM type | FD-CDM2 | FD-CDM2 |
| Size of bitmap for indicating frequency location | 6 | 6 |
| # of starting symbols for indicating time location | 1 | 1 |

To resolve the ambiguity, reusing the existing RRC parameters, we propose to distinguish these two CSI-RS patterns by the contents of the bitmap for indicating frequency location. That is, in the 6-bit bitmap, if there are 4 set bits, CSI-RS pattern in row 6 is used, while CSI-RS pattern in row 7 is adopted when there are 2 set bits.

Proposal 2: Capture in specs that in the 6-bit bitmap for indicating frequency location, when there are {2, 4} set bits, UE shall assume CSI-RS components in row {7, 6} are used, respectively.

Additional periodicity

Considering flexible UL/DL switching periodicity, it was agreed in [1] that additional periodicities of {4, 8, 16, 32, 64} slots and the corresponding slot offsets are supported at least for periodic/semi-persistent CSI-RS. There is a FFS on whether and how to align to additional periodicities as agreed in the frame structure agenda. In our view, introducing additional CSI-RS periodicities will further increase the implementation complexity while the gain is unclear. In addition, as already captured in specs, the scheduled CSI-RS can be dropped if it happens to be located in an UL slot. In other words, the system can work without introducing additional CSI-RS periodicities.

Proposal 3: Preclude any other additional periodicities for CSI-RS transmission in Rel-15.

Summary of proposals

The proposals in this paper are summarized as follows.

Proposal 1: Confirm the WA on CSI-RS sequence mapping.

Proposal 2: Capture in specs that in the 6-bit bitmap for indicating frequency location, when there are {2, 4} set bits, UE shall assume CSI-RS components in row {7, 6} are used, respectively.

Proposal 3: Preclude any other additional periodicities for CSI-RS transmission in Rel-15.

- References

3GPP, “RAN1#AH1801 Meeting Chairman’s Notes”, Vancouver, Canada, January 22nd - 26th, 2018.