3GPP TSG-RAN WG1 Meeting #92	R1-1801487

Athens, Greece, 26th Feb – 2nd Mar 2018

Agenda Item:	6.2.5.7

Source:	Ericsson

Title:	Flexible PDSCH/PUSCH starting PRB for MTC

Document for:	Decision

# Introduction

The revised WID for Rel-15 on ’Even further enhanced MTC for LTE’ [1] has updated objectives on

- Increased PDSCH spectral efficiency [RAN1 lead, RAN2, RAN4]

  - Specify optional support for 64QAM for unicast PDSCH (no UE peak rate increase is intended).

  - Introduce support for more flexible starting PRB for PDSCH resource allocation in connected mode at least for UE operating in CE mode A/B configured with 1.4 MHz max PDSCH channel bandwidth.

- Increased PUSCH spectral efficiency [RAN1 lead, RAN2, RAN4]

- E.g. sub-PRB resource allocation, with no less than 3 subcarriers within a sub-PRB allocation.

- Introduce support for more flexible starting PRB for PUSCH resource allocation in connected mode at least for UE operating in CE mode A/B configured with 1.4 MHz max PUSCH channel bandwidth.

In this contribution we discuss the design of the Resource Allocation (RA) and DCI for the more flexible starting PRB for PUSCH and PDSCH.

# Background

As discussed in [2], it has been observed that the misalignment between the BL/CE narrowband (NB) and the LTE resource block groups (RBG) may cause inefficient utilization of the downlink PRBs in certain traffic conditions. Also for the uplink the allocation of BL/CE UEs with the current narrowbands may lead to inefficient utilization of the uplink PRBs.

The alignment between the CE mode narrowbands and the downlink RBG for system bandwidth 3 to 20 MHz is depicted in Figure 1.

Figure 1: Alignment between CE mode narrowband and downlink resource block groups (RBG).

The effect of the misalignment between the BL/CE NB and the RBG on the possible utilization of PRBs for one higher category UE when scheduling PDSCH on all 6 PRBs of one narrowband is outlined in Table 1.

Table 1: Effect of the unused PRBs due to the misalignment of narrowband and resource block group for PDSCH allocation of one single higher Category UE.

| System BW [MHz] | Total number of PRBs | RBG size | #RBGs per NB |  | # unused PRBs due to one NB |  | Effect of unused PRB |  | Description of NB set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Set #1 | Set #2 | Set #1 | Set #2 | Set #1 | Set #2 | Set #1 | Set #2 |
| 1.4 | 6 | 1 | 6 |  | 0 |  | 0% |  | N/A |  |
| 3 | 15 | 2 | 4 | 3 | 2 | 0 | 33,3% | 0% | Low NB | High NB |
| 5 | 25 | 2 | 3 | 4 | 0 | 2 | 0% | 10,5% | Low NB | High NB |
| 10 | 50 | 3 | 3 |  | 3 |  | 6,8% |  | N/A |  |
| 15 | 75 | 4 | 2 | 3 | 2 | 6 | 2,9% | 8,7% | Even NB | Odd NB |
| 20 | 100 | 4 | 2 |  | 2 |  | 2,1% |  | N/A |  |

To allow large number of consecutive groups of PRBs to be allocated in the uplink, PUCCH and PRACH may be allocated near the edges of the LTE system bandwidth. Thus, in practice, it might be difficult to utilize the narrowbands closest to the edges of the LTE system bandwidth and allocating the BL/CE UE in a narrowband which is not directly adjacent to the resources used for PUCCH/PRACH may lead to a fragmentation of the spectrum. With a more flexible uplink resource allocation, the BL/CE UE may be allocated with resources adjacent to the PUCCH/PRACH resources and other PUSCH resource allocations, thus avoiding spectrum fragmentation.

### PDSCH resource allocation

The Rel-14 resource allocation for PDSCH is described in section 7.1.6 of [3] and is summarized in Table 2.

Table 2: Resource allocation for PDSCH

|  | 1.4 MHz BL/CE UE | 5 MHz BL/CE UE | 20 MHz BL/CE UE |
| --- | --- | --- | --- |
| CE mode A DCI format 6-1A | Resource allocation type 2 with resource block assignment field composed of: - Narrowband index - Resource indication value (RIV) corresponding to a starting PRB and number of contiguously allocated PRBs within the narrowband | Resource allocation type 2 with resource block assignment field composed of: - Narrowband index - Additional consecutively allocated narrowbands - RIV corresponding to a starting PRB and number of contiguously allocated PRBs within each allocated narrowband | Either of: Resource allocation type 0 with resource block assignment field composed of - Bitmap of allocated BL/CE UE resource block groups of 6 contiguous PRBs for system BW less than 20 MHz and 9 PRBs for system BW equal to 20 MHz Resource allocation type 2 with resource assignment field composed of: - Narrowband index - Additional consecutively allocated narrowbands - RIV corresponding to a starting PRB and number of contiguously allocated PRBs within each allocated narrowband |
| CE mode B DCI format 6-1B | A resource block assignment field composed of: - Narrowband index - Indication if RBs with PRB index {0, 1, 2, 3} or all 6 PRBs in the narrowband is allocated | A resource block assignment field composed of: a bitmap of: - Wideband index (four consecutive narrowbands, non-overlapping between widebands) - Bitmap of the allocated narrowbands (all 6 PRBs) within the wideband | A resource block assignment field composed of: - Wideband combination index - Bitmap of the allocated narrowbands (all 6 PRBs) within each allocated wideband |

The design of the resource indication value (RIV) within a narrowband for the CE mode A PDSCH resource allocations in Table 2 is shown in Figure 2. The resource allocation for CE mode B is shown in Figure 3.

Figure 2: PDSCH and PUSCH resource block assignment within a narrowband using resource indication value (RIV).

Figure 3: PDSCH resource block assignment within narrowband for 1.4 MHz BL/CE UE.

A summary of the size of the resource block assignment field for DCI format 6.1A and DCI format 6-1B is given in Table 3.

Table 3: Size of downlink resource block assignemnt field in DCI format 6-1A for different system and BL/CE UE bandwidth.

| System bandwidth | Total number of PRBs | CE mode A, DCI format 6-1A |  |  |  | CE mode B, DCI format 6-1B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UE bandwidth |  |  |  | UE bandwidth |  |  |
|  |  | 1.4 MHz RA type 2 | 5 MHz RA type 2 | 20 MHz |  | 1.4 MHz | 5 MHz | 20 MHz |
|  |  |  |  | RA type 0 | RA type 2 |  |  |  |
| 1.4 MHz | 6 | 5 | 5 | 1 (+4) | 5 | 1 | 1 | 1 |
| 3 MHz | 12 | 6 | 7 | 2 (+ 5) | 7 | 2 | 2 | 2 |
| 5 MHz | 25 | 7 | 9 | 4 (+ 5) | 9 | 3 | 4 | 4 |
| 10 MHz | 50 | 8 | 11 | 8 (+3) | 11 | 4 | 5 | 6 |
| 15 MHz | 75 | 9 | 12 | 12 (+0) | 12 | 5 | 6 | 7 |
| 20 MHz | 100 | 9 | 12 | 11 (+1) | 12 | 5 | 6 | 7 |

### PUSCH resource allocation

The Rel-14 resource allocation for PUSCH is described in section 8.1 of [3] and is summarized in Table 4.

Table 4: Resource allocation for PUSCH

|  | 1.4 MHz BL/CE UE | 5 MHz BL/CE UE |
| --- | --- | --- |
| CE mode A DCI format 6-0A | Uplink resource allocation type 0 with resource block assignment field composed of: - Narrowband index - Resource indication value (RIV) corresponding to a starting PRB and number of contiguously allocated PRBs within the narrowband | Either of: Uplink resource allocation type 0 with resource block assignment field composed of: - Narrowband index - RIV corresponding to a starting PRB and number of contiguously allocated PRBs within the narrowband Uplink resource allocation type 4, specifying a starting PRB at either the first or fourth PRB in a narrowband and a group of contiguously allocated resource blocks in groups of 3 PRBs, via a resource block assignment field composed of - Resource block group indication value (RBGIV) corresponding to a starting resource block group and a set of consecutive uplink resource block groups of resource blocks |
| CE mode B DCI format 6-0B | Uplink resource allocation type 2 with resource block assignment field composed of: - Narrowband index - Indication either 1 or 2 allocated resource blocks within the narrowband in the set {0, 1, 2, 3, 4, 5, 0 and 1, 2 and 3} | N/A |

The uplink resource allocation type 0 used in CE mode A uses the same RIV design as for the downlink resource allocation, as shown in Figure 2. For CE mode B, the uplink resource allocation is as shown in Figure 4.

Figure 4: PUSCH CE mode B type 2 resource block assignment.

The size of the resource block assignment field for DCI format 6.0A and 6-0B is given in Table 5

Table 5: Size of uplink resource block assignemnt field in DCI format 6-0A and 6-0B for different system bandwidth.

| System bandwidth | Total number of PRBs | CE mode A DCI format 6-0A [bits] | CE mode B DCI format 6-0B [bits] |
| --- | --- | --- | --- |
| 1.4 MHz | 6 | 5 | 3 |
| 3 MHz | 12 | 6 | 4 |
| 5 MHz | 25 | 7 | 5 |
| 10 MHz | 50 | 8 | 6 |
| 15 MHz | 75 | 9 | 7 |
| 20 MHz | 100 | 9 | 7 |

### Guard period for narrowband and wideband retuning

When introducing a flexible starting PRB for PDSCH and PUSCH the effect of a potential re-tuning between narrowbands with different center frequencies needs to be considered.

A guard period for re-tuning between downlink narrowbands with different center frequencies is defined in 6.2.8 of [4]. Similarly, a guard period is defined for frame structure type 2 when the UE retunes from an uplink narrowband to a downlink narrowband with a different center frequency.

# Discussion

To minimize the impact on the size of the DCI, it is proposed that the use of flexible starting PRB is introduced as a new resource allocation type that is semi statically configured per UE.

The flexible starting PRB is introduced as a new resource allocation type that is semi statically configured per UE.

To fully utilize the efficiency enhancement for both PDSCH and PUSCH at least for a UE operating in CE mode A and allocations within 1.4 MHz (i.e. an allocation of less than or equal to 6 PRBs) it is proposed that the starting PRB for the resource allocation should be possible to be selected from all PRBs within the system bandwidth.

At least for an allocation within 1.4 MHz (i.e. an allocation of less than or equal to 6 PRBs) and for a UE operating in CE mode A, the starting PRB for the resource allocation for both PDSCH and PUSCH should be possible to be selected from all PRBs within the system bandwidth.

To fully utilize the efficiency enhancement in PUSCH for a UE operating in CE mode B, it is proposed that the starting PRB for the PUSCH resource allocation should be possible to be selected from all PRBs within the system bandwidth.

For PUSCH allocation for a UE operating in CE mode B, the starting PRB for the resource allocation should be possible to be selected from all PRBs within the system bandwidth.

To allow for scheduling multiple PDSCH transmissions with MPDCCH back-to-back with a PDSCH transmission without the need for retuning, it is proposed that at least for 1.4 MHz BL/CE UE the allocated PRBs with flexible starting PRB shall be a superset of the current resource allocation within a narrowband.

At least for 1.4 MHz BL/CE UE the allocated PRBs with flexible starting PRB shall be a superset of the current resource allocations.

## Resource allocation for 1.4 MHz BL/CE UE

This subsection discusses flexible starting PRB for resource allocation of 1.4 MHz BL/CE UEs for PDSCH and PUSCH.

### PDSCH for CE mode A and 1.4 MHz BL/CE UE

It was observed in [2] that an allocation of all possible consecutive groups of {1, 2, 3, 4, 5, 6} PRBs within the system bandwidth may be provided via a resource indication value (RIV) computed as

where  is the starting PRB and is the number of consecutively allocated PRBs {1, 2, 3, 4, 5, 6} at this starting PRB.

The starting PRB is thus computed as  and the number of consecutively allocated PRBs as. The number of bits required for the RIV value is summarized in Table 6.

Table 6: Number of bits required for flexible starting PRB for PDSCH resource allocation for 1.4 MHz BL/CE UE in CE mode A.

| System bandwidth | Total number of PRBs | Maximum RIV value | Required size of resource allocation field for RIV values [bits] | Size of Rel-14 resource block assignment in DCI format 6-1A for 1.4 MHz BL/CE UE [bits] |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 6 | 30 | 5 | 5 |
| 3 MHz | 12 | 66 | 7 | 6 |
| 5 MHz | 25 | 144 | 8 | 7 |
| 10 MHz | 50 | 296 | 9 | 8 |
| 15 MHz | 75 | 504 | 9 | 9 |
| 20 MHz | 100 | 594 | 10 | 9 |

Introduce flexible starting PRB for PDSCH for 1.4 MHz BL/CE UE operating in CE mode A with a resource allocation based on resource indication value  where  is the starting PRB and is the number of consecutively allocated PRBs {1, 2, 3, 4, 5, 6} at this starting PRB.

### PDSCH for CE mode B and 1.4 MHz BL/CE UE

For a 1.4 MHz BL/CE UE in CE mode B, it is proposed to use a resource block assignment based on a resource indication value (RIV) computed as

where the index defines the starting PRB of the allocation as the first PRB of the RBG or narrowband addressed by the index, and = 0 indicates that the 4 consecutive PRBs from the starting PRB are used and = 1 indicates that the 6 consecutive PRBs from the starting PRB are used.

The size of the required resource block assignment field for the proposed allocation is shown in Table 8

Table 7: Number of bits required for flexible starting PRB for PDSCH resource allocation for 1.4 MHz BL/CE UE in CE mode B.

| System BW [MHz] | Total number of PRBs | RBG size | Number of RBGs with valid stating PRB for CE mode B allocation | Number of narrowbands with starting PRB not aligned with RBG | Maximum RIV value | Required size of resource allocation field for RIV values [bits] | Size of Rel-14 resource block assignment field in DCI format 6-1B for 1.4 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 | 6 | 1 | 3 | 0 | 3 | 2 | 1 |
| 3 | 12 | 2 | 6 | 1 | 13 | 4 | 2 |
| 5 | 25 | 2 | 11 | 2 | 25 | 5 | 3 |
| 10 | 50 | 3 | 16 | 8 | 47 | 6 | 4 |
| 15 | 75 | 4 | 18 | 9 | 53 | 6 | 5 |
| 20 | 100 | 4 | 25 | 8 | 65 | 7 | 5 |

Introduce flexible starting PRB for PDSCH for 1.4 MHz BL/CE UE operating in CE mode B, with a resource block assignment based on a resource indication value (RIV) computed as where the starting PRB of the allocation is the first PRB of the RBG or narrowband addressed via the index  to the set of RBGs and narrowbands, and = 0 indicates that the 4 consecutive PRBs from the starting PRB are used and = 1 indicates that the 6 consecutive PRBs from the starting PRB are used.

### PUSCH for CE mode A and 1.4 MHz BL/CE UE

For the uplink resource allocation in CE mode A for a 1.4 MHz BL/CE UE the resource allocation can be similarly defined as for PDSCH CE mode A and 1.4 MHz BL/CE UE via a resource indication value (RIV) computed as

where  is the starting PRB and is the number of consecutively allocated PRBs {1, 2, 3, 4, 5, 6} at this starting PRB.

The starting PRB is thus computed as  and the number of consecutively allocated PRBs as. The number of bits required for the RIV value is summarized in Table 8.

Table 8: Number of bits required for flexible starting PRB for PUSCH resource allocation for 1.4 MHz BL/CE UE in CE mode A.

| System bandwidth | Total number of PRBs | Maximum RIV value | Required size of resource allocation field for RIV values [bits] | Size of Rel-14 resource block assignment in DCI format 6-0A for 1.4 MHz BL/CE UE [bits] |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 6 | 30 | 5 | 5 |
| 3 MHz | 12 | 66 | 7 | 6 |
| 5 MHz | 25 | 144 | 8 | 7 |
| 10 MHz | 50 | 296 | 9 | 8 |
| 15 MHz | 75 | 504 | 9 | 9 |
| 20 MHz | 100 | 594 | 10 | 9 |

Introduce flexible starting PRB for PUSCH for 1.4 MHz BL/CE UE operating in CE mode A with a resource allocation based on resource indication value  where  is the starting PRB and is the number of consecutively allocated PRBs {1, 2, 3, 4, 5, 6} at this starting PRB.

### PUSCH for CE mode B and 1.4 MHz BL/CE UE

For the uplink resource allocation in CE mode B for a 1.4 MHz BL/CE UE the resource allocation can be defined via a resource indication value (RIV) computed as

where  is the starting PRB and = 0 indicates that only the starting PRB is allocated and = 1 indicates that two consecutive PRBs are allocated from the starting PRB.

The number of bits required for the RIV value is summarized in Table 9.

Table 9: Number of bits required for flexible starting PRB for PUSCH resource allocation for 1.4 MHz BL/CE UE in CE mode B.

| System bandwidth | Total number of PRBs | Maximum RIV value | Required size of resource allocation field for RIV values [bits] | Size of Rel-14 resource block assignment in DCI format 6-0B for 1.4 MHz BL/CE UE [bits] |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 6 | 10 | 4 | 3 |
| 3 MHz | 12 | 22 | 5 | 4 |
| 5 MHz | 25 | 48 | 6 | 5 |
| 10 MHz | 50 | 98 | 7 | 6 |
| 15 MHz | 75 | 148 | 8 | 7 |
| 20 MHz | 100 | 198 | 8 | 7 |

Introduce flexible starting PRB for PUSCH for 1.4 MHz BL/CE UE operating in CE mode B with a resource allocation based on resource indication value  where  is the starting PRB and = 0 indicates that only the starting PRB is allocated and = 1 indicates that two consecutive PRBs are allocated from the starting PRB.

# Conclusion

Based on the discussion above we propose the following:

Proposal 1	The flexible starting PRB is introduced as a new resource allocation type that is semi statically configured per UE.

Proposal 2	At least for an allocation within 1.4 MHz (i.e. an allocation of less than or equal to 6 PRBs) and for a UE operating in CE mode A, the starting PRB for the resource allocation for both PDSCH and PUSCH should be possible to be selected from all PRBs within the system bandwidth.

Proposal 3	For PUSCH allocation for a UE operating in CE mode B, the starting PRB for the resource allocation should be possible to be selected from all PRBs within the system bandwidth.

Proposal 4	At least for 1.4 MHz BL/CE UE the allocated PRBs with flexible starting PRB shall be a superset of the current resource allocations.

Proposal 5	Introduce flexible starting PRB for PDSCH for 1.4 MHz BL/CE UE operating in CE mode A with a resource allocation based on resource indication value  where  is the starting PRB and is the number of consecutively allocated PRBs {1, 2, 3, 4, 5, 6} at this starting PRB.

Proposal 6	Introduce flexible starting PRB for PDSCH for 1.4 MHz BL/CE UE operating in CE mode B, with a resource block assignment based on a resource indication value (RIV) computed as where the starting PRB of the allocation is the first PRB of the RBG or narrowband addressed via the index  to the set of RBGs and narrowbands, and = 0 indicates that the 4 consecutive PRBs from the starting PRB are used and = 1 indicates that the 6 consecutive PRBs from the starting PRB are used.

Proposal 7	Introduce flexible starting PRB for PUSCH for 1.4 MHz BL/CE UE operating in CE mode A with a resource allocation based on resource indication value  where  is the starting PRB and is the number of consecutively allocated PRBs {1, 2, 3, 4, 5, 6} at this starting PRB.

Proposal 8	Introduce flexible starting PRB for PUSCH for 1.4 MHz BL/CE UE operating in CE mode B with a resource allocation based on resource indication value  where  is the starting PRB and = 0 indicates that only the starting PRB is allocated and = 1 indicates that two consecutive PRBs are allocated from the starting PRB.

# References

RP-172811, “Revised WID on Even further enhanced MTC for LTE”

R1-1717001, “Flexible PDSCH/PUSCH resource allocation for MTC”, Ericsson

3GPP TS 36.213

3GPP TS 36.211